3GPP TSG-RAN5 Meeting #106	R5-250565

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-2 | CR | 1058 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability update for LTE based 5G broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For introducing the requirement related to LTE based 5G broadcast feature, associated ICS is currently missing in the spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added ICS for new SDO bands 107, 108 Added ICS for new PMCH bandwidth 6/7/8 MHz Added a new table additional UE radio access capabilities for MBMS including LTE based 5G broadcast Added a new Table A.4.5-2c: Additional optional features without UE radio access capability parameters – MBMS features |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.x, Table A.4.5-2b (new), Table A.4.5-2c (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.4	ICS proforma tables

### A.4.1	UE Implementation Types

Table A.4.1-1: UE Radio Technologies

| Item | UE Radio Technologies | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | E-UTRA FDD | 36.101 | Rel-8 |  |
| 2 | E-UTRA TDD | 36.101 | Rel-8 |  |
| 3 | UTRA FDD | 25.101 | R99 |  |
| 3a | UTRA FDD | 25.101 | Rel-9 |  |
| 4 | UTRA TDD | 25.102 | R99 |  |
| 4a | UTRA TDD | 25.102 | Rel-9 |  |
| 5 | GSM | 45.005 | R99 |  |
| 6 | cdma2000 HRPD | C.S0024-A | Rel-8 |  |
| 7 | cdma2000 1xRTT | C.S0002-A | Rel-8 |  |
| 8 | NB-IoT | 36.101 | Rel-13 |  |
| 8a | NB-IoT TDD | 36.101 | Rel-15 |  |
| 8b | NB-IoT NTN | 36.102 | Rel-18 |  |
| 9 | WLAN | IEEE Std 802.11 |  |  |
| 10 | V2X Sidelink Communication | 36.101 | Rel-14 |  |
| 11 | eMTC | 36.101 | Rel-13 |  |
| 12 | eMTC NTN | 36.102 | Rel-18 |  |

### A.4.2	UE Service Capabilities

Table A.4.2-1: UE Radio Technologies

| Item | UE Radio Technologies | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | LTE MBMS | 36.101 | Rel-9 |  |
| 2 | LTE CA | 36.101 | Rel-10 |  |
| 3 | UL-MIMO | 36.306, 4.3.4.6 | Rel-10 |  |
| 4 | Void |  |  |  |
| 5 | Enhanced Dual Layer TDD | 36.306, 4.3.4.5 | Rel-9 |  |
| 6 | EPDCCH | 36.306, 4.3.4.18 | Rel-11 |  |
| 7 | FDD - TDD CA | 36.306, 4.3.4.28 | Rel-12 |  |
| 8 | Support of DC | 36.306, 4.3.5.9 | Rel-12 | The UE supports of synchronous dual connectivity and power control mode 1 |
| 9 | Support of E-UTRAN WLAN Aggregation - LWA | 36.306, 4.3.18, 4.3.25, 4.3.27, 7.10.2 | Rel-13 |  |
| 10 | Support of E-URAN WLAN Aggregation with IPsec Tunnel - LWIP | 36.306, 4.3.18, 4.3.24, 4.3.27, 7.10.2 | Rel-13 |  |

### A.4.3	Baseline Implementation Capabilities

Table A.4.3-1: Supported protocols

| Item | Supported protocols | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | EPS Mobility Management | 24.301, 5 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 2 | EPS Session Management | 24.301, 6 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 3 | GPRS Mobility Management | 23.060 | R99 | For NB-IoT the release is from Rel-13 |
| 4 | Radio Resource Control | 36.331 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 5 | Packet Data Convergence Protocol | 36.323 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 6 | Radio Link Control | 36.322 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 7 | Medium Access Control | 36.321 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 8 | Physical Layer | 36.201 36.302 | Rel-8 | For NB-IoT the release is from Rel-13 |

Table A.4.3-2: Special Conformance Testing Functions

| Item | Special Conformance Testing Functions | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | UE test loop | 36.509 | Rel-8 | For NB-IoT the release is from Rel-13 |
| 2 | Max UE test loop UL RLC SDU size 65535 bits | 36.509 | Rel-8 |  |

Table A.4.3-3: RF Baseline Implementation Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Frequency band: 1920-1980, 2110-2170 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 1 |
| 2 | Frequency band: 1850-1910, 1930-1990 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 2 |
| 3 | Frequency band: 1710-1785, 1805-1880 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 3 |
| 4 | Frequency band: 1710-1755, 2110-2155 MHz | 36.101, 5.5 | Rel-8 | FDD Band 4 |
| 5 | Frequency band: 824-849, 869-894 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 5 |
| 6 | Frequency band: 830-840, 875-885 MHz | 36.101, 5.5 | Rel-8 | FDD Band 6 |
| 7 | Frequency band: 2500-2570, 2620-2690 MHz | 36.101, 5.5 | Rel-8 | FDD Band 7 |
| 8 | Frequency band: 880-915, 925-960 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 8 |
| 9 | Frequency band: 1749.9-1784.9, 1844.9-1879.9 MHz | 36.101, 5.5 | Rel-8 | FDD Band 9 |
| 10 | Frequency band: 1710-1770, 2110-2170 MHz | 36.101, 5.5 | Rel-8 | FDD Band 10 |
| 11 | Frequency band: 1427.9-1447.9, 1475.9-1495.9 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 11 |
| 12 | Frequency band: 699-716, 729-746 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 12 |
| 13 | Frequency band: 777-787, 746-756 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 13 |
| 14 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | Rel-8 | FDD Band 14 |
| 15 | Reserved | 36.101, 5.5 | Rel-8 | FDD Band 15 |
| 16 | Reserved | 36.101, 5.5 | Rel-8 | FDD Band 16 |
| 17 | Frequency band: 704-716, 734-746 MHz | 36.101, 5.5 | Rel-8 | FDD and HD-FDD Band 17 |
| 18 | Frequency band: 815-830, 860-875 MHz | 36.101, 5.5 | Rel-9 | FDD and HD-FDD Band 18 |
| 19 | Frequency band: 830-845, 875-890 MHz | 36.101, 5.5 | Rel-9 | FDD and HD-FDD Band 19 |
| 20 | Frequency band: 832-862, 791-821MHz | 36.101, 5.5 | Rel-9 | FDD and HD-FDD Band 20 |
| 21 | Frequency band: 1447.9-1462.9, 1495.9-1510.9 MHz | 36.101, 5.5 | Rel-9 | FDD and HD-FDD Band 21Band 21 |
| 22 | Frequency band: 3410-3490, 3510-3590 MHz | 36.101, 5.5 | Rel-10 | FDD Band 22 |
| 23 | Frequency band: 2000-2020, 2180-2200 MHz | 36.101, 5.5 | Rel-10 | FDD Band 23 |
| 24 | Frequency band: 1626.5-1660.5, 1525-1559 MHz | 36.101, 5.5 | Rel-10 | FDD Band 24 |
| 25 | Frequency band: 1850-1915, 1930-1995 MHz | 36.101, 5.5 | Rel-10 | FDD and HD-FDD Band 25 |
| 26 | Frequency band: 814-849, 859-894 MHz | 36.101, 5.5 | Rel-11 | FDD and HD-FDD Band 26 |
| 27 | Frequency band: 807-824, 852-869 MHz | 36.101, 5.5 | Rel-11 | FDD Band 27 |
| 28 | Frequency band: 703-748, 758-803 MHz | 36.101, 5.5 | Rel-11 | FDD and HD-FDD Band 28 |
| 29 | Frequency band: N/A, 717-728 MHz | 36.101, 5. 5 | Rel-11 | FDD Band 29 |
| 30 | Frequency band: 2305-2315, 2350-2360 MHz (Note 2) | 36.101, 5.5 | Rel-12 | FDD Band 30 |
| 31 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | Rel-12 | FDD and HD-FDD Band 31 |
| 32 | Frequency band: N/A, 1452-1496 MHz | 36.101, 5.5 | Rel-12 | FDD Band 32 |
| 33 | Frequency band: 1900-1920, 1900-1920 MHz | 36.101, 5.5 | Rel-8 | TDD Band 33 |
| 34 | Frequency band: 2010-2025, 2010-2025 MHz | 36.101, 5.5 | Rel-8 | TDD Band 34 |
| 35 | Frequency band: 1850-1910, 1850-1910 MHz | 36.101, 5.5 | Rel-8 | TDD Band 35 |
| 36 | Frequency band: 1930-1990, 1930-1990 MHz | 36.101, 5.5 | Rel-8 | TDD Band 36 |
| 37 | Frequency band: 1910-1930, 1910-1930 MHz | 36.101, 5.5 | Rel-8 | TDD Band 37 |
| 38 | Frequency band: 2570-2620, 2570-2620 MHz | 36.101, 5.5 | Rel-8 | TDD Band 38 |
| 39 | Frequency band: 1880-1920, 1880-1920 MHz | 36.101, 5.5 | Rel-8 | TDD Band 39 |
| 40 | Frequency band: 2300-2400, 2300-2400 MHz | 36.101, 5.5 | Rel-8 | TDD Band 40 |
| 41 | Frequency band: 2496-2690, 2496-2690 MHz | 36.101, 5.5 | Rel-10 | TDD Band 41 |
| 42 | Frequency band: 3400-3600, 3400-3600 MHz | 36.101, 5.5 | Rel-10 | TDD Band 42 |
| 43 | Frequency band: 3600-3800, 3600-3800 MHz | 36.101, 5.5 | Rel-10 | TDD Band 43 |
| 44 | Frequency band: 703-803, 703-803 MHz | 36.101, 5.5 | Rel-11 | TDD Band 44 |
| 45 | Frequency band: 1447-1467, 1447-1467 MHz | 36.101, 5.5 | Rel-13 | TDD Band 45 |
| 46 | Frequency band: 5150-5925, 5250-5925 MHz | 36.101, 5.5 | Rel-13 | TDD Band 46 |
| 47 | Frequency band: 5855-5925, 5855-5925 MHz | 36.101, 5.5 | Rel-14 | TDD Band 47 |
| ... |  |  |  |  |
| 48 | Frequency band: 3550-3700, 3550-3700 MHz | 36.101, 5.5 | Rel-14 | TDD Band 48 |
| .. |  |  |  |  |
| 53 | Frequency band: 2483.5-2495, 2483.5-2495 MHz | 36.101, 5.5 | Rel-16 | TDD Band 53 |
| 54 | Frequency band: 1670-1675, 1670-1675 MHz | 36.101, 5.5 | Rel-18 | TDD Band 54 |
| ... |  |  |  |  |
| 65 | Frequency band: 1920-2010, 2110-2200 MHz | 36.101, 5.5 | Rel-13 | FDD Band 65 |
| 66 | Frequency band: 1710-1780, 2110-2200 MHz | 36.101, 5.5 | Rel-13 | FDD and HD-FDD Band 66 |
| ... |  |  |  |  |
| 68 | Frequency band: 698-728, 753-783 MHz | 36.101, 5.5 | Rel-15 | FDD Band 68 |
| 69 | Frequency band: N/A, 2570-2620 MHz | 36.101, 5.5 | Rel-14 | FDD Band 69 |
| 70 | Frequency band: 1695-1710, 1995-2020 MHz | 36.101, 5.5 | Rel-14 | FDD and HD-FDD Band 70 |
| 71 | Frequency band: 663-698, 617-652 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 71 |
| 72 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 72 |
| 73 | Frequency band: 450-455, 460-465 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 73 |
| 74 | Frequency band: 1427-1470, 1475-1518 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 74 |
| ... |  |  |  |  |
| 85 | Frequency band: 698-716, 728-746 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 85 |
| ... |  |  |  |  |
| 87 | Frequency band: 410-415, 420-425 MHz | 36.101, 5.5 | Rel-16 | FDD and HD-FDD Band 87 |
| 88 | Frequency band: 412-417, 422-427 MHz | 36.101, 5.5 | Rel-16 | FDD and HD-FDD Band 88 |
| ... |  |  |  |  |
| 103 | Frequency band: 787-788, 757-758 MHz | 36.101, 5.5 | Rel-17 | FDD and HD-FDD Band 103 |
| ... |  |  |  |  |
| 106 | Frequency band: 896-901, 935-940 MHz | 36.101, 5.5 | Rel-18 | FDD and HD-FDD Band 106 |
| 107 | Frequency band: N/A, 612-652 MHz | 36.101, 5.5H | Rel-17 | SDO Band 107 |
| 108 | Frequency band: N/A, 470-698 MHz | 36.101, 5.5H | Rel-17 | SDO Band 108 |
| ... |  |  |  |  |
| 253 | Frequency band: 1668-1675, 1518-1525 MHz | 36.102, 5.2 | Rel-18 | FDD Band 253 |
| 254 | Frequency band: 1610-1626.5, 2483.5-2500 MHz | 36.102, 5.2 | Rel-18 | FDD Band 254 |
| 255 | Frequency band: 1626.5-1660.5, 1525-1559 MHz | 36.102, 5.2 | Rel-18 | FDD Band 255 |
| 256 | Frequency band: 1980-2010, 2170-2200 MHz | 36.102, 5.2 | Rel-18 | FDD Band 256 |
| Note 1: The values indicated in column "Release" are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 36.307 [16] Note 2: The uplink transmission is not allowed at this band for the UE with the externally vehicle-mounted antennas. |  |  |  |  |

Table A.4.3-3a: RF Additional Baseline Implementation Capabilities

| Item | RF Additional Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | Support of 1.4 MHz channel bandwidth | 36.101, 5.6.1 36.102, 5.3A (Band 253, 254, 255 and 256) | Operating bands supporting 1.4 MHz Bandwidth: 2, 3, 4, 5, 8, 12, 23, 25, 26, 27, 31, 35, 36, 53, 54, 65, 66, 72, 73, 74, 87, 88, 253, 254, 106, 255, 256 |
| 2 | Support of 3 MHz channel bandwidth | 36.101, 5.6.1 | Operating bands supporting 3 MHz Bandwidth: 2, 3, 4, 5, 8, 12, 23, 25, 26, 27, 28, 31, 35, 36, 44, 53, 54, 65, 66, 72, 73, 74, 87, 88, 106 |
| 3 | Support of 5 MHz channel bandwidth | 36.101, 5.6.1 | All operating bands support 5 MHz Bandwidth except band 46, 47 and Band 106 |
| 4 | Support of 10 MHz channel bandwidth | 36.101, 5.6.1 | All operating bands support 10 MHz Bandwidth except band 31, 54, 72, 73 and 106 |
| 5 | Support of 15 MHz channel bandwidth | 36.101, 5.6.1 | Operating bands supporting 15 MHz Bandwidth: 1, 2, 3, 4, 7, 9, 10, 18, 19, 20, 21, 22, 23, 25, 26, 28, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 48, 65, 66, 70, 74 |
| 6 | Support of 20 MHz channel bandwidth | 36.101, 5.6.1 | Operating bands supporting 20MHz Bandwidth: 1, 2, 3, 4, 7, 9, 10, 20, 22, 23, 25, 28, 33, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 65, 66, 701, 74 |
| 7 | Support of 20 MHz for both PCell and SCell | 36.101, 5.6A.1 |  |
| 8 | Support of 20 MHz for PCell and 10 MHz for SCell | 36.101, 5.6A.1 |  |
| 9 | Support at most 40 MHz aggregated bandwidth | 36.101, 5.6A.1 |  |
| 10 | Support at most 60 MHz aggregated bandwidth | 36.101, 5.6A.1 |  |
| 11 | Support at most 80 MHz aggregated bandwidth | 36.101, 5.6A.1 |  |
| 12 | Support at most 100 MHz aggregated bandwidth | 36.101, 5.6A.1 |  |
| 13 | Support of 200 kHz channel bandwidth | 36.102, 5.3B | Operating bands supporting 200 kHz Bandwidth: 253, 254, 255, 256 |
| 14 | Support of 6 MHz PMCH bandwidth | 36.101, 5.6H | Operating bands supporting 6 MHz PMCH bandwidth: 107, 108 |
| 15 | Support of 7 MHz PMCH bandwidth | 36.101, 5.6H | Operating bands supporting 7 MHz PMCH bandwidth: 107, 108 |
| 16 | Support of 8 MHz PMCH bandwidth | 36.101, 5.6H | Operating bands supporting 8 MHz PMCH bandwidth: 107, 108 |
| Note 1: 1 For the 20 MHz channel bandwidth, the additional baseline implementation capabilities are restricted to E-UTRA operation when carrier aggregation is configured. |  |  |  |

Table A.4.3-3b: Additional UE Power Class implementation Capabilities

| Item | RF baseline UE Baseline implementation capability | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UE Power Class 1 | 36.101, 6.2.2 | Applicable to Bands 12, 14, 31, 40, 42, 72, 87, 88, 106 |
| 2 | UE Power Class 3 | 36.101, 6.2.2 | All applicable E-UTRA and NB-IoT bands |
|  |  | 36.102, 6.2A.1 and 6.2B.1 | All applicable NB-IoT NTN and eMTC NTN bands |
| 3 | UE Power Class 5 | 36.101, 6.2.2E 36.306, 4.3.5.1 and 4.3.5.1A | All applicable E-UTRA and NB-IoT bands 20dBm |
|  |  | 36.102, 6.2A.1 and 6.2B.1 | All applicable NB-IoT NTN and eMTC NTN bands 20dBm |
| 4 | UE Power Class 2 | 36.101, 6.2.2. 6.2.2E | Applicable to Bands 31, 41, 47, 72 |
| 5 | UE Power Class 6 | 36.101, 6.2.2E 36.306, 4.3.5.1A.2 | All applicable E-UTRA and NB-IoT bands 14dBm |

Table A.4.3-3c: UE Power Class 1 implementation Capabilities per band

| Item | RF Baseline Implementation Capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | Rel-8 | FDD Band 14 |
| 2 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | Rel-12 | FDD and HD-FDD Band 31 |
| 3 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 72 |
| 4 | Frequency band: 410-415, 420-425 MHz | 36.101, 5.5 | Rel-16 | FDD and HD-FDD Band 87 |
| 5 | Frequency band: 412-417, 422-427 MHz | 36.101, 5.5 | Rel-16 | FDD and HD-FDD Band 88 |
| 6 | Frequency band: 699-716, 729-746 MHz | 36.101, 5.5 | Rel-18 | FDD and HD-FDD Band 12 |
| 7 | Frequency band: 2300-2400, 2300-2400 MHz | 36.101, 5.5 | Rel-18 | TDD Band 40 |
| 8 | Frequency band: 3400-3600, 3400-3600 MHz | 36.101, 5.5 | Rel-18 | TDD Band 42 |
| 9 | Frequency band: 896-901, 935-940 MHz | 36.101, 5.5 | Rel-18 | FDD and HD-FDD Band 106 |

Table A.4.3-3d: UE Power Class 2 implementation Capabilities per band

| Item | RF Baseline Implementation Capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Frequency band: 2496-2690, 2496-2690 MHz | 36.101, 5.5 | Rel-10 | TDD Band 41 |
| 2 | Frequency band: 5855-5925, 5855-5925 MHz | 36.101, 5.5 | Rel-14 | TDD Band 47 |
| 3 | Frequency band: 2570-2620, 2570-2620 MHz | 36.101, 5.5 | Rel-15 | TDD Band 38 |
| 4 | Frequency band: 2300-2400, 2300-2400 MHz | 36.101, 5.5 | Rel-15 | TDD Band 40 |
| 5 | Frequency band: 3400-3600, 3400-3600 MHz | 36.101, 5.5 | Rel-15 | TDD Band 42 |
| 6 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | Rel-12 | FDD and HD-FDD Band 31 |
| 7 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | Rel-15 | FDD and HD-FDD Band 72 |

Table A.4.3-4: UE Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category 1 | 36.306, 4.1 | Rel-8 |  |
| 2 | Category 2 | 36.306, 4.1 | Rel-8 |  |
| 3 | Category 3 | 36.306, 4.1 | Rel-8 |  |
| 4 | Category 4 | 36.306, 4.1 | Rel-8 |  |
| 5 | Category 5 | 36.306, 4.1 | Rel-8 | Support for 64QAM in UL |
| 6 | Category 6 | 36.306, 4.1 | Rel-10 |  |
| 7 | Category 7 | 36.306, 4.1 | Rel-10 |  |
| 8 | Category 8 | 36.306, 4.1 | Rel-10 | Support for 64QAM in UL |
| 9 | Category 9 | 36.306, 4.1 | Rel-11 |  |
| 10 | Category 10 | 36.306, 4.1 | Rel-11 |  |
| 11 | Category 11 | 36.306, 4.1 | Rel-11 |  |
| 12 | Category 12 | 36.306, 4.1 | Rel-11 |  |

Table A.4.3-4a: UE Downlink Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category DL 0 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 0 |
| 1a | Category DL 1bis | 36.306, 4.1A | Rel-13 | Only in combination with Category UL 1bis |
| 2 | Category DL 6 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 5 or Category UL 16 |
| 3 | Category DL 7 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 13 or Category UL 18 |
| 4 | Category DL 9 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 5 or Category UL 16 |
| 5 | Category DL 10 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 13 or Category UL 18 |
| 6 | Category DL 11 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 5 or Category UL 16 |
| 7 | Category DL 12 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 13 or Category UL 15 or Category UL 18 or Category UL 20 |
| 8 | Category DL 13 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 16 or Category UL 18 |
| 9 | Category DL 14 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 8 or Category UL 17 |
| 10 | Category DL 15 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 16 or Category UL 18 |
| 11 | Category DL 16 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 |
| 12 | Category DL 17 | 36.306, 4.1A | Rel-13 | Only in combination with Category UL 14 or Category UL 19 |
| 13 | Category DL 18 | 36.306, 4.1A | Rel-13 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 |
| 14 | Category DL 19 | 36.306, 4.1A | Rel-13 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 or Category UL 21 |
| 15 | Category DL 4 | 36.306, 4.1A | Rel-12 | Only in combination with Category UL 5 |
| 16 | Category DL 20 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 or Category UL 21 |
| 17 | Category DL 21 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 3 or Category UL 5 or Category UL 7 or Category UL 13 or Category UL 15 or Category UL 16 or Category UL 18 or Category UL 20 |
| 18 | Category DL 22 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |
| 19 | Category DL 23 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |
| 20 | Category DL 24 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |
| 21 | Category DL 25 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |
| 22 | Category DL 26 | 36.306, 4.1A | Rel-14 | Only in combination with Category UL 20 or Category UL 22 or Category UL 23 or Category UL 24 or Category UL 25 or Category UL 26 |

Table A.4.3-4aa: Additional UE Downlink Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category DL M1 | 36.306, 4.1A | Rel-13 | Only in combination with Category UL M1 |
| 2 | Category DL M2 (NOTE 1) | 36.306, 4.1A | Rel-14 | Only in combination with Category UL M2 |
| NOTE 1: A UE indicating Category M2 shall also indicate Category M1. |  |  |  |  |

Table A.4.3-4b: UE Uplink Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category UL 0 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 0 |
| 1a | Category UL 1bis | 36.306, 4.1A | Rel-13 | Only in combination with Category DL 1bis |
| 2 | Category UL 3 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 13, Category DL 15, Category DL 16, Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 3 | Category UL 5 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 6, Category DL 9, Category DL 11, Category DL 13, Category DL 15 , Category DL 16, Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 4 | Category UL 7 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 13, Category DL 15, Category DL 16, Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 5 | Category UL 8 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 14 |
| 6 | Category UL 13 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL 7, Category DL 10, Category DL 12, Category DL 13, Category DL 15, Category DL 16, Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 7 | Category UL 14 | 36.306, 4.1A | Rel-13 | Only in combination with Category DL 17 |
| 8 | Category UL 15 | 36.306, 4.1A | Rel-13 | Only in combination with Category DL 12 or Category DL 16 or Category DL 18 or Category DL 19 or Category DL 20 or Category DL 21 |
| 9 | Category UL 16 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 6, Category DL 9, Category DL 11, Category DL 13, Category DL 15, Category DL 16. Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 10 | Category UL 17 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 14 |
| 11 | Category UL 18 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 7, Category DL 10, Category DL 12, Category DL 13, Category DL 15, Category DL 16. Category DL 18, Category DL 19, Category DL 20 or Category DL 21 |
| 12 | Category UL 19 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 17 |
| 13 | Category UL 20 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 12, Category DL 16, Category DL 18, Category DL 19, Category DL 20, Category DL 21, Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 14 | Category UL 21 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 19 or Category DL 20 |
| 15 | Category UL 22 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 16 | Category UL 23 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 17 | Category UL 24 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 18 | Category UL 25 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |
| 19 | Category UL 26 | 36.306, 4.1A | Rel-14 | Only in combination with Category DL 22, Category DL 23, Category DL 24, Category DL 25 or Category DL 26 |

Table A.4.3-4ba: Additional UE Uplink Category

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category UL M1 | 36.306, 4.1A | Rel-12 | Only in combination with Category DL M1 |
| 2 | Category UL M2 (NOTE 1) | 36.306, 4.1A | Rel-14 | Only in combination with Category DL M2 |
| NOTE 1: A UE indicating Category M2 shall also indicate Category M1. |  |  |  |  |

Table A.4.3-4c: UE Category NB

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Category NB1 | 36.306, 4.1C | Rel-13 |  |
| 2 | Category NB2 (Note 1) | 36.306, 4.1C | Rel-14 |  |
| NOTE 1: A UE indicating Category NB2 shall also indicate Category NB1. |  |  |  |  |

Table A.4.3-4d: UE Category for V2X Sidelink communication

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | SL-C Category 1 | 36.306, 4.1B | Rel-14 |  |
| 2 | SL-C Category 2 | 36.306, 4.1B | Rel-14 |  |

Table A.4.3-4da: UE Category for reception capabilities for sidelink communication

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | SL-C-RX Category 1 | 36.306, 4.1B | Rel-15 |  |
| 2 | SL-C-RX Category 2 | 36.306, 4.1B | Rel-15 |  |
| 3 | SL-C-RX Category 3 | 36.306, 4.1B | Rel-15 |  |
| 4 | SL-C-RX Category 4 | 36.306, 4.1B | Rel-15 |  |
| NOTE 1: If a UE of this release supports sidelink communication, the UE shall support SL-C-RX Category 1 and SL-C-TX Category 1. If a UE of this release supports V2X sidelink communication, the UE shall support SL-C-RX Category 2 to 4 for reception, and SL-C-TX category 2 to 5 for transmission. |  |  |  |  |

Table A.4.3-4db: UE Category for transmission capabilities for sidelink communication

| Item | UE Category | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | SL-C-TX Category 1 | 36.306, 4.1B | Rel-15 |  |
| 2 | SL-C-TX Category 2 | 36.306, 4.1B | Rel-15 |  |
| 3 | SL-C-TX Category 3 | 36.306, 4.1B | Rel-15 |  |
| 4 | SL-C-TX Category 4 | 36.306, 4.1B | Rel-15 |  |
| 5 | SL-C-TX Category 5 | 36.306, 4.1B | Rel-15 |  |
| NOTE 1: If a UE of this release supports sidelink communication, the UE shall support SL-C-RX Category 1 and SL-C-TX Category 1. If a UE of this release supports V2X sidelink communication, the UE shall support SL-C-RX Category 2 to 4 for reception, and SL-C-TX category 2 to 5 for transmission. |  |  |  |  |

Table A.4.3-5: Void

Table A.4.3-6: Void

Table A.4.3-7: Additional capabilities

| Item | Additional capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Enhanced performance requirements type A for LTE | 36.101, 8 | Rel-11 | Support for Enhanced performance requirements type A |
| 2 | Support of Type B Half-duplex FDD operation | 36.211, 6,2,5 36.306, 4.2.6 | Rel-12 | Support of Half-duplex FDD operation type B for category 0, category M1, category M2, category NB1 and category NB2 UE |
| 3 | Enhanced performance requirements type C for LTE | 36.101, 8 | Rel-12 | Support for Enhanced performance requirements type C |
| 4 | Enhanced performance requirements type B for LTE | 36.101, 8 36.306, 4.3.4.35 | Rel-12 | Support for Enhanced performance requirements type B |
| 5 | Enhanced measurement in high speed scenario | 36.306,4.3.33.1 | Rel-14 | Support measurement enhancements in high speed scenario |
| 6 | Enhanced downlink control channel performance requirements type A for LTE | 36.101, 8 | Rel-13 | Support for Enhanced downlink control channel performance requirements type A for LTE |
| 7 | Enhanced downlink control channel performance requirements type B for LTE | 36.101, 8 | Rel-13 | Support for Enhanced downlink control channel performance requirements type B for LTE |
| 8 | DMRS enhancements for TM9 | 36.306, 4.3.28.4 | Rel-13 | Support for DMRS enhancements in TM9 |
| 9 | aperiodic ZP-CSI-RS reporting | 36.306, 4.3.4.51 | Rel-13 | Support for aperiodic ZP-CSI-RS reporting |

Table A.4.3-8: Void

## A.4.4	Feature group indicators

In Table A.4.4-1a and Table A.4.4-1b, a 'VoLTE capable UE' corresponds to a UE that is capable of the "Voice domain preference for E-UTRAN" defined in TS 24.301 [15] being set to "IMS PS voice only", "IMS PS voice preferred, CS voice as secondary" or "CS voice preferred, IMS PS voice as secondary" (Ref TS 36.331 [14], clause B.1)

When a UE supports E-UTRA FDD only, it’s required to indicate combined FGI capabilities in Table A.4.4-1a, Table A.4.4-2a and Table A.4.4-3a; when a UE supports E-UTRA TDD only, it’s required to indicate combined FGI capabilities in Table A.4.4-1b, Table A.4.4-2b and Table A.4.4-3b; when a UE supports E-UTRA FDD/TDD dual mode with same FGI capabilities on FDD and TDD, it’s required to indicate both FGI capabilities in Table A.4.4-1a, Table A.4.4-2a, Table A.4.4-3a, Table A.4.4-1b, Table A.4.4-2b and Table A.4.4-3b and make sure those FDD and TDD tables are identical.

Note 1:	From Rel-11 onwards 3GPP TSG RAN has discontinued the usage of FGI bits. Instead it has introduced a different mechanism to accomplish the same purposes based on the principles described in TS 36.306 [17] clause 4. This new principles where applicable have been catered for in section A.4.5, e.g. Table A.4.5-2.

Table A.4.4-1:Void

Table A.4.4-1a: Feature group indicators 1-32 for FDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of - Intra-subframe frequency hopping for PUSCH scheduled by UL grant - DCI format 3a (TPC commands for PUCCH and PUSCH with single bit power adjustments) - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PMI - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI | - set to 1 by category M1 UE that has implemented and successfully tested "Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PM" |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_1_F | Corresponding to the Index of Indicator, the leftmost binary bit 1. Set to true if supporting all functionalities in the feature group. |
| 2 | Support of - Simultaneous CQI and ACK/NACK on PUCCH, i.e. PUCCH format 2a and 2b - Absolute TPC command for PUSCH - Resource allocation type 1 for PDSCH - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-0 - UE selected subband CQI without PMI - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_2_F | Corresponding to the Index of Indicator, the leftmost binary bit 2. Set to true if supporting all functionalities in the feature group. |
| 3 | Support of - Semi-persistent scheduling - TTI bundling - 5bit RLC UM SN - 7bit PDCP SN | - can only be set to 1 if the UE has set bit number 7 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_3_F | Corresponding to the Index of Indicator, the leftmost binary bit 3. Set to true if supporting all functionalities in the feature group. |
|  | Support of - 5bit RLC UM SN - 7bit PDCP SN | - can only be set to 1 if the UE has set bit number 7 to 1. | Yes, if UE supports VoLTE | Rel-9, Rel-10 |  |  |  |
|  |  |  | Yes, if UE supports VoLTE. Yes, if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 4 | Support of - Short DRX cycle | - can only be set to 1 if the UE has set bit number 5 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_4_F | Corresponding to the Index of Indicator, the leftmost binary bit 4. Set to true if supporting all functionalities in the feature group. |
| 5 | Support of - Long DRX cycle - DRX command MAC control element |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_5_F | Corresponding to the Index of Indicator, the leftmost binary bit 5. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 6 | Support of - Prioritized bit rate |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_6_F | Corresponding to the Index of Indicator, the leftmost binary bit 6. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 7 | Support of - RLC UM | - can only be set to 0 if the UE does not support voice |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_7_F | Corresponding to the Index of Indicator, the leftmost binary bit 7. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports VoLTE | Rel-9 |  |  |  |
|  |  |  | Yes, if UE supports VoLTE. Yes, if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 8 | Support of - EUTRA RRC_CONNECTED to UTRA CELL_DCH PS handover | - can only be set to 1 if the UE has set bit number 22 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_8_F | Corresponding to the Index of Indicator, the leftmost binary bit 8. Set to true if supporting all functionalities in the feature group. |
|  | Support of - EUTRA RRC_CONNECTED to UTRA FDD or UTRA TDD CELL_DCH PS handover, if the UE supports either only UTRAN FDD or only UTRAN TDD - EUTRA RRC_CONNECTED to UTRA FDD CELL_DCH PS handover, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes (except for category M1 UE), if UE supports UTRA FDD | Rel-9 |  |  |  |
| 9 | Support of - EUTRA RRC_CONNECTED to GERAN GSM_Dedicated handover | - related to SR-VCC - can only be set to 1 if the UE has set bit number 23 to 1 |  | Rel-8, Rel-9, Rel-10 | 36.331, Annex B.1 | pc_FeatrGrp_9_F | Corresponding to the Index of Indicator, the leftmost binary bit 9. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes (except for category M1 UE), if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 10 | Support of - EUTRA RRC_CONNECTED to GERAN (Packet_)Idle by Cell Change Order - EUTRA RRC_CONNECTED to GERAN (Packet_)Idle by Cell Change Order with NACC (Network Assisted Cell Change) |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_10_F | Corresponding to the Index of Indicator, the leftmost binary bit 10. Set to true if supporting all functionalities in the feature group. |
| 11 | Support of - EUTRA RRC_CONNECTED to CDMA2000 1xRTT CS Active handover | - can only be set to 1 if the UE has sets bit number 24 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_11_F | Corresponding to the Index of Indicator, the leftmost binary bit 11. Set to true if supporting all functionalities in the feature group. |
| 12 | Support of - EUTRA RRC_CONNECTED to CDMA2000 HRPD Active handover | - can only be set to 1 if the UE has set bit number 26 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_12_F | Corresponding to the Index of Indicator, the leftmost binary bit 12. Set to true if supporting all functionalities in the feature group. |
| 13 | Support of - Inter-frequency handover (within FDD or TDD) | - can only be set to 1 if the UE has set bit number 25 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_13_F | Corresponding to the Index of Indicator, the leftmost binary bit 13. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes (except for category M1 UE), unless UE only supports band 13 | Rel-9 |  |  |  |
| 14 | Support of - Measurement reporting event: Event A4 - Neighbour > threshold - Measurement reporting event: Event A5 - Serving < threshold1 & Neighbour > threshold2 |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_14_F | Corresponding to the Index of Indicator, the leftmost binary bit 14. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes (except for category M1 UE) | Rel-9 |  |  |  |
| 15 | Support of - Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Measurement reporting event: Event B1 - Neighbour > threshold for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set at least one of the bit number 22, 23, 24, 26 or 39 to 1. - even if the UE sets bits 41, it shall still set bit 15 to 1 if measurement reporting event B1 is tested for all RATs supported by UE - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_15_F | Corresponding to the Index of Indicator, the leftmost binary bit 15. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes for FDD, if UE supports only UTRAN FDD and does not support UTRAN TDD or GERAN or 1xRTT or HRPD | Rel-9 |  |  |  |
| 16 | Support of - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells; - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells, if the UE has set bit number 25 to 1; and - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN, GERAN, 1xRTT or HRPD, if the UE has set bit number 22, 23, 24 or 26 to 1, respectively. NOTE: Event triggered periodical reporting (i.e. with triggerType set to event and with reportAmount > 1) is a mandatory functionality of event triggered reporting and therefore not the subject of this bit. Support of - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells, if the UE has set bit number 25 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively. NOTE: Event triggered periodical reporting (i.e., with triggerType set to event and with reportAmount > 1) is a mandatory functionality of event triggered reporting and therefore not the subject of this bit. | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_16_F | Corresponding to the Index of Indicator, the leftmost binary bit 16. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 17 | Support of Intra-frequency ANR features including: - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 to 1. - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_17_F | Corresponding to the Index of Indicator, the leftmost binary bit 17. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 18 | Support of Inter-frequency ANR features including: - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 to 1. - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_18_F | Corresponding to the Index of Indicator, the leftmost binary bit 18. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, unless UE only supports band 13 | Rel-9 |  |  |  |
| 19 | Support of Inter-RAT ANR features including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, if the UE has set bit number 23 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN, 1xRTT or HRPD, if the UE has set bit number 22, 24 or 26 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN, GERAN, 1xRTT or HRPD, if the UE has set bit number 22, 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set bit number 5 to 1 and the UE has set at least one of the bit number 22, 23, 24 or 26 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_19_F | Corresponding to the Index of Indicator, the leftmost binary bit 19. Set to true if supporting all functionalities in the feature group. |
|  | Support of Inter-RAT ANR features including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, if the UE has set bit number 23 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for 1xRTT or HRPD, if the UE has set bit number 24 or 26 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRANTDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set bit number 5 to 1 and the UE has set at least one of the bit number 22, 39, 23, 24 or 26 to 1. - even if the UE sets bits 33 to 37, it shall still set bit 19 to 1 if inter-RAT ANR features are tested for all RATs for which inter-RAT measurement reporting is indicated as tested |  | Rel-9 |  |  |  |
| 20 | If bit number 7 is set to ' 0': - SRB1 and SRB2 for DCCH + 8x AM DRB If bit number 7 is set to ' 1': - SRB1 and SRB2 for DCCH + 8x AM DRB - SRB1 and SRB2 for DCCH + 5x AM DRB + 3x UM DRB NOTE: UE which indicate support for a DRB combination also support all subsets of the DRB combination. Therefore, release of DRB(s) never results in an unsupported DRB combination. | - Regardless of what bit number 7 and bit number 20 is set to, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB - Regardless of what bit number 20 is set to, if bit number 7 is set to ' 1', UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB + 1x UM DRB |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_20_F | Corresponding to the Index of Indicator, the leftmost binary bit 20. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 21 | Support of - Predefined intra- and inter-subframe frequency hopping for PUSCH with N_sb > 1 - Predefined inter-subframe frequency hopping for PUSCH with N_sb > 1 | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_21_F | Corresponding to the Index of Indicator, the leftmost binary bit 21. Set to true if supporting all functionalities in the feature group. |
| 22 | Support of - UTRAN measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_22_F | Corresponding to the Index of Indicator, the leftmost binary bit 22. Set to true if supporting all functionalities in the feature group. |
|  | Support of - UTRAN FDD or UTRAN TDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports either only UTRAN FDD or only UTRAN TDD - UTRAN FDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes for FDD, if UE supports UTRA FDD | Rel-9 |  |  |  |
| 23 | Support of - GERAN measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_23_F | Corresponding to the Index of Indicator, the leftmost binary bit 23. Set to true if supporting all functionalities in the feature group. |
| 24 | Support of - 1xRTT measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_24_F | Corresponding to the Index of Indicator, the leftmost binary bit 24. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports enhanced 1xRTT CSFB | Rel-9 |  |  |  |
| 25 | Support of - Inter-frequency measurements and reporting in E-UTRA connected mode NOTE: The UE setting this bit to 1 and indicating support for FDD and TDD frequency bands in the UE capability signalling implements and is tested for FDD measurements while the UE is in TDD, and for TDD measurements while the UE is in FDD. | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_25_F | Corresponding to the Index of Indicator, the leftmost binary bit 25. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, unless UE only supports band 13 | Rel-9 |  |  |  |
| 26 | Support of - HRPD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_26_F | Corresponding to the Index of Indicator, the leftmost binary bit 26. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports HRPD | Rel-9 |  |  |  |
| 27 | Support of - EUTRA RRC_CONNECTED to UTRA CELL_DCH CS handover | - related to SR-VCC - can only be set to 1 if the UE has set bit number 8 to 1 and supports SR-VCC from EUTRA defined in TS 24.008 - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_27_F | Corresponding to the Index of Indicator, the leftmost binary bit 27. Set to true if supporting all functionalities in the feature group. |
|  | Support of - EUTRA RRC_CONNECTED to UTRA FDD or UTRA TDD CELL_DCH CS handover, if the UE supports either only UTRAN FDD or only UTRAN TDD - EUTRA RRC_CONNECTED to UTRA FDD CELL_DCH CS handover, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes for FDD, if UE supports VoLTE and UTRA FDD | Rel-9 |  |  |  |
| 28 | Support of - TTI bundling | - If a category M1 UE does not support this feature group, this bit shall be set to 0. | Yes for FDD | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_28_F | Corresponding to the Index of Indicator, the leftmost binary bit 28. Set to true if supporting all functionalities in the feature group. |
| 29 | Support of - Semi-Persistent Scheduling | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_29_F | Corresponding to the Index of Indicator, the leftmost binary bit 29. Set to true if supporting all functionalities in the feature group. |
| 30 | Support of - Handover between FDD and TDD | - can only be set to 1 if the UE has set bit number 13 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_30_F | Corresponding to the Index of Indicator, the leftmost binary bit 30. Set to true if supporting all functionalities in the feature group. |
| 31 | Support of - Indicates whether the UE supports the mechanisms defined for cells broadcasting multi band information i.e. comprehending multiBandInfoList, disregarding in RRC_CONNECTED the related system information fields and understanding the EARFCN signalling for all bands, that overlap with the bands supported by the UE, and that are defined in the earliest version of TS 36.101 [42] that includes all UE supported bands. | - In this release of the protocol, this bit will never be mandated to be set to 1 - This FGI bit concerns an optional release independent feature (as it was difficult to introduce this from REL-8 when using regular UE capability signalling) |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_31_F | Corresponding to the Index of Indicator, the leftmost binary bit 31. Set to true if supporting all functionalities in the feature group. |
|  |  |  |  | Rel-9 |  |  |  |
|  |  |  | Yes | Rel-10 |  |  |  |
| 32 | Undefined |  |  | Rel-8 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 32. |

Table A.4.4-1b: Feature group indicators 1-32 for TDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of - Intra-subframe frequency hopping for PUSCH scheduled by UL grant - DCI format 3a (TPC commands for PUCCH and PUSCH with single bit power adjustments) - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PMI - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI | - set to 1 by category M1 UE that has implemented and successfully tested "Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PM" |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_1_T | Corresponding to the Index of Indicator, the leftmost binary bit 1. Set to true if supporting all functionalities in the feature group. |
| 2 | Support of - Simultaneous CQI and ACK/NACK on PUCCH, i.e. PUCCH format 2a and 2b - Absolute TPC command for PUSCH - Resource allocation type 1 for PDSCH - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-0 - UE selected subband CQI without PMI - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_2_T | Corresponding to the Index of Indicator, the leftmost binary bit 2. Set to true if supporting all functionalities in the feature group. |
| 3 | Support of - Semi-persistent scheduling - TTI bundling - 5bit RLC UM SN - 7bit PDCP SN | - can only be set to 1 if the UE has set bit number 7 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_3_T | Corresponding to the Index of Indicator, the leftmost binary bit 3. Set to true if supporting all functionalities in the feature group. |
|  | Support of - 5bit RLC UM SN - 7bit PDCP SN | - can only be set to 1 if the UE has set bit number 7 to 1. | Yes, if UE supports VoLTE | Rel-9, Rel-10 |  |  |  |
|  |  |  | Yes, if UE supports VoLTE. Yes, if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 4 | Support of - Short DRX cycle | - can only be set to 1 if the UE has set bit number 5 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_4_T | Corresponding to the Index of Indicator, the leftmost binary bit 4. Set to true if supporting all functionalities in the feature group. |
| 5 | Support of - Long DRX cycle - DRX command MAC control element |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_5_T | Corresponding to the Index of Indicator, the leftmost binary bit 5. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 6 | Support of - Prioritized bit rate |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_6_T | Corresponding to the Index of Indicator, the leftmost binary bit 6. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 7 | Support of - RLC UM | - can only be set to 0 if the UE does not support voice |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_7_T | Corresponding to the Index of Indicator, the leftmost binary bit 7. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports VoLTE | Rel-9 |  |  |  |
|  |  |  | Yes, if UE supports VoLTE. Yes, if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 8 | Support of - EUTRA RRC_CONNECTED to UTRA CELL_DCH PS handover | - can only be set to 1 if the UE has set bit number 22 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_8_T | Corresponding to the Index of Indicator, the leftmost binary bit 8. Set to true if supporting all functionalities in the feature group. |
|  | Support of - EUTRA RRC_CONNECTED to UTRA FDD or UTRA TDD CELL_DCH PS handover, if the UE supports either only UTRAN FDD or only UTRAN TDD - EUTRA RRC_CONNECTED to UTRA FDD CELL_DCH PS handover, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes, if UE supports UTRA | Rel-9 |  |  |  |
| 9 | Support of - EUTRA RRC_CONNECTED to GERAN GSM_Dedicated handover | - related to SR-VCC - can only be set to 1 if the UE has set bit number 23 to 1 |  | Rel-8, Rel-9, Rel-10 | 36.331, Annex B.1 | pc_FeatrGrp_9_T | Corresponding to the Index of Indicator, the leftmost binary bit 9. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes (except for category M1 UE), if UE supports SRVCC to EUTRAN from GERAN. | Rel-11 |  |  |  |
| 10 | Support of - EUTRA RRC_CONNECTED to GERAN (Packet_)Idle by Cell Change Order - EUTRA RRC_CONNECTED to GERAN (Packet_)Idle by Cell Change Order with NACC (Network Assisted Cell Change) |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_10_T | Corresponding to the Index of Indicator, the leftmost binary bit 10. Set to true if supporting all functionalities in the feature group. |
| 11 | Support of - EUTRA RRC_CONNECTED to CDMA2000 1xRTT CS Active handover | - can only be set to 1 if the UE has sets bit number 24 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_11_T | Corresponding to the Index of Indicator, the leftmost binary bit 11. Set to true if supporting all functionalities in the feature group. |
| 12 | Support of - EUTRA RRC_CONNECTED to CDMA2000 HRPD Active handover | - can only be set to 1 if the UE has set bit number 26 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_12_T | Corresponding to the Index of Indicator, the leftmost binary bit 12. Set to true if supporting all functionalities in the feature group. |
| 13 | Support of - Inter-frequency handover (within FDD or TDD) | - can only be set to 1 if the UE has set bit number 25 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_13_T | Corresponding to the Index of Indicator, the leftmost binary bit 13. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes (except for category M1 UE), unless UE only supports band 13 | Rel-9 |  |  |  |
| 14 | Support of - Measurement reporting event: Event A4 - Neighbour > threshold - Measurement reporting event: Event A5 - Serving < threshold1 & Neighbour > threshold2 |  |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_14_T | Corresponding to the Index of Indicator, the leftmost binary bit 14. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes (except for category M1 UE) | Rel-9 |  |  |  |
| 15 | Support of - Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Measurement reporting event: Event B1 - Neighbour > threshold for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set at least one of the bit number 22, 23, 24, 26 or 39 to 1. - even if the UE sets bits 41, it shall still set bit 15 to 1 if measurement reporting event B1 is tested for all RATs supported by UE - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_15_T | Corresponding to the Index of Indicator, the leftmost binary bit 15. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes for FDD, if UE supports only UTRAN FDD and does not support UTRAN TDD or GERAN or 1xRTT or HRPD | Rel-9 |  |  |  |
| 16 | Support of - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells; - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells, if the UE has set bit number 25 to 1; and - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN, GERAN, 1xRTT or HRPD, if the UE has set bit number 22, 23, 24 or 26 to 1, respectively. NOTE: Event triggered periodical reporting (i.e. with triggerType set to event and with reportAmount > 1) is a mandatory functionality of event triggered reporting and therefore not the subject of this bit. Support of - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells, if the UE has set bit number 25 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively. NOTE: Event triggered periodical reporting (i.e., with triggerType set to event and with reportAmount > 1) is a mandatory functionality of event triggered reporting and therefore not the subject of this bit. | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_16_T | Corresponding to the Index of Indicator, the leftmost binary bit 16. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 17 | Support of Intra-frequency ANR features including: - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Intra-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 to 1. - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_17_T | Corresponding to the Index of Indicator, the leftmost binary bit 17. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 18 | Support of Inter-frequency ANR features including: - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-frequency periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 to 1. - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_18_T | Corresponding to the Index of Indicator, the leftmost binary bit 18. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, unless UE only supports band 13 | Rel-9 |  |  |  |
| 19 | Support of Inter-RAT ANR features including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, if the UE has set bit number 23 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN, 1xRTT or HRPD, if the UE has set bit number 22, 24 or 26 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN, GERAN, 1xRTT or HRPD, if the UE has set bit number 22, 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set bit number 5 to 1 and the UE has set at least one of the bit number 22, 23, 24 or 26 to 1. |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_19_T | Corresponding to the Index of Indicator, the leftmost binary bit 19. Set to true if supporting all functionalities in the feature group. |
|  | Support of Inter-RAT ANR features including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells for GERAN, if the UE has set bit number 23 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRAN TDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON for 1xRTT or HRPD, if the UE has set bit number 24 or 26 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN FDD or UTRAN TDD, if the UE supports either only UTRAN FDD or only UTRANTDD and has set bit number 22 to 1 - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for UTRAN FDD or UTRAN TDD, if the UE supports both UTRAN FDD and UTRAN TDD and has set bit number 22 or 39 to 1, respectively - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI for GERAN, 1xRTT or HRPD, if the UE has set bit number 23, 24 or 26 to 1, respectively | - can only be set to 1 if the UE has set bit number 5 to 1 and the UE has set at least one of the bit number 22, 39, 23, 24 or 26 to 1. - even if the UE sets bits 33 to 37, it shall still set bit 19 to 1 if inter-RAT ANR features are tested for all RATs for which inter-RAT measurement reporting is indicated as tested |  | Rel-9 |  |  |  |
| 20 | If bit number 7 is set to ‘0’: - SRB1 and SRB2 for DCCH + 8x AM DRB If bit number 7 is set to ‘1’: - SRB1 and SRB2 for DCCH + 8x AM DRB - SRB1 and SRB2 for DCCH + 5x AM DRB + 3x UM DRB NOTE: UE which indicate support for a DRB combination also support all subsets of the DRB combination. Therefore, release of DRB(s) never results in an unsupported DRB combination. | - Regardless of what bit number 7 and bit number 20 is set to, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB - Regardless of what bit number 20 is set to, if bit number 7 is set to ‘1’, UE shall support at least SRB1 and SRB2 for DCCH + 4x AM DRB + 1x UM DRB |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_20_T | Corresponding to the Index of Indicator, the leftmost binary bit 20. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes | Rel-9 |  |  |  |
| 21 | Support of - Predefined intra- and inter-subframe frequency hopping for PUSCH with N_sb > 1 - Predefined inter-subframe frequency hopping for PUSCH with N_sb > 1 | - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_21_T | Corresponding to the Index of Indicator, the leftmost binary bit 21. Set to true if supporting all functionalities in the feature group. |
| 22 | Support of - UTRAN measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_22_T | Corresponding to the Index of Indicator, the leftmost binary bit 22. Set to true if supporting all functionalities in the feature group. |
|  | Support of - UTRAN FDD or UTRAN TDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports either only UTRAN FDD or only UTRAN TDD - UTRAN FDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes for FDD, if UE supports UTRA FDD | Rel-9 |  |  |  |
| 23 | Support of - GERAN measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_23_T | Corresponding to the Index of Indicator, the leftmost binary bit 23. Set to true if supporting all functionalities in the feature group. |
| 24 | Support of - 1xRTT measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_24_T | Corresponding to the Index of Indicator, the leftmost binary bit 24. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports enhanced 1xRTT CSFB | Rel-9 |  |  |  |
| 25 | Support of - Inter-frequency measurements and reporting in E-UTRA connected mode NOTE: The UE setting this bit to 1 and indicating support for FDD and TDD frequency bands in the UE capability signalling implements and is tested for FDD measurements while the UE is in TDD, and for TDD measurements while the UE is in FDD. | - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_25_T | Corresponding to the Index of Indicator, the leftmost binary bit 25. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, unless UE only supports band 13 | Rel-9 |  |  |  |
| 26 | Support of - HRPD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode | - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_26_T | Corresponding to the Index of Indicator, the leftmost binary bit 26. Set to true if supporting all functionalities in the feature group. |
|  |  |  | Yes, if UE supports HRPD | Rel-9 |  |  |  |
| 27 | Support of - EUTRA RRC_CONNECTED to UTRA CELL_DCH CS handover | - related to SR-VCC - can only be set to 1 if the UE has set bit number 8 to 1 and supports SR-VCC from EUTRA defined in TS 24.008 - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_27_T | Corresponding to the Index of Indicator, the leftmost binary bit 27. Set to true if supporting all functionalities in the feature group. |
|  | Support of - EUTRA RRC_CONNECTED to UTRA FDD or UTRA TDD CELL_DCH CS handover, if the UE supports either only UTRAN FDD or only UTRAN TDD - EUTRA RRC_CONNECTED to UTRA FDD CELL_DCH CS handover, if the UE supports both UTRAN FDD and UTRAN TDD |  | Yes for FDD, if UE supports VoLTE and UTRA FDD | Rel-9 |  |  |  |
| 28 | Support of - TTI bundling | - If a category M1 UE does not support this feature group, this bit shall be set to 0 | Yes for FDD | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_28_T | Corresponding to the Index of Indicator, the leftmost binary bit 28. Set to true if supporting all functionalities in the feature group. |
| 29 | Support of - Semi-Persistent Scheduling | - If a category M1 UE does not support this feature group, this bit shall be set to 0 |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_29_T | Corresponding to the Index of Indicator, the leftmost binary bit 29. Set to true if supporting all functionalities in the feature group. |
| 30 | Support of - Handover between FDD and TDD | - can only be set to 1 if the UE has set bit number 13 to 1 |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_30_T | Corresponding to the Index of Indicator, the leftmost binary bit 30. Set to true if supporting all functionalities in the feature group. |
| 31 | Support of - Indicates whether the UE supports the mechanisms defined for cells broadcasting multi band information i.e. comprehending multiBandInfoList, disregarding in RRC_CONNECTED the related system information fields and understanding the EARFCN signalling for all bands, that overlap with the bands supported by the UE, and that are defined in the earliest version of TS 36.101 [42] that includes all UE supported bands. | - In this release of the protocol, this bit will never be mandated to be set to 1 - This FGI bit concerns an optional release independent feature (as it was difficult to introduce this from REL-8 when using regular UE capability signalling) |  | Rel-8 | 36.331, Annex B.1 | pc_FeatrGrp_31_T | Corresponding to the Index of Indicator, the leftmost binary bit 31. Set to true if supporting all functionalities in the feature group. |
|  |  |  |  | Rel-9 |  |  |  |
|  |  |  | Yes | Rel-10 |  |  |  |
| 32 | Undefined |  |  | Rel-8 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 32. |

Table A.4.4-2: Void

Table A.4.4-2a: Feature group indicators 33-64 for FDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | Inter-RAT ANR features for UTRAN including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 22 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_33_F | Corresponding to the Index of Indicator, the leftmost binary bit 33. Set to true if supporting all functionalities in the feature group. |
| 34 | Inter-RAT ANR features for GERAN including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 23 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_34_F | Corresponding to the Index of Indicator, the leftmost binary bit 34. Set to true if supporting all functionalities in the feature group. |
| 35 | Inter-RAT ANR features for 1xRTT including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 24 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_35_F | Corresponding to the Index of Indicator, the leftmost binary bit 35. Set to true if supporting all functionalities in the feature group. |
| 36 | Inter-RAT ANR features for HRPD including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 26 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_36_F | Corresponding to the Index of Indicator, the leftmost binary bit 36. Set to true if supporting all functionalities in the feature group. |
| 37 | Inter-RAT ANR features for UTRAN TDD including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and at least one of the bit number 22 (for UEs supporting only UTRA TDD) or the bit number 39 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_37_F | Corresponding to the Index of Indicator, the leftmost binary bit 37. Set to true if supporting all functionalities in the feature group. |
| 38 | -EUTRA RRC_CONNECTED to UTRA TDD CELL_DCH PS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - can only be set to 1 if the UE has set bit number 39 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_38_F | Corresponding to the Index of Indicator, the leftmost binary bit 38. Set to true if supporting all functionalities in the feature group. |
| 39 | -UTRAN TDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports both UTRAN FDD and UTRAN TDD | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_39_F | Corresponding to the Index of Indicator, the leftmost binary bit 39. Set to true if supporting all functionalities in the feature group. |
| 40 | -EUTRA RRC_CONNECTED to UTRA TDD CELL_DCH CS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - related to SR-VCC - can only be set to 1 if the UE has set bit number 38 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_40_F | Corresponding to the Index of Indicator, the leftmost binary bit 40. Set to true if supporting all functionalities in the feature group. |
| 41 | Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD, if the UE supports UTRAN FDD and has set bit number 22 to 1 | - If a category M1 UE does not support this feature group, this bit shall be set to 0. | Yes for FDD, unless UE has set bit number 15 to 1 | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_41_F | Corresponding to the Index of Indicator, the leftmost binary bit 41. Set to true if supporting all functionalities in the feature group. |
| 42 | DCI format 3a (TPC commands for PUCCH and PUSCH with single bit power adjustments) |  |  | Rel-13 | 36.331, Annex B.1 | pc_FeatrGrp_42_F | Corresponding to the Index of Indicator, the leftmost binary bit 42. |
| 43 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 43. |
| 44 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 44. |
| 45 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 45. |
| 46 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 46. |
| 47 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 47. |
| 48 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 48. |
| 49 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 49. |
| 50 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 50. |
| 51 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 51. |
| 52 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 52. |
| 53 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 53. |
| 54 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 54. |
| 55 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 55. |
| 56 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 56. |
| 57 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 57. |
| 58 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 58. |
| 59 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 59. |
| 60 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 60. |
| 61 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 61. |
| 62 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 62. |
| 63 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 63. |
| 64 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 64. |

Table A.4.4-2b: Feature group indicators 33-64 for TDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | Inter-RAT ANR features for UTRAN including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 22 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_33_T | Corresponding to the Index of Indicator, the leftmost binary bit 33. Set to true if supporting all functionalities in the feature group. |
| 34 | Inter-RAT ANR features for GERAN including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCells - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 23 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_34_T | Corresponding to the Index of Indicator, the leftmost binary bit 34. Set to true if supporting all functionalities in the feature group. |
| 35 | Inter-RAT ANR features for 1xRTT including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 24 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_35_T | Corresponding to the Index of Indicator, the leftmost binary bit 35. Set to true if supporting all functionalities in the feature group. |
| 36 | Inter-RAT ANR features for HRPD including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and bit number 26 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_36_T | Corresponding to the Index of Indicator, the leftmost binary bit 36. Set to true if supporting all functionalities in the feature group. |
| 37 | Inter-RAT ANR features for UTRAN TDD including: - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportStrongestCellsForSON - Inter-RAT periodical measurement reporting where triggerType is set to periodical and purpose is set to reportCGI | - can only be set to 1 if the UE has set bit number 5 and at least one of the bit number 22 (for UEs supporting only UTRA TDD) or the bit number 39 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_37_T | Corresponding to the Index of Indicator, the leftmost binary bit 37. Set to true if supporting all functionalities in the feature group. |
| 38 | -EUTRA RRC_CONNECTED to UTRA TDD CELL_DCH PS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - can only be set to 1 if the UE has set bit number 39 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_38_T | Corresponding to the Index of Indicator, the leftmost binary bit 38. Set to true if supporting all functionalities in the feature group. |
| 39 | -UTRAN TDD measurements, reporting and measurement reporting event B2 in E-UTRA connected mode, if the UE supports both UTRAN FDD and UTRAN TDD | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_39_T | Corresponding to the Index of Indicator, the leftmost binary bit 39. Set to true if supporting all functionalities in the feature group. |
| 40 | -EUTRA RRC_CONNECTED to UTRA TDD CELL_DCH CS handover, if the UE supports both UTRAN FDD and UTRAN TDD | - related to SR-VCC - can only be set to 1 if the UE has set bit number 38 to 1. |  | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_40_T | Corresponding to the Index of Indicator, the leftmost binary bit 40. Set to true if supporting all functionalities in the feature group. |
| 41 | Measurement reporting event: Event B1 - Neighbour > threshold for UTRAN FDD, if the UE supports UTRAN FDD and has set bit number 22 to 1 | - If a category M1 UE does not support this feature group, this bit shall be set to 0. | Yes for FDD, unless UE has set bit number 15 to 1 | Rel-9 | 36.331, Annex B.1 | pc_FeatrGrp_41_T | Corresponding to the Index of Indicator, the leftmost binary bit 41. Set to true if supporting all functionalities in the feature group. |
| 42 | DCI format 3a (TPC commands for PUCCH and PUSCH with single bit power adjustments) |  |  | Rel-13 | 36.331, Annex B.1 | pc_FeatrGrp_42_T | Corresponding to the Index of Indicator, the leftmost binary bit 42. |
| 43 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 43. |
| 44 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 44. |
| 45 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 45. |
| 46 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 46. |
| 47 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 47. |
| 48 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 48. |
| 49 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 49. |
| 50 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 50. |
| 51 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 51. |
| 52 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 52. |
| 53 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 53. |
| 54 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 54. |
| 55 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 55. |
| 56 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 56. |
| 57 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 57. |
| 58 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 58. |
| 59 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 59. |
| 60 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 60. |
| 61 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 61. |
| 62 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 62. |
| 63 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 63. |
| 64 | Undefined |  |  | Rel-9 | 36.331, Annex B.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 64. |

Table A.4.4-3: Void

Table A.4.4-3a: Feature group indicators 101-132 for FDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | - DMRS with OCC (orthogonal cover code) and SGH (sequence group hopping) disabling | - if the UE supports two or more layers for spatial multiplexing in UL, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_101_F | Corresponding to the Index of Indicator, the leftmost binary bit 101. Set to true if supporting all functionalities in the feature group. |
|  |  | - If a category 0 UE does not support this feature, this bit shall be set to 0. |  | Rel-12 |  |  |  |
| 102 | - Trigger type 1 SRS (aperiodic SRS) transmission (Up to X ports) NOTE: X = number of supported layers on given band |  |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_102_F | Corresponding to the Index of Indicator, the leftmost binary bit 102. Set to true if supporting all functionalities in the feature group. |
| 103 | - PDSCH transmission mode 9 when up to 4 CSI reference signal ports are configured | - for Category 8 UEs, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_103_F | Corresponding to the Index of Indicator, the leftmost binary bit 103. Set to true if supporting all functionalities in the feature group. |
|  |  | - for Category 8 UEs, this bit shall be set to 1. - for Category 11 and higher UEs, this bit shall be set to 1. - for DL Category 11 and higher UEs (except for DL Category 13), this bit shall be set to 1. | Yes for the UE categories listed in the column “Notes” | Rel-15 |  |  |  |
| 104 | - PDSCH transmission mode 9 for TDD when 8 CSI reference signal ports are configured | - if the UE does not support TDD, this bit is irrelevant (capability signalling exists for FDD for this feature), and this bit shall be set to 0. - for Category 8 UEs, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_104_F | Corresponding to the Index of Indicator, the leftmost binary bit 104. Set to true if supporting all functionalities in the feature group. |
|  |  | - if the UE does not support TDD, this bit is irrelevant, and this bit shall be set to 0. - this bit is not applicable to FDD (capability signalling exists for FDD for this feature). - for Category 8 UEs, this bit shall be set to 1. - for Category 11 and higher UEs, this bit shall be set to 1. - for DL Category 11 and higher UEs (except for DL Category 13), this bit shall be set to 1. | Yes for TDD, for the UE categories listed in the column “Notes” | Rel-15 |  |  |  |
| 105 | - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-0 - UE selected subband CQI without PMI, when PDSCH transmission mode 9 is configured - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI, when PDSCH transmission mode 9 and up to 4 CSI reference signal ports are configured | - this bit can be set to 1 only if indices 2 (Table B.1-1) and 103 are set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_105_F | Corresponding to the Index of Indicator, the leftmost binary bit 105. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if index 2 is set to 1 for both FDD and TDD, and index 103 is set to 1 either for FDD and TDD. |  | Rel-12 |  |  |  |
| 106 | - Periodic CQI/PMI/RI/PTI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI, when PDSCH transmission mode 9 and 8 CSI reference signal ports are configured | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ' supported') and if index 2 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_106_F | Corresponding to the Index of Indicator, the leftmost binary bit 106. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to' supported', and if index 2 is set to 1 for both FDD and TDD. |  | Rel-12 |  |  |  |
| 107 | - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PMI, when PDSCH transmission mode 9 is configured - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI, when PDSCH transmission mode 9 and up to 4 CSI reference signal ports are configured | - this bit can be set to 1 only if indices 1 (Table B.1-1) and 103 are set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_107_F | Corresponding to the Index of Indicator, the leftmost binary bit 107. Set to true if supporting all functionalities in the feature group. |
| 108 | - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI, when PDSCH transmission mode 9 and 8 CSI reference signal ports are configured | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to' supported') and if index 1 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_108_F | Corresponding to the Index of Indicator, the leftmost binary bit 108. Set to true if supporting all functionalities in the feature group. |
| 109 | - Periodic CQI/PMI/RI reporting on PUCCH Mode 1-1, submode 1 | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to' supported'). |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_109_F | Corresponding to the Index of Indicator, the leftmost binary bit 109. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to ‘supported’. |  | Rel-12 |  |  |  |
| 110 | - Periodic CQI/PMI/RI reporting on PUCCH Mode 1-1, submode 2 | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to 'supported'). |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_110_F | Corresponding to the Index of Indicator, the leftmost binary bit 110. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to ‘supported’. |  | Rel-12 |  |  |  |
| 111 | - Measurement reporting trigger Event A6 | - this bit can be set to 1 only if the UE supports carrier aggregation. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_111_F | Corresponding to the Index of Indicator, the leftmost binary bit 111. Set to true if supporting all functionalities in the feature group. |
| 112 | - SCell addition within3 the Handover to EUTRA procedure | - this bit can be set to 1 only if the UE supports carrier aggregation and the Handover to EUTRA procedure. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_112_F | Corresponding to the Index of Indicator, the leftmost binary bit 112. Set to true if supporting all functionalities in the feature group. |
| 113 | - Trigger type 0 SRS (periodic SRS) transmission on X Serving Cells NOTE: X = number of supported component carriers in a given band combination | - this bit can be set to 1 only if the UE supports carrier aggregation in UL. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_113_F | Corresponding to the Index of Indicator, the leftmost binary bit 113. Set to true if supporting all functionalities in the feature group. |
| 114 | - Reporting of both UTRA CPICH RSCP and Ec/N0 in a Measurement Report | - this bit can be set to 1 only if index 22 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_114_F | Corresponding to the Index of Indicator, the leftmost binary bit 114. Set to true if supporting all functionalities in the feature group. |
| 115 | - time domain ICIC RLM/RRM measurement subframe restriction for the serving cell - time domain ICIC RRM measurement subframe restriction for neighbour cells - time domain ICIC CSI measurement subframe restriction | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_115_F | Corresponding to the Index of Indicator, the leftmost binary bit 115. Set to true if supporting all functionalities in the feature group. |
| 116 | - Relative transmit phase continuity for spatial multiplexing in UL | - this bit can be set to 1 only if the UE supports two or more layers for spatial multiplexing in UL. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_116_F | Corresponding to the Index of Indicator, the leftmost binary bit 116. Set to true if supporting all functionalities in the feature group. |
| 117 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 117. |
| 118 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 118. |
| 119 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 119. |
| 120 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 120. |
| 121 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 121. |
| 122 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 122. |
| 123 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 123. |
| 124 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 124. |
| 125 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 125. |
| 126 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 126. |
| 127 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 127. |
| 128 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 128. |
| 129 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 129. |
| 130 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 130. |
| 131 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 131. |
| 132 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 132. |

Table A.4.4-3b: Feature group indicators 101-132 for TDD

| Item | Additional information | Notes | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Release | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | - DMRS with OCC (orthogonal cover code) and SGH (sequence group hopping) disabling | - if the UE supports two or more layers for spatial multiplexing in UL, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_101_T | Corresponding to the Index of Indicator, the leftmost binary bit 101. Set to true if supporting all functionalities in the feature group. |
|  |  | - If a category 0 UE does not support this feature, this bit shall be set to 0. |  | Rel-12 |  |  |  |
| 102 | - Trigger type 1 SRS (aperiodic SRS) transmission (Up to X ports) NOTE: X = number of supported layers on given band |  |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_102_T | Corresponding to the Index of Indicator, the leftmost binary bit 102. Set to true if supporting all functionalities in the feature group. |
| 103 | - PDSCH transmission mode 9 when up to 4 CSI reference signal ports are configured | - for Category 8 UEs, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_103_T | Corresponding to the Index of Indicator, the leftmost binary bit 103. Set to true if supporting all functionalities in the feature group. |
|  |  | - for Category 8 UEs, this bit shall be set to 1. - for Category 11 and higher UEs, this bit shall be set to 1. - for DL Category 11 and higher UEs (except for DL Category 13), this bit shall be set to 1. | Yes for the UE categories listed in the column “Notes” | Rel-15 |  |  |  |
| 104 | - PDSCH transmission mode 9 for TDD when 8 CSI reference signal ports are configured | - if the UE does not support TDD, this bit is irrelevant (capability signalling exists for FDD for this feature), and this bit shall be set to 0. - for Category 8 UEs, this bit shall be set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_104_T | Corresponding to the Index of Indicator, the leftmost binary bit 104. Set to true if supporting all functionalities in the feature group. |
|  |  | - if the UE does not support TDD, this bit is irrelevant, and this bit shall be set to 0. - this bit is not applicable to FDD (capability signalling exists for FDD for this feature). - for Category 8 UEs, this bit shall be set to 1. - for Category 11 and higher UEs, this bit shall be set to 1. - for DL Category 11 and higher UEs (except for DL Category 13), this bit shall be set to 1. | Yes for TDD, for the UE categories listed in the column “Notes” | Rel-15 |  |  |  |
| 105 | - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-0 - UE selected subband CQI without PMI, when PDSCH transmission mode 9 is configured - Periodic CQI/PMI/RI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI, when PDSCH transmission mode 9 and up to 4 CSI reference signal ports are configured | - this bit can be set to 1 only if indices 2 (Table B.1-1) and 103 are set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_105_T | Corresponding to the Index of Indicator, the leftmost binary bit 105. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if index 2 is set to 1 for both FDD and TDD, and index 103 is set to 1 either for FDD and TDD. |  | Rel-12 |  |  |  |
| 106 | - Periodic CQI/PMI/RI/PTI reporting on PUCCH: Mode 2-1 - UE selected subband CQI with single PMI, when PDSCH transmission mode 9 and 8 CSI reference signal ports are configured | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to' supported') and if index 2 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_106_T | Corresponding to the Index of Indicator, the leftmost binary bit 106. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to 'supported', and if index 2 is set to 1 for both FDD and TDD. |  | Rel-12 |  |  |  |
| 107 | - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-0 - UE selected subband CQI without PMI, when PDSCH transmission mode 9 is configured - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI, when PDSCH transmission mode 9 and up to 4 CSI reference signal ports are configured | - this bit can be set to 1 only if indices 1 (Table B.1-1) and 103 are set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_107_T | Corresponding to the Index of Indicator, the leftmost binary bit 107. Set to true if supporting all functionalities in the feature group. |
| 108 | - Aperiodic CQI/PMI/RI reporting on PUSCH: Mode 2-2 - UE selected subband CQI with multiple PMI, when PDSCH transmission mode 9 and 8 CSI reference signal ports are configured | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to ' supported') and if index 1 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_108_T | Corresponding to the Index of Indicator, the leftmost binary bit 108. Set to true if supporting all functionalities in the feature group. |
| 109 | - Periodic CQI/PMI/RI reporting on PUCCH Mode 1-1, submode 1 | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to' supported'). |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_109_T | Corresponding to the Index of Indicator, the leftmost binary bit 109. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to 'supported'. |  | Rel-12 |  |  |  |
| 110 | - Periodic CQI/PMI/RI reporting on PUCCH Mode 1-1, submode 2 | - this bit can be set to 1 only if the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports (i.e., for TDD, if index 104 is set to 1, and for FDD, if tm9-With-8Tx-FDD-r10 is set to' supported'). |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_110_T | Corresponding to the Index of Indicator, the leftmost binary bit 110. Set to true if supporting all functionalities in the feature group. |
|  |  | - For UEs capable of TDD-FDD CA, this bit can be set to 1 for both FDD and TDD if either index 104 is set to 1 or tm9-With-8Tx-FDD-r10 is set to' supported'. |  | Rel-12 |  |  |  |
| 111 | - Measurement reporting trigger Event A6 | - this bit can be set to 1 only if the UE supports carrier aggregation. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_111_T | Corresponding to the Index of Indicator, the leftmost binary bit 111. Set to true if supporting all functionalities in the feature group. |
| 112 | - SCell addition within the Handover to EUTRA procedure | - this bit can be set to 1 only if the UE supports carrier aggregation and the Handover to EUTRA procedure. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_112_T | Corresponding to the Index of Indicator, the leftmost binary bit 112. Set to true if supporting all functionalities in the feature group. |
| 113 | - Trigger type 0 SRS (periodic SRS) transmission on X Serving Cells NOTE: X = number of supported component carriers in a given band combination | - this bit can be set to 1 only if the UE supports carrier aggregation in UL. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_113_T | Corresponding to the Index of Indicator, the leftmost binary bit 113. Set to true if supporting all functionalities in the feature group. |
| 114 | - Reporting of both UTRA CPICH RSCP and Ec/N0 in a Measurement Report | - this bit can be set to 1 only if index 22 (Table B.1-1) is set to 1. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_114_T | Corresponding to the Index of Indicator, the leftmost binary bit 114. Set to true if supporting all functionalities in the feature group. |
| 115 | - time domain ICIC RLM/RRM measurement subframe restriction for the serving cell - time domain ICIC RRM measurement subframe restriction for neighbour cells - time domain ICIC CSI measurement subframe restriction | - If a category M1 UE does not support this feature group, this bit shall be set to 0. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_115_T | Corresponding to the Index of Indicator, the leftmost binary bit 115. Set to true if supporting all functionalities in the feature group. |
| 116 | - Relative transmit phase continuity for spatial multiplexing in UL | - this bit can be set to 1 only if the UE supports two or more layers for spatial multiplexing in UL. |  | Rel-10 | 36.331, Annex C.1 | pc_FeatrGrp_116_T | Corresponding to the Index of Indicator, the leftmost binary bit 116. Set to true if supporting all functionalities in the feature group. |
| 117 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 117. |
| 118 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 118. |
| 119 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 119. |
| 120 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 120. |
| 121 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 121. |
| 122 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 122. |
| 123 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 123. |
| 124 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 124. |
| 125 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 125. |
| 126 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 126. |
| 127 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 127. |
| 128 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 128. |
| 129 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 129. |
| 130 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 130. |
| 131 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 131. |
| 132 | Undefined |  |  | Rel-10 | 36.331, Annex C.1 |  | Corresponding to the Index of Indicator, the leftmost binary bit 132. |

### A.4.5	Additional information

Table A.4.5-1: Additional UE radio access capabilities

| Item | Additional capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Support of CSG | 36.331, Annex B.2 | Rel-8 |  |
| 2 | Support of intra-frequency SI acquisition for HO in FDD | 36.306, 4.3.11.1 | Rel-9 |  |
| 3 | Support of inter-frequency SI acquisition for HO in FDD | 36.306, 4.3.11.2 | Rel-9 |  |
| 4 | Need for inter-frequency gaps (Note 1) | 36.306, 4.3.6.1 | Rel-8 |  |
| 5 | Need for inter-RAT gaps (Note 1) | 36.306, 4.3.6.1 | Rel-8 |  |
| 6 | Support of E-UTRA bands within band group FDD_Nonly | 36.133, Annex A.3.7.2 | Rel-12 |  |
| 7 | Support of rsrqMeasWideband | 36.306, 4.3.6.2 | Rel-11 |  |
| 8 | Support of Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 | 36.306, 4.3.5.5 | Rel-11 |  |
| 9 | Void |  |  |  |
| 10 | Disable E-UTRA capability if IMSVoIP not supported by the network | 23.221, 7.2a, 24.301, 4.5 | Rel-8 | pc_Disable_E-UTRA_NOIMSVoIP |
| 11 | Support of Maximum CSI processes of Three on a component carrier within a band with PDSCH transmission mode 10 | 36.306, 4.3.5.5 | Rel-11 |  |
| 12 | Support of Maximum CSI processes of Four on a component carrier within a band with PDSCH transmission mode 10 | 36.306, 4.3.5.5 | Rel-11 |  |
| 13 | Support of multiClusterPUSCH-WithinCC-r10 | 36.306, 4.3.4.13 | Rel-10 |  |
| 14 | Support of FDD-TDD CA with PCell in TDD band | 36.306, 4.3.4.28 |  | The UE may not send the IE tdd-FDD-CA-PCellDuplex-r12 |
| 15 | Support of FDD-TDD CA with PCell in FDD band | 36.306, 4.3.4.28 |  | The UE may not send the IE tdd-FDD-CA-PCellDuplex-r12 |
| 16 | Support of interRAT-PS-HO-ToGERAN | 36.306, 4.3.7.11 | Rel-8 |  |
| 17 | Support of 64QAM in UL | 36.306, 4.3.4.39 | Rel-12 |  |
| 18 | Support of 256QAM in DL | 36.306, 4.3.5.7 | Rel-12 |  |
| 19 | Support CRS based discovery signals measurement | 36.306, 4.3.6.9 | Rel-12 |  |
| 20 | Support CSI-RS based discovery signals measurement | 36.306, 4.3.6.10 | Rel-12 |  |
| 21 | Support the behaviour on DL signals and physical channels when SCell is deactivated and discovery signals measurement is configured | 36.306, 4.3.4.38 | Rel-12 |  |
| 22 | Support of 4Rx antenna ports | 36.101, 7.2 | Rel-13 |  |
| 23 | Support of ProSe direct communication | 36.306, 4.3.21.1 | Rel-12 |  |
| 24 | Support of ProSe direct discovery | 36.306, 4.3.21.3 | Rel-12 |  |
| 25 | Support of CE mode A | 36.306, 4.3.8.3 | Rel-13 | Mandatory for CAT M1 UE |
| 26 | Support of CE mode B | 36.306, 4.3.29.1 | Rel-13 |  |
| 27 | Support of DC ASYNCH | 36.306, 4.3.29.2 | Rel-12 | The UE supports asynchronous dual connectivity and power control mode 2 |
| 28 | Support of DC SCG DRB | 36.306, 4.3.20.2 | Rel-12 | The UE supports dual connectivity and DRB type of SCG bearer |
| 29 | Support of DC Split DRB | 36.306, 4.3.20.1 | Rel-12 | The UE supports dual connectivity and DRB type of Split bearer |
| 30 | Support of MPR for intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation | 36.306, 4.3.5.10 36.101, H.1 | Rel-10 | ModifiedMPR_Behavior bit 0 (leftmost bit) |
| 31 | Support of A-MPR associated with NS_05 for Band 1 | 36.306, 4.3.5.10 36.101, H.1 | Rel-10 | ModifiedMPR_Behavior bit 1 |
| 32 | supports downlink LAA operation | 36.306, 4.3.23.1 | Rel-13 |  |
| 33 | supports measurement and reporting for RSSI and channel occupancy | 36.306, 4.3.6.19 | Rel-13 |  |
| 34 | Support of User plane CIoT | 24.301, 5.3.15 | Rel-13 |  |
| 35 | Support of EMM-REGISTERED without PDN | 24.301, 5.3.15 | Rel-13 |  |
| 36 | Support of EMM-REGISTERED with PDN | 24.301, 5.3.15 | Rel-13 |  |
| 37 | Support of 4Rx antenna ports in at least one FDD frequency band | 36.101, 7.2 | Rel-13 |  |
| 38 | Support of 4Rx antenna ports in at least one TDD frequency band | 36.101, 7.2 | Rel-13 |  |
| 39 | Support of FDD-TDD CA with PCell in FDD band and SCell with 4Rx supported TDD RF band | 36.306, 4.3.4.28, 36.101, 7.2 | Rel-13 |  |
| 40 | Support of 4Rx antenna ports on all supported FDD operating bands | 36.101, 8.1.2.6.1, 36.133, A.3.8.1 | Rel-13 | UE with same FDD band support declared in tables A.4.3-3 and A.4.5-5 |
| 41 | Support of 4Rx antenna ports on all supported TDD operating bands | 36.101, 8.1.2.6.1, 36.133, A.3.8.1 | Rel-13 | UE with same TDD band support declared in tables A.4.3-3 and A.4.5-5 |
| 42 | Support of A-MPR associated with NS_04 for Band 41 | 36.306, 4.3.5.10 36.101, H.1 | Rel-12 | ModifiedMPR_Behavior bit 2 |
| 43 | Support of RSSI and Channel occupancy reporting | 36.306, 4.3.6.19 | Rel-13 | Support of RSSI and Channel Occupancy. |
| 44 | Support of intra-frequency SI acquisition in TDD for HO | 36.306, 4.3.11.1 | Rel-9 |  |
| 45 | Support of inter-frequency SI acquisition in TDD for HO | 36.306, 4.3.11.2 | Rel-9 |  |
| 46 | Support of 4-layer spatial multiplexing with transmission mode 3 and transmission mode 4 | 36.306, 4.3.5.14. | Rel-10 |  |
| 47 | Void |  |  |  |
| 48 | Support of autonomous resource selection mode with full sensing for V2X sidelink communication | 36.306, 4.3.21.15 | Rel-14 |  |
| 49 | Support of SLSS transmission and reception for V2X sidelink communication | 36.306, 4.3.21.17 | Rel-14 |  |
| 50 | Support of maximum transmit power associated with Power class 2 V2X UE | 36.306, 4.3.21.22 | Rel-14 |  |
| 51 | Support of TM-9 in CE Mode A | 36.306 4.3.29.10 | Rel-13 |  |
| 52 | Support of TM-9 in CE Mode B | 36.306 4.3.29.11 | Rel-13 |  |
| 53 | Support of 4-layer spatial multiplexing with transmission mode 9 and transmission mode 10 | 36.306, 4.3.4.7 | Rel-10 |  |
| 54 | Support of TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a TDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations | 36.306 4.3.4.31 | Rel-12 |  |
| 55 | Support of Rel-12 DL CSI subframe set configuration | 36.306 4.3.4.29 | Rel-12 |  |
| 56 | Support of tm9 operation on LAA cell(s). | 36.306, 4.3.23.6 | Rel-13 |  |
| 57 | Supports of RRM measurements on LAA cell(s) based on CSI-RS-based DRS. | 36.306, 4.3.23.3 | Rel-13 |  |
| 58 | Support of 256QAM in UL | 36.306 4.3.4.73 | Rel-14 |  |
| 59 | Support of SRS switching between a band pair | 36.306, 4.3.5.24, 4.3.5.25 | Rel-14 |  |
| 60 | Support of Make-Before-Break handover | 36.306, 4.3.30.1 | Rel-14 |  |
| 61 | Support of csi-RS-DiscoverySignalsMeas-r12 | 36.331, 6.3.6 | Rel-12 |  |
| 62 | Support of UL LAA | 36.306, 4.3.23.8 | Rel-14 |  |
| 63 | Support of TM-6 in CE Mode A | 36.306 4.3.29.12 | Rel 13 |  |
| 64 | Support of high speed measurement enhancements | 36.306 4.3.33.1 | Rel-14 |  |
| 65 | Support of PUCCH transmission on SCell in CA | 36.306 4.3.4.47 | Rel-13 |  |
| 66 | Support of simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | 36.306 4.3.5.27 | Rel-14 |  |
| 67 | Support of transmitting PSCCH/PSSCH using dynamic scheduling | 36.306 4.3.21.14 | Rel-14 |  |
| 68 | Support of Channel Busy Ratio measurement and reporting of Channel Busy Ratio measurement to eNB for V2X sidelink communication | 36.306 4.3.21.18 | Rel-14 |  |
| 69 | Support of transmission and reception in the configuration of non-adjacent PSCCH and PSSCH for V2X sidelink communication | 36.306 4.3.21.21 | Rel-14 |  |
| 70 | Support of reception of 20 PSCCH in a subframe and decoding of 136 RBs per subframe counting both PSCCH and PSSCH in a band for V2X sidelink communication | 36.306 4.3.21.13 | Rel-14 |  |
| 71 | Support of Increased UE carrier monitoring E-UTRA | 36.306 4.3.6.6 | Rel-12 |  |
| 72 | Support of codebookConfig 1 for non-precoded EBF/ FD-MIMO operation in TM9 | 36.306 4.3.28.6 | Rel-13 |  |
| 73 | Support of codebookConfig 2 for non-precoded EBF/ FD-MIMO operation in TM9 | 36.306 4.3.28.6 | Rel-13 |  |
| 74 | Support of codebookConfig 3 for non-precoded EBF/ FD-MIMO operation in TM9 | 36.306 4.3.28.6 | Rel-13 |  |
| 75 | Support of codebookConfig 4 for non-precoded EBF/ FD-MIMO operation in TM9 | 36.306 4.3.28.6 | Rel-13 |  |
| 76 | Support of Increased UE carrier monitoring UTRA | 36.306 4.3.6.7 | Rel-12 |  |
| 77 | Support of multi-carrier operation | 36.306 4.3.4.115 | Rel-13 |  |
| 78 | Support of reception ending with a subframe occupied for a DwPTS-duration on LAA cell(s) | 36.306 4.3.23.4 | Rel-13 |  |
| 79 | Support of reception of subframes with second slot starting position on LAA cell(s) | 36.306 4.3.23.5 | Rel-13 |  |
| 80 | Support of enhanced 4Tx codebook | 36.306 4.3.4.33 | Rel-12 |  |
| 81 | Support of PUSCH feedback mode 3-2 | 36.306 4.3.4.34 | Rel-12 |  |
| 82 | Support for channel measurement restrictions in TM9 and TM10 | 36.306, 4.3.28.3 | Rel-13 |  |
| 83 | Support for interference measurement restrictions | 36.306 4.3.28.5 | Rel-13 |  |
| 84 | Support of 64QAM for non-repeated unicast PDSCH in RRC_CONNECTED when operating in coverage enhancement mode A | 36.306 4.3.4.126 | Rel-15 |  |
| 85 | Support of alternative CQI table in RRC_CONNECTED when operating in coverage enhancement mode A | 36.306 4.3.4.127 | Rel-15 |  |
| 86 | Support of eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode | 36.306, 4.3.6.31 | Rel-15 |  |
| 87 | Support of having SCell configured in dormant SCell state | 36.306, 4.3.19.18 | Rel-15 |  |
| 88 | Support of asynchronous DAPS handover in source PCell and intra-frequency target PCell | 36.306, 4.3.5.39 | Rel-16 |  |
| 89 | Support of DAPS handover in source PCell and intra-frequency target PCell | 36.306, 4.3.5.40 | Rel-16 |  |
| 90 | Support of asynchronous DAPS handover in source PCell and inter-frequency target PCell | 36.306, 4.3.5.42 | Rel-16 |  |
| 91 | Support of DAPS handover in source PCell and inter-frequency target PCell | 36.306, 4.3.5.43 | Rel-16 |  |
| 92 | Support of conditional handover | 36.306, 4.3.30.3 | Rel-16 |  |
| 93 | Support of conditional handover between FDD and TDD cells | 36.306, 4.3.30.5 | Rel-16 |  |
| 94 | Support of short TTI and/or short processing time | 36.306 4.3.4.150 | Rel-15 | pc_sTTI_SPT |
| 95 | Support of sTTI combination {slot, slot} | 36.306 4.3.4.103 | Rel-15 |  |
| 96 | Support of sTTI combination {subslot, subslot} | 36.306 4.3.4.103 | Rel-15 |  |
| 97 | Support of sTTI combination {subslot, slot} | 36.306 4.3.4.103 | Rel-15 |  |
| 98 | Support of short processing time for the corresponding frame structure types | 36.306 4.3.4.100 | Rel-15 | pc_spt_Parameters |
| 99 | Support NTN access | 36.306 4.3.38.1 | Rel-17 |  |
| 100 | Support Timing advance reporting in NTN cell | 36.306 4.3.38.2 | Rel-17 |  |
| 101 | Support delaying the start of the pur-ResponseWindowTimer for NTN operation | 36.306 4.3.38.3 | Rel-17 |  |
| 102 | Support timing relationship enhancements using Differential Koffset | 36.306 4.3.38.4 | Rel-17 |  |
| 103 | Support NTN features in NGSO scenario | 36.306 4.3.38.5 | Rel-17 |  |
| 104 | Support NTN features in GSO scenario | 36.306 4.3.38.5 | Rel-17 |  |
| 105 | Support to report the supported gap length between segments for PUSCH and PUCCH required by a UE supporting ce-ModeA-r13 or for NPUSCH required by a UE supporting ue-category-NB, for TA pre-compensation. | 36.306 4.3.38.6 | Rel-17 |  |
| 106 | Support DL channel quality reporting of the configured carrier for FDD in RRC_CONNECTED | 36.306 4.3.6.37 | Rel-16 |  |
| 107 | Support UE specific DRX | 24.301 9.9.3.63 | Rel-16 |  |
| 108 | Support of WUS (wake up signal) for FDD | 36.306 4.3.4.113 | Rel-15 |  |
| 109 | Support DL channel quality reporting for a non-anchor carrier for FDD in Msg3 | 36.306 6.17.8 | Rel-16 |  |
| 110 | Support NPRACH on non-anchor carrier | 36.306 4.3.4.75 | Rel-14 |  |
| 111 | Support DL channel quality reporting for an anchor carrier for FDD in Msg3 | 36.306 6.17.2 | Rel-16 |  |
| 112 | Support Relaxed RRM measurements | 36.306 6.17.11 | Rel-16 |  |
| Note 1: Need for inter-frequency gaps or inter-RAT gaps indicates that the UE does not support corresponding measurement without gaps. |  |  |  |  |

Table A.4.5-2: Additional UE radio access capabilities (Mandatory for Rel-11 and onward)

| Item | Additional capabilities | Ref. | Release | Status (Note 1) | Support (Note 2) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | UE supports CRS interference handling | 36.306, 4.3.4.15 | Rel-11 | O.01 |  | This is a Rel-11 Mandatory feature |
| 2 | UE supports ss-CCH interference handling | 36.306, 4.3.4.20 | Rel-11 | O.01 |  | This is a Rel-11 Mandatory feature |
| 3 | UE supports multiple timing advances for each band combination supported by the UE | 36.306, 4.3.5.3 | Rel-11 | O.01 |  | This is a Rel-11 Mandatory feature (Note 3) |
| Note 1: From Rel-11 onwards 3GPP TSG RAN has discontinued the usage of FGI bits (see A.4.4). Instead it has introduced a different mechanism to accomplish the same purposes based on the following principles (TS 36.306 [17] clause 4): 'For optional features, the UE radio access capability parameter indicates whether the feature has been implemented and successfully tested. For mandatory features with the UE radio access capability parameter, the parameter indicates whether the feature has been successfully tested.’ Reflecting this situation, in the present table the status for Mandatory features would be indicated as conditional Optional (O.xx) until IOT testing availability is ensured. The decision when IOT testing availability can be considered ensured is made by 3GPP TSG RAN. After the 3GPP TSG RAN decision that IOT testing is available, the status of the capability parameter will be changed to Mandatory (M) and the release from which this requirement apply would be explicitly stated. Note 2: If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release. Note 3: It is mandatory for UEs of this release of the specification to support this capability for band combinations having an UL on multiple FDD bands (see 36.306, 4.3.5.3). In the context of evaluating the status of the capability this would depend on the indication for UL support provided in Table A.4.3.3.3-3 i.e. if for at least one CA configurations for Inter-band CA the UE indicates A-A then the Support of multiple timing advances for this CA configuration is Mandatory. |  |  |  |  |  |  |

Table A.4.5-2a: Additional UE radio access capabilities Conditions

O.01 IF The feature has been IOT-ed THEN Support shall be indicated ELSE Support shall not be indicated

Table A.4.5-2b: Additional UE radio access capabilities for MBMS including LTE based 5G Broadcast

| Item | Additional capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Support of MBMS reception with SCS 15kHz from MBMS-dedicated cells | 36.306, 4.3.17.5 | Rel-14 |  |
| 2 | Support of MBMS SCS 1.25kHz and 7.5kHz on MBMS-dedicated cells | 36.306, 4.3.17.6 | Rel-14 |  |
| 3 | Support of MBMS SCS 0.37kHz and 2.5kHz on MBMS-dedicated cells | 36.306, 4.3.17.6a | Rel-16 |  |
| 4 | Support of indicating the maximum supported bandwidth for MBMS reception | 36.306, 4.3.17.7 | Rel-14 |  |
| 5 | Support for different MBMS scaling factor for SCS 1.25 kHz and 7.5 kHz | 36.306, 4.3.17.8 | Rel-14 |  |
| 6 | Support for different MBMS scaling factor for SCS 0.37 kHz and 2.5 kHz | 36.306, 4.3.17.9 | Rel-16 |  |
| 7 | Support for time staggering length of 2 or 4 slots for MBSN-RS associated with PMCH of SCS 0.37 kHz | 36.306, 4.3.17.10 | Rel-16 |  |
| 8 | Support for MBMS reception for PMCH Bandwidth 40/35/30 PRBs | 36.306, 4.3.17.11 | Rel-17 | Corresponds to PMCH Bandwidth of 8/7/6 MHz |

Table A.4.5-2c: Additional optional features without UE radio access capability parameters – MBMS features

| Item | Additional capabilities | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | Support of PBCH repetition in CAS | 36.306, 6.3.3 | Rel-14 |  |
| 2 | Support of PDCCH AL 16 for CAS in MBMS-dedicated cell | 36.306, 6.3.4 | Rel-14 |  |

Table A.4.5-3: UL MIMO Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | Frequency band: 1920-1980, 2110-2170 MHz | 36.101, 5.5 | FDD Band 1 |
| 2 | Frequency band: 1850-1910, 1930-1990 MHz | 36.101, 5.5 | FDD Band 2 |
| 3 | Frequency band: 1710-1785, 1805-1880 MHz | 36.101, 5.5 | FDD Band 3 |
| 4 | Frequency band: 1710-1755, 2110-2155 MHz | 36.101, 5.5 | FDD Band 4 |
| 5 | Frequency band: 824-849, 869-894 MHz | 36.101, 5.5 | FDD Band 5 |
| 6 | Frequency band: 830-840, 875-885 MHz | 36.101, 5.5 | FDD Band 6 |
| 7 | Frequency band: 2500-2570, 2620-2690 MHz | 36.101, 5.5 | FDD Band 7 |
| 8 | Frequency band: 880-915, 925-960 MHz | 36.101, 5.5 | FDD Band 8 |
| 9 | Frequency band: 1749.9-1784.9, 1844.9-1879.9 MHz | 36.101, 5.5 | FDD Band 9 |
| 10 | Frequency band: 1710-1770, 2110-2170 MHz | 36.101, 5.5 | FDD Band 10 |
| 11 | Frequency band: 1427.9-1447.9, 1475.9-1495.9 MHz | 36.101, 5.5 | FDD Band 11 |
| 12 | Frequency band: 699-716, 729-746 MHz | 36.101, 5.5 | FDD Band 12 |
| 13 | Frequency band: 777-787, 746-756 MHz | 36.101, 5.5 | FDD Band 13 |
| 14 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | FDD Band 14 |
| 15 | Reserved | 36.101, 5.5 | FDD Band 15 |
| 16 | Reserved | 36.101, 5.5 | FDD Band 16 |
| 17 | Frequency band: 704-716, 734-746 MHz | 36.101, 5.5 | FDD Band 17 |
| 18 | Frequency band: 815-830, 860-875 MHz | 36.101, 5.5 | FDD Band 18 |
| 19 | Frequency band: 830-845, 875-890 MHz | 36.101, 5.5 | FDD Band 19 |
| 20 | Frequency band: 832-862, 791-821MHz | 36.101, 5.5 | FDD Band 20 |
| 21 | Frequency band: 1447.9-1462.9, 1495.9-1510.9 MHz | 36.101, 5.5 | FDD and HD-FDD Band 21 |
| 22 | Frequency band: 3410-3490, 3510-3590 MHz | 36.101, 5.5 | FDD Band 22 |
| 23 | Frequency band: 2000-2020, 2180-2200 MHz | 36.101, 5.5 | FDD Band 23 |
| 24 | Frequency band: 1626.5-1660.5, 1525-1559 MHz | 36.101, 5.5 | FDD Band 24 |
| 25 | Frequency band: 1850-1915, 1930-1995 MHz | 36.101, 5.5 | FDD Band 25 |
| 26 | Frequency band: 814-849, 859-894 MHz | 36.101, 5.5 | FDD Band 26 |
| 27 | Frequency band: 807-824, 852-869 MHz | 36.101, 5.5 | FDD Band 27 |
| 28 | Frequency band: 703-748, 758-803 MHz | 36.101, 5.5 | FDD Band 28 |
| 29 | Frequency band: N/A, 717-728 MHz | 36.101, 5.5 | FDD Band 29 |
| 30 | Frequency band: 2305-2315, 2350-2360 MHz (Note 1) | 36.101, 5.5 | FDD Band 30 |
| 31 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | FDD Band 31 |
| ... |  |  |  |
| 33 | Frequency band: 1900-1920, 1900-1920 MHz | 36.101, 5.5 | TDD Band 33 |
| 34 | Frequency band: 2010-2025, 2010-2025 MHz | 36.101, 5.5 | TDD Band 34 |
| 35 | Frequency band: 1850-1910, 1850-1910 MHz | 36.101, 5.5 | TDD Band 35 |
| 36 | Frequency band: 1930-1990, 1930-1990 MHz | 36.101, 5.5 | TDD Band 36 |
| 37 | Frequency band: 1910-1930, 1910-1930 MHz | 36.101, 5.5 | TDD Band 37 |
| 38 | Frequency band: 2570-2620, 2570-2620 MHz | 36.101, 5.5 | TDD Band 38 |
| 39 | Frequency band: 1880-1920, 1880-1920 MHz | 36.101, 5.5 | TDD Band 39 |
| 40 | Frequency band: 2300-2400, 2300-2400 MHz | 36.101, 5.5 | TDD Band 40 |
| 41 | Frequency band: 2496-2690, 2496-2690 MHz | 36.101, 5.5 | TDD Band 41 |
| 42 | Frequency band: 3400-3600, 3400-3600 MHz | 36.101, 5.5 | TDD Band 42 |
| 43 | Frequency band: 3600-3800, 3600-3800 MHz | 36.101, 5.5 | TDD Band 43 |
| 44 | Frequency band: 703-803, 703-803 MHz | 36.101, 5.5 | TDD Band 44 |
| 45 | Frequency band: 1447-1467, 1447-1467 MHz | 36.101, 5.5 | TDD Band 45 |
| ... |  |  |  |
| 48 | Frequency band: 3550-3700, 3550-3700 MHz | 36.101, 5.5 | TDD Band 48 |
| … |  |  |  |
| 53 | Frequency band: 2483.5-2495, 2483.5-2495 MHz | 36.101, 5.5 | TDD Band 53 |
| 54 | Frequency band: 1670-1675, 1670-1675 MHz | 36.101, 5.5 | TDD Band 54 |
| … |  |  |  |
| 65 | Frequency band: 1920-2010, 2110-2200 MHz | 36.101, 5.5 | FDD Band 65 |
| 66 | Frequency band: 1710-1780, 2110-2200 MHz | 36.101, 5.5 | FDD Band 66 |
| ... |  |  |  |
| 68 | Frequency band: 698-728, 753-783 MHz | 36.101, 5.5 | FDD Band 68 |
| … |  |  |  |
| 70 | Frequency band: 1695-1710, 1995-2020 MHz | 36.101, 5.5 | FDD Band 70 |
| … |  |  |  |
| 72 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | FDD Band 72 |
| 73 | Frequency band: 450-455, 460-465 MHz | 36.101, 5.5 | FDD Band 73 |
| 74 | Frequency band: 1427-1470, 1475-1515 MHz | 36.101, 5.5 | FDD Band 74 |
| Note 1: The uplink transmission is not allowed at this band for the UE with the externally vehicle-mounted antennas. |  |  |  |

Table A.4.5-4: nonContiguousUL-RA-WithinCC-Info-r10 Capabilities
(required for MultiClusterPUSCH-WithinCC-r10)

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | Frequency band: 1920-1980, 2110-2170 MHz | 36.101, 5.5 | FDD Band 1 |
| 2 | Frequency band: 1850-1910, 1930-1990 MHz | 36.101, 5.5 | FDD Band 2 |
| 3 | Frequency band: 1710-1785, 1805-1880 MHz | 36.101, 5.5 | FDD Band 3 |
| 4 | Frequency band: 1710-1755, 2110-2155 MHz | 36.101, 5.5 | FDD Band 4 |
| 5 | Frequency band: 824-849, 869-894 MHz | 36.101, 5.5 | FDD Band 5 |
| 6 | Frequency band: 830-840, 875-885 MHz | 36.101, 5.5 | FDD Band 6 |
| 7 | Frequency band: 2500-2570, 2620-2690 MHz | 36.101, 5.5 | FDD Band 7 |
| 8 | Frequency band: 880-915, 925-960 MHz | 36.101, 5.5 | FDD Band 8 |
| 9 | Frequency band: 1749.9-1784.9, 1844.9-1879.9 MHz | 36.101, 5.5 | FDD Band 9 |
| 10 | Frequency band: 1710-1770, 2110-2170 MHz | 36.101, 5.5 | FDD Band 10 |
| 11 | Frequency band: 1427.9-1447.9, 1475.9-1495.9 MHz | 36.101, 5.5 | FDD Band 11 |
| 12 | Frequency band: 699-716, 729-746 MHz | 36.101, 5.5 | FDD Band 12 |
| 13 | Frequency band: 777-787, 746-756 MHz | 36.101, 5.5 | FDD Band 13 |
| 14 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | FDD Band 14 |
| 15 | Reserved | 36.101, 5.5 | FDD Band 15 |
| 16 | Reserved | 36.101, 5.5 | FDD Band 16 |
| 17 | Frequency band: 704-716, 734-746 MHz | 36.101, 5.5 | FDD Band 17 |
| 18 | Frequency band: 815-830, 860-875 MHz | 36.101, 5.5 | FDD Band 18 |
| 19 | Frequency band: 830-845, 875-890 MHz | 36.101, 5.5 | FDD Band 19 |
| 20 | Frequency band: 832-862, 791-821MHz | 36.101, 5.5 | FDD Band 20 |
| 21 | Frequency band: 1447.9-1462.9, 1495.9-1510.9 MHz | 36.101, 5.5 | FDD Band 21 |
| 22 | Frequency band: 3410-3490, 3510-3590 MHz | 36.101, 5.5 | FDD Band 22 |
| 23 | Frequency band: 2000-2020, 2180-2200 MHz | 36.101, 5.5 | FDD Band 23 |
| 24 | Frequency band: 1626.5-1660.5, 1525-1559 MHz | 36.101, 5.5 | FDD Band 24 |
| 25 | Frequency band: 1850-1915, 1930-1995 MHz | 36.101, 5.5 | FDD Band 25 |
| 26 | Frequency band: 814-849, 859-894 MHz | 36.101, 5.5 | FDD Band 26 |
| 27 | Frequency band: 807-824, 852-869 MHz | 36.101, 5.5 | FDD Band 27 |
| 28 | Frequency band: 703-748, 758-803 MHz | 36.101, 5.5 | FDD Band 28 |
| 29 | Frequency band: N/A, 717-728 MHz | 36.101, 5.5 | FDD Band 29 |
| 30 | Frequency band: 2305-2315, 2350-2360 MHz (Note 1) | 36.101, 5.5 | FDD Band 30 |
| 31 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | FDD Band 31 |
| ... |  |  |  |
| 33 | Frequency band: 1900-1920, 1900-1920 MHz | 36.101, 5.5 | TDD Band 33 |
| 34 | Frequency band: 2010-2025, 2010-2025 MHz | 36.101, 5.5 | TDD Band 34 |
| 35 | Frequency band: 1850-1910, 1850-1910 MHz | 36.101, 5.5 | TDD Band 35 |
| 36 | Frequency band: 1930-1990, 1930-1990 MHz | 36.101, 5.5 | TDD Band 36 |
| 37 | Frequency band: 1910-1930, 1910-1930 MHz | 36.101, 5.5 | TDD Band 37 |
| 38 | Frequency band: 2570-2620, 2570-2620 MHz | 36.101, 5.5 | TDD Band 38 |
| 39 | Frequency band: 1880-1920, 1880-1920 MHz | 36.101, 5.5 | TDD Band 39 |
| 40 | Frequency band: 2300-2400, 2300-2400 MHz | 36.101, 5.5 | TDD Band 40 |
| 41 | Frequency band: 2496-2690, 2496-2690 MHz | 36.101, 5.5 | TDD Band 41 |
| 42 | Frequency band: 3400-3600, 3400-3600 MHz | 36.101, 5.5 | TDD Band 42 |
| 43 | Frequency band: 3600-3800, 3600-3800 MHz | 36.101, 5.5 | TDD Band 43 |
| 44 | Frequency band: 703-803, 703-803 MHz | 36.101, 5.5 | TDD Band 44 |
| 45 | Frequency band: 1447-1467, 1447-1467 MHz | 36.101, 5.5 | TDD Band 45 |
| ... |  |  |  |
| 53 | Frequency band: 2483.5-2495, 2483.5-2495 MHz | 36.101, 5.5 | TDD Band 53 |
| 54 | Frequency band: 1670-1675, 1670-1675 MHz | 36.101, 5.5 | TDD Band 54 |
| … |  |  |  |
| 65 | Frequency band: 1920-2010, 2110-2200 MHz | 36.101, 5.5 | FDD Band 65 |
| 66 | Frequency band: 1710-1780, 2110-2200 MHz | 36.101, 5.5 | FDD Band 66 |
| ... |  |  |  |
| 68 | Frequency band: 698-728, 753-783 MHz | 36.101, 5.5 | FDD Band 68 |
| … |  |  |  |
| 70 | Frequency band: 1695-1710, 1995-2020 MHz | 36.101, 5.5 | FDD Band 70 |
| … |  |  |  |
| 72 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | FDD Band 72 |
| 73 | Frequency band: 450-455, 460-465 MHz | 36.101, 5.5 | FDD Band 73 |
| 74 | Frequency band: 1427-1470, 1475-1518 MHz | 36.101, 5.5 | FDD Band 74 |
| Note 1: The uplink transmission is not allowed at this band for the UE with the externally vehicle-mounted antennas. |  |  |  |

Table A.4.5-5: 4 Rx antenna ports Capabilities

| Item | Ref. | Release | Band | Supported | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | 36.101, 7.2 | Rel-13 | FDD Band 1 |  |  |
| 2 | 36.101, 7.2 | Rel-13 | FDD Band 2 |  |  |
| 3 | 36.101, 7.2 | Rel-13 | FDD Band 3 |  |  |
| 4 | 36.101, 7.2 | Rel-15 | FDD Band 4 |  |  |
| ... |  |  |  |  |  |
| 7 | 36.101, 7.2 | Rel-13 | FDD Band 7 |  |  |
| ... |  |  |  |  |  |
| 20 | 36.101, 7.2 | Rel-13 | FDD Band 20 |  |  |
| 21 | 36.101, 7.2 | Rel-14 | FDD Band 21 |  |  |
| ... |  |  |  |  |  |
| 25 | 36.101, 7.2 | Rel-14 | FDD Band 25 |  |  |
| ... |  |  |  |  |  |
| 30 | 36.101, 7.2 | Rel-15 | FDD Band 30 |  |  |
| ... |  |  |  |  |  |
| 34 | 36.101, 7.2 | Rel-15 | FDD Band 34 |  |  |
| ... |  |  |  |  |  |
| 39 | 36.101, 7.2 | Rel-13 | TDD Band 39 |  |  |
| 40 | 36.101, 7.2 | Rel-14 | TDD Band 40 |  |  |
| 41 | 36.101, 7.2 | Rel-13 | TDD Band 41 |  |  |
| 42 | 36.101, 7.2 | Rel-13 | TDD Band 42 |  |  |
| 66 | 36.101, 7.2 | Rel-15 | FDD Band 66 |  |  |

Table A.4.5-6: Void

Table A.4.5-6a: E-UTRA ProSe Communication Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | Frequency band: 1710-1785, 1805-1880 MHz | 36.101, 5.5 | FDD Band 3 |
| 2 | Frequency band: 2500-2570, 2620-2690 MHz | 36.101, 5.5 | FDD Band 7 |
| 3 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | FDD Band 14 |
| 4 | Frequency band: 832-862, 791-821MHz | 36.101, 5.5 | FDD Band 20 |
| 5 | Frequency band: 814-849, 859-894 MHz | 36.101, 5.5 | FDD Band 26 |
| 6 | Frequency band: 703-748, 758-803 MHz | 36.101, 5.5 | FDD Band 28 |
| 7 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | FDD Band 31 |
| 8 | Frequency band: 698-728, 753-783 MHz | 36.101, 5.5 | FDD Band 68 |
| 9 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | FDD Band 72 |
| 10 | Frequency band: 450-455, 460-465 MHz | 36.101, 5.5 | FDD Band 73 |

Table A.4.5-6b: E-UTRA ProSe Discovery Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | Frequency band: 1850-1910, 1930-1990 MHz | 36.101, 5.5 | FDD Band 2 |
| 2 | Frequency band: 1710-1785, 1805-1880 MHz | 36.101, 5.5 | FDD Band 3 |
| 3 | Frequency band: 1710-1755, 2110-2155 MHz | 36.101, 5.5 | FDD Band 4 |
| 4 | Frequency band: 2500-2570, 2620-2690 MHz | 36.101, 5.5 | FDD Band 7 |
| 5 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | FDD Band 14 |
| 6 | Frequency band: 832-862, 791-821MHz | 36.101, 5.5 | FDD Band 20 |
| 7 | Frequency band: 814-849, 859-894 MHz | 36.101, 5.5 | FDD Band 26 |
| 8 | Frequency band: 703-748, 758-803 MHz | 36.101, 5.5 | FDD Band 28 |
| 9 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | FDD Band 31 |
| 10 | Frequency band: 2496-2690, 2496-2690 MHz | 36.101, 5.5 | TDD Band 41 |
| 11 | Frequency band: 698-728, 753-783 MHz | 36.101, 5.5 | FDD Band 68 |
| 12 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | FDD Band 72 |
| 13 | Frequency band: 450-455, 460-465 MHz | 36.101, 5.5 | FDD Band 73 |

Table A.4.5-7: E-UTRA V2X Sidelink Communication

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | Frequency band: 5855-5925, 5855-5925 MHz | 36.101, 5.5 | TDD Band 47 |

Table A.4.5-7a: Supported Inter-band con-current V2X configurations

| Inter-band con-current V2X configurations | Release | Comments |
| --- | --- | --- |
| V2X_3A-47A | Rel-14 | - |
| V2X_5A-47A | Rel-14 | - |
| V2X_7A-47A | Rel-14 | - |
| V2X_8A-47A | Rel-14 | - |
| V2X_20A-47A | Rel-14 | - |
| V2X_34A-47A | Rel-14 | - |
| V2X_39A-47A | Rel-14 | - |
| V2X_41A-47A | Rel-14 | - |

Table A.4.5-7b: Supported V2X intra-band multi-carrier configurations

| V2X intra-band multi-carrier configurations | Release | Comments |
| --- | --- | --- |
| V2X_47B | Rel-14 | - |

Table A.4.5-8: Supported CA configurations with multi layer spatial multiplexing

| Item | Configuration | Release | Ref. | Comments |
| --- | --- | --- | --- | --- |
| 1 | 2DL FDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-10 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 2 | 2DL TDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-10 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 3 | 2DL FDD-TDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-12 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 4 | 3DL FDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-10 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 5 | 3DL TDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-10 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 6 | 3DL FDD-TDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-12 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 7 | 4DL FDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-11 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 8 | 4DL TDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-11 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 9 | 4DL FDD-TDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-12 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 10 | 5DL FDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-11 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 11 | 5DL TDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-11 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| 12 | 5DL FDD-TDD CA, with 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4 | Rel-12 | 36.101, 5.6A 36.306, 4.3.5.14 | - |
| NOTE: At least one component carrier in a CA configuration shall support 4Rx antenna ports and 4-layer spatial multiplexing with TM 3 and TM 4. |  |  |  |  |