3GPP TSG RAN Meeting #107										RP-250756

Incheon, South Korea, March. 12-14, 2025

Status Report to TSG

Agenda item:			9.2.4

| WI / SI Name | Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_AIML_Mob |  |  |  |  |
| Unique ID | 1020084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242393 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 09/2025 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 63 % | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Zhongda Du |
|  | Company | OPPO |
|  | Email | duzhongda@oppo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#129 agreements:

Text proposal in R2-2500287 is endorsed

- Agreements on priority

RLF prediction will not be studied in Rel-19

Further RRM prediction simulations are not expected.  For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction.   FFS if L3 beam level needs to be considered.  For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

Continue with generalization across cell configurations for RRM prediction

Continue with measurement events

Agreements

- For Case B temporal domain prediction, RAN2 to capture that AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold) and the gain increases as MRRT increases.

- Regarding temporal domain prediction case B, the gain that AI can provide is more obvious in high speed scenario when compared to sample and hold.

- Regarding the temporal domain prediction case A, the gain that AI can provide increases with the increase of prediction window length (up to a certain window length) when compared to sample and hold.

We will capture something about model complexity.  FFS how and what to capture it

Average L3 cell RSRP difference and last predicted point L3 cell RSRP difference of measurement results within PW is captured in TR

For both temporal domain case A and case B, simulation result with different filtering approach is listed separately.

Take the optimal result, e.g., the one with the lowest RSRP difference, from each company.

Agreements on generalization over UE speed

- For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across all UE speeds

- Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size).

- For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline.

Agreements on generalization over frequency

Generalization using GC#2 always outperform that of GC#1

Training and generalization using the knowledge about the input & output frequency (or even an indication) outperforms the case where the model cannot recognize which frequency at the input or output.

Considering the generalization parameter of different predicted frequencies in inter-freq prediction, the GC#1 case without any preprocessing based on the information of predicted frequency suffers from significant performance loss.

For GC#2 case, the prediction accuracy is acceptable and close to the baseline

Agreements

1: two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study.

2: FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2).

3: agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2).

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 2: generalization parameters for FR1

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 3: generalization parameters for FR2

Agreements on Measurement event case A

- Most of the results show that the F1 score for prediction of measurement event is very good.

- For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate.

Agreements on Measurement event case B

- Measurement event case B can have very good F1 score depending on filtering approach/PW

- F1 score is good for the PW window for case B (i.e. low PW)

- F1 score decreases with an increasing MRRT value

- We will focus on finding cases that bring benefits to the system rather than trying to compare

- With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

Other agreements

Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required.

Agreements on direct measurement event prediction

-	F1 score for direct measurement is very good based on the current simulation results.

2.2.2	Remaining Open issues

For RRM measurement use case:

1, To collect simulation results based on updated template and conclude further statistics observation based on collected simulation result

2, Generalization study on cell configuration

For Measurement event use case:

1, Evaluation based on Simulation result by clarifying the F1 score methodology for temporal domain case A

Issues covered by following objectives in the SID:

- Potential specification impacts of AI/ML aided mobility [RAN2]

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#114 meeting agreements:

Issue 1-1-1: Measurement event prediction use case

Agreement:

- RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

Issue 2-1-1a: Relative RSRP Prediction Accuracy

Agreement:

- Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2)

  - FFS: Beam 1 and Beam 2 can be from same cell or different cells.

  - FFS: whether ground truth can be reported or ideal

  - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy

Agreement:

- The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy

Agreement:

Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered.

- Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy

- Agreement:

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

  - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as:

    - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction

    - Impact of L3 filtering

    - Number of samples for L1 filtering

    - Timing window, if it exists, where UE can’t measure the ground truth in Alt2

    - channel condition

    - Avoid UE cheat in the test

    - S(I)NR variations in the test at different time instances (for Alt2)

    - Other aspects are not precluded

  - Companies are encouraged to provide analysis on all or some of the aspects in the list

Issue 2-1-4: Impacts of measurement error on prediction accuracy

Agreement:

For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

Issue 2-1-9: Impacts from observation and prediction windows

Agreement:

- RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

2.4.2	Remaining Open issues

For General Aspects: Assessments of sub-use-case priorities.

For RAN4 requirements: Analyses on performance metrics and factors that potentially impact core/performance requirements.

For Testability aspects: Evaluations on the testing setup, further study new testability aspects, e.g., FR1/FR2, consistency in time domain, and the influence of inter-carrier scenarios.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

Contributions submitted to RAN2#129 meeting:

R2-2500161 	Simulation results on RRM measurement prediction	Spreadtrum, UNISOC, BUPT

R2-2500167 	Discussion on RRM measurement predictions and prediction-based mobility events	Sharp

R2-2500214	 Simulation results of RRM measurement prediction	vivo

R2-2500215	 Simulation results of model generalization for RRM measurement prediction	vivo

R2-2500216	 Simulation results of measurement event prediction and SLS	vivo

R2-2500243	 Simulation results of Model Generalization	CATT, Turkcell

R2-2500244	 Simulation results of event A3 prediction	CATT, Turkcell

R2-2500245	 Simulation results of AI based Handover performance	CATT, Turkcell

R2-2500287	 Text proposal of 38.744	OPPO

R2-2500288	 Discussion on work plan of AI mobility SI	OPPO,MediaTek,Nokia,Interdigital

R2-2500289	 Discussion on model generalization of RRM measurement prediction	OPPO

R2-2500290	 Analysis on simulation results of RRM measurement prediction	OPPO

R2-2500291	 Discussion on simulation results of measurement event prediction	OPPO

R2-2500299	 Simulation assumption for Measurement event prediction	NEC

R2-2500313	 Discussions on simulation results of the RRM measurement prediction	NTT DOCOMO, INC.

R2-2500314 	Discussions on generalization of RRM measurement prediction	NTT DOCOMO, INC.

R2-2500315 	Discussions on measurement event prediction	NTT DOCOMO, INC.

R2-2500318	 Model Generalization for AIML RRM Prediction	MediaTek Inc.

R2-2500319 	Preliminary results of indirect prediction and simulation assumption for direct prediction	MediaTek Inc.

R2-2500324	 Simulation result for RRM prediction and system level simulation	MediaTek Inc.

R2-2500327	 AI-ML based RLF/HO failure prediction	Rakuten Mobile, Inc

R2-2500399	 Simulation results for measurement event prediction	Xiaomi

R2-2500406	 Simulation results for RRM measurement predictions	Interdigital Inc.

R2-2500407	 Generalization of AIML models for RRM measurement prediction	Interdigital Inc.

R2-2500408	 Measurement event predictions	Interdigital Inc.

R2-2500520	 Simulation results for RRM measurement prediction	China Telecom Corporation Ltd.

R2-2500542	 Discussion on Generalization Issues for AI/ML Mobility	Samsung

R2-2500559	 Email discussion summary for [Post128][018][AI Mob] generalization  (Apple)	Apple

R2-2500560	 On measurement event prediction	Apple

R2-2500601	 Simulation results on Model Generalization for RRM measurement prediction	Spreadtrum, UNISOC, BUPT

R2-2500639	 Discussion on simulation results for RRM measurement prediction 	Samsung

R2-2500640	 Simulation results for measurement event prediction and system level performance	Samsung

R2-2500734	 Discussion on simulation metric for RLF prediction	KDDI Corporation

R2-2500800	 Discussion on generalization performance over UE speed of GC1	Xiaomi

R2-2500801	 Discussion on generalization performance over UE speed of GC2	Xiaomi

R2-2500851	 Generalization of the AI/ML models for RRM prediction	Ericsson

R2-2500852	 Simulation results for spatial domain RRM measurement predictions	Ericsson

R2-2500890	 Additional simulation results for RRM measurement prediction	Huawei, HiSilicon

R2-2500891 	Model generalization evaluation for RRM measurement prediction	Huawei, HiSilicon

R2-2500892	 Simulation results for measurement event prediction	Huawei, HiSilicon

R2-2500899	 Discussion on RRM measurement prediction	ZTE Corporation

R2-2500900 	Discussion on model generalization  of RRM measurement prediction	ZTE Corporation

R2-2500919	 Evaluation of measurement event prediction	Ericsson

R2-2500946	  Simulation results for RRM measurement predictions	Nokia

R2-2500947	 On the measurement event prediction	Nokia

R2-2500948	 Generalization of ML mobility use-cases	Nokia, Nokia Shanghai Bell

R2-2501040	 Potential specification impact for RRM measurement prediction	CMCC

R2-2501041	 Simulation results for Measurement event predictions	CMCC

R2-2501052	 Discussion on  model generalization	CMCC

R2-2501070	 Measurement Event prediction and handover modelling	Qualcomm Incorporated

R2-2501071	 Simulation Results for Model Generalization	Qualcomm Incorporated

R2-2501072	 Simulation Results for Measurement Event Predictions	Qualcomm Incorporated

R2-2501241	 Discussion on system level simulation	ETRI

R2-2501293	 Discussion on measurement event prediction	ZTE Corporation

Contributions submitted to RAN4#114 meeting:

R4-2500231 On Testability and Interoperability Issues for AIML Mobility in NR	Apple

R4-2500232 General aspects on AI/ML for mobility in NR	Apple

R4-2500233 Study of impacts on RAN4 requirements for AIML Mobility in NR	Apple

R4-2500533 Topic summary for [114][219] FS_NR_AIML_Mob	Moderator (Nokia)

R4-2500594 Discussions on RAN4 requirement impacts of AIML for mobility	NTT DOCOMO, INC.

R4-2500619 Discussion on impacts on RAN4 requirement for AI mobility	Xiaomi

R4-2500628 Discussion on study of testability and interoperability for AIML mobility	Xiaomi

R4-2500696 Discussion impacts on requirements of AI/ML mobility	ZTE Corporation, Sanechips

R4-2500697 Discussion on the Interoperability and testability aspects	ZTE Corporation, Sanechips

R4-2500890 Discussion on impact on RRM requirements of AI mobility	MediaTek Inc.

R4-2500891 Discussion on testability and interoperability of AI mobility	MediaTek Inc.

R4-2501040 Discussion on impacts of AIML mobility on RRM requirements	CATT

R4-2501041 Discussion on testability and interoperability issues for AIML mobility	CATT

R4-2501099 Discussion on general part for AI/ML for mobility	CMCC

R4-2501100 Discussion on impacts on RAN4 requirements for AI/ML for mobility	CMCC

R4-2501101 Discussion on testability and interoperability for AI/ML for mobility	CMCC

R4-2501133 Study of impacts on RAN4 requirements for AI mobility	OPPO

R4-2501134 Study of testability and interoperability for AI mobility	OPPO

R4-2501233 Discussion on genereal aspects in AIML mobility	Huawei, HiSilicon

R4-2501234 Discussion on impacts of RAN4 requirements in AIML mobility	Huawei, HiSilicon

R4-2501235 Discussion on testability and interoperability issues in AIML mobility	Huawei, HiSilicon

R4-2501339 Discussion on general aspects for AI mobility	vivo

R4-2501340 Discussion on impacts on RAN4 requirements for AI mobility	vivo

R4-2501341 Discussion on testability and interoperability for AI mobility	vivo

R4-2501674 Discussion on general aspects and requirements impact	Samsung

R4-2501675 General discussion on AI mobility regarding testability and interoperability	Samsung

R4-2501694 Study of impacts on RAN4 requirements for AIML Mobility in NR	Tejas Network Limited

R4-2501759 General discussion on AI/ML for mobility	Ericsson

R4-2501760 On requirements for AI/ML based mobility	Ericsson

R4-2501761 On testability issues related to AI/ML for mobility	Ericsson

R4-2501961 General aspects of AIML Mobility	Nokia

R4-2501962 On RRM Requirement Impacts of AIML Mobility	Nokia

R4-2502072 Testability and Interoperability Issues for AI/ML Mobility	Nokia

R4-2502219 (FS_NR_AIML_Mob) Impact of AI based mobility on RRM requirements	Qualcomm Incorporated

R4-2502220 (FS_NR_AIML_Mob) Testability and data consistency	Qualcomm Incorporated

R4-2502574 Ad-hoc minutes for FS_NR_AIML_Mob SI	Nokia

R4-2502594 WF on FS_NR_AIML_Mob	Nokia