3GPP TSG RAN Meeting #107										RP-250782

Incheon, Korea, March 12-14, 2025

Agenda Item:	16

Source:	Moderator (CMCC)

Title:	Moderator's summary for IMT-2030 TPRs

Document for:	discussion & approval

- Background

In RAN #106, 3GPP TSG RAN has made initial considerations of possible TPRs as following, the definitions of these items will be further discussed.

1.	Peak data rate

2.	Peak spectral efficiency

3.	User experienced data rate

4.	5th percentile user spectral efficiency

5.	Average spectral efficiency

6.	Sustainability/Energy efficiency

7.	Area traffic capacity

8.	User plane latency

9.	Control plane latency

10.	Connection density

11.	Reliability

12.	Mobility

13.	Mobility interruption time

14.	Bandwidth

According to the workplan, we should aim to finalize the candidate set of TPRs and have initial discussion on the associated values of the identified TPRs.

Following is the list of TPR produced at WP5D February

| MTPR | Method |
| --- | --- |
| AI-related capabilities | (TBD) |
| Area traffic capacity | (Quantitative) |
| Average Spectral efficiency | (Quantitative) |
| Peak spectral efficiency | (Quantitative) |
| 5th percentile user Spectral efficiency | (Quantitative) |
| Bandwidth | (Quantitative) |
| Connection density | (Quantitative) |
| Control plane latency | (Quantitative) |
| User plane latency | (Quantitative) |
| Energy Efficiency | (Quantitative) |
| Mobility | (Quantitative) |
| Mobility interruption time | (Quantitative) |
| Peak data rate | (Quantitative) |
| User Experienced Data Rate | (Quantitative) |
| Sensing-related capabilities | (Quantitative) |
| Positioning | (Quantitative) |
| Reliability | (Quantitative) |
| [Joint/composite requirement] |  |
| [Coverage] |  |
| [Security] |  |
| [Resilience] |  |
| [Sustainability] |  |
| [Interoperability] |  |

- Items agreed in ITU

| MTPR | Method | Comment |
| --- | --- | --- |
| AI-related capabilities | (TBD) | Not included in RAN#106 LS |
| Area traffic capacity | (Quantitative) |  |
| Average Spectral efficiency | (Quantitative) |  |
| Peak spectral efficiency | (Quantitative) |  |
| 5th percentile user Spectral efficiency | (Quantitative) |  |
| Bandwidth | (Quantitative) |  |
| Connection density | (Quantitative) |  |
| Control plane latency | (Quantitative) |  |
| User plane latency | (Quantitative) |  |
| Energy Efficiency | (Quantitative) |  |
| Mobility | (Quantitative) |  |
| Mobility interruption time | (Quantitative) |  |
| Peak data rate | (Quantitative) |  |
| User Experienced Data Rate | (Quantitative) |  |
| Sensing-related capabilities | (Quantitative) | not included in RAN#106 LS |
| Positioning | (Quantitative) | not included in RAN#106 LS |
| Reliability | (Quantitative) |  |

  - Positioning

16 contributions support defining positioning as a quantitative TPR, about 7 contributions support defining both vertical and horizontal accuracy, about 6 contributions support to define positioning accuracy for both indoor and outdoor scenarios

Moderator Proposal 1-1: Define positioning as a quantitative TPR evaluated by simulation

- Define requirements for vertical positioning accuracy and horizontal positioning accuracy

- Define requirements for both indoor and outdoor scenarios

  - Sensing-related capabilities

Over 10 contributions support defining sensing-related capabilities as a quantitative TPR. Almost all contributions support Detectability, localization accuracy, about 6 contributions support sensing resolution and velocity accuracy. There are also other requirements proposed by contributions.

Moderator Proposal 1-2: Define Sensing-related capabilities as a quantitative TPR evaluated by simulation and analysis, candidate requirements are:

- Detectability (Detection / False Alarm Probability):

- Localization accuracy

- Sensing Resolution

- Velocity accuracy

- [Sensing capacity]

- [Correct classification probability]

- [RMSE]

- [Motion rate accuracy]

- [Displacement accuracy]

  - AI-related capabilities

Regarding network for AI, 10 contributions support defining AI-related capabilities as qualitative TPR evaluated by inspection, while 3 contributions support defining quantitative TPRs, e.g. AI service accuracy, AI service latency, capacity.

Regarding AI for network, 2 contributions support comparison of 6G with and without AI, while 1 contribution does not support defining TPR.

Moderator Proposal 1-3: Network for AI: define AI-related capabilities as qualitative TPR evaluated by inspection.

  - Values of legacy TPRs

| ITU IMT-2030 TPR |  | IMT-2020 Value | Values proposed in this meeting | Moderator proposal |
| --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | TBD |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | [2x] of IMT-2020 |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 2 contributions propose to revisit the evaluation methodology before setting values: - Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. | DL: TBD UL: Revisit the evaluation methodology before setting values: Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | [10] |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] | eMBB: [2] HRLLC: TBD |
|  | Control plane latency (ms) | 20 | [5~20] | [10] |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 | [10x] of IMT-2020 |
| Reliability |  | - 10⁻⁵ | [1~100x] of IMT-2020 | [1-10-5] |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 |
| Mobility interruption time (ms) |  | 0 | 0 ms | 0 ms |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | [400MHz] |

- Items in [ ] in ITU

| MTPR | Method |
| --- | --- |
| [Joint/composite requirement] |  |
| [Coverage] |  |
| [Security] |  |
| [Resilience] |  |
| [Sustainability] |  |
| [Interoperability] |  |

  - Joint/composite requirement

10 contributions support defining joint/composite requirement as a quantitative TPR. Almost all contributions support immersive communication as the applicable usage scenarios, meanwhile 2 contributions support HRLLC usage scenario.

Moderator Proposal 2-1: Define joint/composite requirement on data rate, latency, reliability and capacity as a candidate IMT-2030 TPR.

- Usage scenario: immersive communication, [HRLLC]

  - Energy efficiency

14 contributions support defining energy efficiency as a quantitative TPR. Meanwhile some details (e.g. usage scenario, evaluation method and etc.) still need further discussion.

Moderator Proposal 2-2: Define energy efficiency as a quantitative TPR. The requirement is the relative power saving gain between the selected load case(s) and a reference case, further discuss the following aspects:

- Applicable usage scenario and use cases: immersive communication, [massive communication]

- Evaluation method for network:

  - For unloaded case: analysis without user data traffic

  - For loaded case if agreed: simulation or analysis with user data traffic

- Evaluation method for device:

  - For unloaded case: analysis without user data traffic

  - For loaded case: simulation or analysis with user data traffic

- How to deal with communication performance impact due to energy saving

  - Others

Regarding other candidates including coverage, security, resilience, interoperability and sustainability:

Security: 3 contributions propose not to define security as a TPR, meanwhile 2 contributions mentioned that security can be covered by submission template for IMT-2030.

Resilience: 1 contribution supports defining resilience as TPR, while 3 contributions propose not to define resilience as TPR. Meanwhile 1 contribution mentioned resilience can be covered by submission template for IMT-2030.

Interoperability: 4 contributions propose not to define interoperability as TPR. Meanwhile 1 contribution mentioned resilience can be covered by submission template for IMT-2030.

Sustainability: At least 8 contributions support sustainability to be covered by energy efficiency.

Coverage: 4 contributions support defining coverage as a quantitative TRP, while 5 contributions do not support defining coverage as TRP, instead link budget evaluation (same as IMT-2020) approach should be used.

Proposal 2-3: Do not define security, resilience, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030.

Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability.

Proposal 2-5: For coverage, further discuss the following two options:

- Option 1: define coverage as a quantitative TPR

- Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020)

- Conclusion

To be added after offline session.

# Reference

- RP-250070	View on IMT-2030 Technical Performance Requirements (TPRs)	Samsung

- RP-250082	Technical Performance Requirements (TPR) for IMT-2030	Ericsson

- RP-250083	Discussion of new TPR for IMT-2030	Ericsson

- RP-250084	TR 38.8xx/9xx text proposal for IMT-2030 TPR	Ericsson

- RP-250114	Proposals on Technical Performance Requirements (TPRs) of IMT-2030	SoftBank Corp.

- RP-250145	Target Values for IMT-2030 Technical Performance Requirements	Nokia

- RP-250153	Views on the study of IMT-2030 requirements	Indian Institute of Tech (M), CEWiT, IIT Kanpur, Tejas Networks, Indian Institute of Tech (H), WiSig Networks

- RP-250172	Discussion on 6G Scenarios and Requirements	OPPO

- RP-250182	Views on Candidate Set of TPRs	Spreadtrum, UNISOC

- RP-250189	View on RAN Study on 6G Scenarios and Requirements	SK Telecom

- RP-250210	Discussion on IMT-2030 technical performance requirements (TPR)	vivo

- RP-250263	Discussion on IMT-2030 TPRs	CMCC

- RP-250293	Operator’s view on realistic TPR definition for IMT-2030 / “6G”	Deutsche Telekom, Orange, Telefonica, Telenor, Spark NZ, TeliaCompany

- RP-250298	Views on IMT-2030 TPR	Qualcomm

- RP-250354	Discussion on 6G SI and IMT-2030 TPR	Xiaomi

- RP-250365	Discussions on minimum technical performance requirements for IMT-2030 radio interface(s)	NTT DOCOMO, INC.

- RP-250388	Views on IMT-2030 Technical Performance Requirements (TPRs)	China Telecom

- RP-250431	Further considerations on IMT-2030 TPRs	Huawei, HiSilicon

- RP-250434	On IMT-2030 Technical Performance Requirements	Apple Computer Trading Co. Ltd

- RP-250461	Views on IMT-2030 Performance Requirements for RAN's 6G Study	MITRE Corporation

- RP-250484	IMT2030 TPR considerations	MediaTek Inc.

- RP-250512	Consideration and proposals on IMT-2030 TPRs	CATT

- RP-250567	Views on RAN study for IMT-2030	ZTE Corporation, Sanechips

- RP-250571	Considerations on candidate IMT2030 TPRs for sustainability	China Unicom

# Annex: Summary of proposed TPR values in RAN#107

| ITU IMT-2030 TPR |  | IMT-2020 Value | Comments |
| --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | - OPPO, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN: Same as IMT-2020 - Samsung: [1.5x] IMT-2020 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [1.5-5x] IMT-2020 - Ericsson: [2.5x] IMT-2020 - CATT: [2.5-5x] IMT-2020 - Xiaomi: [3-4x] IMT-2020 - CMCC: [3-5x] IMT-2020 - Vivo: [4x] IMT-2020 - Nokia, ZTE, China Telecom: [5x] IMT-2020 - Huawei: [5-10x] IMT-2020 - Apple: - DL: [2.5x] IMT-2020 - UL: [2x] IMT-2020 - Qualcomm: - DL: [5-10x] IMT-2020 - UL: [5x] IMT-2020 |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | - OPPO: Same as IMT-2020 - Samsung: [1.3x] IMT-2020 - Nokia, Apple: [1.5x] IMT-2020 - Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN: [1.5-2x] IMT-2020 - China Telecom, IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [1.5-3x] IMT-2020 - Ericsson: [1.7x] IMT-2020 - Vivo, Xiaomi: [2x] IMT-2020 - Huawei: [2-3x] IMT-2020 - CATT, ZTE: [2.5x] IMT-2020 - CMCC: [2.5-4x] IMT-2020 - Qualcomm: [3x] IMT-2020 |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | - CMCC: [1.25-2x] IMT-2020 (DL) - OPPO: [1.5-2x] IMT-2020 - CATT, IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [2-3x] IMT-2020 - China Telecom: [3x] IMT-2020 - Xiaomi: [3-4x] IMT-2020 - Samsung: [4x] IMT-2020 - vivo : [4-6] IMT-2020 (DL) - Nokia, ZTE: [5x] IMT-2020 - Huawei: [5-10x] IMT-2020 - Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN: - DL: [0.5-5x] IMT-2020 - UL: [0.04-4x] IMT-2020 - Apple: - DL: [3x] IMT-2020 - UL: [4x] IMT-2020 - Qualcomm: - DL: [10x] IMT-2020 - UL: [4x] IMT-2020 |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | - Apple: [1.5x] IMT-2020 - Xiaomi, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN: [1.5-2x] IMT-2020 - China Telecom: [1.5-3x] IMT-2020 - Samsung, Nokia, OPPO: [2x] IMT-2020 - Ericsson, CATT, CMCC (DL): [2-3x] IMT-2020ZTE: [2.5-3x] IMT2020 - Huawei: [3-5x] IMT-2020 - Vivo: - DL: [2-3x] IMT-2020 - UL: [1.5-2x] IMT-2020 |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | - Apple: [1.5x] IMT-2020 - OPPO, Xiaomi, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN: [1.5-2x] IMT-2020 - China Telecom: [1.5-3x] IMT-2020 - Nokia: [2x] IMT-2020 - Ericsson, CATT, CMCC: [2-3x] IMT-2020 - ZTE: [2.5x] IMT-2020 - Samsung, Qualcomm: [3x] IMT-2020 - Huawei: [3-5x] IMT-2020 - Vivo: - DL: [2-3x] IMT-2020 - UL: [1.5-2x] IMT-2020 |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | - Apple, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN: Same as IMT-2020 - CMCC: [1.25-2x] IMT-2020 - OPPO: [2x] IMT-2020 - CATT, IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [2-3x] IMT-2020 - Ericsson, Qualcomm: [3x] IMT-2020 - Xiaomi: [3-4x] IMT-2020 - Vivo: [4-6x] IMT-2020 - Samsung, ZTE: [5x] IMT-2020 - Huawei, Nokia: [5-10x] IMT-2020 - China Telecom: [10x] IMT-2020 |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | - OPPO, Samsung, Ericsson, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN: Same as IMT-2020 - CMCC: - IC: 2ms - HRLLC: 0.5ms - China Telecom: - HRLLC: 0.1-0.5ms - Qualcomm, Apple, Nokia: - IC: 1ms - HRLLC: 1ms - Xiaomi, ZTE: - IC: 1ms - HRLLC: 0.5ms - Huawei: - IC: 1ms - HRLLC: 0.5-1ms - Vivo: - IC: 1ms - HRLLC: 0.1ms - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 0.5-1ms |
|  | Control plane latency (ms) | 20 | - Ericsson, OPPO: Same as IMT-2020 - Qualcomm, Apple: 15ms - Samsung, Nokia, CMCC, China Telecom, Huawei, CATT: 10ms - Xiaomi: 5-10ms - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 5-20ms - ZTE: 5ms or 10ms - Vivo: - with device state transition: 10ms - without device state transition: 4ms |
| Connection density (devices/km²) |  | 10⁶ | - Samsung, Ericsson, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN: Same as IMT-2020 - Nokia, Qualcomm, Apple, CATT: [10x] IMT-2020 - Vivo, Xiaomi, IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [1-10x] IMT-2020 - Huawei, CMCC, China Telecom: [10-100x] IMT-2020 - ZTE: [100x] IMT-2020 |
| Reliability |  | 1 - 10⁻⁵ | - OPPO, Samsung, Ericsson, Nokia, vivo, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, Xiaomi: 1-10−5 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 1-10-5 ~ 1-10-7 - Huawei, CMCC, China Telecom: 1-10-6 ~ 1-10-7 - Qualcomm, Apple: 1-10−6 |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | - CMCC, Qualcomm, Xiaomi, China Telecom propose to introduce a new class of mobility (up to 1000/1200 km/h) for ATG. - Samsung, Nokia, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, Apple, MTK propose that same mobility classes as in IMT-2020 are sufficient, but the associated data rates should/may be increased, e.g., [1.5x] IMT-2020 proposed by Samsung. - Vivo proposes to add the practical Urban Grid deployment scenario as a test environment. |
| Mobility interruption time (ms) |  | 0 | - Samsung, CMCC, Xiaomi, China Telecom, Huawei, Apple, CATT, ZTE, Nokia, vivo, IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 0ms |
| Bandwidth (MHz) |  | 100 | - Samsung, Nokia, vivo, China Telecom, Apple, ZTE: 400MHz - Ericsson, CMCC, Xiaomi, Huawei, China Telecom, IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 200MHz |