3GPP TSG RAN meeting #107										RP-250791

Incheon, Republic of Korea, March 12-14, 2025					(Revision of RP-250146)

## Status Report to TSG

Agenda item:			9.2.5

| WI / SI Name | Study on spatial channel model for demodulation performance requirements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: N/A | Performance part: N/A |  | Testing part: N/A |
| Acronym | FS_NR_demod_SCM |  |  |  |  |
| Unique ID | 1040100 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-250557 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 09/2025 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 45 % | Core part: N/A | Performance part: N/A | Testing part: N/A |  |

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Alexander Hamilton |
|  | Company | Nokia |
|  | Email | Alexander.hamilton@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

Note: Study Item is extended by 1 quarter to 09/2025, new TU spreadsheet attached to RP-250557.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	Agreements from RAN4#114

#### Title of Section 6

Agreement:

Rename Chapter 6 to “Evaluation and Comparison of Channel Models”

Note: This chapter will include comparison of all channel models including legacy channels.

#### Legacy channel models

Agreement:

Evaluation and comparison with legacy channel models should be included in Section 6 of TR.

#### CSI-RS Periodicity

Agreement:

CSI-RS is transmitted with a periodicity of 5ms in the first slot of the TDD pattern. PDSCH is not transmitted in the slots carrying CSI-RS nor S slot. CQI/PMI delay of 7ms.

#### Regarding EPRE Ratio:

Agreement:

Average EPRE of PDSCH data is equal to the average EPRE of CSI-RS, no power boosting is allowed for CSI-RS with respect to the number of CDM-groups in an OFDM symbol.

#### Receiver Type for SU-MIMO

Agreement:

A linear MMSE-IRC receiver to be used for the SCM study

#### Receiver Type for 8 Layer 8Rx

Agreement:

For 8Rx, “Baseline” 8Rx Receiver to be assumed

#### Cases for MU-MIMO

Agreement:

SU-MIMO to take priority over MU-MIMO.

Simulation alignment spreadsheet will only include agreed alignment cases.

#### SNR Definition – ‘long term average’

Agreement:

Normalization factor is constant over the duration of the simulation

#### Comparison with Legacy TDL

Agreement:

The following Legacy TDL models for comparison:

- Option 1: TDLC 300-10/100 with following correlation:

  - Option 1a: Low

  - Option 1b: XP Medium

  - Option 1c: XP High

  - Option 1d: Medium

#### 2.4.2	Remaining Open issues

##### 2.4.2.1	Open Issues following RAN4#114

#### Capturing of Channel Model Description

Way Forward:

(Keep Prior Agreement) High level descriptions of channel models in Chapter 5, comparison using cases and metrics in Chapter 6, alignment in chapter 7, with detailed description in Annex [B]

Note: Further details for each section to be clarified with contributions and pCRs at future meetings.

#### Capturing of Alignment and Comparison in the TR

Way Forward:

Company views on how to capture alignment and comparison in the TR to be brought to RAN4#114-bis.

#### PDSCH PMI Selection

Way Forward:

Random PMI already agreed. Regarding potential fixed PMI:

- Option 1: Random PMI from sub-set

- Option 2: Collectively chosen fixed PMI

- Option 3: Independently chosen fixed PMI

Note: Companies to propose detailed method description for interested options for RAN4#114-bis.

#### PMI for alignment of SCM

Way Forward:

First check alignment with PMI Random (type I)

#### SU-MIMO PMI configurations

Way Forward:

Applicable parameters reused from Rel-16 configuration for eType II, Table 6.3.3.2.6-1 (for 16Tx), requires change to 8Tx

See Annex A for further information.

#### Speed/Doppler

Way Forward:

For Comparison:

TDL Channels use 10 and 100Hz

CDL Channels use 3kph and 30kph

For Alignment:

TDL Channels use 10Hz

CDL Channels use 3kph

#### TDL Model Extensions

Way Forward:

Companies to use following two enhanced TDL channel for study.

- Option 1: Multi-cluster TDL channel with implementation details in R4-2500337:

  - For proposed simulation configurations see Annex C, Table C.1 and Table C.2.

  - For cluster powers settings there are several examples simulated in R4-2500335. We suggest trying 2 different setups

    - Un-even layer powers and even CW-powers.

    - Even layer powers and un-even CW-powers.

- Option 2: Multi-beams TDL channel with implementation details in R4-2501738:

  - 4T4R,4Layers: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table C.3

  - PMI test: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table C.3

  - 8T8R, 8Layers: Enhanced TDLC300-100, XPL, high with 4 Tx-Rx beams as defined in Table C.4 and 6 Tx-Rx beams as defined in Table C.5

#### Angles of Departure and Angles of Arrival

Way Forward:

For all 4 direction combinations (AoA, AoD, azimuth, zenith) the following parameters need to be configured.

Angle spread scaling: ASdesired

Angle rotation: μϕ,desired

as a starting point.

For CDL Option A1: Based on 38.827

For CDL Option A2, for each (AoA, AoD) and θ,ϕ, the parameters in the formula are the following:

ASmodel is computed according to TR 38.901, Annex A.1, Equation A-1

μϕ,model  is computed according to TR 38.901, Annex A.2, Equation A-2

For the purposes of simulation alignment:

μϕ,desired =  μϕ,model

ASdesired

|  | ASdesired, ASD | ASdesired, ASA | ASdesired, ZSD | ASdesired, ZSA |
| --- | --- | --- | --- | --- |
| CDL Option A1 | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| CDL Option A2 | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| CDL Option A3 | 25.7620 | 74.1138 | 4.8978 | 18.2050 |

As a starting point: cASA,ASD,ZSD,ZSA according to the table below

| Per-Cluster Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value | 2 | 15 | 3 | 7 | 7 |

#### SNR Normalisation with CDL

Way Forward:

Options:

- Option 1: Interested companies to provide information on long-term pre-EQ power measurements, for discussion whether a formula, or value, is required during RAN4#114-bis.

- Option 2: Define a normalisation method

  - Option 2a:

RAN4 to use following as normalization factor: C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2} Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

  - Option 2b:

Let normalized AAV beamformer vector of length Ms∙Ns (i.e. sub-array size) for one antenna port be α1,1⋯αMs,Ns=1GBF1Ms∙Nsexp-j2π∙θ1,1 ⋯ exp⁡(-j2π∙θMs,Ns) Here, amplitude normalization is separated into two factors - 1Ms∙Ns : split transmit power of each AP evenly over Ms∙Ns transmit antenna elements - 1GBF : normalize with measured transmit beamforming (power) gain GBF

#### TDL and CDL Methodologies

Way Forward:

Options:

CDL from TR 38.753 will include randomness reduction for the purpose of the SI

- CDL (TR 38.753)

  - Option A1 : with Table 7.2.1-8 in TR 38.827

  - Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation

  - Option A3 : based on Table 7.7.1-3 in TR 38.901 with truncation 
According to the updated truncation procedure described in Annex B, truncated table to be shared before RAN4#114-bis.

- Option B1: TDL (MediaTek)

- Option B2: TDL (Huawei)

#### Antenna Array Virtualisation (for CDL options)

Way Forward:

Downselection to be based upon (1. Best performance, 2. Ease of alignment, 3. Practicality)

Options:

- Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration

  - [(M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only]

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

#### Performance Comparison Methodologies

Way Forward:

Companies welcome to bring views at next meeting

#### Channel Properties Comparison Methodologies

Way Forward:

Companies welcome to bring views at next meeting.

#### Metric for PMI tests comparison

Way Forward:

Options:

- Option 1: Gamma value to random PMI configuration.

- Other options are not preconcluded.

#### SNR Throughput Percentage measurements (for PDSCH)

Way forward:

At least 70% and 30% for alignment of PDSCH and interested companies can provide full curves if desired.

Simulation results collection excel sheet format will be updated accordingly in the next meeting allowing to capture input SNR values for target % of the max throughput for PDSCH alignment.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

## 4.	References

From RAN4#112

- RP-241610	SID, Study on spatial channel model for demodulation performance requirements	Nokia, BT plc

- R4-2411557	Scenarios and Requirements for Spatial Channel Modelling	BT plc

- R4-2411044	Work Plan for Spatial Channel Model Study Item	Nokia

- R4-2412320	General view on SCM for demodulation	Ericsson

- R4-2412321	Discussion on SCM methodology	Ericsson

- R4-2412322	Simulation results for SCM	Ericsson

- R4-2412328	Discussion on Spatial Channel Modelling	MediaTek inc.

- R4-2413272	On Rel-19 spatial channel model for demodulation performance requirements	Keysight Technologies UK Ltd

- R4-2413056	Initial Considerations on the study on SCM for demodulation performance requirements	Qualcomm Incorporated

- R4-2412793	Discussion on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2412762	Overviews on spatial channel model for demodulation requirements	Huawei,HiSilicon

- R4-2411119	Discussion on spatial channel modelling methodology	CATT

- R4-2411300	Discussion on Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2411301	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2411390	On Spatial Channel Modeling for Demodulation Requirements	Apple

- R4-2412535	Initial view on spatial channel modelling SI	Samsung

- R4-2413427	Topic summary for [112][327] NR_SCM	Moderator (Nokia)

- R4-2413568	Offline meeting minutes for [112][327] NR_SCM	Nokia

- R4-2413606	Way Forward for [112][327] NR_SCM	Nokia

From RAN4#112-bis

- R4-2414957	Views on spatial channel modeling study	China Telecom

- R4-2415078	Discussion on spatial channel modelling methodology	CATT

- R4-2415282	Discussion on Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2415283	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2415285	TDL and CDL comparison with respect to the CSI resolution feedback 	Orange, BT, Telecom Italia, Vodafone, Bouygues Telecom, Fraunhofer, Deutsche Telekom

- R4-2415296	Discussion on General Items on SCM SI	Nokia

- R4-2415382	Discussion on spatial channel modelling	MediaTek inc.

- R4-2415383	Simulation results of spatial channel modelling	MediaTek inc.

- R4-2415506	On Spatial Channel Modeling for Demodulation Requirements	Apple

- R4-2415636	Discussion on spatial channel modelling methodology	Huawei,HiSilicon

- R4-2415673	Further observations from deployment and MU-MIMO use cases for SCM evaluation	BT plc

- R4-2415682	Discussion on spatial channel model for demodulation performance requirements	Samsung

- R4-2416093	Discussion on general issue of NR SCM for demodulation	Ericsson

- R4-2416094	Discussion on methodology of NR SCM for demodulation	Ericsson

- R4-2416095	Simulation results for NR SCM for demodulation	Ericsson

- R4-2416343	Discussion on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2416344	Simulation results on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2416380	Study of SCM Model Parameters for demodulation performance requirements	Qualcomm Incorporated

- R4-2416451	On Rel-19 spatial channel model for demodulation performance requirements	Keysight Technologies UK Ltd

- R4-2416539	Topic summary for [112bis][318] NR_SCM	Moderator (Nokia)

- R4-2416549	TR 38.753 Skeleton	Nokia

- R4-2416581	Ad-hoc meeting minutes for [112bis][318] NR_SCM	Nokia

- R4-2416591	Way Forward for [112bis][318] NR_SCM	Nokia

From RAN4#113

- R4-2417556	Discussion on General Items on SCM SI	Nokia

- R4-2417801	Discussion on spatial channel modelling	MediaTek inc.

- R4-2417802	Simulation results of spatial channel modelling	MediaTek inc.

- R4-2417827	Further discussion on spatial channel modelling methodology	CATT

- R4-2418028	Further considerations on the study on SCM	Qualcomm Incorporated

- R4-2418042	On Rel-19 Spatial channel model CDL Based Methodologies aspects	Keysight Technologies UK Ltd

- R4-2418043	Discussion on Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2418044	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2418550	On Spatial Channel Modeling for Demodulation Requirements	Apple

- R4-2418620	Discussion on spatial channel model for demodulation performance requirements	Samsung

- R4-2419166	Discussion on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2419251	General issue for Rel-19 SCM for demodulation	Ericsson

- R4-2419252	Discussion on methodology for Rel-19 SCM for demodulation	Ericsson

- R4-2419253	Simulation results for SCM for demodulation	Ericsson

- R4-2419338	Further observations on the properties of MIMO channels in typical deployments	BT plc, Ericsson

- R4-2419347	Initial Simulation Results for Performance Comparison	BT plc

- R4-2419607	Topic summary for [113][320] NR_SCM	Moderator (Nokia)

- R4-2419781	Ad-hoc meeting minutes for [113][320] NR_SCM	Nokia

- R4-2419782	Way Forward for [113][320] NR_SCM	Nokia

- R4-2419895	Discussion on spatial channel model for demodulation requirements	Huawei, HiSilicon

From RAN4#113

- R4-2500818	Discussion on General Aspects of SCM SI	Nokia

- R4-2501745	Measurement results for MIMO channels in band n78	BT plc, Ericsson

- R4-2501785	Discussion on Rel-19 NR SCM general aspects	Ericsson

- R4-2501786	TP for TR38.753 on content of chapter 7	Ericsson

- R4-2501990	On Spatial Channel Modeling study and TR scope	Apple

- R4-2500217	Performance comparison of high and low-resolution codebook under different CDL models and AAV configurations	Orange Romania

- R4-2500262	On Spatial Channel Modeling	Apple

- R4-2500263	Simulation results Spatial Channel Modeling study	Apple

- R4-2500264	Draft TP to TR on SCM study - 5.1 CDL approach	Apple, Qualcomm

- R4-2500331	TP to TR 38.753: Scope and reference	China Telecom

- R4-2500335	Discussion on spatial channel modelling	MediaTek inc.

- R4-2500336	Simulation results of spatial channel modelling	MediaTek inc.

- R4-2500337	TP to TR38.753: TDL approaches and related Annex	MediaTek inc.

- R4-2500427	Discussion on Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2500428	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements	Nokia

- R4-2500993	Further discussion on spatial channel modelling methodology	CATT

- R4-2501428	Discussion on spatial channel model for demodulation performance requirements	Samsung

- R4-2501429	Initial simualtion results on spatial channel model	Samsung

- R4-2501430	TP for 38.753 on chapter 4	Samsung

- R4-2501503	Discussion on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2501504	Simulation results on spatial channel model for demodulation performance requirements	ZTE Corporation, Sanechips

- R4-2501505	TP for 38.753 introduction of spatial channel models for SU-MIMO PMI cases	ZTE Corporation, Sanechips

- R4-2501517	Discussions on spatial channel model for demodulation requirements	Huawei, HiSilicon

- R4-2501518	Simulation results for spatial channel model	Huawei, HiSilicon

- R4-2501519	Draft TP introduction of comparision of spatial channel models for SU-MIMO PDSCH cases	Huawei, HiSilicon

- R4-2501738	Discussions on spatial channel model for demodulation requirements	Huawei, HiSilicon

- R4-2501743	Results collection table for NR SCM	BT plc

- R4-2501744	Simulation Results for SCM Comparison and Alignment	BT plc

- R4-2501787	Discussion on Rel-19 NR SCM methodology for demodulation	Ericsson

- R4-2501788	Simulation results for Rel-19 NR SCM for demodulation	Ericsson

- R4-2501934	Study of SCM for demodulation performance requirements	Qualcomm Incorporated

- R4-2501954	Performance comparison of high and low-resolution codebook under different CDL models and AAV configurations	Orange, Fraunhofer

- R4-2501998	Simulation Results for SCM Demod	Qualcomm Incorporated

- R4-2502377	Results collection table for NR SCM	BT plc

- R4-2500583	Topic summary for [114][322] NR_SCM	Moderator (Nokia)

- R4-2502276	Ad-hoc meeting minutes for [114][322] NR_SCM	Nokia

- R4-2502277	Way Forward for [114][322] NR_SCM	Nokia

- R4-2502378	Way Forward for [114][322] NR_SCM	Nokia