3GPP TSG-RAN Meeting #107	RP-250121

Incheon, Korea, 12 - 14 March 2025

Agenda Item:		15.2.4

Source:			NEC

Title:			Views on Rel-20 AI/ML Mobility

Document for:		Discussion and decision

# Introduction

In Rel-19, the potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover was extensively studied and evaluated for NR. The study focuses on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases in the study focus on standalone NR PCell change. The following use cases were studied and evaluated:

- AI/ML based RRM measurement prediction

- AI/ML based measurement event prediction

- AI/ML based RLF prediction

This paper provides our views on the work scope of Rel-20 AI/ML Mobility according to the outcome of the Rel-19 study on AI/ML aided mobility captured in TR38.744[3].

# Discussion

## 2.1 AI/ML based RRM measurement prediction

During the study, AI/ML based RRM measurement prediction was discussed for both UE-side and NW-side model. The study for RRM measurement prediction was carried out in terms temporal domain, frequency domain and spatial domain.

In intra-frequency temporal domain case A, continuous measurement results in PW (Prediction Window) are predicted by continuous historical measurement result(s) in OW (Observation Window). Then OW and PW slide forward with either sampling period(s) or measurement period(s), where measurement result(s) are actually measured before sliding. In intra-frequency temporal domain case B, measurement results in PW are predicted by historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) or measurement period(s) and measurement result(s) in previous PW is/are skipped during window sliding. The examples of case A and case B are depicted in Figure-1 below.

Figure-1: Temporal domain case A and case B for RRM measurement prediction

During the study, both beam-level RRM measurement prediction and cell-level RRM measurement prediction were studied.

There are three sub-use cases considered during the study for beam-level RRM measurement prediction:

- Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

- Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

For above sub-use case 4,5,6 (in the TR 38.744), we suggest to focus on L3 beam-level measurement prediction in terms of Intra-frequency intra-cell temporal domain Case A.

There are three sub-use cases considered for cell-level RRM measurement prediction during the study:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

For above sub-use case 1,2,3 (in the TR 38.744) for cell-level RRM measurement prediction, we suggest to focus on the following scenarios:.

- Intra-frequency intra-cell temporal domain Case A;

- Intra-frequency intra-cell temporal domain Case B;

- Intra-frequency intra-cell spatial domain (as depicted in Figure-2);

- Inter-frequency inter-cell spatial domain, i.e., frequency domain(as depicted in Figure-2).

Figure-2: Intra-cell spatial domain and Inter-cell spatial domain prediction

The evaluation work in RAN2 for AIML based RRM measurement prediction is still ongoing. Based on the current preliminary simulation result submitted to RAN2 and also the RAN2 discussion, the gain was observed compared to non-AI case. For example, as concluded in TR38.744, for intra-frequency temporal domain case A, AI can provide gains (in terms of L3 cell RSRP difference) compared to sample and hold, and the gain improves with UE speed increase. In addition, for Case A, the gain that AI can provide increases with the increase of prediction window length (up to a certain window length) when compared to sample and hold. For FR1 intra-frequency temporal domain Case B temporal domain prediction, it was observed that AI can provide gain (i.e., lower L3 cell-level RSRP difference) compared to Non-AI (i.e., Sample and Hold) and the gain increases as MRRT increases. For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold.

As the current RAN2 evaluation focuses on intra-frequency intra cell temporal domain cases (Case A and Case B as defined by TR38.744) and also Inter-frequency inter-cell frequency domain case for cell level RRM measurement prediction, we would like to suggest the work item prioritize them as the objective.

Proposal-1: Focus on Cell-level RRM measurement prediction.

Proposal-2: Prioritize intra-frequency intra cell temporal domain cases (Case A and Case B as defined by TR38.744) and also Inter-frequency inter-cell frequency domain case for RRM measurement prediction.

## 2.2 AI/ML based measurement event prediction

The existing handover mechanism is triggered based on reported measurement event, i.e., it is kind of reactive scheme. However, with the development of network, such kind of reactive handover mechanism may not be optimal considering high mobility, high density and future services, e.g., XR, such reactive scheme may result in unintended event, e.g., HOF, RLF, Ping-Pong, throughput loss etc.

With the aid of AIML, proactive scheme can be achieved for mobility. Proactive mobility may be based on the prediction of the occurrence of a certain measurement event based on AIML. As shown in Figure 3, with the prediction of the fulfilment of the A3 event, there may be less delay for the handover decision made at the network and accordingly the handover performance may be improved in terms of handover preparation in advance.

Figure-3: Example of prediction of measurement event by AIML

During the study in Rel-19, both direct prediction and indirect prediction were studied for AI/ML based measurement event prediction based on UE-side model. Indirect prediction is reusing AI outputs from RRM measurement prediction to determine the occurrence of measurement events.

The evaluation work in RAN2 for measurement event prediction is still ongoing. According to the current observation, the AIML model based measurement event predictions give a possibility to reduce the measurement effort of the UE without impacting the handover performance. The handover performance gain was also observed with measurement event prediction (e.g. A3 event) comparing to non-AI case based on the discussion in RAN2. Hence, the support of measurement event prediction should be specified for Rel-20.

Proposal-3: Specify AI/ML based event prediction for UE-side model, to predict events occurrence time instance.

## 2.3 AI/ML based RLF prediction

Radio link failure (RLF) can be a problem in 5G networks because it can reduce communication reliability and increase latency.  RLF prediction system in Radio Access Networks (RANs) is critical for ensuring seamless communication and meeting the stringent requirements of high data rates, low latency, and improved reliability in 5G networks.

In NR, T310 timer is used to govern the process of RLF detection and declaration. The T310 timer gives the user equipment (UE) a chance to re-sync. If the UE can't re-sync within the T310 timer, the UE declares a radio link failure. Hence the prediction of the occurrence of T310 start and T310 expiration can help to determine if there is RLF ahead. As shown in the example of Figure-4, the left side shows an example of the prediction of the RLF with T310 expiration and the right side shows an example of the prediction of the RLF with T310 stop before expiration.

Figure-4: Example of RLF prediction by AIML

Even though the study in RAN2 on AI/ML based RLF prediction is completed yet, we see the value of the RLF prediction to improve the system performance from UE side.

Proposal-4: Specify AI/ML based RLF prediction for UE-side model, to predict T310 and/or RLF occurrence time instance.

# Proposals

In this contribution, we shared our further considerations on the scope of Rel-20 AI/ML for Mobility and the following proposals were drawn from the analysis.

Proposal-1: Focus on Cell-level RRM measurement prediction.

Proposal-2: Prioritize intra-frequency intra cell temporal domain cases (Case A and Case B as defined by TR38.744) and also Inter-frequency inter-cell frequency domain case for RRM measurement prediction.

Proposal-3: Specify AI/ML based event prediction for UE-side model, to predict events occurrence time instance.

Proposal-4: Specify AI/ML based RLF prediction for UE-side model, to predict T310 and/or RLF occurrence time instance.

# Reference

[1] RP-242393, SID on AIML for mobility in NR

[2] RP‑242506, AIML for Mobility, NEC

[2] TR38.744 v0.0.5