3GPP TSG-RAN Meeting #107	RP-250123

Incheon, Korea, 12 - 14 March 2025

Agenda Item:		15.2.6

Source:		NEC

Title:			Views on Rel-20 AI/ML for NG-RAN Phase 3

Document for:		Discussion and decision

# Introduction

AI/ML is an advanced tool to help enhance network performance and maintain operation cost-effectiveness. For AI/ML for NG-RAN topic, in Rel-18 and Rel-19, RAN3 covered the following use cases for both non-split and split architecture:

- Load Balancing

- Energy Saving

- Mobility Optimization

- Network slicing

- Capacity and Coverage Optimization

According to majority opinion, Rel-20 is likely to be the last release for 5G, therefore, a Rel-20 study item is needed to finalize Rel-19 leftover topic(s), which have already been studied in Rel-19 to some extent, but still need further discussion on solution down selection. Besides, more new AI/ML based use cases is preferred to be investigated to enrich 5G AI/ML feature.

In RAN#106 meeting, an initial discussion was taken place on Rel-20 studies and AI/ML for NG-RAN was listed as one of the priority items for Rel-20.

In this contribution, based on our contribution submitted and presented in RAN#106 meeting [1], we further provide our views on Rel-20 AI/ML for NG-RAN study, including the detailed study scopes.

# Discussion

## 2.1	Rel-20 baseline

As mentioned above, considering Rel-20 is likely to be the last release for 5G, in this release, we should entirely finalize the ongoing specification tasks for 5G if possible. Therefore, for Rel-20 study, we suggest considering Rel-19 leftovers as 1st priority items, and then we can discuss whether or what new use cases can be included.

Proposal 1: For Rel-20 AI/ML for NG-RAN Phase 3, Rel-19 leftovers should be considered as 1st priority items.

In Rel-18 and Rel-19 AI/ML for NG-RAN, some high-level principles and the following functional framework were developed.

Figure 1. Functional Framework for RAN Intelligence [2]

The following deployment scenarios are supported.

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB (gNB-CU).

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB (gNB-CU).

And two procedures, i.e., Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, were defined to support AI/ML data collection and reporting.

For the Rel-20 study, we suggest to still follow the already defined high-level principles and functional framework and  take into account the already defined procedures.

Proposal 2: Rel-20 study should take the Rel-18 and Rel-19 discussions into account.

## 2.2	SI objectives

Below we analyse our preferred Rel-20 use cases that has already brought up in [1] one by one.

- Finer granularity energy saving

Nowadays operators ranked sustainability as the most important area for their network development. Energy saving is an importance feature for operators contributing to operational cost savings and commitments to net zero. With the continuous growth of 5G builds, high energy consumption is an urgent issue to resolve. To address this issue, AI/ML is a powerful tool to help network achieve energy optimization.

In Rel-19 Enhancements for AI/ML for NG-RAN SI, a conclusion was drawn after study [3]:

“Enhancements to the node-level EC to a finer granularity could be beneficial, assuming that such enhancements enable network-level energy saving”.

However, due to the limited study time, it was difficult to reach a consensus on specific work tasks of this topic for the following specification development, ES topic was excluded from Rel-19.

Considering the high interest of this use case, in Rel-20, we suggest continuing the study on AI/ML based finer granularity energy saving, taking into account the discussion and achieved agreements in Rel-19. For example, several NES technologies have already been developed in previous releases and AI/ML can facilitate these mechanisms to achieve best performance. To be specific, talking about cell-level energy saving, cell DTX/DRX was introduced to facilitate reducing gNB transmission/reception activity time, and for this mechanism, using AI/ML, an optimum cell DTX and cell DRX pattern can be obtained based on the future resource needs.

Proposal 3: Study on AI/ML based finer granularity energy saving, e.g. cell-level, needs to be continued in Rel-20.

- RAN-CN/OAM collaboration for AI/ML based network slicing

In previous releases, RAN and SA conducted AI/ML studies separately and thus end-to-end network performance optimization which involves different network domains may not be guaranteed.

For example, in 5GC, SA2 defined a network data analytics function (NWDAF), which supports efficient data collection from UEs, network functions and OAM as well as advanced data analytics. Therefore, in order to enhance overall network performance and user experience, the valuable data from NWDAF can be used in RAN as an input to or feedback data to train AI/ML model to enable better actions. As a starting point, we can first study the RAN-CN collaboration on network slicing feature. The detailed tasks can include enhancements to the NGAP interface to support AI/ML-driven information exchange between RAN and CN for network slice-level resource status updates.

Besides, we can also consider transmitting RAN feedback, e.g. slice-level SLA performance metrics, to CN/OAM. SLA is a contract between the Network Service Provider and the customer that defines the performance standards that to be met by the provider, and it is broken down into a set of attributes, e.g. PDCP throughput, PRB usage in RAN domain.

Since OAM manages the life cycle of the end-to-end slice across the network domains, including configuration, resource allocation, and performance management, RAN can provide SLA-related performance metrics (e.g., PDCP throughput, PRB usage) as feedback to OAM. AI/ML models can then use this feedback for inference to dynamically adjust slice configurations and resource allocation, ensuring better SLA compliance.

Proposal 4: Study on RAN-CN/OAM collaboration for AI/ML based network slicing is suggested to be conducted in Rel-20 SI.

- AI/ML based RVQoE prediction

5G QoE measurement collection a mechanism targeting at collecting application layer measurements from UEs. QoE metrics includes average throughput, initial playout delay, buffer level.

In order to optimize resource allocation per UE to improve user experience, QoE prediction by AI/ML model can be used in gNB for better resource reallocation. In case of not impacting legacy QoE mechanism, only the QoE metrics that are visible to NG-RAN can be considered as input data for AI/ML model in gNB, and therefore we suggest to focus on RVQoE prediction in this use case. The study can include e.g. RVQoE measurement collection for AI/ML model training and inference, future RVQoE prediction for specific UE(s), QoE prediction exchange over Xn interface for better handover decision or whole network optimization and actual RVQoE measurements feedback for model performance monitoring.

Proposal 5: Study on RVQoE predication is suggested to be conducted in Rel-20 SI.

Based on the analysis above, the objectives of the new Rel-20 SI are summarized as below:

The aim is to complete Rel-19 leftovers and investigate new AI/ML based use cases and identify enhancements to support more AI/ML functionality with existing NG-RAN interfaces and architecture. The detailed objectives are listed as follows: - Rel-19 leftovers, based on Rel-18 and Rel-19 principles, as follows: - Finer granularity energy saving, e.g. cell-level - Other leftovers (TBD depend on Rel-19 outcome) - Study RAN-CN/OAM collaboration to support AI/ML based network slicing - Study AI/ML based RVQoE Note: RAN3 should take the Rel-18 and Rel-19 discussions into account. Note: Coordination with RAN2, SA2, SA5 if needed.

Proposal 6: RAN to approve a new Rel-20 SI on AI/ML for NG-RAN Phase 3 and the SI objective is captured as above.

## 2.3	Expected time scale

Besides Rel-19 leftovers, if any new use case(s) will be included in the new Rel-20 SI, a short study phase is needed. Considering more TUs are needed for 6G studies at the end of Rel-20, in this case, we suggest allocating at least 3 month for SI phase, which can be followed by a 9-month WI phase.

Proposal 7: RAN to allocate 3 months for Rel-20 SI on AI/ML for NG-RAN Phase 3 and reserve another 9 months for the following up WI.

# Proposals

In this contribution, we share our further considerations on Rel-20 AI/ML for NG-RAN study and the following proposals were drawn from the analysis.

Proposal 1: For Rel-20 AI/ML for NG-RAN Phase 3, Rel-19 leftovers should be considered as 1st priority items.

Proposal 2: Rel-20 study should take the Rel-18 and Rel-19 discussions into account.

Proposal 3: Study on AI/ML based finer granularity energy saving, e.g. cell-level, needs to be continued in Rel-20.

Proposal 4: Study on RAN-CN/OAM collaboration for AI/ML based network slicing is suggested to be conducted in Rel-20 SI.

Proposal 5: Study on RVQoE predication is suggested to be conducted in Rel-20 SI.

Proposal 6: RAN to approve a new Rel-20 SI on AI/ML for NG-RAN Phase 3 and the SI objective is captured as below.

The aim is to complete Rel-19 leftovers and investigate new AI/ML based use cases and identify enhancements to support more AI/ML functionality with existing NG-RAN interfaces and architecture. The detailed objectives are listed as follows: - Rel-19 leftovers, based on Rel-18 and Rel-19 principles, as follows: - Finer granularity energy saving, e.g. cell-level - Other leftovers (TBD depend on Rel-19 outcome) - Study RAN-CN/OAM collaboration to support AI/ML based network slicing - Study AI/ML based RVQoE Note: RAN3 should take the Rel-18 and Rel-19 discussions into account. Note: Coordination with RAN2, SA2, SA5 if needed.

Proposal 7: RAN to allocate 3 months for Rel-20 SI on AI/ML for NG-RAN Phase 3 and reserve another 9 months for the following up WI.

# Reference

[1] RP‑242507, AIML for NG-RAN Phase 3, NEC

[2] TR 37.817, Study on enhancement for Data Collection for NR and EN-DC

[3] TR 38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN