3GPP TSG RAN Meeting #107					RP- 250128

Incheon, Korea, 12 - 14 March 2025

Agenda item:		15.2.9.1

Title:		View on Rel-20 ISAC study

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The channel modelling study on Integrated Sensing And Communication (ISAC) for NR was triggered from RAN#102 [1], which including the following use cases and sensing mode in Rel-19:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic).

In addition, in the RAN#106 meeting, the sensing/ISAC was listed as a RAN3-led candidate topic for Rel-20 [2] that:

Sensing/ISAC - A gNB based (mono-static only?) sensing/RAN3-led item targeting UAV? How about other use cases, e.g., smart transportation? RAN1 impact (e.g., performance evaluation, etc.)? - A study phase is necessary before any potential normative work - The scope and the expected specification impact should be minimized - Tight coordination with SA is expected - Important to avoid overlapping with the related study in 6G

This contribution would like to discuss the potential impacts of ISAC based on 5G-Adv system.

# 2. Discussion

2.1 Supporting on Rel-20 5G-Adv ISAC study

Integrated Sensing and Communication (ISAC) is recognized as a critical 6G feature requiring in-depth research within the Rel-20 6G study. However, the necessity and methodology for incorporating ISAC into Rel-20 5G-Advanced (5G-Adv) should be discussed and decided.

The rapid growth of the low-altitude economy, driven by Unmanned Aerial Vehicle (UAV) applications, necessitates foundational capabilities such as UAV detection, tracking, and navigation—functions essentially enabled by sensing technologies.

Considering the long-term nature of 6G development and implantation, it cannot address the urgent requirements of emerging low-altitude economy scenarios. Furthermore, the widespread deployment of NR systems and their expected longevity in existing networks, combined with ongoing RAN channel modeling investigations in Rel-19, strongly justify integrating ISAC studies into 5G-Adv systems under Rel-20.

Proposal 1: Rel-20 should initiate ISAC study based on 5G-Adv systems.

To avoid redundant efforts between 5G-Adv and 6G ISAC studies, clear differentiation in scope is required. For 5G-Adv, we propose prioritizing TRP-based sensing modes (TRP-TRP bi-static or TRP mono-static mode) due to their lower standardization impact compared to UE-based or TRP-UE-based sensing modes. The primary use case should focus on UAV detection/tracking/navigation to fulfill baseline requirements of low-altitude applications. Meanwhile, 6G studies should explore broader ISAC dimensions, including advanced use cases [3], more sensing modes, unified communication-sensing waveforms, and new network architectures.

Both TRP-TRP bi-static and TRP mono-static modes should be included in the study, as their technical solutions exhibit significant overlap while offering complementary deployment flexibility.

Proposal 2: The 5G-Adv study should focus on TRP-based sensing and UAV detecting/tracking/navigation use case.

Proposal 3: Both the TRP-TRP bi-static and TRP mono-static sensing modes are preferred in 5G-Adv study.

2.2 Standardization impact of Rel-20 5G-Adv ISAC study

Network architecture to support ISAC

Two potential architectural options for sensing control and data collection are proposed:

- Option 1: RAN sensing nodes are controlled by the 5G Core (5GC), with sensing data transmitted to the application server via the 5GC.

- Option 2: RAN sensing nodes are controlled by OAM, with sensing data sent to the application server via OAM, bypassing the 5GC.

Option 1: via 5G Core Network

Option 2: via OAM and bypass 5G Core Network

Option 1 may introduce standardization impacts on the 5GC and the 5GC-RAN interface, whereas Option 2 is primarily an implementation-specific approach with minimal impact on RAN and SA.

Proposal 4: The ISAC 5G-Adv study should evaluate the following two architectural options for sensing control and data collection:

- Option 1: RAN sensing nodes are controlled by the 5GC, with sensing data transmitted to the application server via the 5GC.

- Option 2: RAN sensing nodes are controlled by OAM, with sensing data sent to the application server via OAM, bypassing the 5GC.

For Option 1, some companies have proposed introducing a Sensing Function (SF) as a new 5GC entity to support sensing services and process sensing data from RAN node(s). In our understanding, the SF's essential role is to provide sensing service control. However, whether the SF should also support sensing data processing requires further discussion in RAN3 and SA2. This is because integrating data processing capabilities would increase the SF's complexity, and sensing data could alternatively be offloaded to trusted third-party servers for processing.

Proposal 5: If Option 1 is adopted, the necessity of introducing an SF within the 5GC to support sensing service control should be discussed during the SI phase.

Proposal 6: The extent and methodology of SF's involvement in sensing data processing should be discussed during the SI phase.

Additionally, two connection types between the RAN node and SF are proposed:

- The RAN node connects to the SF directly.

- The RAN node connects to the SF indirectly via AMF.

These connection types directly impact the 5GC-RAN interface design for ISAC support.

Proposal 7: The direct or indirect connection between the RAN node and SF should be proposed and evaluated during the SI phase, with consideration of its impact on the 5GC-RAN interface design.

The necessary signaling to support ISAC

For the Option 1, the necessary signaling should include the following to support ISAC services:

1. A sensing measurement request message from the SF to the RAN node to initiate the sensing procedure for the target(s).

2. The exchange of sensing assistance information between the SF and RAN node to facilitate the sensing process.

3. A sensing data reporting message from the RAN node to the SF to transmit the collected sensing data.

Proposal 8: The sensing measurement request, sensing assistance information, and sensing data collection are the basic signaling to support ISAC over 5GC-RAN node interface, which should be discussed and agreed in SI phase.

If the TRP bi-static sensing is performed by pair RAN nodes case, the transmitting RAN node should send the sensing signal configuration to the receiving RAN node(s), so that the receiving RAN node(s) can do the collection of corresponding sensing data. And it will has some impact on the interface between RAN nodes. If the TRP bi-static sensing is performed by single RAN node, the sensing signal configuration coordination is based on RAN node implementation and has no RAN impact.

Proposal 9: If the TRP bi-static sensing is performed by pair RAN nodes, the transmitting RAN node should provide the sensing signal configuration to the receiving RAN node(s).

Based on the analysis above, the architecture and the signaling are only having impact on RAN3, so the ISAC 5G-Adv study should only focus on RAN3.

Proposal 10: The ISAC 5G-Adv study should only focus on RAN3.

2.3 Expected time scale

Considering the parallel study of ISAC for both 5G-Adv and 6G during Rel-20, the time allocation must be carefully considered to ensure the solid study on both 5G-Adv and 6G and avoid duplication of efforts. Given the limited study scope proposed for Rel-20—specifically, TRP-based sensing modes for UAV use cases—we recommend a 9-month study period for 5G-Adv ISAC. This timeframe allows RAN to evaluate whether the study outcomes are sufficient to transition to a normative work within the remaining time of Rel-20.

Proposal 11: A 9-month study period should be allocated for 5G-Adv ISAC in Rel-20. RAN is suggested to evaluate the 9-month study outcomes to determine if it is sufficient to transition to a normative work within the remaining time of Rel-20.

# 3. Conclusion

Proposal 1: Rel-20 should initiate ISAC research based on 5G-Adv systems.

Proposal 2: The 5G-Adv study should focus on TRP-based sensing and UAV detecting/tracking/navigation.

Proposal 3: The TRP-TRP bi-static and TRP mono-static sensing modes are preferred in 5G-A5G-Adv study.

Proposal 4: The ISAC 5G-Adv study should evaluate the following two architectural options for sensing control and data collection:

- Option 1: RAN sensing nodes are controlled by the 5GC, with sensing data transmitted to the application server via the 5GC.

- Option 2: RAN sensing nodes are controlled by OAM, with sensing data sent to the application server via OAM, bypassing the 5GC.

Proposal 5: If Option 1 is adopted, the necessity of introducing an SF within the 5GC to support sensing service control should be discussed during the SI phase.

Proposal 6: The extent and methodology of SF's involvement in sensing data processing should be discussed during the SI phase.

Proposal 7: The direct or indirect connection between the RAN node and SF should be proposed and evaluated during the SI phase, with consideration of its impact on the 5GC-RAN interface design.

Proposal 8: The sensing measurement request, sensing assistance information, and sensing data collection are the basic signaling to support ISAC over 5GC-RAN interface, which should be discussed and agreed in SI phase.

Proposal 9: If the TRP bi-static sensing is performed by pair RAN nodes, the transmitting RAN node should provide the sensing signal configuration to the receiving RAN node(s).

Proposal 10: The ISAC 5G-Adv study should only focus on RAN3.

Proposal 11: A 9-month study period should be allocated for 5G-Adv ISAC in Rel-20. RAN is suggested to evaluate the 9-month study outcomes to determine if it is sufficient to transition to a normative work within the remaining time of Rel-20.

# References

- RP-234069, New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR.

- RP-243292, RAN Chair’s Summary of 5G-Advanced in Rel-20.

- TR 22.837, Feasibility Study on Integrated Sensing and Communication.