3GPP TSG-RAN Meeting #107	RP-250130

Incheon, Korea, 12 - 14 March 2025

Agenda Item:	15.2.10

Source:	NEC

Title:	Views on Rel-20 SBFD for 5G-Adv

Document for:	Discussion and Decision

1	Introduction

Subband Full Duplex (SBFD) has been proposed in Release 18 with a follow-up specification Work Item in Release 19. The support from various companies on the topic has demonstrated its necessity. Looking forward to Release 20, additional enhancements on duplex evolution are needed to facilitate its real-world deployment for 5GA. The commercial deployment of a 5GA duplex is vital as it paves a smoother path to a duplex for 6G as well.

This paper discusses and gives our views on Rel-20 Duplex Enhancement for 5G-Adv.

2	Discussion

For the Rel-20 5G-Adv scope, there is strong market interest in deploying SBFD features following some additional enhancements. The current SBFD frame established in Release 19 has inflexibilities that will hinder the real-world deployment of SBFD. The additional flexibility enhancements to duplex include the following areas:

- Dynamic On-Off SBFD

- Dynamic BW of SBFD

- Conversion of UL slots to SBFD slots

2.1	Dynamic On-Off SBFD

During the discussions in Release 18 duplex, the following was captured in TR 38.858 [1]:

For dynamic SBFD, the following observations are agreed. - Compared to semi-static SBFD, dynamic SBFD can better adapt to the UL/DL resource requirements based on UL/DL traffic loads. - Dynamic SBFD may increase gNB implementation complexity due to dynamic antenna/panels switching and filters/RF tuning, may incur loss of resources due to transition time, may increase inter-gNB CLI, may increase scheduling complexity, and can result in additional specification impact on top of semi-static SBFD - UE implementation complexity may be increased if the UE supports dynamic SBFD and dynamic SBFD may result in increased UE-to-UE CLI If dynamic SBFD is supported, the following options can be considered. - Option 1: Dynamic SBFD is achieved by scheduling DCI which is used to schedule DL receptions outside semi-statically configured SBFD DL subband and/or UL transmission outside semi-statically configured SBFD UL subband. - Option 2: Dynamic SBFD is achieved by non-scheduling DCI which indicates whether a symbol is SBFD symbol or not. - Option 3: Dynamic SBFD is achieved by MAC-CE which indicates whether a symbol is SBFD symbol or not. Note 1: Whether or not dynamic SBFD is beneficial from a performance and complexity perspective is a separate discussion. Note 2: The possibility of introducing flexible subband type for Option 1 to achieve DL receptions outside semi-statically configured SBFD DL subband and/or UL transmission outside semi-statically configured SBFD UL subband is not precluded. Note 3: None of the above options imply that there is a dynamic change in the DL/UL subband sizes.

While there was no consensus to support dynamic On-OFF SBFD in Release 19, there should be normative work to support this use case to allow for additional flexibilities in real-world deployment scenarios. The ability to turn off SBFD cells as needed can be a useful case when the DL traffic needs to be preserved. This allows the interference-free spectrum to be used for pure downlink (URLLC, for instance). Figure 1 outlines what this may look like.

Fig.1 Dynamic On-OFF SBFD

2.2	Dynamic BW of SBFD

In addition to the dynamic On-Off capabilities of SBFD, additional discussions can be had regarding note 3 of the TR 38.858 agreements. The changing SBFD bandwidth can be beneficial to the overall system as it offers some additional flexibility to increase the uplink traffic dynamically in order to meet the additional traffic demands.

Fig.2 Dynamic On-OFF SBFD

2.2	Conversion of UL slot to SBFD slot

While the enhanced uplink is an important aspect of SBFD, the DL must not be forgotten either. A key benefit of duplex is the ability to have a pseudo-FDD pattern within a TDD slot. This can only be realized when the uplink slots can be changed to downlink slots as well. This offers the benefit of a dedicated DL subband that is present for all slots and can be useful for emergency service signals where the availability of the DL needs to be ensured at all times

Fig.3 Conversion of UL slot to SBFD slot

3	Conclusion

In this contribution, we give our views on Rel-20 duplex for 5G-Adv. We propose that:

Proposal 1: Support duplex for Rel-20 5G-Adv for RAN1-led work, including:

- Dynamic SBFD [RAN1, RAN2, RAN4]

  - On-Off

  - BW size

- Conversion of UL slot to SBFD slot [RAN1, RAN2, RAN3, RAN4]

- Additional cases are not precluded

References

[1]	TR 38.858, Study on Evolution of NR Duplex Operation (Release 18)