3GPP TSG RAN Meeting #107	RP-250145

Incheon, Korea, March 12-14, 2025

Agenda item:	16

Source:	Nokia

Title:	Target Values for IMT-2030 Technical Performance Requirements

Document for:	Decision

# 1	Introduction

ITU-R Working Party 5D (WP 5D) has developed Report ITU-R M.2516 titled “Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond”, and Recommendation ITU-R M.2160 titled “Framework and overall objectives of the future development of IMT for 2030 and beyond” in the previous WRC cycle. In their LS in [1], ITU-R WP 5D request input from external organizations such as 3GPP on aspects including but not limited to:

- Candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (Feb 2025 TBC).

- Associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49, (June/July 2025 TBC)

The Framework includes definition of capabilities of IMT-2030, where some are currently listed with a range of research target values. The IMT-2030 Capabilities are illustrated in Figure 1. WP 5D is now expected to define (TPRs) for the capabilities, which then need to be mapped to usage scenarios and test environments, if applicable (refer to Section 4 of Rec. ITU-R M.2160). It should be recognized that some of these TPRs can be expressed and evaluated quantitatively, whereas others can only be expressed and evaluated qualitatively, i.e. as in IMT-2020, a target value may not always be defined for a TPR. Some capabilities that will not be evaluated may be left out of the ITU-R Report (without having associated TPRs); instead, IMT-2030 systems support for those Capabilities may be described verbally using the “Submission Template”, which is expected to be similar to ITU-R Report M.2411 defined for IMT-2020.

Figure 1. Capabilities of IMT-2030.

At the ITU-R WP 5D meeting #48 SGW Radio Aspects agreed to develop minimum technical performance requirements for the items listed in Table 1. The associated high-level assessment methodology is indicated as well.

Table 1. The Items for minimum technical performance requirements (MTPR) and high-level assessment methodologies agreed at ITU-R WP 5D meeting #48.

| MTPR | Method |
| --- | --- |
| AI-related capabilities | (TBD) |
| Area traffic capacity | (Quantitative) |
| Average Spectral efficiency | (Quantitative) |
| Peak spectral efficiency | (Quantitative) |
| 5th percentile user Spectral efficiency | (Quantitative) |
| Bandwidth | (Quantitative) |
| Connection density | (Quantitative) |
| Control plane latency | (Quantitative) |
| User plane latency | (Quantitative) |
| Energy Efficiency | (Quantitative) |
| Mobility | (Quantitative) |
| Mobility interruption time | (Quantitative) |
| Peak data rate | (Quantitative) |
| User Experienced Data Rate | (Quantitative) |
| Sensing-related capabilities | (Quantitative) |
| Positioning | (Quantitative) |
| Reliability | (Quantitative) |

Additionally, discussion continues on a few other items including a Joint requirement on data rate, latency, reliability and capacity, Coverage, Resilience, Interoperability, Security, and Sustainability.

In this contribution we provide our views on the IMT-2030 Capabilities, including items for which Technical Performance Requirements should be developed, and what the values for minimum performance requirements should be. It is proposed that TSG RAN discusses and agrees target values for the minimum technical performance requirements and sends a reply LS to ITU-R WP 5D at RAN#107 and/or RAN#108.

# 2	Discussion

The IMT-2030 capabilities can be roughly split into two groups:

- “Legacy” Capabilities that were already present in IMT-2020 (green colour in Figure 1), and

- Capabilities that are new

In the following we discuss these two groups of capabilities. Proposals related to the TPRs are provided in Section 3.

## 2.1	Legacy Capabilities Derived from IMT-2020

As IMT-2030 is based on evolution IMT-2020, ITU-R WP 5D has agreed to support the Capabilities defined for IMT-2020 in IMT-2030 as well. The associated performance numbers are expected to be more ambitious, when necessary. The same evaluation methodology can and should be largely re-used as well, to allow for performance comparisons between different generations of radio standards, and to avoid unnecessary work in both ITU-R and 3GPP. However, in some cases there is a need to consider extension of the IMT-2020 TPRs to better reflect the target Usage Scenarios of IMT-2030. In some other cases, IMT-2020 already provides more than satisfactory performance, and the requirements will not need to be stretched further. Below we discuss further these legacy TPRs. The related proposals are summarized in Table 4 in Section 3.

### 2.1.1 Spectral Efficiency

IMT-2020 requirement for peak spectral efficiency is 30bits/s/Hz for downlink and 15 bits/s/Hz for uplink. These values were defined assuming an antenna configuration to enable eight spatial layers (streams) in the downlink and four spatial layers (streams) in the uplink. Peak spectral efficiency is calculated analytically, taking into account the maximum number of spatial layers, the largest supported transport block sizes, as well as overhead due to e.g. DL-UL switching.

The main components affecting peak spectral efficiency are the number of spatial layers in SU-MIMO and the modulation order. With an increased number of transmitter and receiver antennas and spatial layers, as well as high-order modulation, peak spectral efficiency can be increased to some extent. We propose to set the requirements 50% higher than in IMT-2020, i.e.:

- Downlink peak spectral efficiency: 45 bits/s/Hz

- Uplink peak spectral efficiency: 22.5 bits/s/Hz

Average Spectral Efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP. IMT-2020 requirements for average spectral efficiency are listed in the table below.

Table 2. Spectral efficiency in IMT-2020.

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 9 | 6.75 |
| Dense Urban – eMBB | 7.8 | 5.4 |
| Rural – eMBB | 3.3 | 1.6 |

As increasing bandwidth does not significantly improve spectral efficiency, the main gain components are:

-	Advanced MIMO configurations provide beam forming / MU-MIMO gain​

- 	​Higher order modulation improves the data rates of high-SINR users

- 	Increased number of RX antennas improves link budget

5th Percentile Spectral Efficiency, aka cell-edge spectral efficiency is obtained in a similar way as average spectral efficiency. The gain mechanisms are largely the same, except that higher order modulation does not benefit cell-edge users having low SINR. On the other hand, interference management techniques may provide benefits to the users at the low end of the CDF curve. We propose to increase requirements for the 5th percentile spectral efficiency in the same way as average spectral efficiency, i.e. by a factor of two compared to IMT-2020.

Both average and 5th percentile spectral efficiency are heavily dependent on the evaluation scenario, and in particular on carrier frequency, number of TX/RX antennas, and the geographical environment. These assumptions need to be taken into account when setting the exact targets for spectral efficiency. As a starting point we propose to aim for 2x increase in spectral efficiency when compared to IMT-2020. The exact values may be adjusted later as the detailed evaluation scenario is developed.

### 2.1.2 Data rate related capabilities

Peak data rate is calculated analytically based on the peak spectral efficiency as a product of Peak Spectral Efficiency and the bandwidth. In IMT-2020, the assumption on the bandwidth for achieving the peak rata rate was not restricted, which allows for extensive use of carrier aggregation. The IMT-2020 requirement for DL and UL peak data rates are 20 Gbits/s and 10 Gbits/s, respectively.

As the bandwidth required for meeting peak data rates is unrestricted, it is in principle possible to set the requirements very high and the 6G standards would still meet them. On the other hand, it is widely acknowledged that that peak data rates are not met in practical deployments e.g. due to availability of spectrum for a single operator, as well as UE complexity related constraints. Therefore, we propose to increase the peak data rate requirements moderately by a factor of 5. Even such data rates are very hard to achieve in practice. The following IMT-2030 minimum technical requirements are proposed for peak data rate.

- Downlink peak data rate = 100 Gbits/s

- Uplink peak spectral efficiency = 50 Gbits/s

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. In case bandwidth is aggregated across multiple bands, the user experienced data rate will be summed over the bands. IMT-2020 requirements for user experienced data rate are 100 Mbit/s (downlink) and 50 Mbit/s (uplink). Key components in meeting the requirements for user experienced data rate are MU-MIMO and carrier aggregation.

With increased number of TX/RX antennas and additional spectrum, 6G should be able to exceed the IMT-2030 requirements by a fair margin. We propose to set the requirements for user experienced data rate five times higher than in IMT-2030, i.e.

- Downlink user experienced data rate: 500 Mbits/s

- Uplink user experienced data rate: 250 Mbits/s

### 2.1.3 Area Traffic Capacity and Connection Density

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2 ). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time. IMT-2020 requirement for area traffic capacity is 10 Mbits/s/m2.

In line with the increased spectral efficiency, we propose that the requirement for area traffic capacity is increased by a factor of five compared to IMT-2020.

Connection Density is the total number of devices fulfilling a specific quality of service (QoS) per km2. Connection density should be achieved for a limited bandwidth and number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability, as specified in Report ITU-R M.2412-0. The minimum requirement for connection density in IMT-2020 is 1 000 000 devices per km2.

Connection density depends heavily on the deployment/evaluation scenario: in smaller cells such as indoor hotspots very high connection density can be achieved, while is larger cells the achievable numbers are smaller. Assuming similar evaluation methodology as in IMT-2020, the requirement for connection density can be set 10 times higher than in IMT-2020.

### 2.1.4 Mobility

IMT-2020 TPRs include requirements for mobility classes up to 500 km/h with varying data rates, as shown in the table below. The requirements cover a wide rang of use cases ranging from pedestrian to high-speed railway scenarios in various geographical scenarios.

Table 3. Mobility requirements in IMT-2020

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 1.5 | 10 |
| Dense Urban – eMBB | 1.12 | 30 |
| Rural – eMBB | 0.8 | 120 |
|  | 0.45 | 500 |

We see that the same use cases are still the most important ones in 6G, and there is no need to add requirements for further mobility classes (i.e. velocities). However, the associated data rate requirements should be moderately increased to reflect the enhanced spectral efficiency.

### 2.1.5 Latency and Reliability

Reliability TPR in IMT-2020 was targeting in particular the URLLC Usage Scenario, with very low User Plane latency (down to 1ms) supported for small application data (e.g. 20 bytes). Given that radio air interface latency is only one of the components impacting the end-to-end latency, and other delay components such as propagation times to servers and related processing times are of the same or larger order, the IMR-2020 requirements for URLLC are sufficient also to meet the needs of IMT-2030. That is, the target for UP latency is 1 ms, and the target for reliability is 10-5 success probability.

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). In IMT-2020 the minimum requirement for control plane latency was 20 ms, and proponents were encouraged to consider lower value as well, e.g. 10 ms. For IMT-2030 we propose to set the requirement for CP latency as 10 ms.

### 2.1.6 Energy Efficiency

Sustainability is a key design target for the forthcoming 6G deployments and devices. Sustainability is a very broad field, including aspects related to manufacturing, deployment and operation during the life-time of the 6G networks and devices. Most of the aspects affecting overall sustainability of 6G are not directly related to standards, but rather on how the equipment is manufactured and operated and are hence not within the scope of IMT-2030 TPRs.

Energy efficiency is a key aspect for operation of IMT-2030 networks and UEs, affecting overall sustainability as well as operational cost and user experience. It is also possible to define quantifiable technical performance requirements for energy efficiency. Therefore, it makes sense to focus on EE, and in particular network EE when considering IMT-2030 related TPRs for sustainability.

When considering evaluation of EE, one should focus on defining requirements in a way that is agnostic to the exact type of implementation, which is not dictated by the standards. Instead, the requirements should verify the fundamental properties of radio standards that facilitate energy efficient operation. In IMT-2020 there were no numerical requirements set for EE, but instead network and UE energy efficiency were verified by inspection of support for high sleep ratio and duration, i.e. the network’s transmission rate of signals in case of an unloaded cell. We see that similar approach can be the starting point for IMT-2030 as well.

While IMT-2020 did not set specific requirements for sleep ratio and sleep duration, considering the constantly increasing importance of energy efficiency, IMT-2030 should adopt a more advanced approach and set quantifiable TPRs for EE. There are two main alternatives considered at ITU-R WP 5D:

Approach A: An approach similar to IMT-2020, but with specific requirements set for sleep ratio (e.g. > 95 %) and sleep duration (e.g. > 20 ms). Analytical approach can be used to verify that and RIT/SRIT meets the set requirements.

Approach B: Setting limit for the relative energy consumption in an unloaded case when compared to a fully loaded case. The requirement may be defined e.g. in terms of a percentage, such as 5%. This approach involves use of base station power models, that model the relative power consumption of different types of transmissions as well as transitions between them. The TPR can be evaluated analytically or with a relatively simple simulation, where the power model is applied on top a minimum set of transmissions in a case where there is no data traffic in a cell.

The two approaches described above are both ultimately analyzing the same thing, i.e. the minimum amount of transmissions occurring in a cell with no data traffic. Approach A is simpler in terms of definition in ITU-R, as well as analysis. Approach B provides more insight on how the actual network energy consumption varies between the fully loaded and unloaded cases; the downside is more complicated definition (especially the power model) and evaluation. Regarding the power model, 3GPP has developed and applied power models in past studies, but such models are not universally applicable nor technology agnostic, since they assume certain specific types of signals and protocols to be applied, that may not be present in other standards. Therefore, for Approach B to work, a simple and technology agnostic power model should be agreed upon at ITU-R WP 5D, or the power model should be left for the technology proponent to provide. This is unlikely to be a trivial discussion at ITU-R.  Given that approach A avoids this extra hurdle while capturing the same amount of information about the energy efficiency of the different systems, our preference is to adopt approach A in ITU-R.

### 2.1.7 Security and Resilience

Security and Resilience are key targets for the design of both 5G s well as 6G. Both Security and Resilience are broad concepts, including aspects going beyond radio standards and scope of ITU-R WP 5D. It is clearly impractical to define numerical requirements for security and resilience, and hence these two capabilities were not having specific TPRs in IMT-2020; instead, the related aspects were considered in the Submission Template, where technology proponents were tasked to verbally provide information on how the RAT in question supports these aspects.

As in 5G, it remains impossible to define numerical targets for Security and Resilience. However, verbal description on the supported security mechanisms and means to guarantee resilient operation should be provided, either using the Submission template as in IMT-2020, or by defining a TPR that is evaluated by “Inspection”. The latter approach may be considered especially for Resilience.

### 2.1.8 Bandwidth

The minimum requirement for bandwidth in IMT-2020 was 100 MHz for frequencies corresponding to (approximately) FR1. In IMT-2030 the requirement for bandwidth should be larger.   We propose to set the requirement for bandwidth to be at least 400 MHz. This can be achieved by e.g. a single carrier with 400 MHz channel bandwidth, or with carrier aggregation of two 200 MHz carriers.

## 2.2	New Capabilities of IMT-2030

In addition to the Capabilities that were already present in IMT-2020, IMT-2030 also defines a set of new Capabilities, extending the use of 6G to new use cases. These are discussed in the following.

### 2.2.1 Positioning

While Positioning is a new capability to ITU-R, present 3GPP standards for NR already provide support for multiple positioning techniques. Technical performance requirements should be defined for selected use cases (e.g. indoor and outdoor), considering also that practically achievable positioning accuracy is largely deployment dependent. The current IMT-2030 research target for positioning accuracy (sub-meter level) is only feasible with certain specific scenarios, and IMT-2030 TPRs should not be limited to the most demanding use cases only. We propose to set requirements for two cases:

- Indoor: sub-meter level accuracy

- Outdoor: meter level accuracy

### 2.2.2 Sensing

Sensing is one of the fundamentally new capabilities in IMT-2030. Unlike e.g. positioning, sensing is not presently supported in the existing 3GPP NR standards. Sensing encompasses a large variety of techniques and use cases, and it is hard to foresee which ones will be the most important applications in the future.

Some of the mentioned potential use cases for sensing include e.g. intrusion detection​, environment monitoring​, detection and tracking in factories​, UAV flight trajectory tracking, traffic sensing, automotive use, health checking and environment monitoring​. These potential use cases may further have different targets in terms of what is the sensing object (e.g. human, automobile, drone, factory goods…). It is apparent that technical performance requirements cannot be set to optimally cater for all envisioned use cases; instead, it is necessary to focus on key aspects that are common for most of the anticipated sensing applications.

Given that sensing is still largely in a research phase, and anticipated applications are rather diverse, it is preferred to focus in IMT-2030 on requirements that are not tied to a specific type of sensing object or use case. Most of the foreseen sensing applications assume at least localization of the sensing object to be performed. Therefore, it is proposed that IMT-2030 focuses on this aspect. By setting requirements for localization accuracy or resolution, evaluated analytically or by simulations, it is possible to verify the main boundaries for the sensing performance of an IMT-2030 standard, while avoiding complex prioritization of largely varying use cases. The setting of the requirements depends on the related channel models, and may need to be postponed.

### 2.2.3 Joint Requirement for Data Rate, Latency, Reliability and Capacity

IMT-2020 defines joint requirements for data rate, reliability and latency for the purpose of supporting URLLC services. While URLLC/HRLLC remains to be an important use case for IMT-2030, there are also new applications, where low latency (in the order of a few ms) is combined with larger data packet sizes (e.g. a few MBs) and moderate reliability. Use cases for such data rates and latencies include e.g. XR, video conferencing, and support for emerging AI based services. Therefore IMT-2030 TPRs should account for also such types of services, and defining new performance requirements for moderately low latency in combination with larger packet sizes should be considered. Additionally, the requirement should take into account the capacity, i.e. how many users can be served with the necessary QoS.

We propose to define a joint requirements for data rate, latency, reliability and capacity for a limited number of cases to reflect the required values for key use cases such as XR, AI, and videoconferencing. It needs to be further discussed what are the exact use cases that this requirement should target for, and what are the associated values for the requirements.

### 2.2.4 AI-related Capabilities

Use of AI in radio networks can be roughly categorized as follows:

- AI for network: using AI in operating the network, to e.g. improve performance

- Network for AI: providing network connectivity for AI services

AI for network:  AI will likely be widely used in IMT-2030 systems to improve the performance of radio interface functions such as symbol detection/decoding, channel estimation, scheduling, network optimization etc. Corresponding improvements in network performance are expected to be reflected in TPRs such as Spectrum Efficiency, User Experienced Data Rate, and Reliability. There is no need to define separate TPRs for “AI for Network”.

Network for AI: AI services are one of the major use cases for IMT-2030. IMT-2030 should therefore provide sufficient performance to support AI related tasks. Many of the existing TPRs are already good metrics for determining how well the network can support AI services. Moreover, an extension of the “Reliability” TPR as discussed in Section 2.1 can also ensure that necessary support for AI is present in IMT-2030.

### 2.2.5 Coverage

Coverage is listed as one of the capabilities of IMT-2030. Network coverage is a complex aspect to measure, and it depends on multiple factors including network deployment (inter-site distance, TX power, antenna configuration, BS antenna height, type of geographical environment, etc.). Coverage is also not a binary measure (yes/no), but instead it varies depending on the assumption on the required minimum bit rate and maximum allowed latency. Therefore, statistical properties, such as spectrum efficiency CDF, are the best way of characterizing network’s coverage in various types of deployment scenarios. The legacy TPRs discussed in Section 2.1. meet this purpose already, and there is no need to define additional TPRs for coverage.

Another aspect directly impacting coverage is the link budget. Provisioning of a link budget along with Maximum Coupling Loss (MCL) may be considered, as in IMT-2020.

### 2.2.6 Interoperability

Interoperability refers to the radio interface being based on member-inclusivity and transparency, so as to enable functionality(ies) between different entities of the system. It is clearly hard to define technical performance requirements for interoperability. Related aspects of an IMT-2030 radio standard can be covered in the submission template for IMT-2030.

# 3	Conclusions

In this contribution we have provided our views on Technical Performance requirements for IMT-2030. The following Table 4 summarizes the proposed items for which TPRs should be developed, as well as the values for minimum TPRs. We make the following proposals.

Proposal 1: TSG RAN discusses and agrees target values for candidate items for IMT-2030 minimum technical performance requirements and sends a reply LS to ITU-R WP 5D.

Proposal 2: Quantitative Technical Performance Requirements with are developed for the items listed in Table 4.

Table 4. The proposed items for Quantitative Technical Performance requirements (TPRs) of IMT-2030. The TPRs in green are carried over from IMT-2020 with some updates, while the TPRs in blue are completely new.

| TPR | IMT-2030 Requirement | Comments |
| --- | --- | --- |
| Peak Data Rate | DL 100 Gbits/s UL: 50 Gbits/s | 5 x IMT-2020 |
| Peak Spectral Efficiency | DL: 45 bit/s/Hz UL: 22.5 bit/s/Hz | 50% higher than in IMT-2020 |
| User Experienced Data Rate | DL: 500 Mbits/s UL: 250 Mbits/s | 5 x IMT-2020 |
| Average Spectral Efficiency | [2] x IMT-2020 | The exact requirements need to take into account the associated evaluation scenario, such as the carrier frequency. |
| 5%tile Spectral Efficiency | [2] x IMT-2020 | The exact requirements need to take into account the associated evaluation scenario, such as the carrier frequency. |
| Area Traffic Capacity | 5 – 10 M devices/km2 | 5-10 x IMT-2020 |
| User Plane Latency | 1 ms |  |
| Control Plane Latency | 10 ms |  |
| Connection Density | 107 devices per km2 | 10 x IMT-2020 |
| Energy Efficiency | 95% sleep ratio, 20 ms sleep duration | Requirements are set for sleep ratio and sleep duration. Alternatively, relative energy savings may be analysed using a power model. |
| Reliability | 10-5 |  |
| Mobility | Up to 500 km/h | Same mobility classes as in IMT-2020 are sufficient, but the associated data rates should be increased. |
| Mobility Interruption Time | 0 |  |
| Bandwidth | 400 MHz | 4 x IMT-2020 |
| Positioning (New) | Indoor: sub-meter Outdoor: a few meters | Define separate requirements for vertical and horizontal positioning accuracy. The requirements need to take into account the deployment scenarios, and same accuracy cannot be assumed in all cases. |
| Sensing (New) | FFS | Focus on TPRs that are use-case agnostic, e.g. localization accuracy and/or resolution, while considering detection probability and false alarm rate as side conditions. |
| Joint requirement for data rate, latency, reliability and capacity | FFS | The requirements are defined for a limited number of scenarios to reflect the required values for key use cases such as XR, AI, and videoconferencing |

# References

- SP-241439 , “LIAISON STATEMENT TO EXTERNAL ORGANIZATIONS: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, ITU-R WP 5D