3GPP TSG RAN Meeting #107			RP-250167

Incheon, Korea, March 12-14, 2025

Agenda item:	15.2.4

Title:	Discussion on normative working scope of AIML mobility in Rel-20

Source:	OPPO, ZTE, MediaTek, Nokia, CATT,CAICT

Document for:	Discussion

- Introduction

At RAN plenary#106 there is a workshop discussing potential Rel20 package in 5G-A thread. As one of the important conclusions, AI mobility is listed as one of the potential WIs in Rel20 as following in [2]:

The discussion on AI mobility mainly based on contribution [1], which elaborate the motivation of the normative work. In this paper we update the progress made so far to justify the normative work in Rel20 further. The initial version of potential working scope is also listed in the Annex.

- Discussion

There are mainly 4 use cases in Rel19 study item on AI Mobility and RAN2 focus on RRM prediction, measurement event prediction. HOF and RLF prediction are down prioritized.

Table 5.2.1-1: Prioritization of evaluation scenarios [3]

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

The RRM prediction use case is the core part of the study since other use cases are built on it basically e.g. the measurement event prediction with indirect methodology. Extensive simulations are evaluated on high priority scenarios as indicated in Table 5.2.1-1 [3]. Before evaluation two goals of the study are captured as following in the TR [3]:

The use cases in this study focus on RRC_CONNECTED mode and cover RRM measurement prediction, measurement event prediction and RLF/HOF prediction for PCell change procedure in standalone NR scenario. The study of the use cases is driven mainly by two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

The study on temporal domain case A is to verify the 2nd goal i.e. whether a proactive handover mechanism is feasible and if yes, how is the performance. For purely RRM prediction, RAN2 has observed that the performance i.e. the L3 RSRP prediction accuracy within a prediction window e.g. 480ms is around 1dB in average. And obviously AI algorithm outperform non-AI algorithm e.g. sample and hold. The performance gain against non-AI increases in more challenging scenarios e.g. with higher UE speed and longer prediction window etc. The further simulation on measurement event prediction based on temporal domain case A show high F1 score, on top of which SLS indicates that HOF rate decreased quite many compared to legacy handover procedure. The assumed handover model shows promising potential.

Observation 1: The evaluation on intra-frequency temporal domain case A shows that proactive handover is feasible with good performance gain.

The study on temporal domain case B is to verify the 1st goal i.e. whether the degradation of handover performance is acceptable while partial measurement is skipped. For purely RRM prediction, RAN2 has observed that prediction accuracy e.g. with 50% MRRT (measurement reduction rate in temporal domain) of less than 1 dB is observed among majority company’s simulation result. RAN2 also observed high F1 score for the measurement event prediction on top of temporal domain case B and concluded in relevant SLS that minor or even no handover performance degradation is observed. Agreements on

Observation 2: The evaluation on intra-frequency temporal domain case B shows that reducing measurement in temporal domain is feasible with acceptable or even no performance degradation.

The study on frequency domain prediction experience similar prediction accuracy situation and measurement event prediction. It means network could get measurement result of one frequency layer without any measurement as long as another frequency with correlation is measured and reported. And thus, the demand on measurement gap can be relaxed and user throughput can be improved.

Observation 3: The evaluation on inter-frequency domain shows that it is feasible to relax measurement gap configuration to improve user throughput and reduce measurement efforts.

RAN2 also study generalization aspect of the AI mobility model for RRM measurement prediction and observed that the generalization issue is minor across UE speeds. The inter-frequency prediction has generalization issue. However, the model trained with mixed dataset i.e. GC#2 with knowledge of prediction direction bears close performance to baseline. It means generalization issue can be resolved with proper way.

Observation 4: The evaluation on generalization aspects shows that model for AI mobility could be well generalized

Observations on high priority scenarios of RRM measurement prediction as well as their generalization evaluation are captured in section 5.2.2 [3]. Observations on measurement event prediction is captured in section 5.3.2. Conclusions of related SLS is capture din section 5.5.2 [3]. Those observations and conclusions are being updated via RAN2 post email discussion [5]. You can find the draft version here.

Based on [4] and online discussion, RAN2 agree to shift the focus from evaluation to spec impact study from April meeting and on.  Some offline discussion happened during offline of last RAN2#129 meeting in Athens regarding scope of spec impact and working scope of normative work. The objectives in Annex reflects what could be potential working scope of normative work of AI mobility for NR based on views of interested companies.

- Conclusion

Observation 1: The evaluation on intra-frequency temporal domain case A shows that proactive handover is feasible with good performance gain.

Observation 2: The evaluation on intra-frequency temporal domain case B shows that reducing measurement in temporal domain is feasible with acceptable or even no performance degradation.

Observation 3: The evaluation on inter-frequency domain shows that it is feasible to relax measurement gap configuration to improve user throughput and reduce measurement efforts.

Observation 4: The evaluation on generalization aspects shows that model for AI mobility could be well generalized

Based on above observations, we can conclude that RAN2 can proceed with normative work after spec impact study in this SI. The potential working scope based on current progress and views from interested companies so far is listed in the Annex for your information. The objectives are subject to WGs further progress.

Proposal 1: RAN2 proceeds with normative work after further study on spec impact aspects

- References

[1] RP-242793 The motivation to specify AI mobility.pptx, OPPO, CAICT, China Unicom, China Telecom, Interdigital

[2] RP-243292 RAN Chair's summary of 5G Advanced in Rel-20.pptx

[3] R2-2500287 Discussion on text proposal for TR, OPPO

[4] R2-2500288 Discussion on work plan of AI mobility SI, OPPO, MediaTek, Nokia, Interdigital

[5] [POST129][021][AI Mob] TR update (OPPO)

- Annex

Objective of SI or Core part WI or Testing part WI

This WID focus on AI/ML-assisted mobility in RRC_CONNECTED mode wherein RRM measurement prediction and measurement event prediction could be applied e.g. L3 handover procedure and SCell addition or change.

Provide specification support for RRM measurement prediction use case: (RAN2)

	RRM measurement prediction can be performed in temporal domain, frequency domain and spatial domain

	Specify signaling and protocol aspect to enable reporting inference outcome of UE sided model and other information, if needed

	Specify signaling and protocol aspect to enable reporting information from UE for inference operation of network sided model

Provide specification support for measurement event prediction use case: (RAN2)

	Specify signaling and protocol aspect to enable reporting inference outcome of UE sided model e.g. measurement event based on prediction at UE side

	Measurement event type could be measurement event A1, A2, A3, A4, A5 and A6

For both RRM measurement prediction and measurement event prediction: (RAN2)

	For UE sided model, specify signaling and protocol to facilitate functionality-based LCM procedures including but not limited to applicability framework, model management framework, performance monitoring, data collection triggering , configuration and reporting etc.

	For network sided model, specify signaling and protocol to facilitate functionality-based LCM procedures including but not limited to performance monitoring(if any) ，data collection configuration and reporting etc.

	The agreement(s) from WI NR_AIML_air should be reused as much as possible, if applicable

	This WI should focus on AI mobility specific aspects

Note: Data transfer scheme and model delivery for UE sided model is subject to the progress made in WI NR_AIML_air

For both RRM measurement prediction and measurement event prediction: (RAN4)

	Specify core requirement identified in study phase

	Specify LCM-related requirements, if any

Objective of Performance part WI

For both RRM measurement prediction and measurement event prediction: (RAN4)

	Specify RRM performance requirement and tests identified in study phase

	Specify necessary test cases and performance requirements for LCM procedure, if any