3GPP TSG RAN Meeting #107				RP-250172

Incheon, Korea, March 12-14, 2025

Source:	OPPO

Title:	Discussion on 6G Scenarios and Requirements

Agenda Item:	16

Document for:	Discussion

Introduction

In 4G and 5G period, the technical requirements of IMT-Advanced (4G) and IMT-2020 (5G) were defined in ITU-R. Since Feb. 2024, ITU-R Working Party 5D (WP 5D) has started to draft a new Report on Minimum requirements related to technical performance for IMT-2030 (6G) radio interface(s). In this paper, we discuss the Technical performance requirements (TPR) for IMT-2030, i.e., the KPI targets. Also, we will introduce our views on 3GPP RAN requirements for 6G, which may not be included in the scope of IMT-2030 TPR.

Figure 1: Usage scenarios and KPIs for IMT-2030 [1]

Discussions on IMT-2030 (6G) TPR

Candidate sets of TPRs

In the ITU-R Recommendation “Framework and overall objectives of the future development of IMT for 2030 and beyond” [1], the in IMT-2030 TPR includes 9 enhanced capabilities and 6 enhanced capabilities:

Enhanced Capabilities for IMT-2030:

- Peak data rate

- User experience data rate

- Spectrum efficiency

- Area traffic capacity

- Connection density

- Mobility

- Latency

- Reliability

- Security and resilience

New Capabilities for IMT-2030:

- Positioning

- Interoperability

- Sustainability

- AI-related capabilities

- Sensing-related capabilities

- Coverage

We think the 15 capability items can be the baseline for IMT-2030 TPR study. Among them, the 9 enhanced capability items are generally inherited from IMT-2020 (5G) TPR and their definition are clear. However, for some KPI items are defined only for extreme use cases. For example, the latency and reliability in 5G are defined assuming zero payload in packet. But the extreme latency (e.g. 1ms) and reliability (e.g. 99.999%) does not make much sense for service provision in real market. Even for URLLC/IIoT services, a moderate size of packet needs to be transferred.

Hence, to evaluate the performance and the real use experience of a service requiring other KPIs than data rate, a joint KPI combining latency, reliability and capacity should be adopted. For example, in Study on XR (Extended Reality) Evaluations for NR (TR 38.838 [4]), the joint KPI is used to evaluate performance of XR system. For example, 11 XR users/cell can be supported with 30Mbps data rate, 10ms PDB (Packet Delay Budget) and 1% PER (Packet Error Rate). 16 XR users/cell can be supported with 30Mbps data rate, 10ms PDB and 5% PER.

We suggest to adopt the evaluation methodology in TR 38.838 [4] as reference, and define a joint KPI combining data rate, PDB, PER and capacity for typical services, as shown in Table 1.

Table 1: Example of joint KPI requirements for different services

|  | Data rate (Mbps) | PDB (ms) | PER (%) | Capacity (UEs/cell) |
| --- | --- | --- | --- | --- |
| Service 1 |  |  |  |  |
| Service 2 |  |  |  |  |
| Service 3 |  |  |  |  |
| …… |  |  |  |  |

For the first release of 6G, the primarily essential application scenarios are Immersive and Massive communication. For massive communication, the joint KPI can be with low data rate, large PDB, basic PER and large number of UEs/cell. We can also define the joint KPI for HRLLC. But from our perspective, the HRLLC can be supported for 6G Day1 by reusing 5G URLLC standards.

Proposal 1: The 9 enhanced capability items defined on IMT-2020 (5G) are reused for IMT-2030 (6G). Consider joint KPI combining data rate, PDB (Packet Delay Budget), PER (Packet Error Rate) and capacity (number of UEs/cell).

- Adopt XR evaluation methodology in TR 38.838 as reference.

- At least used for Immersive and Massive communication application scenarios.

KPI values for the communication TPRs

It should be noted that these KPIs are defined to evaluate the 6G capability to provide traditional communication services, e.g., eMBB, industrial IoT, low-end IoT. The question is that to what extent the KPIs need to be improved over IMT-2020 values. Considering that 6G targets on a more successful technology on market, the performance improvements should be helpful for the success of the services. In this sense, we can categorize the KPIs into three groups:

- Group 1: The KPIs are not the key to make the 6G communication services successful.

- Group 2: The KPIs are the key to make the 6G communication services successful, but have been met by 5G system. No substantial enhancement is needed.

- Group 3: The KPIs are the key to make the 6G communication services successful, and have not well met by 5G system. Enhancement is needed.

From our perspective, the Group 1 include Peak data rate and Peak spectral efficiency. The two KPIs are only theoretical, and are not directly related to service provision capability, even for Immersive/eMBB services. In IMT-2020 (5G) TPR, DL Peak data rate = 20Gbps, UL Peak data rate = 10Gbps. The data rate is not actually needed by any 5G service. It will not be needed by any 6G service either, including XR or AIaaS application. It may be required by holographic display. But we do not see the need to provide holographic display services in mobile communications system in near future. Another consideration is that IMT-2030 may not have new spectrum with much wider bandwidth than IMT-2020. Only 800MHz spectrum are newly provided to IMT services below 7GHz in WRC-23. There is no need to significantly expand the bandwidth. Hence Peak data rate should not be the focus for 6G improvement. We see no need to increase DL/UL Peak data rate to 50~100/25~50Gbps.

We have similar observation on Peak spectral efficiency. In IMT-2020 (5G) TPR, DL Peak spectral efficiency = 30bit/s/Hz, UL Peak spectral efficiency = 15bit/s/Hz, which has been a very high value. We see no need to further increase them.

Group 2 may include Control plane latency, Reliability and Connection density. Latency and reliability are the essential performance requirements for URLLC/IIoT applications. Connection density is the essential performance requirement for massive IoT. The performance has been supported by 5G/5G-A system, e.g., 1ms air interface latency, 10-5~10-6 reliability and 106 devices/km2 connection density. Compared to the actual commercial deployment of URLLC/IIoT and massive IoT, the 5G system performance is far beyond the market needs. No actual justification that 1ms latency and 10-6 reliability cannot support HRLLC/IIoT applications in market. No actual justification that 106 devices/km2 connection density cannot support massive IoT applications in market. We suggest to firstly deploy the system capabilities in market, and consider the further KPI improvements after the first-wave commercial deployment. We suggest to design 6G low-end IoT in 1st release of 6G. But the motivation is to design a low-end IoT which share the single RAT with 6G eMBB/immersive, not to pursue a higher KPI, e.g., connection density. 6G connection density does not need to be improved over 5G.

Group 3 include User experienced data rate, Average spectral efficiency, 5%th percentile user spectral efficiency and Area traffic capacity. The KPIs are related to actual user experience in eMBB/Immersive services. And the values are below the actual market expectation, at least for 5G smartphone market. Among them the most essential performance is the 5th percentile user spectral efficiency. In 5G system, the difference between user experience in cell center and at cell edge became larger. Smartphone users feel that 5G coverage degrades over 4G coverage. However, the feasibility of applicable coverage enhancement techniques needs to be considered. 2 performance over 5G can be considered for 5%th percentile user spectral efficiency. For Average spectral efficiency, we think the performance should be compared between 5G and 6G assuming using the same frequency band. In this sense, the performance gain should be practically estimated considering the improvement of MIMO and AI for air interface techniques. We suggest to consider 1.5~2 performance over 5G for Average spectral efficiency. Considering 6G new spectrum is limited, we should not expect an obvious wider bandwidth to improve User experienced data rate and Area traffic capacity. So, the gain mainly relies on the improvement of spectral efficiency. We suggest to consider 1.5~2 performance over 5G for them.

Proposal 2: For the KPI items already defined on IMT-2020 (5G), consider necessary KPI improvements for IMT-2030 (6G), which really help the success of the services in market.

- No need to further increase Peak data rate and Peak spectral efficiency.

- Not pursue immediate KPI improvements on Latency, Reliability and Connection density on 6G Day1. Wait for market feedback on 5G system, and justify need of further enhancements taking lessons from market.

- Consider 5%th percentile user spectral efficiency 2 over 5G.

- Consider Average spectral efficiency 1.5 ~ 2 over 5G.

- Consider User experienced data rate and Area traffic capacity 1.5 ~ 2 over 5G.

KPIs for AI and communication

AI and communication is the most important usage scenario for 6G. Mobile AI (AIaaS) is likely to be the first 6G billion-user-level market. So, it is essential that the 6G system should be designed to support AIaaS on NW as well as UE side.

Firstly, no need to define dedicate KPIs on AI for 6G. AI for 6G is used to enhance performance of other usage scenarios, e.g., Immersive communication (eMBB), Integrated Sensing and Communication. AI for 6G is not dedicate service.

Regarding 6G for AI, at least the functionalities should be supported for AI/ML LCM, e.g., AI/ML model activation/selection, inference, training, performance monitoring, and data collection. The question is that whether numerical KPIs should be defined for AI and communication scenario. Defining dedicate KPIs, e.g., AI/ML inference accuracy, AI/ML inference latency, AI/ML training accuracy, AI/ML training latency, is very helpful for promoting 6G AIaaS market deployment. The challenge is that the required accuracy/latency depends on the different AI/ML use cases. One way is to select several typical AI/ML use cases for reference, e.g., image recognition, voice recognition. Since 3GPP standards may not define AI/ML models, some well-known AI/ML models can be used for reference to define the KPI values. The 6G technology and system design (e.g. optimized transmission for AI/ML traffic) can be evaluated to meet the required AI/ML inference accuracy, AI/ML inference latency, AI/ML training accuracy and AI/ML training latency.

Proposal 3: For 6G AI and communication,

- No need to define dedicate KPIs for “AI for 6G”.

- Consider dedicate KPIs for “6G for AI”, e.g., AI/ML inference accuracy, AI/ML inference latency, AI/ML training accuracy, AI/ML training latency.

  - Some reference use case (e.g., image recognition, voice recognition) and reference model can be defined to define KPI values.

KPIs for Integrated Sensing and Communication

Integrated Sensing and Communication is another area requiring new KPI system. One discussion point is which level KPIs are defined for 6G sensing. Generally, three options can be considered.

Option 1: KPIs for basic physical parameters, such as distance, velocity and angle. Similar as defined in TR22.837, accuracy and resolution of distance/velocity/angle estimation and related general requirement, Confidence level, latency, refreshing rate, missed detection under constant false alarm. Option 1 could cover some fundamental use cases but it is still restricted within these fundamental use cases. Some popular use cases recent years cannot be covered.

Option 2: KPIs for both basic and advanced physical parameters, such as accuracy of model parameters. Comparing with option 1, option 2 extends to some advanced parameters to cover some popular use cases recent years, such as motion monitoring. For motion monitoring, accuracy of model parameter, i.e. repetition rate can be defined as KPI.

Option 3: KPIs for service parameters, such as probability of rainfall. Comparing with option 1 and option 2, option 3 means that BS/UE will not only perform signal processing in physical layer but also provide service-oriented data analysis. For each service, service-oriented data analysis is very professional, which is not our advantage currently but it gives us opportunity to extend our business.

Comparing with option 1/2/3, Option 2 is a good tradeoff between extensiveness and complexity. So, option 2 can be a starting point at current stage. As sensing function and application develops, option 3 can be considered in later stage.

Proposal 4: For 6G Integrated Sensing and Communication, KPIs for both basic and advanced physical parameters can be defined.

KPIs for Energy saving

Energy efficiency was in principle defined in IMT-2020 (5G) TPR under capability of “sustainability”, without defining numerical KPIs values. It can be considered to define the KPIs for IMT-2030 (6G) TPR.

One way is to define the comparative KPI from 5G energy consumption. This approach has been employed in 5G enhancement for UE power saving and NW energy saving from Rel-16 to Rel-19. We think this approach can be used as a baseline, i.e., how many percent energy 6G NW and UE can save over 5G NW and UE.

A further way is to define the energy efficiency, e.g., consumed energy per transmitted bit (in Joule/bit). However, it needs to be further studied what service/traffic model is used for the definition. In principle, a high-rate data service has a higher energy efficiency than low-rate service, although the energy consumption of high-rate service is much higher.

Proposal 5: For 6G Energy saving, use power/energy saving percentage over 5G as the baseline KPI.

- Study the feasibility to define energy efficiency formulation.

Discussions on RAN requirements for 6G

Observation on 4G/5G operations

4G LTE unlocked the era of mobile Internet, and 5G NR targeted wider market segments with more flexible options.  In the dawn of 6G, some observations are made on 4G and 5G against the new service requirements of IMT-2030 and market trends:

- Majority of 4G/5G services are valued against quantity of data delivered, however there is an upper limit for both data consumption and revenue per capita. As a sign, usage saturation has been observed in some markets.

- Seamlessly covered basic services (voice, low-speed data…) are still essential, while service-specific capabilities vary with deployment scenarios

- Large-area full-coverage broadband networks can be costly because of big chunks of spectrum needed in mid-to-high frequency ranges and subsequent denser BS deployment required. And the RAN occupies the major part in the cost breakdown.

- Vertical markets needs application-specific optimization for better cost and efficiency

To summarize, 4G and 5G are focused on delivery of information and have not extended to the processing of information. Service-awareness and service-oriented optimization for the RAN may be new directions for the next generation of mobile information system to evolve.

Capabilities required by 6G RAN

Targeted as a new generation of mobile communication and possibly also an information system, 6G may have certain goals to satisfy the evolving social trends and needs of end users. A few examples are listed:

Goal 1: to provide substantial value to users and industry by fostering new value-adding services by:

- creating new distinguished intelligent services utilizing new capabilities like AI, sensing, etc.

- to facilitate service participants the access to new capabilities through service interfaces and APIs

Goal 2: to reduce total cost of ownership (TCO) of both network and UE by adopting high-efficient, sustainable and extensible technologies to:

- substantially reduce the cost and complexity of migration from earlier generations to with minimum number of options

- adaptively support service requirements with minimized cost & complexity of both network and UE

Goal 3: to support IMT-2030 and advanced user scenarios in a future-proof manner

- essential scenarios are addressed with Day One features, while other features/functionalities may be added in later phases according to market needs, and without introducing complexities due to backward/forward compatibility

- requirements like ubiquitous connectivity are met with technologies like spectrum sharing and relay-like to balance CAPEX/OPEX

Support different services with high flexibility and low complexity

6G is expected to support a large variety of services ranging from LPWA to the extremely high data-rate and low-latency mobile broadband required by e.g. virtual reality applications. Thus, a system framework supporting high flexibility while at the same time minimizing incurred complexity will be the key for a multi-purpose 6G.

Different services may have diverse requirements in e.g. traffic patterns, propagation conditions and coverage goals. Additionally, life cycles of different services may also mismatch greatly, for example some verticals ask for 20 years of service period, which is longer than the time span of a single generation. Thus a “one for all” system design accommodating all services will introduce unnecessary interdependencies and make the system design very complex. And this problem will become more outstanding with time when more demands come and more features are introduced in the specifications in the later releases. A prohibitively complex set of specifications, where only a small portion of which are actually implemented here and there, will greatly increase the cost of the whole industry.

In sum, the ability to respond to diverse service requirements with “just enough” resources and minimum interdependency between service “tracks” may be the key for a cost effective and profitable 6G system.

High energy efficiency for both network and UEs

Energy cost has been a very substantial part of the network OPEX. Battery life of smart phones is also essential to end user experience. However, during 5G timeframe, the energy saving mechanisms of the network and UE e.g. NES and LPWUS were not specified in the first release but are standardized independently in later releases, which leads to the lack of coordination between the network and UE and difficulty to achieve jointly optimal results. Late introduction of these features also makes network deployments and UE implementation difficult and fragmented.

6G should develop a holistic, end-to-end energy saving framework from Day One, which means the energy efficiency for both the network and the UE are jointly optimized without sacrificing either side.

Fundamental support for new intelligent services

New intelligent services are essential to increase the revenue of the industry. One example of new intelligent services is a broad sense mobile Metaverse, where interactions between cyber world and physical world happen on different levels. Another example is mobile AI applications, where AI helps to improve communication performance and enable “endogenous” intelligent services of 6G. Low-altitude economy and self-driving cars are also hot topics.

For these new services, higher peak data rate and spectrum efficiency may not always be the deciding factors. New capabilities like compute, sensing and data life cycle management (Data LCM) may be more relevant. The capability to provide rightful capabilities like data and compute at the right place and time could be essential for user experience, which gives an edge to a “modular” system architecture where the acquisition, transportation, storage, processing and consumption of data are somehow “decoupled” from specification aspects and thus could be independently optimized.

Smooth migration from earlier generations

Well-deployed 4G and 5G infrastructures and the prevalence of 4G/5G capable smart devices have laid a good foundation for current mobile Internet services. And the smooth and simple migration from these earlier generations will be a key to ensure the quality of existing service while new services are being introduced.

Technologies like advanced spectrum sharing are anticipated to play an important role, and the minimization of migration path options will be critical for the large-scale deployment of 6G.

Potential RAN functionality grouping to fulfil the requirements

As described in the previous Section, 6G should not only enhance peak performance, but also optimize aspects like the support of new services and energy efficiency. A flexible and extensible “modular” system design is depicted as an example to support these requirements, which is especially beneficial when the requirements are “on-demand” and have different life cycles and limited interdependencies.

As depicted, 6G RAN functions may be divided into Functionality Sets, where a basic functionality set is common and fundamental to all services, and other sets could be seen as “plug-in” or “add-on” modules to support more advanced services like high-performance eMBB and various verticals. In this way, the balance between demand and cost will be able to achieve.

Figure 1.  6G“modular” system design

Proposal 6: 6G should provide substantial value to users and industry by fostering new value-adding, intelligent services.

Proposal 7: 6G should reduce total cost of ownership (TCO) of both network and UE.

Proposal 8: 6G should be able to support different services with high flexibility and low complexity.

Proposal 9: 6G should achieve holistic, end-to-end high energy efficiency.

Proposal 10: 6G should ensure smooth migration from earlier generations, as well as minimize the cost and complexity.

Conclusion

6G should be able to support new services with high efficiency and low complexity from Day 1, and by introducing a modular and extensible system architecture, 6G will be able to introduce new capabilities in the later phases to respond to emerging market needs, while at the same time by avoiding unnecessary interdependencies, does not interfere with existing services. Cost-effectiveness could also be achieved since only the necessary technologies will be applied according to emerging service demands.

Our proposals for 6G RAN requirements are listed below:

Proposal 1: The 9 enhanced capability items defined on IMT-2020 (5G) are reused for IMT-2030 (6G). Consider joint KPI combining data rate, PDB (Packet Delay Budget), PER (Packet Error Rate) and capacity (number of UEs/cell).

- Adopt XR evaluation methodology in TR 38.838 as reference.

- At least used for Immersive and Massive communication application scenarios.

Proposal 2: For the KPI items already defined on IMT-2020 (5G), consider necessary KPI improvements for IMT-2030 (6G), which really help the success of the services in market.

- No need to further increase Peak data rate and Peak spectral efficiency.

- Not pursue immediate KPI improvements on Latency, Reliability and Connection density on 6G Day1. Wait for market feedback on 5G system, and justify need of further enhancements taking lessons from market.

- Consider 5%th percentile user spectral efficiency 2 over 5G.

- Consider Average spectral efficiency 1.5 ~ 2 over 5G.

- Consider User experienced data rate and Area traffic capacity 1.5 ~ 2 over 5G.

Proposal 3: For 6G AI and communication,

- No need to define dedicate KPIs for “AI for 6G”.

- Consider dedicate KPIs for “6G for AI”, e.g., AI/ML inference accuracy, AI/ML inference latency, AI/ML training accuracy, AI/ML training latency.

  - Some reference use case (e.g., image recognition, voice recognition) and reference model can be defined to define KPI values.

Proposal 4: For 6G Integrated Sensing and Communication, KPIs for both basic and advanced physical parameters can be defined.

Proposal 5: For 6G Energy saving, use power/energy saving percentage over 5G as the baseline KPI.

- Study the feasibility to define energy efficiency formulation.

Proposal 6: 6G should provide substantial value to users and industry by fostering new value-adding, intelligent services.

Proposal 7: 6G should reduce total cost of ownership (TCO) of both network and UE.

Proposal 8: 6G should be able to support different services with high flexibility and low complexity.

Proposal 9: 6G should achieve holistic, end-to-end high energy efficiency.

Proposal 10: 6G should ensure smooth migration from earlier generations, as well as minimize the cost and complexity.

References

- Recommendation ITU-R M.2160-0 (11/2023), Framework and overall objectives of the future development of IMT for 2030 and beyond.

- RP-243327, New SID: Study on 6G scenarios/use cases and requirements, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

- RP-243276, RAN Chair’s Guidance for 6G Study: ITU Focused, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17), V1.0.0 (2021-12)