3GPP TSG-RAN WG4 Meeting #82bis	        R4-1704220

Spokane, WA, US, 3 - 7 April, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 38.803 | CR | 0005 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.803: Relation with the existing specifications (section 7) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-03-20 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During RAN#75, the first version of the RAN4 TR 38.803 for NR SI was agreed. Still, section 7 of that TR (i.e. Relation with the existing specifications) was empty. As inter-relations among RAN4 BS specifiaction will be important to properlyy structure the work in Rel-15, it is proposed to introduce the descriptive text for the foreseen relations among MSR, AAS BS and NR BS specifications. This Cat. B CR introduces the missing text for section 7 of TR 38.803. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduction of the missing text for section 7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Empty section in the internal RAN4 NR TR would exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------------------------- Modified section ----------------------------------

# 7	Relation with the existing specifications

The scope of the existing MSR BS, AAS BS and UE specifications in RAN4 was identified to be as follows:

- MSR BS specification

  - MSR specification in TS 37.104 [x] captures BS RF requirements for Rx and Tx for GSM, UTRA and E-UTRA.

  - MSR BS specification captures BS RF conducted requirements.

  - BS demodulation requirements for MSR BS are captured by referring to single RAT UTRA TDD, single RAT UTRA FDD, and single RAT E-UTRA BS demodulation requirements in TS 25.105, TS 25.104 and TS 36.104, respectively.

  - MSR BS specification is consider as non-AAS BS specification (same as single RAT UTRA BS and single RAT E-UTRA BS specifications).

- AAS BS specification

  - AAS BS specification in TS 37.105 captures single RAT UTRA, single RAT E-UTRA and MSR operation.

  - AAS BS specification captures BS RF requirements for Hybrid AAS BS and for OTA AAS BS.

  - AAS BS specification covers single RAT UTRA, single RAT E-UTRA and MSR operation.

  - The Rel-13/14 version of the AAS BS specification comprises a single set of Hybrid requirements, composed of conducted requirements as well as OTA requirements on EIRP and EIS.

  - Rel-15 version of the AAS BS specification will be extended with at least the full set of OTA requirements and possibly a further set of hybrid requirements.

  - BS demodulation requirements for AAS BS are captured by referring to single RAT UTRA TDD, single RAT UTRA FDD, and single RAT E-UTRA BS demodulation requirements in TS 25.105, TS 25.104 and TS 36.104, respectively.

- EMC requirements

  - Conducted EMC requirements for the MSR BS in TS 37.113 cover emissions and immunity requirements.

  - Conducted EMC requirements for the AAS BS in TS 37.114 are captured by referring to the EMC specifications for single RAT UTRA, single RAT E-UTRA and MSR BS in TS 25.113, 36.113 and 37.113, respectively.

- UE specifications are capturing the following:

  - RF and demodulation requirements, single RAT UTRA and E-UTRA

  - RRM requirements, UTRA and E-UTRA

The following is identified as needed to be captured in the specification structure around NR:

- NSA and SA NR, Range 1

  - Conducted and OTA requirements to be defined

  - Single RAT NR BS requirements may be needed; the need for these is FFS

  - MSR NR BS requirements will be needed, aligned to the existing MSR requirements

  - Hybrid and OTA NR BS requirements will be needed, aligned to the existing AAS requirements

  - It is FFS which (if any) BS requirements need to differ between SA and NSA

  - UE conducted RF, RRM and demodulation requirements

- NSA and SA NR, Range 2

  - OTA requirements to be defined

  - Single RAT BS RF requirements, OTA

  - UE RF requirements, OTA

  - BS and UE demodulation requirements, scope FFS

  - RRM requirements, OTA

---------------------------------- End of modified section ----------------------------------