3GPP TSG RAN Plenary Meeting RAN#78                                               	RP-172392

Lisbon, Portugal, December 18 - 21, 2017

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title:  	NTN NR impacts on the HARQ Operation

Document for: 	Discussion

Agenda Item: 	9.3.1  (RAN SI: FS_NR_nonterr_nw)

Release: 	Rel-15

Specification: 	38.811

# 1 Abstract

The objective of this document is to propose a text for the 7th clause “Potential key impact areas on NR to support NTN” of the TR 38.811 “Study on NR to support Non-Terrestrial Networks”. The main issue of the text proposal is to identify the impact on NR HARQ process due to the induced propagation delay of the satellite channel. Since every satellite constellation, e.g., GEO MEO, or LEO, have different round-trip time (RTT, e.g., the time needed to connect a ground gateway to a terminal via a bent-pipe satellite), the impacts of each of these constellations on the HARQ operation needs to be identified separately.

# 2 Discussion

This clause identifies the specific constraints associated with Non-Terrestrial Network support in NR, where the NR radio interface will need to cope with regard to HARQ process. Different impact needs to be identified for different satellite scenarios, i.e., for GEO and non-GEO satellites, and different technology solutions needs to be identified and agreed upon by the companies. Since the extended RTT is major key performance indicator in satellite, the text proposes a support of a flexible (mainly extendable) number of HARQ process handle, transmission bundling, slot aggregation, asynchronous HARQ feedback, memory/latency efficient adaptive HARQ, K-repetition in DL/UL, and (finally) distributed HARQ process utilizing dual-connectivity (DC) for terrestrial-in-coverage scenarios. These solutions are mapped to the consensus approved in NR work-item.

# Proposed text for discussion

It is proposed to add the following texts to TR 38.811 “Study on NR to support Non-Terrestrial Networks”, v0.2.0.

* * * Start of changes * * * * (new text)

## 2	References

[1]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies (Release 14)".

[2]	3GPP TS 36.101: "Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 14)".

[3]	3GPP TR 38.801: “Technical Specification Group Radio Access Network; Study on new radio access technology: Radio access architecture and interfaces (Release 14)”

[4]	3GPP TR 38.804: "Technical Specification Group Radio Access Network; Study on New Radio Access Technology; Radio Interface Protocol Aspects (Release 14) ".

[6]	3GPP TS 38.211, “Technical Specification Group Radio Access Network; NR; Physical channels and modulation (Release 15)”, v1.0.0 (2017-09)

[7]	3GPP TS 38.214, “Technical Specification Group Radio Access Network; NR; Physical layer procedures for data (Release 15)”, v1.0.0 (2017-09)

[8]	3GPP TS 38.321, “Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification (Release 15)”, v1.0.0 (2017-09)

[9]	3GPP TSG RAN WG1 Meeting #91, agreements in AI 7.3

[10]	3GPP TS 38.321, “Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification (Release 15)”, v1.0.0 (2017-09)

[11]	3GPP TS 38.331, “Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC); Protocol specification (Release 15)”, v1.0.0 (2017-09)

[12]	3GPP TSG RAN WG2 Meeting #100, agreements in AI 10.3.1

[13]	O. Kodheli, A. Guidotti, and A. Vanelli-Coralli. “Integration of Satellites in 5G through LEO Constellations,” CoRR abs/1706.06013 (2017): pp. 1-6

[14]	R. Ali Ahmad, J. Lacan, F. Arnal, M. Gineste and L. Clarac, “Enhancing satellite system throughput using adaptive HARQ for delay tolerant services in mobile communications,” 2015 Wireless Telecommunications Symposium (WTS), New York, NY, 2015, pp. 1-7.

[15]	Tae Chul Hong, Kunseok Kang, Bon-Jun Ku, Dae-Ig Chang, “Receiver memory management method for HARQ in LTE-based satellite communication system,” International Journal of Satellite Communications and Networking, 2017, 35, 1, 3

[16]	3GPP TR 38.802 – “Study on New Radio (NR) Access Technology; Physical Layer Aspects (Release 14)” (2017-09)

* * * End of Changes * * * *

* * * Start of changes * * * * (new text)

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACK:	Acknowledgement

ARQ:	Automatic repeat request

AWGN:	Additive White Gaussian Noise

BLOS:	Beyond Line of Sight

BS:	Base Station

BW:	Bandwidth

C2:	Control and Command

CBG:	Code block group

eMBB:	enhanced Mobile Broadband

FOTA:	Firmware Over The Air services

FSS:	Fixed Satellite Services

GEO:	Geostationary Earth Orbiting

gNB:	next Generation Node B

GW:	Gateway

HARQ:	Hybrid ARQ

HD:	High Definition

HEO:	Highly Elliptical Orbiting

IoT:	Internet of Things

KPI:	Key Performance Indicator

IMUX:	Input MUltipleXer

ISL:	Inter-Satellite Links

LEO:	Low Earth Orbiting

LMS:	Land Mobile Satellite

LoS:	Line of Sight

Mbps:	Mega bit per second

MEO:	Medium Earth Orbiting

mMTC:	massive Machine Type Communications

MS:	Mobile Services

MSS:	Mobile Satellite Services

NACK:	Negative Acknowledgement

NGSO:	Non Geostationary Satellite Orbit

NT:	Non Terrestrial

NTN: 	Non Terrestrial Network

OMUX:	Output MUltipleXer

POI:	Point of Interest

RAN:	Radio Access Network

RAT:	Radio Access Technology

RB:	Resource Block

RV:	Redundancy Version

RTT:	Round Trip Time

SOTA:	Software Over The Air services

SPS:	Semi-persistent Scheduling

TB:	transport block

TV:	Television

UAS:	Unmanned Aerial System

UE:	User Equipment

URLCC:	Ultra-Reliable Low latency Communications

* * * End of Changes * * * *

* * * Start of changes * * * * (new text)

# 7 Potential key impact areas on NR to support NTN

7.<X> Constraints associated to NTN for HARQ adaptation

The HARQ operation is a time-critical mechanism. This means that if HARQ-acknowledgment (ACK or NACK) is misaligned, this leads to false decision [12]. HARQ operation becomes more critical when the round-trip-time (RTT) is becoming extremely long, i.e., as in the case of NTN and the extreme coverage scenarios.

In Satellite communication, the RTT normally exceeds the agreed upon maximum conventional known timers or the maximum possible number of HARQ processes [12]. This is also true even for low-earth orbit (LEO) constellations, where the RTT varies between 15 to 63 times longer than the terrestrial situation [13]. Moreover, simply extending the HARQ process number linearly to the amount of delay induced by the satellite channel might not be feasible on some UEs due to memory restrictions and the maximum possible parallelism [14, 15]. Therefore, different solutions need to be agreed up on for NR HARQ operation to support NTN. Figure 1 depicts the HARQ feedback RTT, i.e., from acknowledge instance to retransmission time.

Figure 1 : Bent-pipe Satellite HARQ feedback operation with a maximum RTT (from sending a NACK until receiving a retransmission RV)

7.<X>.1 Impact on the HARQ process with long RTT

Assuming LTE as a base-line, the system efficiency is improved using multiple parallel HARQ processes, e.g., this is 8 up to Rel. 14 and extended to 16 in Rel. 15 to accommodate eMBB and HRLLC multiplexing [9]. For NR NTN satellite transmission, the number of HARQ processes has to be extended flexibly according to the induced RTT delay. Thus, the minimum required number of HARQ process can be computed directly from the RTT delay of each satellite constellation, e.g., LEO, MEO and GEO, using the following formula [13]

NHARQ, min≥THARQTslot                                                          ( 1 )

where NHARQ, min is the minimum number of HARQ process, Tslot is 1ms for the 15kHz* subcarrier spacing numerology, and THARQ is the time duration between the initial transmission of one transport block (TB) and the corresponding ACK/NACK complete decoding. The latter (THARQ) is depicted carefully in Figure 2, where the total RTT for a single HARQ process, i.e., THARQ, is illustrated considering processing timer Tproc1 and Tproc2 for decoding the TB and the ACK/NACK frame, respectively. In Figure 2, the propagation time, Tp, is computed as in Figure 1.

Figure 2: Timing diagram of a single HARQ process for a NTN with a single bent-pipe satellite in the link (similar to [13]).

The following table gives an overview of the number of the HARQ processes, NHARQ, min, based on the different satellite constellations, LEO, MEO, and GEO and their RTT delays.

Table 1: Possible required number HARQ processes with a normal 1ms slot operation for the reference numerology 15 kHz* and referring to Eqn. (1)

| constellation | RTT | Possible # of HARQ Process for 1 ms slot operation | UE side feasibility | Possible # of HARQ Process for 10 x 1 ms slots-aggregation | UE side feasibility |
| --- | --- | --- | --- | --- | --- |
| Terrestrial | 8ms | 8 | Feasible | -- | -- |
| LEO | 50ms | 50 | Possible | >= 5 | Feasible |
| MEO | 180ms | 180! | Unfeasible! | 18 | Feasible |
| GEO/HEO | 500ms | 500! | Unfeasible! | 50 | Possible |

Note*: For higher subcarrier spacing (SCS) numerology 2k * 15 kHz, the possible number of the HARQ process might be scaled by 2k. This will lead to much higher numbers than the reference numerology, which requires further studies on the optimum number of HARQ processes.

In order to handle such a high number of parallel HARQ process, different solutions have been proposed in literature to enhance processing and memory feasibility [13-15]. Some proposals are discussed in the next subsection. FFS RLC ARQ mechanisms as well.

7.<X>.2 Suggested solutions supporting HARQ with long RTT

In LTE, both asynchronous and synchronous HARQ are supported. Synchronous HARQ is easier since it does not require encoding the HARQ process ID, e.g., in the DCI. However, asynchronous HARQ has better flexibility and lower latency for selected transmissions. Also in LTE, adaptive HARQ is used; however, adaptation is simply done by requesting a specific HARQ redundancy version (RV) in each retransmission. Additionally, TTI bundling is used as well, which allows transmission of an initial redundancy version (i.e., RV0) in a subframe and the remaining RVs in consecutive frames, i.e., without waiting for the HARQ retransmission.

In NR, asynchronous and adaptive DL HARQ is supported at least for eMBB and URLLC. From UE perspective, HARQ ACK/NACK feedback for multiple DL transmissions in time can be transmitted in one UL data/control region. Similarly, for NR, the TTI bundling in DL and the K-repetition in UL are supported as well. These can be (among other techniques) the key solution to reduce the complexity HARQ operation in NTN. The following section describes in details some of the solutions fitting NTN:

- Flexible number of HARQ process for NTN and long-range communication

It is agreed in NR to supports flexible number of HARQ process. This will lead to efficient solutions for scenarios with long RTT such as in NTN, i.e., satellite with GEO, MEO, and LEO constellations. Similar solutions can be used also for extreme coverage scenarios. Table 1 suggests multiple values for the minimum number of the parallel HARQ processes. Therefore, it is necessarily to further study the optimal extension of the NHARQ, min and study whether Eqn. (1)  is still optimum or not. Moreover, it is important to further study the number of HARQ process for the higher SCS numerology, i.e., 2k * 15 kHz, k=1, 2, 3, ….

For the extended number of HARQ processes, asynchronous transmission can reduce the latency requesting only the necessarily retransmissions and acknowledgements [12]. However, it is for further study how to simplify the control channel feedback with such an enlarged number of HARQ processes to be easily enumerated.

- Memory efficiency for the NTN extended number of HARQ process

To keep the quality-of-service of NR maintained, the previous multiple NHARQ, min processes need to be stored and handled in the NTN terminal memory. This might lead to extensive use of the device memory. Multiple schemes have been introduced to enhance the terminal memory usage to handle the difficult HARQ operation in NTN, e.g., a memory split has been introduced in [15]. This has claimed to meet the system throughput; however, it requires the terminal to feedback their memory status to the transmitter. Thus, it might be useful to further study such a proposal or similar ones to better manage the NTN terminals memory for accommodating large NHARQ, min.

- Slot aggregation for reduced Number of HARQ Processes

In TR 38.802 [16], slot aggregation is supported, where data transmission can be scheduled to span one or multiple slots. For each of this long aggregation, one HARQ feedback is sufficient to be transmitted for all aggregated transport blocks. This can be handled as a code block group (CBG) or in other ways [9]. This can reduce the number of the HARQ processes significantly; however, without much enhancing the used receivers’ memory. To further enhance the performance, the aggregation of slots can be combined with K-repetitions in frequency, i.e., to reduce the long repetition time of the whole aggregated slots.

- Transmission bundling and K-repetition in DL/UL

As stated before, TTI or slot bundling (similar to LTE) might be a solution for reducing the retransmission probability and; therefore, a solution for the large number of required HARQ processes. In NR release 15 –December freeze – it was agreed upon a similar mechanism for UL, the K-repetition [9, 12]. Those K repetitions can be similar to the initial transmission or following certain RV sequence [9]. Inhere, bundling and repetition are supported in time domain only, causing additional delay over NTN. However, in order to further enhance the performance of NTN transmission and avoid long retransmission, it is important to consider for further study the repetition in both time and frequency domain.

- Fine adaptive HARQ operation

In NR or LTE, in adaptive HARQ process, the transmitter is able to request a certain redundancy version number, i.e., RV1, RV2, or RV3. Currently, the required RV number is contained in DCI0 for adaptive DL HARQ operation. However, this might not be enough to avoid further retransmissions or achieving the targeted probability. Hence, some proposals in literature suggest a finer adaptive HARQ when the terminal is able to estimate the exact number of bits needed to decode the failed code word. This requires a feedback of the estimated number of bits to the transmitter [14]. Although this mechanism guarantees minimum retransmission trials; however, it is important to study and evaluate the amount of feedback (indicating the estimated number of bits) needs to be transmitted in the ACK/NACK message and the suitability to specify limited RVs.

- HARQ utilizing terrestrial dual-connectivity

Herewith, the HARQ process delays (including the system and frame delivery latency shown in Figure 2) can be extremely reduced utilizing a dual-connectivity (DC)/multi-connectivity (MC) with a ground station (gNB or Relay gNB). This ground station might be connected to the NR core network via cables or backhauled directly using the satellite’s more reliable link, see Figure 3 for more details. Once the UE is also in the coverage of the terrestrial node, the HARQ retransmission and feedback of the initial transmission (RV0, sent via the bent-pipe satellite to the UE) can simply flow from the terrestrial links (i.e., rather than the longer delay NTN links). In case of buffering to a terrestrial node from satellite, the RVs can be directly buffered at the terrestrial gNB (routed via a bent-pipe satellite) or regenerated simply from the initial transmission, i.e., assuming a correct decoding of an initial transmission utilizing the more reliable satellite-to-ground link connecting the terrestrial gNB. Therefore, it is important to further study the impact on NR HARQ operation when a DC/MC with terrestrial gNB and relay-node gNB exists. This study has to be adapted once the Relay-node gNB is considered and discussed in NR Rel. 16.

Figure 3: The HARQ RVs are transmitted to the gNB via a satellite (or non-satellite) backhauling network. The gNB transmits the RVs to the UE upon HARQ negative acknowledgement NACK

* * * End of Changes * * * *

This document will focus on the first satellite architecture option in TR 38.811; the bent-pipe satellite connection.

Agreed on a timer T after which an ACK is assumed; also, a flexible pool of parallel HARQ process similar to LTE is a base-line