3GPP TSG RAN Meeting #78	RP- 172537

Lisbon, Portugal, 18 – 21 December 2017

Agenda item:	9.3.2

Source:	Huawei, HiSilicon

Title:	Consideration on IMT-2020 self evaluation: Bandwidth and Scalability

Document for:	Information

1	Introduction

At RAN#77 meeting, the preliminary work plan of self evaluation was agreed in [1], where a three-step plan is made. In [2], it is proposed that 3GPP starts Step 2 activity for self evaluation study, which includes the performance evaluation against the eMBB, mMTC and URLLC technical performance requirements as defined in Report ITU-R M.2410 (see [3]).

In this document, we focus on one of the technical performance requirement, i.e., bandwidth and scalability, and provide the consideration on this aspect.

2	Definition and requirements of bandwidth and scalability

In Report ITU-R M.2410, the following definition and requirements of bandwidth and scalability are defined in subsection 4.3.

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2020 evaluation. The requirement for bandwidth is at least 100 MHz. The RIT/SRIT shall support bandwidths up to 1 GHz for operation in higher frequency bands (e.g. above 6 GHz). The RIT/SRIT shall support scalable bandwidth. Scalable bandwidth is the ability of the candidate RIT/SRIT to operate with different bandwidths.

Observation 1: The requirement of bandwidth is at least 100 MHz.

Observation 2: The requirement of scalable bandwidth is measured by the ability of operating with at least two different bandwidths.

3	Evaluation methodology and initial evaluation results

From Report ITU-R M.2412 (see [4]), we can see the following evaluation methodology.

The support of maximum bandwidth required in § 4.13 of Report ITU-R M.2410, is verified by inspection of the proposal. The scalability requirement is verified by demonstrating that the candidate RITs/SRITs can support multiple different bandwidth values. These values shall include the minimum and maximum supported bandwidth values of the candidate RITs/SRITs. The requirements for bandwidth or the bandwidth numbers demonstrated by the proponent do not pose any requirements or limitations for other Technical Performance Requirements that depend on bandwidth. If any other requirement requires a higher bandwidth, the capability to reach that bandwidth should be described as well.

From the description we can see that bandwidth and scalability should be evaluated by inspection. For NR, In [2], the bandwidth capability and scalability of NR are includedare evaluated in the following sub-sections.

3.1	Initial evaluation results for bandwidth

According to Section 5.3.2 in [5], the maximum bandwidth of one component carrier is related to subcarrier spacing (SCS) and frequency range. Besides, in [6], it is mentioned that at least from RAN1 perspective, carrier aggregation of up to sixteen component carriers is supported for NR. Based on these values, the maximum bandwidth for a specific SCS in NR is shown in Table 1.

Table 1 – Bandwidth capability for NR

|  | SCS [kHz] | Maximum bandwidth for one component carrier (MHz) | Maximum number of component carriers for carrier aggregation | Maximum bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| FR1 | 15 | 50 | 16 | 800 |
|  | 30 | 100 | 16 | 1600 |
|  | 60 | 100 | 16 | 1600 |
| FR2 | 60 | 200 | 16 | 3200 |
|  | 120 | 400 | 16 | 6400 |

It is observed that the bandwidth requirement is met under all frequency ranges for all sub-carrier spacing values.

Observation 1: The bandwidth requirement is met under all frequency ranges for all sub-carrier spacing values for NR. FR1 can support larger than 1 GHz bandwidth, while FR2 can support up to 6.4 GHz bandwidth.

3.2	Initial evaluation results for scalability

According to Section 5.3.2 in [5], multiple component carrier bandwidths are supported for a given SCS for NR. This is listed in Table 2.

Table 2 – Multiple bandwidth suport in NR

(a) For FR1

| SCS [kHz] | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 40MHz | 50MHz | 60MHz | 80MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | - | - | - |
| 30 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| 60 | - | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

(b) For FR2

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- |
| 60 | Yes | Yes | Yes | - |
| 120 | Yes | Yes | Yes | Yes |

Based on Table 2, the bandwidth scalability capability of NR is summarized in Table 3.

Table 3 –Bandwidth scalability capability for NR

|  | SCS [kHz] | Minimum component carrier bandwidth (MHz) | Maximum component carrier bandwidth (MHz) | Supported number of different bandwidth |
| --- | --- | --- | --- | --- |
| FR1 | 15 | 5 | 50 | 7 |
|  | 30 | 5 | 100 | 10 |
|  | 60 | 10 | 100 | 9 |
| FR2 | 60 | 50 | 200 | 3 |
|  | 120 | 50 | 400 | 4 |

Observation 2: Bandwidth scalability is met under all frequency ranges for all sub-carrier spacing values for NR. FR1 supports more bandwidth options than FR2.

4	Conclusion

In this document, the evaluation of bandwidth and scalability is considered. The following observations are obtained.

Observation 1: The bandwidth requirement is met under all frequency ranges for all sub-carrier spacing values for NR. FR1 can support larger than 1 GHz bandwidth, while FR2 can support up to 6.4 GHz bandwidth.

Observation 2: Bandwidth scalability is met under all frequency ranges for all sub-carrier spacing values for NR. FR1 supports more bandwidth options than FR2.

Reference

- RP-172101, “WF on Work plan of Self Evaluation SI”, Huawei, Ericsson, Telecom Italia, September 2017.

- RT-172532, “Work plan for self evaluation towards 3GPP Update Submission to ITU-R”, Huawei, Ericsson, Telecom Italia, December 2017.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R4-1714544, TS 38.104 v0.5.0, Ericsson, Nov. 2017.

- 3GPP TR38.802, “Study on New Radio Access Technology Physical Layer Aspects”, 2017.