3GPP TSG-RAN Meeting #83	RP-190425

# Shenzhen, China,  March 18 – 21, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0025 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on QCL assumption for receiving PDSCH for RAR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-3-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current specification describes that the UE may assume that the DM-RS port of the received PDCCH order and the DM-RS ports of the corresponding PDSCH scheduled with RA-RNTI are quasi co-located with the same SS/PBCH block or CSI-RS in case of non-contention based random access procedure triggered by a PDCCH order. However, this behavior is appropriate only when the non-contention based random access procedure is for SpCell where the PDSCH for RAR and the PDCCH order are sent on the same SpCell. Note that TS 38.213 CR in R1-1903749 was agreed to clarify the QCL assumption for receiving PDCCH for RAR correspondingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that UE assumes QCL properties of PDCCH order for receiving PDSCH for RAR only in case that PDCCH order and PDSCH for RAR are sent on the same SpCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When PDCCH order and PRACH are sent on SCell, UE may assume QCL properties of PDCCH order on SCell for receiving PDSCH for RAR on SpCell, which may cause a misdetection of PDSCH for RAR due to inappropriate QCL assumption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact analysis: This CR has an isolated impact to the PDSCH reception for RAR for non-contention based random access triggered by PDCCH order on SCell. - If the NW implements this CR while the UE does not When NW triggers non-contention based random access on SCell, UE may mis-detect the PDSCH of SpCell for RAR due to inappropriate QCL assumption based on PDCCH order on SCell. - If the NW does not implements this CR while the UE does The NW will avoid an operation which requires non-contention based random access on SCell, such as intra-band non-contiguous CA and inter-band CA operations with multiple TAG. |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

5.1	UE procedure for receiving the physical downlink shared channel

<omitted parts>

When receiving PDSCH scheduled with RA-RNTI the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE used for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. When receiving a PDSCH scheduled with RA-RNTI in response to a random access procedure triggered by a PDCCH order which triggers non-contention based random access procedure for the SpCell [10, TS 38.321], the UE may assume that the DM-RS port of the received PDCCH order and the DM-RS ports of the corresponding PDSCH scheduled with RA-RNTI are quasi co-located with the same SS/PBCH block or CSI-RS with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

<omitted parts>

----------------------------------------------------- end of Change ------------------------------------------------------------