3GPP TSG-RAN WG1 Meeting #96	R1-1903554

# Athens, Greece, February 25 – March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0010 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on uplink resource allocation type 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction on wrong description on uplink resource allocation type 1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The uplink type 1 resource allocation field of DCI 0_0 in USS shall indicate an uplink resource block assignment. In current version of specification, the resource allocation field of DCI 0_0 in USS, with DCI size derived from DCI 0_0 in CSS, is mistakenly associated with downlink. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong interpretation of frequency domain allocation of DCI 0_0 in USS will cause transmission failure or configuration limitation on USS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR corrects the mistake of using downlink resource allocation in the uplink resource allocation description. No impact to current implementation due to common understanding in RAN1. |  |  |  |  |  |  |  |  |

##### 6.1.2.2.2	Uplink resource allocation type 1

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in any common search space in which case the size of the initial bandwidth part  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

When the DCI size for DCI format 0_0 in USS is derived from the initial BWP with size  but applied to another active BWP with size of , an uplink type 1 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block and a length in terms of virtually contiguously allocated resource blocks .

The resource indication value is defined by

if  then

else

where, and where shall not exceed .

If , K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1.