3GPP TSG-RAN WG1 Meeting #96	R1-1903563

# Athens, Greece, February 25th - March 1st, 2019

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0019 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on CSI reporting on PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 28-Feb-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is not clear how the code rate for droping CSI-part 2 reports is calculated and if the code rate should excluded the coded modulation symbols punctured by HARQ-ACK or not |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide the equation for the calculating the code rate and clarify that the code rate for CSI part 2 is calculated before puncturing. This ensures that the code rate is the same at the gNB and UE, even with missing DCIs at the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the gNB and UE calculate the CSI part 2 code rate differently depending on whether to include or excluded coded modulation symbols punctured by HARQ-ACK, there could be a potentinal misalignment between the CSI part 2 payload size at the gNB and UE. As a consequence, the gNB will not be able to decode CSI part 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: The impact of this CR is limited to the case when CSI part 2 information bits are transmitted on PUSCH with no UL-SCH, and HARQ-ACK punctures CSI-part 2 coded modulation symbols. Without this CR, or if only one side (e.g. UE or gNB) implements this CR, it is possible that the other side interpreted the specifications differently, and gNB will not be able to receive CSI-part2 reports that are sent on PUSCH without UL-SCH and that are punctured by HARQ-ACK. To avoid ambiguity in determining the number of coded modulation symbols, with potentially missing DCIs at the UE, a reasonably implemented UE and gNB would calculate the code rate before HARQ-ACK pucturing. Therefore, it is RAN1’s understanding that this CR has no implementation impact on the UE and gNB. |  |  |  |  |  |  |  |  |

### 5.2.3	CSI reporting using PUSCH

<omitted parts>

When part 2 CSI is transmitted on PUSCH with no transport block, lower priority bits are omitted until Part 2 CSI code rate, which is given by OCSI-2+LCSI-2/(NL⋅Q'CSI,2⋅Qm) where OCSI-2, LCSI-2, NL,Q'CSI,2,Qm are given in subclause 6.3.2.4 of [5, 38.212] before HARQ-ACK puncturing part 2 CSI if any, is below a threshold code rate lower than one, where

-	is the CSI offset value from Table 9.3-2 of [6, TS 38.213]

-	R is signaled code rate in DCI

<omitted parts>