3GPP TSG-RAN Meeting #85		RP-191949

Newport Beach, USA, September 16-20, 2019

Agenda item:	9.4

Source:	Panasonic

Title:	Scope adjustment of Release 16 work items

Document for:	Discussion

Introduction

This document shows our view on the scope adjustment of Release 16 work items for timely finalization.

Discussion

Our view is following on each of work items.

Two step RACH for NR

- To limit same configuration periodicity for Msg A PRACH and PUSCH

- To limit only up to two Msg.A PUSCH configuration for RRC_INACTIVE/IDLE state

- Not to support DFT-s-OFDM when multiple DMRS sequence combination

NR-based Access to Unlicensed Spectrum

- Not to support more than 2 PDSCH groups for HARQ ACK retransmission for dynamic codebook

5G V2X with NR sidelink

- Only one Tx resource pool per UE

- Not to support Mode 2 resource allocation of HARQ feedback option 2 for groupcast: Receiver UE transmits HARQ ACK/NACK

Physical Layer Enhancements for NR URLLC

- Enhanced inter UE TX prioritization/multiplexing

- Not to support UE specific DCI for UE TX prioritization

- UCI enhancement

- Not to support Semi-static (Type I) HARQ-ACK codebook for sub-slot-based HARQ-ACK feedback procedure

- Only up to 2 level priority at PHY

- Configured grant

- Other solutions to reduce the transmission alignment delay and ensure the reliability (to limit only multiple configured grant configuration) is not supported.

Enhancements on MIMO for NR

- During Multi-TRP/Panel transmission, how common channel related behaviour like SIBs and random access are carried out has not discussed. In order to finalize multi-TRP properly, these aspects should be discussed.

Power saving

- Not to support UE specific DCI outside of Active time

- Not to support group common DCI inside of Active time

- No enhancement on RS enhancement

Conclusions

This document shows our view on the scope adjustment of Release 16 work items. We'd like to discuss further in the meeting for the final adjustment.