3GPP TSG RAN Meeting #88-e	RP-201316

E-meeting, June 29 - July 3, 2020                                         	(revision of RP-201038)

- Source:	Huawei, HiSilicon

- Title:	WID revision: NR Multicast and Broadcast Services

- Document for:	Approval

Agenda Item:	9.10.8

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Revised Work Item on NR Multicast and Broadcast Services

## Acronym: NR_MBS

## Unique identifier: 	 860048

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830030 | Architectural enhancements for 5G Multicast-Broadcast Services | The outcome of this SI needs to be taken into account |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

## 3	Justification

A discussion on 5G Broadcast evolution in RAN took place between RAN #78 and RAN #80, summarizing the technical attributes of “terrestrial broadcast” and “mixed mode multicast”, leading to a recommendation to proceed with a study on “terrestrial broadcast” in Rel-16, while leaving the standardization of “mixed mode” multicast/broadcast to further releases. The LTE Rel-16 WI on enhanced EN-TV was approved in RAN #83, targeting the introduction of new frame structures with new CPs and the related designs. The main attributes of “terrestrial broadcast” are broadcast only, DL-only, large and static transmission areas typically achieved with High-Power High-Tower deployments.

No broadcast/multicast feature support is specified in the first two NR releases, i.e. Rel-15 and Rel-16. Nevertheless, there are important use cases for which broadcast/multicast could provide substantial improvements, especially in regards to system efficiency and user experience.

A study item on the Architectural enhancements for 5G multicast-broadcast services has been approved in SP-190625 and it is ongoing.

Objective A of the SA2 SI is about Enabling general MBS services over 5GS and the uses cases identified that could benefit from this feature include (but are not limited to) public safety and mission critical,. V2X applications, transparent IPv4/IPv6 multicast delivery, IPTV, software delivery over wireless, group communications and IoT applications.

This WI aims to provide the support in RAN for Objective A and objective B, consistently with TR 23.757.

In particular, for public safety and mission critical, we should take into account as much as possible design goals identified during the SA6 Study on enhanced Mission Critical (MC) services over 5G multicast-broadcast system (SP-190726) as captured in TR 23.774 and requirements identified by SA1 in TS22.261, clause 6.13.2, provided that the RAN system complexity is manageable.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The set of objectives includes:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3]

  - Specify support for basic mobility with service continuity [RAN2, RAN3]

  - Assuming that the necessary coordination function (like functions hosted by MCE, if any) resides in the gNB-CU, specify required changes on the RAN architecture and interfaces, considering the results of the SA2 SI on Broadcast/Multicast (SP-190625) [RAN3]

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

  - Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3]

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]:

  - Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1].

  - Specify RAN basic functions for reception of broadcast, including receive-only mode and free-to-air services, as defined by SA2.

- Note: the possibility of receiving multicast Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Broadcast/Multicast service while in RRC CONNECTED state beforehand, is subject to verification of service subscription and authorization assumptions during the WI.

- Specify NG-RAN functions for interworking with existing E-UTRA-based broadcast (NG-RAN Option 5) [RAN3]

Restrictions and assumptions:

Architecture: it is the one in Figure 4.1-1 in TR 23.757 v0.2.0: High level MBS architecture, with the further restriction that only NR in NG-RAN (i.e. connected to 5GC) is considered as RAT. Consequently, in addition to in NR SA, there should be no reasons preventing the use of the feature standardized in this WI in case of MR DC configurations in the MCG when the MN is a gNB (NE-DC, NR DC).

Physical layer: limit the scope of this WI to current Rel-15 numerologies, physical channels (PDCCH/PDSCH) and signals.

FR2: we assume that there are no issues to provide Multicast / Broadcast transmissions in FR2. If any enhancements is needed it should be treated with lower priority compared to the minimum set of objectives above.

In order to facilitate implementation and deployment of the feature, the overall implementation impact should be limited, and the UE complexity should be minimized (e.g. device hardware impact should be avoided).

SFN provides synchronized delivery of user plane packets over the air from different cells. No standardized support specifically for SFN, is provided in this WI. Any SFN operation is transparent to the UE, and any related synchronization is left to network implementation. The existing QCL framework (based on SSB and CSI-RS) is reused.

Flexible resource allocation between Unicast and Broadcast/Multicast services should be possible in this WI, but resource allocation up to 100% to Broadcast/Multicast is not guaranteed requirement in this WI.

Any design decisions taken for this WI in Release 17 shall not prevent introducing the following features in future Releases:

- Standardised support of SFN over multiple cells above gNB-DU level;

- Resource allocation up to 100% to Broadcast/Multicast service.

Note: collaboration with SA2 is expected in due course.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {E.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {E.g. "TSG#87"} | {E.g. "TSG#89"} | {e.g.: rapporteur: <FamilyName>, <GivenName>, <Company>, <email address>} |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.300 | NR; NR and NG-RAN Overall Description | RAN#93 | Core part |
| 38.331 | NR; Radio Resource Control (RRC) Protocol Specification | RAN#93 | Core part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#93 | Core Part |
| 38.322 | NR; Radio Link Control (RLC) protocol specification | RAN#93 | Core Part |
| 38.323 | NR; Packet Data Convergence Protocol (PDCP) specification | RAN#93 | Core Part |
| 38.304 | NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state | RAN#93 | Core Part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#93 | Core Part |
| 38.423 | NG-RAN; Xn Application Protocol (XnAP) | RAN#93 | Core part |
| 38.413 | NG-RAN; NG Application Protocol (NGAP) | RAN#93 | Core part |
| 38.401 | NG-RAN; Architecture description | RAN#93 | Core part |
| 38.473 | NG-RAN; F1 application protocol (F1AP) | RAN#93 | Core part |
| 38.463 | NG RAN; E1 Application Protocol (E1AP) | RAN#93 | Core part |
| 38.211 | NR; Physical channels and modulation | RAN#92 | Core part |
| 38.212 | NR; Multiplexing and channel coding | RAN#92 | Core part |
| 38.213 | NR; Physical layer procedures for control | RAN#92 | Core part |
| 38.214 | NR; Physical layer procedures for data | RAN#92 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Zhenzhen Cao, Huawei, caozhenzhen@huawei.com [RAN2, RAN3]

Fei Wang, CMCC, wangfei@chinamobile.com [RAN1]

## 7	Work item leadership

Primary: RAN2

Secondary: RAN1, RAN3

## 8	Aspects that involve other WGs

Architectural aspects needs to be studied and specified in coordination with SA2

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

Ericsson

Qualcomm

Futurewei

ZTE Corporation

Sanechips

Lenovo

Motorola Mobility

Sharp

CATT

Nokia

Nokia Shanghai Bell

KT Corp.

ITRI

CBN

EBU

BBC

IRT

Blackberry

BT

Enensys

Telenor

CMCC

China Telecom

China Unicom

AT&T

Bell Mobility

LG Uplus

Interdigital

LG Electronics

Samsung

CBN

ICS

KPN

ABS

Vivo

Spreadtrum Communications

OPPO

Telus

CAICT