3GPP TSG-RAN Meeting #89-e	draft RP-202000

Electronic Meeting, 14 - 18 September, 2020

- Source:	China Telecom

- Title:	New WID: Further enhancement on NR demodulation performance

- Document for:	Approval

Agenda Item:	9.1.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Further enhancement on NR demodulation performance

## Acronym: NR_demod_enh2-Perf

## Unique identifier: 	{A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part |  |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 840094 | NR performance requirement enhancement |  |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

## 3	Justification

NR Rel-15 demodulation performance requirements was started from April 2018 and completed by December 2019. Several NR features specified in Rel-15 core part are not covered when developing the Rel-15 performance requirements.

A Rel-16 WI on Rel-15 demod leftovers was approved in June 2019, which specifies:

- UE demod/CSI requirements including CA normal demod, CA CQI reporting, CA power imbalance demod, EN-DC power imbalance demod, PMI reporting for Tx ports larger than 8 and up to 32, TDD LTE-NR co-existence demod requirements;

- BS demod requirements including 30% throughput PUSCH, testable PUSCH 2T2R 16QAM requirements for FR2.

However, due to the limited time unit, the performance requirements for some important features are still not covered in Rel-16. Therefore, it is proposed to open a new Rel-17 WI to define performance requirements for the selected NR features such as: PDSCH and PUSCH advanced receivers for handling inter-cell, inter-user and/or inter-stream interference, PUSCH 256QAM requirements for FR1.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

### 4.2	Objective of Performance part WI

UE demodulation and CSI reporting requirements:

- MMSE-IRC receiver for suppressing inter-cell interference and intra-cell inter-user interference

    - Scenario 1: Inter-cell interference scenario

    - Type of requirements:

      - Define PDSCH demodulation requirements

      - Further decide whether to introduce the corresponding CQI reporting requirements during the WI

    - SCS and slot duration

      - Scenario 1a: Slot-based transmission and aligned SCS among cells (first priority)

      - Scenario 1b: Non-slot-based transmission and/or different SCSs among cells (second priority)

      - Further discuss the assumptions for requirements definition

    - Reference receiver:

      - MMSE-IRC with DMRS based interference covariance estimation

        - Note: use the DRMS for target UE’s PDSCH.

    - Target frequency: FR1

    - Rx antenna number: 2Rx and 4Rx

    - Interference profile

      - Reuse LTE interference profiles as a starting point

      - Other interference profiles are not precluded

    - Scenario 2: Intra-cell inter-user interference scenario

    - Phase I: Evaluate the performance under practical MU-MIMO interference profile for the candidate reference receiver

      - Identify practical MU-MIMO interference modelling methodology

      - Reference receiver: MMSE-IRC receiver. Further discuss and decide the interference covariance estimation method during the WI

    - Phase II: Define the requirements if needed based on the outcome of phase I

    - Target frequency: FR1

    - Rx antenna number: 2Rx and 4Rx for FR1

BS demodulation requirements:

- MMSE-IRC receiver for suppressing inter-cell interference

    - Type of requirements: PUSCH requirement

    - Reference receiver:

    - MMSE-IRC with DMRS based interference covariance estimation

    - Target frequency: FR1

    - Rx antenna number: further discuss and decide in the WI

    - Identify the interference profiles for requirements definition

    - LTE interference profiles can be used as a starting point

- PUSCH demodulation requirements for FR1 256QAM

    - Note 1: low mobility is considered for requirements definition

    - [Note 2: Realistic phase noise modelling is left up to the contributing entities]

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-4 | NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements | TSG-RAN #96 | Performance part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | TSG-RAN #96 | Performance part |
| 38.141-1 | NR; Base Station (BS) conformance testing Part 1: conducted conformance testing | TSG-RAN #96 | Performance part |
| 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | TSG-RAN #96 | Performance part |
| 38.307 | NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band | TSG-RAN #96 | Performance part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

China Telecom

## 7	Work item leadership

RAN WG4

## 8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

China Telecom