3GPP TSG RAN Meeting #89-e							RP-201984

Electronic Meeting, Sept 14th - 18th, 2020

Agenda Item:		9.2.13

Source:			Nokia, Nokia Shanghai Bell

Title:				Summary for WI NR RF requirement enhancements for frequency range 2 (FR2)

WI code(s):			NR_RF_FR2_req_enh

leading WG:		RAN4

Release:			Rel-16

### 1	Introduction

This work item developed various UR RF enhancements for FR2. Basis for the WI was the features that were not developed in REL15 due to lack of time.

### 2	Description

During this WI following enhancements were introduced.

- MPE: UE can report to network how much P-MPR it uses or is going to use because of MPE. Based on this reporting network can take actions to prevent sudden radio link failures.

- Beam correspondence: Requirements for SSB-based and CSI-RS based BC were developed. This allows network  to efficiently deploy different beam widths for SSB  and CSI-RS. This enables e.g. to deploy wider SSB based beams for idle mode UEs and narrower CSI-RS based beams for RRC connected mode UEs.

- Interband DL CA: Requirements were developed for band combination CA_n260-n261 based on independent beam management.

- DL Intra-band CA BW Enhancement: New frequency separation classes Fs were introduced for up to 2400 MHz separation. Furthermore frequency separation classes for DL-only spectrum Fsd were introduced. The DL-only frequency spectrum is the width of UE frequency spectrum available to network to configure DL CCs only, and it extends on one-side of the bidirectional spectrum in contiguous manner with no frequency gap between the two.

- Non-contiguous intra-band uplink CA: Requirements were developed for n260 for up to three uplink sub-blocks.

- MPR enhancements: Zero dB MPR range was extended to cover more allocations.

- Output power boost when in-band emissions are suspended: 1 dB boost was introduced for QPSK modulation.

- Multiband relaxation framework enhancement: Based on RAN5 LS MBR framework was modified. REL15 retains original MBR concept but introduced a relaxation cap. REL16 adopted new concept where relaxation defined per band basis.

### 3	References

[1]	RP-201606	Revised WID on NR RF Requirement Enhancements for FR2

[2] RP-201607	SR of NR RF requirement enhancements for frequency range 2 (FR2), RAN#89

[3] RP-201608	TR 38.831 v1.0.0

[4] RP-200666	SR of NR RF requirement enhancements for frequency range 2 (FR2), RAN#88

[5] RP-200247	SR of NR RF requirement enhancements for frequency range 2 (FR2), RAN#87

[6] RP-192652	SR of NR RF requirement enhancements for frequency range 2 (FR2), RAN#86

[7] RP-192226	SR of NR RF requirement enhancements for frequency range 2 (FR2), RAN#85