3GPP TSG-RAN Meeting #89-e	Revised RP-201589

Electronic Meeting, 14 - 18 September, 2020

- Source:	China Telecom

- Title:	New WID: Further enhancement on NR demodulation performance

- Document for:	Approval

Agenda Item:	9.1.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Further enhancement on NR demodulation performance

## Acronym: NR_demod_enh2-Perf

## Unique identifier: 	{A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part |  |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 840094 | NR performance requirement enhancement |  |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

## 3	Justification

NR Rel-15 demodulation performance requirements was started from April 2018 and completed by December 2019. Several NR features specified in Rel-15 core part are not covered when developing the Rel-15 performance requirements.

A Rel-16 WI on Rel-15 demod leftovers was approved in June 2019, which specifies:

- UE demod/CSI requirements including CA normal demod, CA CQI reporting, CA power imbalance demod, EN-DC power imbalance demod, PMI reporting for Tx ports larger than 8 and up to 32, TDD LTE-NR co-existence demod requirements;

- BS demod requirements including 30% throughput PUSCH, testable PUSCH 2T2R 16QAM requirements for FR2.

However, due to the limited time unit, the performance requirements for some important features are still not covered in Rel-16. Therefore, it is proposed to open a new Rel-17 WI to define performance requirements for the selected NR features such as: PDSCH and PUSCH advanced receivers for handling inter-cell, inter-user and/or inter-stream interference, PUSCH 256QAM requirements for FR1.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

### 4.2	Objective of Performance part WI

Candidate objectives for UE interference-aware receivers

- Scenario a): Inter-cell interference

    - Type of requirements:

    - Define PDSCH demodulation requirements

    - Further decide whether to introduce the corresponding CQI reporting requirements during the WI

    - SCS and slot duration

    - Scenario 1: Slot-based transmission and aligned SCS among cells

    - [Scenario 2: Non-slot-based transmission and/or different SCSs among cells]

    - Further discuss the assumptions for requirements definition

    - Reference receiver:

    - For scenario 1: MMSE-IRC with DMRS based interference covariance estimation

      - Note: use the DRMS for target UE’s PDSCH.

    - [For scenario 2: MMSE-IRC. Interference covariance estimation method is FFS]

    - Target frequency: FR1

    - Rx antenna number: 2Rx and 4Rx

    - Interference profile

    - Reuse LTE interference profiles as a starting point

    - Other interference profiles are not precluded

    - TRS/CSI-RS/DMRS configuration

    - Further discuss and decide whether TRS/CSI-RS are collided among cells during the WI

    - Further discuss and decide whether DMRS is colliding with TRS/CSI-RS of interfering cell during the WI

    - [Further discuss and decide whether suppression or cancellation on interfering cell’s CSI-RS/TRS is needed during the WI]

    - As baseline, avoid network assistance and/or restriction.

- Scenario b): Inter-layer interference for SU-MIMO

  - Phase I: Study on the reference receiver structure for (iterative) soft IC and evaluate the performance benefit over R-ML, by taking into account the implementation complexity and processing delay

    - Phase II: Define the requirements if needed based on the outcome of phase I

    - Target frequency: FR1, FR2

    - Number of data layers: up to 4 for FR1; 2 for FR2

    - Rx antenna number: 2Rx and 4Rx for FR1; 2Rx for FR2

    - Modulation order and spatial correlation level: further discuss and decide during the WI

- Scenario c): Intra-cell inter-user interference for MU-MIMO

    - Phase I: Evaluate the performance under practical MU-MIMO interference profile for the candidate reference receiver including MMSE-IRC receiver.

    - Further discuss and decide the interference covariance estimation method during the WI

    - Phase II: Define the requirements if needed based on the outcome of phase I

    - Target frequency: FR1

    - Rx antenna number: 2Rx and 4Rx for FR1

- [Decide the applicable release for new UE requirements]

Note: Prioritization of different objectives will be discussed during the RAN plenary.

Candidate objectives for BS interference-aware receivers

- Scenario a): Inter-cell interference

    - Type of requirements: PUSCH requirement

    - Reference receiver:

    - As starting point, use MMSE-IRC with DMRS based interference covariance estimation

    - FFS Soft-IC/CW-IC for cell sectors belonging to the same site

    - Target frequency: FR1

    - Rx antenna number: further discuss and decide in the WI

    - Interference profile

    - Reuse LTE interference profiles as a starting point

    - Other interference profiles are not precluded

    - As baseline, avoid network assistance and/or restriction.

- Scenario b): Inter-layer interference for SU-MIMO

    - Type of requirements: PUSCH requirement

    - Candidate reference receiver:

    - Soft IC

    - SL-IC

  - Evaluate the performance gain for different reference receivers, by taking into account the implementation complexity

    - Target frequency: FR1, FR2

    - Number of data layers: 2 for FR1 and FR2

    - Rx antenna number: 2Rx, 4Rx and 8Rx for FR1; 2Rx for FR2

- Scenario c): Intra-cell inter-user interference for MU-MIMO

    - Type of requirements: PUSCH requirement

    - Reference receiver:

    - As starting point, use CWIC

    - FFS hybrid-IC  (mixing hard-IC and soft-IC)

    - Target frequency: FR1

    - Rx antenna number: 2Rx, 4Rx and 8Rx for FR1

Note: Prioritization of different objectives will be discussed during the RAN plenary.

BS FR1 PUSCH 256QAM

- Define PUSCH demodulation requirements for FR1 256QAM

Other candidate objectives for UE demodulation and CSI reporting requirements

- Requirements for Rel-15 multi-TRP DPS TX scheme under normal propagation conditions

- NR PDSCH demodulation requirements for neighboring cell CRS-IC in LTE-NR coexistence scenarios

    - Further discuss and decide if network assistance information including cell ID, number of ports, MBSFN configuration of the neighbouring cell is required

- Demodulation/CSI reporting requirements for downlink 8Rx antennas

    - Define channel model for downlink 8Rx antennas

    - Specify the antenna configuration and MIMO channel correlation matrices for 8Rx antennas

    - Specify the static channel model

    - Define PDSCH demodulation requirements for the rank lower than or equal to 4.

    - Further discuss and decide whether to define PDSCH demodulation requirements for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

    - Define SDR requirements for 8Rx

    - Define CSI reporting requirements for 8Rx.

    - No PDCCH demodulation requirement is expected for 8Rx.

    - Considering the test coverage of 8Rx, test applicability rule is needed to define

    - Define applicability rule of existing performance requirements for 8Rx capable UEs.

    - After finalizing the single carrier requirements, the following CA scenarios will be specified

    - Intra-band contiguous CA with 2CC and 8Rx with up to 8-layers supported per CC

    - Inter-band CA with 8Rx/8-layers supported on one or two contiguous CCs on the identified operating bands, and with 2Rx or 4Rx supported on the other CC(s).

- Requirements for non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA (e.g. band 42, n77/n78)

  - First investigate the applicable MRTD and power imbalance level, considering the network deployment scenario and UE implementation feasibility.

  - Define PDSCH demodulation performance requirement based on the applicable MRTD and power imbalance values.

  - Introduce a UE capability report that indicates the support of dual Rx chain in a band of interest

Note 1: For downlink 8Rx and non-colocated scenario for intra-band non-contiguous EN-DC/NR-CA, if agreeable to be included in Rel-17 package, the RF/RRM impact needs to be included in the FR1 RF/RRM work areas.

Note 2: For Rel-16 performance leftovers: In Dec and/or 2021 March RAN Plenary, update the WID to further consider potential new objectives for Rel-16 performance left over issues if justified.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
|  | TR |  |  |  |  |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-4 | NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements | TSG-RAN #96 | Performance part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | TSG-RAN #96 | Performance part |
| 38.141-1 | NR; Base Station (BS) conformance testing Part 1: conducted conformance testing | TSG-RAN #96 | Performance part |
| 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | TSG-RAN #96 | Performance part |
| 38.307 | NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band | TSG-RAN #96 | Performance part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

China Telecom

## 7	Work item leadership

RAN WG4

## 8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

China Telecom