3GPP TSG-RAN Meeting #89e	RP-201734

Electronic Meeting, 14th – 18th Sep, 2020

- Source:	Moderator (Huawei)

- Title:	New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1) in Rel-17

- Document for:	Approval

Agenda Item:	9.1.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	RF requirements enhancement for NR frequency range 1 (FR1) in Rel-17

## Acronym:

## Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

## 1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 Work Item is presented or e.g. the related Study Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. List previous Work Items of earlier releases if relevant.}

{This section is mandatory to be filled out by the rapporteur.}

{Not applicable for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

## 3	Justification

This work includes two parts: Rel-17 new topics for RF requirement enhancement and Rel-16 FR1 leftover topics.

Regarding Rel-17 new topics, we would like to consider the further enhancement of Tx switching between carriers with UL-MIMO supported on carrier(s), and support of the 26dBm high power UE for intra-band contiguous UL CA.

In Rel-16 the mechanism and RAN4 requirements are specified for Tx switching between case 1 and case 2. The motivation is to select the better uplink carrier and better transmission mode in a given slot for transmission assuming that there are only two con-current transmitters. In Rel-16 it is assumed that only one transmitter can be supported on one carrier and two transmitters can be supported on the other carrier. But recently two transmitters are enabled for more NR bands. Thus it would be possible for UE to support two transmitter on both carriers. In this case, the Tx switching would be conducted between two carriers which are both capable of 2Tx transmissions. The uplink throughput could be further improved. In the meanwhile, UL-MIMO requirements for interested SUL/TDD/FDD bands are to be further specified to enable UE 2Tx transmissions on both carriers when configured switching with SUL or UL-CA band combinations.

The second topic is to support 26dBm high power UE for intra-band contiguous UL CA. The motivation is quite straightforward. But more study would be needed. Because the much wider channel bandwidth for NR is supported compared to LTE. It would be non-trivial to investigate the UE architecture, e.g., 23dBm+23dBm PA and 23dBm+26dBm PA, to support high power as well as the large aggregated channel bandwidth. And the supported channel bandwidth for PA also needs be studied. If the supported channel bandwidth of a single PA is less than the available channel bandwidth, then some tradeoff among the supported channel bandwidth, supported antenna ports per CC and achievable uplink power per CC should be considered. Based on the outcome of discussion on UE architecture, the RF requirements can be specified.

Regarding Rel-16 leftover topics, we identify some enhancement for 2Tx UE requirements based on the previous discussion in RAN4. Those topics need extensive discussions and cannot be finalized within one or two quarters/meetings, including per-layer EVM requirements for UL-MIMO, impacts of power imbalance on the RF requirements, 2Tx RF requirement enhancement for EN-DC and UL CA.

In Rel-15 and Rel-16, DC location is indicated by RRC signaling which cannot be dynamically reported to the network. From UE implementation perspective, DC location is shifting dynamically by external factor, e.g. temperature. Once DC location is not correctly indicated to the network and gNB cannot puncture the RE which carries the LO leakage, it will cause the uplink performance degradation especially for high order modulation. In Rel-17, DC location indication mechanism can be enhanced to solve the problem.

Besides, we think more Rel-16 leftover issues could be added in this work item as the RF requirement enhancement after RAN4 August meeting in order to timely complete the essential Rel-16 RF core requirements.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objectives for Rel-17 RF requirement continuation work for FR1 include:

- Specify UE requirements to enable Tx switching between 2Tx@carrier#1 and 2Tx@carrier#2 based on SUL and NR uplink CA for UE supporting maximum two concurrent transmission

  - Remove the RAN2 and RAN4 restriction on UL MIMO configuration for SUL bands, after

    - Specify the UL-MIMO requirements for example SUL configuration with SUL band n80, n84, and n95, i.e., SUL_n41A-n80A, SUL_n78A-n84A, and SUL_n79A-n95A

  - Specify RAN4 requirement for UL Tx switching between 2Tx@carrier#1 and 2Tx@carrier#2

    - Length of switching period

    - Time mask RF requirements

    - Uplink interruption and downlink interruption (RRM) requirements, if needed

  - Minimize the impacts on RAN1

    - Update RAN1 uplink switching for carrier aggregation and supplementary uplink to accommodate the new switching cases

  - Minimize the impacts on RAN2

    - Update the RRC signaling to indicate the switching period location and switching period length

    - Update the UE capabilities

Note 1:  Only addressing the case of co-located and synchronized network deployment for the two UL carriers.

Note 2:  Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA.

Note 3:  The UE is configured with two different uplink carrier frequencies.

- [Specify UE requirements to enable Tx switching between 1 carrier in one band and 2 contiguous carriers within the other band, under the condition that there are only 2 simultaneous Tx at maximum]

- MPR enhancement for power class 3 TDD intra-band contiguous and non-contiguous UL CA

  - Contiguous CA: MPR enhancement for dual PA architecture when aggregated channel bandwidth >100MHz

  - Non-contiguous CA: study whether Rel-16 MPR definition can be reused for single PA architecture

    - If not, define MPR requirements for single PA architecture

- Enhance the 2Tx RF requirements

  - Investigate the impact of power imbalance between Tx antennas for UL-MIMO, and enhance the RF requirements, e.g., EVM and MOP, and the test method

Note: Tx diversity scheme may be concluded in Rel-16, remaining issues can be discussed in TEI16

  - Investigate impact of RIMD on MPR, and enhance 2Tx RF measurement for UL CA and EN-DC testing

    - 2Tx Antenna isolation assumption for RF requirement measurement: 10dB

    - Study the measurement method enhancement: e.g. How to injecting interference into the antenna connector

- HPUE for TDD intra-band contiguous UL CA

  - Take n41 intra-band contiguous UL CA for an example band

  - Investigate and specify the 26dBm power class for n41 intra-band contiguous UL CA

    - Study the impact of different UE architectures on the requirements

      - [Use 1Tx architecture as baseline]

      - [The other architecture is not precluded]

      - [Power class Relation between single CC and intra-band contiguous CA on HPUE band can be studied in this phase(TBD)]

    - [Specify the mechanism to meet SAR requirements

      - Mechanism for HPUE on single carrier can be a start point considering the same UL-DL configuration assumption

      - Power class fallback enhancement is specified for TDD intra-band contiguous UL CA]

    - A-MPR requirement

  - Specify MPR requirements [and optimize the requirement, if necessary]

- [DC location enhancement]

- [Other Rel-16 WI leftover FR1 topics]: FFS

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

This Perf. Part WI has to standardize the Perf. Part requirements:

- Required changes are to be added to release independence TS 38.307.

- Specify RRM performance requirements if needed.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | Add FR1 RF Requirements to User Equipment (UE) radio transmission and reception; Part 1 |  | Core part |
| 38.101-3 | Add FR1 RF Requirements to User Equipment (UE) radio transmission and reception; Part 3 |  | Core part |
| 38.307 | Add FR1 RF Requirements to User Equipments (UEs) supporting a release-independent frequency band |  | Perf. part |
| 38.133 | Add RRM core requirements if needed |  | Core part |
| 38.133 | Add RRM performance requirements if needed |  | Perf. part |
| 36.133 | Add RRM core requirements if needed |  | Core part |
| 36.133 | Add RRM performance requirements if needed |  | Perf. part |
| 38.306 | Add impacts on 38.306 User Equipment (UE) radio access capabilities, if needed |  | Core part |
| 38.331 | Add impacts on 38.331 Radio Resource Control (RRC) Protocol specification, if needed |  | Core part |
| 38.213 | Add impacts on 38.213 Physical layer procedures for control, if needed |  | Core part |
| 38.214 | Add impacts on 38.214 Physical layer procedures for data, if needed |  | Core part |
| 36.306 | Add impacts on 36.306 User Equipment (UE) radio access capabilities, if needed |  | Core part |
| 36.331 | Add impacts on 36.331 Requirements for support of radio resource management, if needed |  | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

## 7	Work item leadership

R4

## 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name