3GPP TSG RAN#88e	RP-202009

e-Meeting, 14th – 18th September, 2020

Agenda item:	9.2.2

Source: 	Nokia

Title: 	Moderator's summary for email discussion [89E][09][R16_NR-U] Initial round

Document for:	Discussion/Decision

Introduction

The initial phase of discussion on this topic took place between 07:00 UTC on Monday 14th September and 11:59 UTC on Tuesday 15th September.

This document summarises the main points of the initial discussion and provides a proposal for the way forward, to be further discussed in the intermediate phase of the discussion.

Summary of Initial Discussion

The main points raised in the discussion were as follows:

- 6 GHz:

  - n96 UE Rx REFSENS

    - 1.1dB relaxation compared to n46: Mediatek

    - 0.5dB: Charter, Skyworks, AT&T

    - TBD: Huawei

  - Band plan

    - US focus: Charter, Intel, Samsung, AT&T

    - Include EU band when regulations are available: BT, DT, Skyworks, Nokia

  - 60kHz SCS:

    - Downscope:  Apple, Charter

    - Not TBD for SU:  Huawei

  - 20MHz SU:

    - Separate table from wideband operation

  - MR BS: Samsung, Nokia

  - A-MPR

    - More simulations/measurements needed: Huawei

    - Already provided: Skyworks, Qualcomm

  - UE coexistence table for standalone operation and CA operation: Huawei

  - Reference measurement channels: Huawei

  - ΔfOBUE and ΔfOOBB: Huawei

  - Co-existence requirements between 5GHz and 6GHz: Huawei

  - Wideband capability

  - Bandwidth classes

  - Channel raster study: Huawei

- 5GHz:  Huawei:

  - UE coexistence table for standalone operation and CA operation

  - Reference measurement channels

  - 38.101-3 CR

- Handling of remaining work:

  - Approve CRs this week and handle remaining issues in maintenance: Qualcomm, Charter, Verizon, Skyworks, AT&T, Nokia, Samsung

  - Exception sheet: Mediatek, CHTTL

Full details of company comments are included in the Annexe.

Intermediate Discussion

Overall, quite a number of open issues are raised, although a clear majority of contributing companies prefer to approve the set of CRs submitted to RAN#89-e.

Taking these views into account, the moderator makes the following proposal for a way forward:

Proposal:

- Approve the CRs in RP-201913 (38.104), RP-201914 (37.107), RP-201915 (37.106), RP-201917 (37.104), RP-201918 (37.105) with existing square brackets

- Modify CRs in RP-201890 (38.101-1) and RP-201916 (36.104) to remove 60kHz SCS, and approve the modified CRs

- Keep the work item open until RAN#90-e by means of an exception sheet to address the following open issues:

  - Resolve all parameters in square brackets

  - CR for UE coexistence for 38.101-3

  - Medium range base station requirements

  - If European regulations for 5925-6425 MHz are approved by CEPT ECC at least 1 week before the submission deadline for the Q4 RAN4 meeting, the 5925-6425 MHz band may be included if possible, using either different NS values for the UE and regional requirements for the BS, or a new band, depending on the details of the European regulatory requirements

Companies are invited to provide their constructive comments on the above proposal in the table below. If agreeing with an earlier comment, please simply add your company name against that comment.

| Company | Comments |
| --- | --- |
| Charter Communications Inc | - We agree with bullets one and two. We have a very strong opinion that revised CRs RP-201890 and RP-201916 need to be approved in this meeting as these are critical for Operators in the US to execute deployment and time to market plans - On bullet three, we also agree but we will like to emphasize that the remaining items need to be completed in the next RAN4 meeting and a final CR removing all square brackets and addressing UE co-existence and MR BS requirements should also be completed in the next meeting. - I believe the proposal by moderator on how to handle 5925-6425 Mhz is a good compromise approach to reach consensus agreement |
| Qualcomm | In general, we can agree to the moderator’s proposal. The concerns raised have already been discussed extensively in RAN4 and compromises reflected in the company CR’s. Refsens: Only MediaTek has trouble meeting the already relaxed requirement. We are not sure what MediaTek means by a fair average and how the existing value is “unfair”, but the average was taken based on numerical average of proposals provided by all companies who stated a numerical opinion. Channelization: The channel raster is consistent with the latest draft from IEEE. It is not expected to change any further but if it does, then 3GPP can consider a change at that time as already agreed. WiFi companies have already announced products so they are not going to engage in a conversation with 3GPP to study an optimized channelization. 60 kHz SCS and SU: The previous agreement referred to by some companies R4-1910537 states “It is agreed to increase the number of PRBs to 25 for 20 MHZ channel bandwidth with 60 kHz SCS with the condition awareness of relaxation on NR-U emission requirements compared with R15 NR.” It is our understanding that while the ACLR is relaxed, the NR-U emission mask is actually more stringent than the NR SEM, so the conditions of the agreement are not fully met. Due to the disagreement among companies here and the expectation that 60 kHz SCS will not be deployed initially in 6 GHz, we can accept the proposal to remove 60 kHz SCS from the CR’s. A-MPR: A-MPR has been studied extensively by at least two companies including simulations as well as measurements. The late modification to the channel raster does not impact indoor operation at all (NS_53) since indoor is limited by in-band PSD, not by spurious emissions. For outdoor operation (NS_54), the guard band is increased by 10 MHz at the low edge of the band so low edge channels have been adjusted accordingly and placed in square brackets. No other channels are impacted since there is guard band to the emission requirement. UE coexistence: This has already been agreed in RAN4 #96e that UE coexistence for SA NR-U is not required. This topic was discussed in both first round and second round discussions based on a paper that was submitted on time to the meeting giving companies ample opportunity to express their views. No objections were received and the meeting instructions from the RAN4 chairman is “Delegates are strongly encouraged to provide comments/concerns asap: Silence within a reasonable timeframe means no objection”. 3GPP cannot accommodate companies who are not willing to follow the meeting procedures or intentionally try to circumvent them to obstruct progress. Reference measurement channels: These are defined during the performance phase of the work item. Band plan: There is no question from any company that the regulations are completed in the US for the band. We believe the band should be defined accordingly in 3GPP without further delay for the US. When regulations are completed in Europe or any other country, 3GPP can then decide in the usual way how to add that band into the specifications either as a new band or by leveraging Band n96. |
| ZTE | For BS related CRs, there are still some issues not addressed. - For co-location spurious emission for n96 and co-location requirements between n46 and n96 were not addressed, the proposed value is not acceptable for us. -52dBm/MHz should be related with UE NF based on background how this requirement was derived. - We disagree with LO leakage exception requirement specified for NR-U BS and the formula is not correctly which has mentioned in last RAN4#96e meeting and has been corrected in WF. - For IBB and OOBB requirements for NR-U BS is not agreed and it should be kept as TBD instead of values in square bracket. - some requirement table is not put in the right order, this should also be corrected. - 60KHz is still kept in NR-U BS spec, if it was agreed to remove from UE spec, then the same approach should be used for BS spec. - For MSR spec, as proposed in the previous meeting, CS configuration for NR-U with other RAT should be addressed firstly and we didn't see the discussion and conclusion yet. |

In addition, the WI Summary in RP-201840 was flagged. This can be postponed until RAN#90-e, when the Rapporteur should provide an update taking the new status of the work and the comments into account.

Annexe

The following detailed comments were made during the initial phase of the discussion:

| Open issue |  |  | Proposed solution |  |
| --- | --- | --- | --- | --- |
| Company | Issue | CR affected | Company | Solution |
| RP-201699 Mediatek | Rx REFSENS MediaTek requests min 1.1dB relaxation vs. n46 (5GHz) Band Plan US focus vs. US + EU focus | RP-201890 RP-201913, 914,915,916,917, 918 | Charter Communications | Allow 0.5dB difference as it is an average from other proposals, then approve revised CR in this meeting and leave it in [ ] for further maintenance discussion in R4 Q4 or approve revised value without [ ] Band Plan for this CR should be US focus as regulatory framework is complete |
|  |  |  | Skyworks Solutions | The offered 0.5dB increase vs n46 for 6GHz band is sufficient to cover the 20% increase in frequency and BW increase from 14 to 18% fractional BW. Note that n46 REFSENS is already 3dB higher than any other FR1 band |
|  |  |  | Intel | For band plan, we prefer to focus on US band. Once EU or other countries finalize their regulation, then 3GPP can further define additional band as business usual. |
|  |  |  | Samsung | Rel-16 n96 Band plan should focus on the US. When other regional regulations are completed later, e.g., Korea, then it can be applied by using NS or regional requirements if needed. |
|  |  |  | AT&T | We support the REFSENS increase proposed by Charter Comm and Skyworks and leaving it in [ ] for this meeting. For the band plan, the focus for the CRs should be on the US band per the FCC R&O and NPRM. Other bands would be considered once the regulations are defined based on normal RAN4 operating procedures. |
|  |  |  | MediaTek | 1.1dB is the fair average of all different proposals made. 0.5dB relaxation is unacceptable to MediaTek If 1.1dB cannot be agreed this week in [] we request this issue be captured in the exception sheet. |
|  | Proposal: Exception sheet |  | MediaTek, CHTTL | Exception sheet is required given half of the work is basically incomplete – this is clearly not maintenance. |
|  |  |  | LG Electronics | We support the proposal from Chartered. In addition, as pointed out also by Samsung other regions are also finalizing their band plans and this work needs to be addressed in future meeting(s) through different band numbers or NS approach. |
| RP-201854 Apple | Proposal 1:De-scope 60kHz SCS related discussions from Rel-16. Proposal 2:Complete NR-U wide-band capabilities discussion in Rel-16. Proposal 3: Complete NR-U bandwidth classes discussion in Rel-16. | RP-201890 RP-201913, 914,915,916,917, 918 | Charter Communications | Agree with proposal 1 but can further discuss for reaching consensus agreement on CRs, neutral in proposals 2 and 3 |
|  |  |  | Intel | Regarding 20 MHz SU, controversial issue is for wideband operation. A single carrier 20 MHz SU has the agreement (R4-1910537) and this should be captured in the CR. Table 5.3.3-2 in RP-20189 might be misinterpreted as 20 MHz is listed as a wideband operation. Since 20 MHz is a minimum channel BW as a single carrier, we suggest to have a separate table to differentiate from the wideband operation. |
| RP201889 Qualcomm | Proposal  As described in the previous slides, all of the open issues are relatively simple and can be addressed by: • regular maintenance of the approved specs (also considering the approved specs would include some square brackets) or • with minimal modifications of the CRs to be done this week • Due to the high industry interest in the North America region for the spectrum corresponding to n96, • we propose to address the remaining issues during RAN #89e, with the intention to solve them • This will allow closure of the project in the Rel-16 timeframe | RP-201890 RP-201913, 914,915,916,917, 918 | Charter Communications, Verizon, Skyworks Solutions Inc, AT&T, Nokia, LG Electronics | We agree with this proposal |
|  |  |  | Intel | As the rapporteur (Qualcomm) pointed out, our concern might not be a major issue as 20 MHz SU was already agreed. Please note that the current controversy is for wideband operation and not a single carrier. As commented above, we suggest to have a separate table for 20 MHz as a single carrier and put the existing agreement, i.e., 25 RB (R4-1910537). |
|  |  |  | Samsung | We agree with the proposals in RP-201889. For BS related items, the proposed values for n96 LA BS RF requirements in square brackets is the reasonable at this stage. The requirements of n96 MR BS can be considered with different way those of LA BS depending on the ongoing process of FCC in the future as proposed by rapporteur. |
|  |  |  | MediaTek, CHTTL | The outstanding work (BS) is clearly not a maintenance effort. It is critical that the views of all infra vendors be carefully taken into account in this discussion to ensure implementable and deployable specifications. Rushing the specs out for the sake of rushing them is not ok. The statement “with minimal intention to change them” is particularly worrying in view of the above |
| RP-201699 Mediatek | Band plan - US focus vs. US+EU focus: EU Regulations incomplete - Plenary decision needed Proposal 5: Decision required whether to focus only on US band in Rel-16 | RP-201890 RP-201913, 914,915,916,917, 918 | BT plc, Deutsche Telekom | We believe that the European regulations for 5925 – 6425 MHz will be completed and agreed by the end of October, in which case it would be possible for these to be added at the next RAN4 meeting. So, in response to “Proposal 5” we believe that it may be possible to also include the European regulations in Release 16. (We have no objection to the existing US regulations and are not proposing to amend them.) |
| RP-201834 Qualcomm | This work item will specify NR enhancements for a single global solution framework for access to unlicensed spectrum which enables operation of NR in the 5GHz and the 6GHz |  | BT plc, Deutsche Telekom | The WID states that the work item “will specify NR enhancements for a single global framework”, although with regard to the 6 GHz band at present it only specifies enhancements for a single country (the USA). We understand that in practice it may not be possible to achieve a single global framework, but we believe that adding the regulations for Europe (which should be completed by the end of October) would take this much closer to achieving the WI objective. (We have no objection to the existing US regulations and are not proposing to amend them.) |
|  |  |  | Skyworks Solutions Inc. | Regarding the band plan, as usual for many features, a feature can use an example band to develop the framework. With current CRs it is the case for n46 and n96 where regulation is available. We support covering the European band in the same way for R16 as soon as regulation is available and this can be introduced with NS or a new band based on the regulation requirement. this should not prevent to accept the current bands that are complete from a regulation prospective. |
|  |  |  | Nokia | Whenever regulation is available for Europe, we support to include that work to be covered either by new band or NS/additional regional requirements. Agree with Skyworks that work should not prevent agreement on band that is completed for other region where regulations are finalized. |
|  |  |  | LG Electronics | Fully aligned with Skyworks. In addition to Europe 6GHz band plan is under finalization also in Korea. |
| Huawei | 5GHz band UE co-existence UE co-existence table for standalone operation and CA operation should be added. If we check 36.101, for LAA UL used for Scell, it needs to protect other bands. Hence when NR-U support stand-alone operation, it needs to protect other bands as well. Reference measurement channels Reference measurement channels for NR-U are not defined The CR for 38.101-3 is missing for core part WI 6GHz band Channel raster/sync raster On channel/sync raster, due to the uncertainty of IEEE draft standard, 3GPP needs some study on the optimized channelization or to have some coordination with IEEE to minimize the risk on the misalignment between NR-U and WIFI. SU for 60k SCS On SU for 60 KHz, TBD is not acceptable since it does not include the previous agreements (R4-1910537). REFSENS On reference sensitivity, we support TBD since the difference of proposals (2.6 dB from original QC and MKT proposal) is too large. A-MPR On A-MPR, A-MPR for NS_54 are based on previous channelization. there is are 10 MHz shift for guard band. New simulation or measurements are needed UE co-existence UE co-existence table for standalone operation and CA operation should be added. If we check 36.101, for LAA UL used for Scell, it needs to protect other bands. Hence when NR-U support stand-alone operation, it needs to protect other bands as well. Reference measurement channels Reference measurement channels for NR-U are not defined BS ΔfOBUE and ΔfOOBB On ΔfOBUE and ΔfOOBB, TBD can not provide the guidance to implementation design. BS co-existence On co-existence between 5GHz and 6 GHz, the requirements need to be defined, only a note( -52 dBm is not applicable) is not enough. | RP- 201890, RP-201913, RP-201914, RP-201915, RP-201916, RP-201917, RP-201918 | Huawei Skyworks MEDIATEK, CHTTL Nokia | Focus on the 5GHz NR-U band and corresponding requirements in Rel-16. Once the requirements for band n46 are finished, the NR-U WI can be closed. 6GHz band(s) are further studied in Rel-17, and adopt release independent manner to support these bands from Rel-16. Skyworks also provided input for REFSENS and justified that the difference between n46 and n96 can only be 0.5dB to account for +20% freq and BW increased from 14% and 16%. The+2.6dB has never been justified vs n46 especially with n46 NF already being 3dB higher than any other FR1 bands We have also provided measurements for A-MPR on top of QCOM simulations and NS54 now benefits from 10MHZ higher guard band and is in brackets. We do sympathize with the technical issues outlined in this document. On UE Rx REFSENS, please see above – MediaTek is proposing a fair average of the different values proposed, which today equals to 1.1dB, not 0.5dB. Nokia: Remaining items for the BS (including Medium Range BS class) shall be completed in the next RAN4 meeting. We encourage companies to actively contribute to this item with clear solutions! |