|  | ASIA-PACIFIC TELECOMMUNITY |  |
| --- | --- | --- |
|  | The 26th Meeting of the APT Wireless Group (AWG-26) |  |
|  | 14 – 18 September 2020, Virtual Meeting | 18 September 2020 |

Source: AWG-26/OUT-22

APT Wireless Group

liaison statement to 3GPP RAN, 3GPP RAN4

| Title of the Liaison Statement: | FREQUENCY ARRANGEMENTs FOR IMT in the BAND 470 – 703 MHz |
| --- | --- |
| Objective: | For information and comments |
| Origin: | APT Wireless Group |
| Contacts: | Mr. Le Van Tuan, Chairman, AWG |

APT/AWG has published the APT/AWG/REP-79 “APT Report on Frequency Arrangements for IMT in the Band 470-698 MHz” in Sep. 2017, in which the recommended harmonised band plan for 470-698 MHz band aligns with that of 3GPP Band 71.

During AWG-26 meeting (14-18th of September), the following two proposals were received and considered about the frequency arrangements of IMT on 600 MHz frequency band

- the 1st proposal urged AWG to look into spectrum arrangements for the 600 MHz band that provide further alignment with the APT 700 MHz band if possible.

- the 2nd proposal is to modify the APT Report 79 to enhance the spectrum efficiency for those countries in APAC that wish to implement both the APT700 and 600 MHz frequency arrangements. The addition of an option B suits those APT Members that have 8 MHz broadcasting channels, and there are two suggestion proposed, B1 and B2, with some pros and cons considered that needs discussion and a decision by AWG#26 which of B1 or B2 that is preferred. It is further proposed that depending on outcome the result of decision should be forwarded to 3GPP and possible other standard developing organizations to secure that adequate specifications are developed.

AWG-26 agreed to undertake the study to revise the APT/AWG/REP-79 to develop frequency arrangements in the band 470-703 MHz for APT Members that wish to implement both the APT700 and a 600 MHz frequency arrangements that is optimal for APT Members. The workplan see page 4 in the attachment.

AWG would appreciate 3GPP to provide any feedback on the technical feasibility of B1 and B2 respectively in the Annex before the next 27th APT Wireless Group (AWG-27) meeting, scheduled in Q1 2021, if any. Specifically, for option B2, there are in the Fig. 2 in Annex four different suggested sizes of the second duplexer overlapping with the US600 band 71/n71, and AWG appreciate 3GPP views on the different suggested sizes of the second duplexer from a feasibility and cost-efficient perspective.

For technical queries please contact:

Dr Mansoor Shafi

Spark Fellow Wireless

Spark New Zealand Ltd

+64 274515186

Email Mansoor.shafi@spark.co.nz

AWG will keep 3GPP informed of the progress and outcomes of relevant studies in future AWG meetings.

Le Van Tuan
Chairman

APT Wireless Group

Annex

Option B1

This option is based on an extension to band n71 and is shown in Fig 1 below:

Figure 1: Harmonized band plan for 470-698 MHz band- Option B1

In this option the frequency arrangement in Option A (i.e. existing n71 frequency arrangement) is extended upwards and downwards by 5 MHz being a better utilization of spectrum that is achieved with existing APT band that is 703- 748 (UL) paired with 758- 803 MHz (DL) (otherwise the spectrum 698- 703 MHz is not fully utilized).

Some countries in Region 3 have Radio Astronomy Service (RAS) on a co-primary basis (Radio Regulation No. 5.305 and No. 5.307). And as a consequence, co-existence/sharing with these other services will need to be studied.

Duplex Direction

The extended band proposal is a reverse duplex and in the lower part of the FDD band (below the duplex gap) is down link as compared to up link that is the ordinary convention but this is better suited for co-existence with APT 700 MHz band (band 28) at the upper edge of the band but also with possible broadcasting at the lower edge.

Channel Bandwidth

The revised proposal will result in 2x 40 MHz of FDD spectrum.

Duplex distance

The duplex distance is 51MHz. The frequency plan can support full band or split duplexer as is already the case for 3GPP band 28

Carrier Bandwidths supported

The extended bandwidth will support NR carrier bandwidths up to 30 MHz and LTE bandwidths up to 20 MHz

Co-existence with existing services

Co-existence with the following services will need study:

- Broadcast television below 612 MHz and size of guard band required. We expect this will be similar to band n71

- Radio Astronomy.

Radio astronomy is a receive only service that uses highly sensitive receivers. We expect IMT services using this band will need geographical separation with radio astronomy service receivers, but this will an Administration specific issue.

- Unlicensed white space devices and Wireless microphones

Wireless microphones are permitted to operate even with band 71 in part of the duplex gap. This has certain limits proposed in 3GPP TR 36 755. These limits will not change. This proposal is not making any changes to the duplex gap.

Option B2

This option uses two duplexers as shown in Fig 2 below and the duplex distance is 46 MHz as is the case with option A. Also the bottom duplexer is the same as that of option A, with an additional upper duplexer that should have as large possible overlap as possible with the lower duplexer in option A but at the same time being able to handle the duplex gap of 6 MHz. The size of this upper duplexer needs to be studied and decided prior to being specified. It can be considered to have variable bandwidths ranging from 35 MHz to 10 MHz respectively. Like option B1, this arrangement B2 is also a reversed duplex arrangement.

Fig 2: Option B2- two duplexers

This option B2 retains economies of scale advantages by adopting one duplexer being the same as that of option A and the duplex distance same as option A (i.e. existing n71 frequency arrangement) but the duplex gap is now reduced to 6 MHz. Further study of this is required to determine if this is feasible.

Also in this option there is no issue with Radio Astronomy Service (RAS) co-existence as is the case with option A.

https://www.apt.int/sites/default/files/Upload-files/AWG/APT-AWG-REP-79_APT_Report_Arrangement_470-698_MHz.docx