3GPP TSG RAN Meeting #90e						RP-202153

Electronic Meeting, December 7 - 11, 2020

3GPP TSG RAN WG1 #103-e		R1-2009841

e-Meeting, October 26th – November 13th, 2020

# Title:	LS on Mode 2 enhancements in NR sidelink

# Response to:

# Release:	Rel-17

# Work Item:	NR_SL_enh-Core

Source:                  RAN WG1

To:                          RAN

Cc:

Contact Person:

Name:	Seungmin Lee

E-mail Address:	<edison.lee@lge.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

For NR sidelink enhancement WI, the objective of Mode 2 enhancements is as follows:

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#90. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#90 is to be decided in RAN#90. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after RAN#89.

On the objective mentioned above, RAN1 made the following conclusions on inter-UE coordination. RAN1 has categorized schemes for inter-UE coordination in mode 2 sidelink resource allocation, and concluded that further study is required from RAN1 perspective to complete the assessment of their feasibility and benefits.

Conclusion: - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B: - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission - e.g., based on its sensing result - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission - e.g., based on its sensing result and/or expected/potential resource conflict - UE-A sends to UE-B the set of resource where the resource conflict is detected - FFS: details of resource conflict, e.g., including type of resource conflict - FFS: details of sensing operation at UE-A side - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s) - Note: these different types may be used in combination with each other - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required - Send an LS to RAN plenary Conclusion: - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed. - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it - How UE-A and UE-B are determined - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

2. Actions:

To RAN: RAN1 kindly asks RAN to take the above conclusions into account.

3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #104-e	25 Jan – 5 Feb 2021				e-Meeting

TSG RAN WG1 Meeting #104-bis-e	12 Apr – 20 Apr 2021				e-Meeting