3GPP TSG RAN Meeting #90e						RP-202156

Electronic Meeting, December 7 - 11, 2020

3GPP TSG RAN WG4 Meeting #97-e		R4-2016369

Electronic Meeting, 2 – 13 November, 2020

Title:	LS on Spurious emission limits for AAS BS in 6.425 – 7.125 GHz and 10-10.5 GHz

Response to:	-

Source:	TSG RAN WG4

To:	CEPT/ECC SE21

Cc:	TSG RAN

Contact person:	Johan Sköld

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

In response to a request from ITU-R WP5D (Att. 7.4 to 5D/134), 3GPP RAN WG4 is conducting a study on Parameters of terrestrial component of IMT (specifically NR), to be used for sharing and compatibility studies in preparation for WRC-23. The request covers frequency ranges from 470 MHz up to 10.5 GHz, of which the ranges 6.425-7.125 GHz and 10.0-10.5 GHz are presently not covered by 3GPP specifications. For these new ranges, 3GPP RAN WG4 is developing IMT technology related parameters for an LS response to be finalized in January 2021.

One of the essential parameters to include in the response is spurious emissions. It is noted however that in ERC Recommendation 74-01 on Unwanted emissions in the spurious domain, there are presently no limits specified for AAS BS operating in the range 6 - 24.25 GHz. In Annex 2 of the recommendation, Table entries 2.1.6 and 2.1.7 give limits below and above that range, but no limits are given for 6 - 24.25 GHz.

3GPP RAN WG4 will respond to the ITU-R WP5D request for AAS BS also for the frequency ranges 6.425 - 7.125 GHz and 10.0 - 10.5 GHz, which are presently not covered in ERC Recommendation 74-01. For spurious emissions, limits will be taken from entries 2.1.6 and 2.1.7 in Table 2 of the recommendation, based on the new frequency ranges seen in relation to the existing frequency ranges specified, with observations of the foreseen implementation of AAS BS and how they are to be deployed and used:

- In the range 6.425 - 7.125 GHz, limits according to 2.1.6 will be used, which aligns with the present limits applied for operation below 6 GHz.

- In the range 10.0 - 10.5 GHz, limits according to 2.1.7 will be used. The limits and related frequency offsets in the referenced Figure 7 (or 6) will however not be applied.

For point (2) above, it is noted that the break point at 18 GHz is applied in a way consistent with what is done for operation above 24.25 GHz and that no limits for frequencies below 18 GHz are modified. The 10 MHz reference bandwidth applied above 18 GHz will also ensure efficient OTA measurements for conformance testing purposes and type approval when measuring at these considerably higher frequencies.

TSG RAN WG4 welcomes any feedback on this topic and looks forward to future co-operation on defining spurious emission limits for the mobile service.

# 2	Dates of next TSG RAN WG4 meetings

RAN4#98-e		25 Jan 2021 – 5 Feb 2021		Online meeting

RAN4#98-bis-e		12 – 20 Apr 2021			Online meeting