ATTACHMENT 3.8

Source:	Document 5D/TEMP/224

WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT 
NEW REPORT ITU-R M.[IMT.C-V2X]

The use of the Terrestrial Component of IMT 
for Cellular-Vehicle-to-Everything Application

(Question ITU-R 262/5)

(YYYY)

TABLE OF CONTENTS

Page

1	Introduction		2

2	Relevant ITU-R Recommendations and Reports		2

3	Acronyms		2

4	C-V2X as specific application supported by IMT		5

4.1	Overview on Usage of IMT technology to C-V2X		5

4.2 	Use Cases

Relationship between IMT and C-V2X

6	Characteristics and Capabilities of C-V2X supported by IMT		6

6.1	Service requirements		6

6.2	IMT capabilities		6

6.3	Operational Spectrum		6

7 	Case Study

7.1 	Country AAA

7.2 	Country BBB

8	Summary		3

[Editor’s note: This Working document is consolidated version: Japan, China, Korea, and 3GPP]

[Editor’s note: On 8 October 2020, it is reviewed for scope. The other parts were not reviewed]

Scope

This report addresses the mutual relationship between IMT technologies and Cellular-Vehicle-to-Everything (C-V2X) as a specific application and elements of functions in IMT technologies that are used to realize C-V2X application.

Further, the report provides details on Overview on Usage of IMT technology, use cases, Relationship between IMT and C-V2X, Characteristics and Capabilities supported by IMT, and Case Study associated with C-V2X for the various scenarios including eMBB, mMTC, and URLLC of terrestrial component of IMT.

[Editor’s note: This document should be improved in terms of ITU-R view point with referring the scope and in the sense of more general]

# 1	Introduction

[Editor’s note: This section addresses overall information on C-V2X as emerging applications by IMT systems.]

# 2	Relevant ITU-R Recommendations and Reports

Recommendation ITU-R M.1890	Operational radiocommunication objectives and requirements for advanced Intelligent Transport Systems

Recommendation ITU-R M.2083	IMT Vision – Framework and overall objectives of the future development of IMT for 2020 and beyond

Recommendation ITU-R M.2084	Radio interface standards of vehicle-to-vehicle and vehicle-to-infrastructure two-way communications for Intelligent Transport System applications

Recommendation ITU-R M.2121	Harmonization of frequency bands for Intelligent Transport Systems in the mobile service

Report ITU-R M.2228	Advanced intelligent transport systems (ITS) radiocommunications

Report ITU-R M.2441	Emerging usage of the terrestrial component of International Mobile Telecommunication (IMT)

Report ITU-R M.2444	Examples of arrangements for Intelligent Transport Systems deployments under the mobile service

Report ITU-R M.2445	Intelligent transport systems (ITS) usage

[Editor’s note: More to be added]

# 3	Acronyms

[Editor’s note: To be added]

# 4	C-V2X as specific application supported by IMT

[Editor’s note: This section addresses overview and use cases for C-V2X using IMT technologies from relevant industry and external organizations as well as ITU-R members.]

## 4.1	Overview on Usage of IMT technology to C-V2X

[Editor’s note: This section summarizes how to use IMT systems for C-V2X applications in several groups and organizations outside ITU-R as well as ITU-R Members. It will clarify the relationship between C-V2X and IMT]

## [Editor’s note: this general overview comes from 3GPP TS 22.185, it is needed to have the reference number]

## 4.1	Types of V2X application support in 3GPP

### 4.1.1	General

The V2X applications in the present specification, referred to as Vehicle-to-Everything (V2X), contain the following four different types:

-	Vehicle-to-Vehicle (V2V)

-	Vehicle-to-Infrastructure (V2I)

-	Vehicle-to-Network (V2N)

-	Vehicle-to-Pedestrian (V2P)

Figure 4.1.1-1

Types of V2X applications (V2V, V2P, V2N and V2I)

These four types of V2X applications can use “co-operative awareness” to provide more intelligent services for end-users. This means that entities, such as vehicles, roadside infrastructure, application server and pedestrians, can collect knowledge of their local environment (e.g., information received from other vehicles or sensor equipment in proximity) to process and share that knowledge in order to provide more intelligent services, such as cooperative collision warning or autonomous driving.

These intelligent transportation services and the associated message sets have been defined in automotive SDOs outside 3GPP. Three basic classes of applications for providing ITS services: road safety, traffic efficiency, and other applications can be found in e.g., [3].

3GPP only handles the transport of these messages to support different types of V2X applications. The message transport expectations are described in requirements defined in this specification.

### 4.1.2	Vehicle-to-Vehicle (V2V) application

V2V applications expect UEs that are in proximity of each other to exchange V2V application information. 3GPP transport of messages containing V2V application information requires the UE to have a valid subscription and authorization from a network operator. Transport for a valid subscriber is provided whether the UE is served or not served by E-UTRAN.

The UE supporting V2V applications transmits messages containing V2V application information (e.g. location, dynamics, and attributes). The message payloads may be flexible in order to accommodate varying amount of information.

3GPP transport of message containing V2V application information is predominantly broadcast-based as illustrated in Figure 4.1-2. Such 3GPP transport includes the transport between UEs directly and/or, due to the limited direct communication range, the transport between UEs via infrastructure supporting V2X communication, e.g., RSU, application server, etc.

Figure 4.1.2-1

Broadcast-based V2V communications

### 4.1.3	Vehicle-to-Infrastructure (V2I) application

The UE supporting V2I applications transmits messages containing V2I application information to an RSU or locally relevant application server. The RSU and/or the locally relevant application server transmit messages containing V2I application information to one or more UEs supporting V2I applications.

A locally relevant application server serves a particular geographic area. There can be multiple application servers serving overlapping areas, providing the same or different applications.

### 4.1.4	Vehicle-to-Network (V2N) application

The UE supporting V2N applications communicates with an application server supporting V2N applications. Both parties communicate with each other via EPS.

### 4.1.5	Vehicle-to-Pedestrian (V2P) application

V2P applications expect UEs that are in proximity of each other to exchange V2P application information.  3GPP transport of messages containing V2P application information requires the UE to have a valid subscription and authorization from a network operator. Transport for a valid subscriber is provided whether the UE is served or not served by E-UTRAN.

The UE supporting V2P applications transmits messages containing V2P application information. It is expected that V2P application information can be transmitted either by a UE supporting V2X application in a vehicle (e.g., warning to pedestrian), or by a UE supporting V2X application associated with a vulnerable road user (e.g., warning to vehicle).

3GPP transport of messages containing V2P application information includes the transport between UEs directly and/or, due to the limited direct communication range, the transport between UEs via infrastructure supporting V2X communication, e.g., RSU, application server, etc.

NOTE: 	The main difference between 3GPP transport of messages with V2P and V2V application information is due to the properties of the UE. A UE supporting V2P applications used by pedestrian might, for example, have lower battery capacity, the radio sensitivity might be limited, e.g. due to antenna design, and therefore it may not be able to send messages with the same periodicity as UEs supporting V2V application, and/or receive messages.

{Editor’s note: V2V, V2I, V2P applications are supported by PC5 interface and V2N applications are supported by Uu interface. It may be needed to describe in the overview section}

## 4.2	Use Cases

[Editor’s note: This section summarizes use cases for C-V2X]

{Editor’s note: LTE-V2X and NR-V2X use cases come from 3GPP TR 22.885 and 22.886}

### 4.2.1	3GPP use cases

In the 3GPP TR 22.885 and TR 22. 886 have recommended use cases of C-V2X. they describe C-V2X use cases, pre-conditions, service flows, post-conditions, and potential Requirements.

In Table XX, the use case and the description are listed for TR 22.885

Table XX

Use cases in TR 22.885

| Use cases | Description |
| --- | --- |
| Forward Collision Warning | The FCW application is intended to warn the driver of the HV in case of an impending rear-end collision with a RV ahead in traffic in the same lane and direction of travel. Using the V2V Service, FCW is intended to help drivers in avoiding or mitigating rear-end vehicle collisions in the forward path of travel. |
| Control Loss Warning | The CLW application enables a HV to broadcast a self-generated control loss event to surrounding RVs. Upon receiving such event information, a RV determines the relevance of the event and provides a warning to the driver, if appropriate. |
| V2V Use case for emergency vehicle warning | Emergency vehicle warning service enables each vehicle to acquire the location, speed and direction information of a surrounding emergency vehicle (e.g. ambulance) to assist safety operation like allowing ambulance path to get free. |
| V2V Emergency Stop Use Case | This use case describes vehicles V2V communication used in case of emergency stop to trigger safer behaviour for other cars in proximity of the stationary vehicle. |
| Cooperative Adaptive Cruise Control | This use case describes the scenario whereby a vehicle with V2V capability joins and leaves a group of corporative-adaptive-cruise-control (CACC) vehicles. This provides convenience and safety benefits to participating vehicles and also has societal benefits to improve road congestion and fuel efficiency. |
| V2I Emergency Stop Use Case | This use case describes V2I communication where a Service RSU notifies vehicles in vicinity in case of emergency stop to trigger safer behaviour. |
| Queue Warning | In a lot of situations, a queue of vehicles on the road may pose a potential danger and cause delay of traffic, e.g. when a turning queue extends to other lanes. Using the V2I Service, the queue information can be made available to other drivers beforehand. This minimizes the likelihood of crashes and allows for mitigation actions. |
| Road safety services | V2X messages are delivered from one UE supporting V2I Service to other UEs supporting V2I Service via an RSU which may be installed on the road side. The RSU receives V2X messages transmitted from UEs supporting V2I Service and transmits the received V2X messages to UEs within a local area. A UE receives V2X messages transmitted by the RSU. After processing the received V2X messages, the UE notifies the driver of relevant information. |
| Automated Parking System | The Automated Parking System (APS) contains a database which provides real-time information to vehicles in a metropolitan area on availability of parking spots, be it on the street or in public parking garages. Connected vehicles help maintain the real-time database of the occupancy of parking spaces, which can be accessed by means of smartphones and connected vehicles. APS allows a driver to reserve an available parking space, be guided to it via a navigation application, and make a hands-free payment for parking. |
| Wrong way driving warning | This use case describes V2V communication used between 2 vehicles driving in opposite directions warning wrong way driving and trigger safer behaviour for cars in proximity. |
| V2X message transfer under MNO control | This use case describes the scenario where a given UE supporting V2V Service sends V2X messages to other surrounding UEs and the given UE is under E-UTRAN coverage. |
| Pre-crash Sensing Warning | The pre-crash sensing warning application provides warnings to vehicles in imminent and unavoidable collision by exchanging vehicles attributes after non-avoidable crash is detected. |
| V2X in areas outside network coverage | This use case describes V2X communication when one or more vehicles are located in an area not served by E-UTRAN which supports V2X Service. |
| V2X Road safety service via infrastructure | This use case describes the scenario where infrastructure nodes such as RSUs and traffic safety servers generate and distribute traffic safety-related messages for road safety. |
| V2N Traffic Flow Optimisation | This use case describes vehicles V2N (Vehicle-to-Network) communication to a centralised ITS server referred here to as “entity” to optimise traffic flow when approaching intersections. This use case addresses the situation when approaching the vehicle has to stop even though there are no other cars around at an intersection or has to slow down because of explicit traffic lights signal absence. Depending on the traffic situation which is based on the vehicles' periodically transmitted messages this entity will provide, via LTE network entity, a green light to a car when approaching the intersection and an indication of speed at which the green light will be met without having to stop or miss the green light phase. |
| Curve Speed Warning | Curve speed warning application alerts the driver to manage the curve at an appropriate speed. |
| Warning to Pedestrian against Pedestrian Collision | This use case is to provide information to vulnerable road users, e.g. pedestrian or cyclist, of the presence of moving vehicles in case of dangerous situation. As a result, warnings are provided to vulnerable road users to avoid collision with the moving vehicle. |
| Vulnerable Road User (VRU) Safety | This use case describes the scenario whereby a vehicular and a pedestrian are both equipped with V2P capabilities, and the vehicle detects the pedestrian's presence and alerts the driver, if an imminent threat is present. This capability extends the safety benefit of V2X to pedestrians and other vulnerable road users, e.g. bicyclists, wheelchair users, etc. |
| V2X by UE-type RSU | This use case describes the scenario where UE supporting V2X discovers and communicates with UE-type RSU. |
| V2X Minimum QoS | This use case describes the scenario where E-UTRA(N) resource is not enough for every UEs 10 Hz V2X message transmission. In addition, this use case includes the scenario where emergency vehicle is supported. |
| Use case for V2X access when roaming | Mary is taking a road trip across the country. She has a car equipped with V2X capability, with service from her home network operator. On her journey, Mary encounters a traffic jam in town not served by her home network provider. The town has deployed V2X capabilities to redirect traffic jams caused by a major infrastructure construction project. The V2X capabilities must be able to communicate with devices associated with multiple service providers. |
| Pedestrian Road Safety via V2P awareness messages | A pedestrian carries a UE, which is able to transmit awareness and safety related V2P broadcast messages. |
| Mixed Use Traffic Management | There are a number of variables to be taken into account in a scenario involving different types of vehicular traffic. |
| Enhancing Positional Precision for traffic participants | To obtain their position vehicles usually use a GNSS such as GPS, Galileo, Beidou, and Glonass. However, the publicly available precision for a position fix for the most common system GPS is just around 15 m, better values can be obtained and dependent on the radio conditions and are thus not guaranteed. |
| Privacy in the V2V communication environment | The privacy or anonymity in the V2V communication environment is a requirement deemed very important for user adoption of the V2V system. |
| V2N Use Case to provide overview to road traffic participants and interested parties | This use case describes a general use case for V2N communication that exercises the strength of 3GPP networks of providing excellent coverage. |
| Remote diagnosis and just in time repair notification | A road side unit (RSU) having the capability to access an internet will enable any passing by vehicle to report about its current functional state to a local/remote diagnosis center and to receive “Just in time repair notification” if having subscribed to such service. |

In 3GPP Rel. 15, eV2X use cases had been developed and are recommended in TR. 22.886. In Table YY, the use cases and description of TR22.886 are listed.

Table YY

Use cases in TR 22.886

| Use cases | Description |
| --- | --- |
| eV2X support for vehicle platooning | Platooning is operating a group of vehicles in a closely linked manner so that the vehicles move like a train with virtual strings attached between vehicles. |
| Information exchange within platoon | When the vehicles are travelling on the road, they can dynamically form a platoon. The platoon creator is responsible for platoon management. |
| Automotive: sensor and state map sharing | Sensor and state map sharing (SSMS) enables sharing of raw or processed sensor data to build collective situational awareness. |
| eV2X support for remote driving | Remote driving is a concept in which a vehicle is controlled remotely by either a human operator or cloud computing. |
| Automated cooperative driving for short distance grouping | Cooperative driving allows a group of vehicles to automatically communicate to enable lane changing, merging, and passing between vehicles of the group and inclusion/removal of vehicle in the group in order to improved safety and fuel economy. |
| Collective perception of environment | Vehicles can exchange real time information (based on vehicle sensors information or sensor data from a capable UE-type RSU) among each other in the neighbour area. |
| Communication between vehicles of different 3GPP RATs | Depending on the choice of OEMs, while some vehicles are equipped with modules supporting only LTE, other vehicles may be equipped with modules supporting NR (New Radio). If a vehicle of NR cannot talk to a vehicle supporting LTE, the vehicle supporting LTE can be regarded as another vehicle of no V2X capability. |
| Multi-PLMN environment | Although required communication condition, e.g. for immediate message transfer, is set high for some of eV2X use cases, such condition needs to be met regardless of whether or not all the involved UEs and UE-type RSUs are subscriber of the same PLMN |
| Cooperative collision avoidance (CoCA) of connected automated vehicles | To enable vehicles to better evaluate the probability of an accident and to coordinate manoeuvres in addition to usual CAM, DENM safety messages, data from sensors, list of actions like braking and accelerating commands, lateral as well as longitudinal control are exchanged amongst vehicles to coordinate in the application the road traffic flow through 3GPP V2X communication. |
| Information sharing for partial/ conditional automated driving | This use case is interpreted as an automated driving at the level of SAE Level 3 and Level 2 automation. |
| Information sharing for high/full automated driving | This use case is interpreted as an automated driving at the level of SAE Level 4 and Level 5 automation. |
| Information sharing for high/full automated platooning | This use case is interpreted as an automated driving at the level of SAE Level 4 and Level 5 automation. |
| Dynamic ride sharing | This use case enables a vehicle to advertise willingness to share capacity with another road user and for a pedestrian to indicate intent to travel in a ride share. |
| Use case on multi-RAT | The user starts a V2X application, and a message from that application needs to be transmitted to other cars nearby. |
| Video data sharing for assisted and improved automated driving (VaD) | The visual range of the driver is in some road traffic situations obstructed, for instance by trucks driving in front [26]. Video data sent from one vehicle to the other can support drivers in these safety-critical situations. |
| Changing driving-mode | According to a vehicle cooperation level, driving-mode can be classified generally into three classes. |
| Tethering via Vehicle | This use case enables a vehicle to provide network access to occupants, pedestrians etc. |
| Use case out of 5G coverage | A UE supporting V2X application is equipped with a multi-RAT modem (5G, LTE). |
| Emergency trajectory alignment | Emergency Trajectory Alignment (EtrA) messages complement cooperative automated driving. |
| Teleoperated support (TeSo) | While traffic safety as well as accident-free driving is the task of each connected autonomous vehicle, |
| Intersection safety information provisioning for urban driving | The traffic accident occurs at the intersection where the vehicle and pedestrians are crowded. |
| Cooperative lane change (CLC) of automated vehicles | On a multi-lane road, a lane change manoeuvre could be initiated by a vehicle. |
| Proposal for secure software update for electronic control unit | A car Electronic Control Unit (ECU) is a generic term for a software module that controls the electronics within a car system; |
| 3D video composition for V2X scenario | This use case consists of multiple UEs supporting V2X application moving in an area. |
| QoS aspects of advanced driving | Based on the implementation or approach taken by each manufacturer or the environment where each vehicle is located, whether to engage automated driving or not needs to be controlled by the V2X service |
| QoS aspect of remote driving | Remote Driving applications allow a remote driver that is not sitting in the vehicle to undertake the control of the vehicle and drive remotely the vehicle, in an efficient and safe manner, from the current location to the destination. |
| QoS Aspect for extended sensor | The extended sensor use cases are composed of sensor data collection to construct local dynamic map and the state map sharing, sensor data shared to extend sensor range, different all round video data shared for automatic drive. |
| Different QoS estimation for different V2X applications | QoS estimation will help V2X applications e.g. automation driving, intelligent traffic system, to get the communication system connection capability in advance which is very important for them to compute and adjust in advance to right working mode to guarantee safety and service availability |

## 4.2	Use cases

[Editor’s note: This section addresses potential use cases of C-V2X by IMT systems.]

The typical C-V2X use cases can be divided into information services, traffic efficiency, driving safety from usage object perspective. And among them, some use cases are not only for one object, it is also aiming for multiple objects.

With the evolution from vehicle-road to vehicle-road- pedestrian -cloud coordination, it is expected that during future 1-3 years, the following C-V2X typical use cases will be fully developed.

Note: In the following use cases, the mentioned IMT system includes two kinds of wireless connections: Uu and sidelink.

### 4.2.1	Information service

#### 4.2.1.1	OTA (Over The Air)

The use case is based on the V2X OTA platform and IMT system. When the Uu signal is good, the vehicle obtains the upgrade software package through the wireless network; when the Uu signal is poor; the vehicle obtains the upgrade software package through sidelink connection from the proximity RSU which is the smart roadside equipment and can be upgraded with V2N.

OTA through sidelink connection, on the one hand, can shorten the time for downloading software packages; on the other hand, it can extend the coverage scope of the upgrade.

#### 4.2.1.2	In-Vehicle Entertainment (IVE)

The use case concerns the entertainment content delivery to the passengers of a moving or stationary vehicle. It is applicable to both automated and non-automated vehicles, where in the latter the driver is restricted in the contents she or he is allowed to consume. The contents may include video, gaming, virtual reality (VR), office work, online education, advertisement, etc.

#### 4.2.1.3	High definition map collecting & sharing

The use case is for vehicles to get HD map which is real-time updated and more accurate. Vehicles equipped with LIDAR or other HD sensors can collect environment around themselves, and share the information with a HD map provider (e.g., cloud server). The HD map provider analyzes the collected information and merges or combines them to build a regional HD map. This allows the construction of HD maps that are dynamically updated and also with more accurate information.

#### 4.2.1.4	ETC (Electrnic Toll Collection)

This use case is proposed a new service method that utilizes the on-board unit (OBU) as the payment terminal, through sidelink connection of IMT system, to pay for the expenses incurred by the vehicle on the road and the goods or services consumed by the vehicle owner.

### 4.2.2	Traffic efficiency

#### 4.2.2.1	Vehicle Inbound and outbound

Vehicle inbound and outbound is suitable for vehicle entrance or exit on highways, expressways and other road sections, as well as lane changing scenario. It can assist the road management. Under the premise of ensuring safety, by selecting reasonable time, position and speed of inbound and outbound and lane changing, the impact of vehicles on the main line traffic flow is reduced, and the traffic efficiency of the road is improved.

When the host vehicle (HV) and the remote vehicle (RV) are located on both sides of the ramp entrance/exit respectively:

–	in the case of the roadside unit (RSU) and multi-edge computing platform (MEC) are deployed:

•	MEC and RSU calculate the driving policy and RSU delivers the instructions to guide the vehicles on both sides through sidelink connection of IMT system. HV and RV then follow the instructions;

•	RSU delivers the perception information of the driving environment through sidelink connection of IMT system, and HV and RV make their own decisions based on the perception information received.

–	In the case of no RSU deployed, HV and RV transmit vehicle information to each other through sidelink connection of IMT system. The vehicle calculates the driving policy by itself and broadcasts the result.

#### 4.2.2.2	Intersection traffic

In intersection traffic use case, the host vehicle (HV) drives to the intersection. The HV sends vehicle driving information to the corresponding V2X application server (either in the MEC or in the cloud). The V2X server generates traffic dispatch information for the HV according to the received vehicle information, the traffic control phase information of the target intersection, the driving information reported by other vehicles, and the perception information reported by the roadside sensors, etc. and send the traffic dispatch information to the HV;

Or HV obtains the perception information of roadside sensors, other vehicle information, and cloud information of the V2X server through IMT system, and generates scheduling information by itself.

Then the HV controls the HV to pass through the intersection according to the traffic dispatch information, combined with the surrounding environment information sensed by the V2X function and other on-board sensors.

In this use case, the vehicle reasonably adjusts its driving state according to surrounding vehicle information, traffic light information, road condition information, maps and other information to avoid idle roads and wasting unnecessary time. Under the condition of ensuring safety, the traffic efficiency of intersections is maximized.

#### 4.2.2.3	Dynamic Lane Management

This application is aiming for release the congestion problem of intersections. The dynamic lane division at the intersection is utilized to realize the real-time and reasonable allocation of the space resources of the intersection entrance lane. Through the real-time communication between the vehicles within the intersection range and the roadside unit (RSU), the roadside unit collects the status data of the connected vehicle (CV, Connected Vehicle), including position, speed, steering, etc., to determine the traffic demand in each direction of the intersection in real time. Based on this, the edge computing plateform (MEC) adjusts the lane function of the entrance lane, and sends the result to the vehicle, which improves the operation efficiency of the intersection through dynamic lane management.

It is mainly for the scenarios where the intersection demand changes frequently and fluctuations in each flow direction are large. Compared with the existing fixed lane management method, dynamic lane management can match the traffic demand of each flow direction in real time, provide sufficient queuing space for each flow direction vehicle to the greatest extent, and reduce the queue length and the number of secondary queuing.

#### 4.2.2.4	Dynamic optimization of traffic signal timing

It refers to the V2X service server at the intersection that optimizes the phase duration or phase sequence of signal timing based on real-time networked vehicle data. If possible, it can be optimized based on the background data and plan of the central subsystem.

It is suitable for signal control optimization of signal control intersections and signal control ramp entrances of ordinary roads and highways in cities and suburbs.

#### 4.2.2.5	Flexible management of dedicated highway lanes

In order to meet the rapid traffic demand of emergency vehicles, a dynamic dedicated lane is generated to improve the travel condition of emergency vehicles. When an emergency vehicle drives on the dedicated lanes, it broadcasts the current status and clearing distance, in some case, RSU can either broadcasts the occupancy status of the section. After the nearby vehicles receive the relevant information, it will detect whether it is within the clearing distance of the emergency vehicle, if it is yes, it will leave the dedicated lane. This application is suitable for road traffic management of highways, expressways and other sections.

#### 4.2.2.6	Group start

Self-driving or semi-automated vehicles form a group to jointly start at traffic light. A traffic control center provides tactical and strategic information to coordinate the activity.

A traffic control center identifies several vehicles which intend to cross an intersection on a similar path at similar time. It assigns the role of the group leader to the first vehicle approaching the intersection and instructs this vehicle and other participating vehicles about which spot at the intersection to take. The traffic control center takes vehicle capabilities in terms of dynamics, sensor capabilities, planned route, and communications properties into account when compiling the groups.

#### 4.2.2.7	Fleet management

In this application, the leading vehicle is usually in the front of the fleet which obtains information such as traffic environment, information from the V2X service cloud and surrounding vehicles to form a fleet driving strategy, thereby implements the dynamic management of the entire fleet and ensuring safe and efficient travel of the fleet. This application is suitable for urban and suburban roads under network coverage.

#### 4.2.2.8	Platooning

Platooning is to connect trucks traveling in the same direction through wireless technology. The leading vehicle is responsible for fleet management and driving. The trailing vehicle can receive driving information such as acceleration, braking of the preceding vehicle, and improve driving efficiency. Usually the leader of the fleet is driven by a man, followed by multiple unmanned member vehicles based on real-time information interaction and maintaining a stable inter-vehicle distance at a certain speed.

#### 4.2.2.9	Real-time Navigation

It provides accurate, real-time, and efficient travel path planning for traveling vehicles through IMT communication system, including driving sections, driving lanes, etc., thus traveling vehicles can drive on the "best" route, improving travel efficiency. It can also provide real-time Eco-driving suggestions and parameters such as vehicle speed, real-time acceleration, optimized following plan, and better lane change plan which enable traveling vehicles to drive in the "best" driving mode and improve the efficiency of vehicle energy consumption and travel safety. On the whole, it can optimize the overall and local transportation system to enhance user experience.

When traveling with an electric vehicle (EV), it also need to consider battery power, starting point and destination location, charging station (CS) information, and traffic information to plan and dynamically adjust the travel route and charging route of the electric vehicle.

### 4.2.3	Driving safety

#### 4.2.3.1	VRU recognization

Vulnerable Road User recognization refers to recognize VRU nearby and warn traffic accidents as earlier as possible. The types of VRUs are including pedestrians, cyclists, and power-driven two-wheelers etc.

According to the communication capability the VRU has, it can be divided into the following categories:

1	VRU with communication capability;

2	VRU without communication capability.

For different VRUs, there are several ways (not limited to) for recognization and warning:

1	VRU with communication capability: The VRU actively sends its own information, then Vehicle nearby and RSU receive the information to recognize the VRU and detect the VRU risk.

2	VRU without communication capability:

•	Vehicles equipped with sensors (such as cameras, radars, lidars, etc.) perform VRU recognization based on the data collected by the sensors, and send relevant VRU recognization information to surrounding vehicles and RSUs;

•	For RSU equipped with sensors, if it has the ability to analyze perceptual data, it can perform VRU recognization based on the data collected by the sensors and send relevant VRU recognization information to surrounding vehicles;

•	MEC performs VRU recognization based on the data collected by RSU sensors, and then RSU sends relevant VRU recognization information to surrounding vehicles.

### 4.2.3.2	See-through for passing

Driver of Host Vehicle that signals an intention to pass a Remote Vehicle (RV) using the oncoming traffic lane is provided a video stream showing the view in front of the RV.

HV approaches from behind or follows RV with the intention to pass using the oncoming lane. Video stream of the front view of RV is shown to the HV driver during the passing maneuver.

### 4.2.3.3	Obstructed view assist

This case is usually happened in the intersections, e.g., in rural and urban environments. Host Vehicle (HV) is provided an alternate when faced with an obstructed view. A video stream is provided to HV, circumventing the obstructed view from the HV perspective.

### 4.2.3.4	High definition sensor sharing

Vehicle uses its own sensors (e.g. HD camera, lidar), and sensor information from other vehicles, to perceive its environment (e.g. come up with 3D model of world around it) and safely performs an automated driving lane change.

### 4.2.3.5	Infrastructure assisted environment perception

When an automated vehicle enters a section of the road that is covered by infrastructure sensors it enrols to receive information from the infrastructure containing environment data of dynamic and static objects on the road. This data is used to increase the trust level of the cars own sensor observations and extends its viewing range.

### 4.2.3.6	Tele-operated driving

A remote driver undertakes the control of the vehicle and drives remotely the vehicle, in an efficient and safe manner, from the current location to the destination. Based on perceived environment the remote driver provides to the vehicle that is remotely driven the appropriate trajectory and maneuver instructions for the efficient and safe navigation to the destination.

### 4.2.3.7	Autonomouse parking

In this use case, when the vehicle arrives at the entrance of the parking lot, with IMT assistance, it can drive autonomously and drive into the right parking space. It includes functions such as the remote driving into the parking space, auto-driving into the parking space by itself, and obtaining parking space information and parking space status information, applying for parking space and parking navigation etc.

# 5	Relationship between IMT and C-V2X

[Editor’s note: This section addresses technical and operational characteristics  for C-V2X by IMT systems, based on information from ITU-R members and external organizations, with making clear the relationship between them.]

# 6	Characteristics and capabilities of C-V2X supported by IMT

[Editor’s note: This section summarizes service requirements, technical and operational characteristics and operation spectrum for C-V2X in several groups and organizations outside ITU-R as well as ITU-R Members.]

## 6.1	Service Requirements

[Editor’s note: This section addresses service requirements of C-V2X by IMT systems.]

### 6.1.1	V2X Service Requirements

[Editor’s note: Service requirements for V2X Services; TS 22.185]

[Editor’s note: The below paragraph is a summary of servies requirements of LTE-V2X]

To satisfy the V2X service requirements, there are overall, specific service, and security requirements. For the specific service requirements, there are latency/Reliability, Message size, Frequency, range, and speed requirements.

### 6.1.2	Enhanced V2X Service Requirements

[Editor’s note: Service requirements for enhanced V2X Services; TS 22.186]

Different V2X scenarios require the transport of V2X messages with different performance requirements for the 3GPP system. This Technical Specification specifies service requirements to enhance 3GPP support for V2X scenarios in the following six areas:

−	General Aspects: interworking, communication-related requirements valid for all V2X scenarios

−	Vehicles Platooning

−	Advanced Driving

−	Extended Sensors

−	Remote Driving

−	Vehicle quality of service Support

Vehicles Platooning enables the vehicles to dynamically form a group travelling together. All the vehicles in the platoon receive periodic data from the leading vehicle, in order to carry on platoon operations. This information allows the distance between vehicles to become extremely small, i.e., the gap distance translated to time can be very low (sub second). Platooning applications may allow the vehicles following to be autonomously driven.

Advanced Driving enables semi-automated or fully-automated driving. Longer inter-vehicle distance is assumed. Each vehicle and/or RSU shares data obtained from its local sensors with vehicles in proximity, thus allowing vehicles to coordinate their trajectories or maneuvers. In addition, each vehicle shares its driving intention with vehicles in proximity. The benefits of this use case group are safer traveling, collision avoidance, and improved traffic efficiency.

Extended Sensors enables the exchange of raw or processed data gathered through local sensors or live video data among vehicles, RSUs, devices of pedestrians and V2X application servers. The vehicles can enhance the perception of their environment beyond what their own sensors can detect and have a more holistic view of the local situation.

Remote Driving enables a remote driver or a V2X application to operate a remote vehicle for those passengers who cannot drive themselves or a remote vehicle located in dangerous environments. For a case where variation is limited and routes are predictable, such as public transportation, driving based on cloud computing can be used. In addition, access to cloud-based back-end service platform can be considered for this use case group.

Vehicle quality of service support enables a V2X application to be timely notified of expected or estimated change of quality of service before actual change occurs and to enable the 3GPP System to modify the quality of service in line with V2X application’s quality of service needs. Based on the quality of service information, the V2X application can adapt behaviour to 3GPP System’s conditions. The benefits of this use case group are offerings of smoother user experience of service.

There are requirements for General, Vehicles Platooning, Advanced Driving, Extended Sensors, Remote Driving, and Vehicle Quality of Service in {Editor’s note: it needs the reference}.

## 6.2	IMT capabilites

[Editor’s note: This section addresses capabilities to meet the service, technical and operational requirements for C-V2X by IMT systems, based on information from ITU-R members and external organizations.]

### 6.2.1	Radio Access Network

[Editor’s note: This section comes from TR 37.985]

The LTE V2X sidelink supports broadcast transmission of messages in the physical layer, since this is a suitable approach for delivery BSM, CAM, DENM and similar traffic. In the MAC layer, a broadcast address can be mapped to a single UE or a group of UEs by implementation. Such implementation techniques have no particular specification support in LTE, and are transparent to the physical layer.

Whereas LTE-V2X supports broadcast transmission in the physical layer and any finer-grained addressing is handled according to MAC layer ID implementation, NR V2X has physical layer support for broadcast, unicast, and groupcast sidelink operation. The addition of unicast and groupcast is linked with the introduction of sidelink HARQ feedback [Editor’s note: TR 37.985 (clause 6.2.4)], high order modulation, sidelink CSI, and PC5-RRC [Editor’s note: TR 37.985(clause 6.5.7)], amongst other points.

The resource allocation, congestion control, and higher-layer protocols are described in [Editor’s note: TR 37.985]

### 6.2.2	Architecture Enhancements

[Editor’s note: This section comes from TS 23.285 and TS 23.287]

There are two modes of operation for V2X communication, namely V2X communication over PC5 reference point and V2X communication over Uu reference point. These two operation modes may be used by a UE independently for transmission and reception.

V2X communications over PC5 reference point are supported by LTE and/or NR.

V2X communications over Uu reference point are supported by E-UTRA connected to 5GC and/or NR connected to 5GC. In this release, V2X communication over Uu reference point is only unicast.

The Architecture reference models for LTE-V2X are

1	PC5 and LTE-Uu based V2X architecture reference model

a)	Non-roaming architecture for PC5 and LTE-Uu based V2X communication

b)	Roaming architecture for PC5 and LTE-Uu based V2X communication

c)	Inter-PLMN architecture for PC5 and LTE-Uu based V2X communication

2	MBMS for LTE-Uu based V2X architecture reference model

And further, for the 5G-V2X, the architecture reference models are

1	PC5 and Uu based V2X architecture reference model

a)	Non-roaming 5G System architecture for V2X communication over PC5 and Uu reference points

b)	Roaming 5G System architecture for V2X communication over PC5 and Uu reference points

c)	Inter-PLMN 5G System architecture for V2X communication over PC5 reference point

2	AF-based service parameter provisioning for V2X communications

3	interworking with EPS V2X

The functional entities, functionality and features are described in {Editor’s note: TS 23.285 and TS 23.286}

### 6.2.3	Application Layer Support

In order to ensure efficient use and deployment of V2X applications on 3GPP networks an architecture for V2X application layer consisting of V2X application enabler should be specified {Editor’s note: TS 23.286}.

The V2X application enabler capabilities takes into consideration for

- specifies the functional architecture, procedures and information flows for V2X application enabler layer

- capabilities of the application layer support for V2X services that are necessary to ensure efficient use and deployment of V2X services over 3GPP systems

- The VAE capabilities applies to EPS.

## 6.3	Operational Spectrum

[Editor’s note: This section summarizes information on operational spectrum for C-V2X, based on input contributions.]

LTE-V2X operation is defined in band 47 in TS 36.101 [11, clause 5.5G], which supports single-carrier and multi-carrier operation: It is summarized in Table KKK.

Table KKK

LTE V2X operating band

| E-UTRA Operating Band | E-UTRA V2X Operating Band | V2X UE transmit |  |  | V2X UE receive |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| 47 | 47 | 5 855 MHz |  | 5 925 MHz | 5 855 MHz |  | 5 925 MHz | HD | PC5 |

NR V2X communication is designed to operate in the operating bands in FR1 defined in Table ZZZ.

Table ZZZ

NR V2X operating bands in FR1

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n381 | 2 570 MHz | - | 2 620 MHz | 2 570 MHz | - | 2 620 MHz | HD | PC5 |
| n47 | 5 855 MHz | - | 5 925 MHz | 5 855 MHz | - | 5 925 MHz | HD | PC5 |
| Note 1: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. |  |  |  |  |  |  |  |  |

{Editor’s note: Uu interface band consideration is needed in this section}

### 6.3.1	Uu interface

### 6.3.2	PC5 interface

# 7	Case Study

## 7.1	Country AAA

## 7.2	Country BBB

# 8	Summary

[Editor’s note: To be described.]

____________________