3GPP TSG RAN meeting #91e										RP-210677

Electronic Meeting, March 16-26, 2020

## Status Report to TSG

Agenda item: 9.2.7

| WI / SI Name | NR RRM enhancement in R16 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_Enh-Perf |  |  |  |  |
| Unique ID | 840195 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201883 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2020 | Performance part: 03/2021 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 100% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Hua Li |
|  | Company | Intel Corporation |
|  | Email | hua.li@intel.com |
| Rapporteur | Name | Richie Leo |
|  | Company | ZTE Corporation |
|  | Email | richie.leo@zte.com.cn |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple Inc. |
|  | Email | jie_cui@apple.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN4

#### 2.1.1	Agreements

RAN4 #98-e meeting (Feb 2021)

- SRS carrier-based switching requirements

The following agreements were reached and captured in WF R4-2104070:

- Issue 2-1-1: FR1 + FR2 test cases

  - Agreement: Further check the feasibility of SRS carrier-based switching between different frequency ranges. Do not define test cases for SRS carrier-based switching between FR1 and FR2 in Rel-16.

- Issue 2-1-2:

  - Agreement: Do not specify UE type in the tests.

- Issue 2-1-3:

  - Agreement: Set the parameter usage to ‘antennaSwitching’ in SRS configuration. The parameter applies to aperiodic SRS case only.

- Issue 2-1-4:

  - Agreement: There is no need to add additional FR1+FR2 CA tests for

    - E-UTRAN – NR interruptions at NR SRS carrier based switching

      - NR PSCell in FR1, NR SCell in FR1, adding an NR SCell in FR2

      - NR PSCell in FR2, NR SCell in FR2, adding an NR SCell in FR1

    - E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching

      - E-UTRA SCell, NR PSCell in FR1, adding an NR SCell in FR2

      - E-UTRA SCell, NR PSCell in FR2, adding an NR SCell in FR1

- Issue 2-1-5：

  - Agreement: Keep previous definition for TC1 and TC2.

    - TCs of SA interruptions at NR SRS carrier based switching:

      - TC1: Do not add FR2 Scell. Keep the configurations as agreed in RAN4#97e: PCell in FR1, SCell in FR1.

      - TC2: Do not add FR1 Scell. Keep the configurations as agreed in RAN4#97e: PCell in FR2, SCell in FR2

- Issue 2-1-6:

  - Agreement: Specify SRS configuration in Annex A.3.

The following CRs / Draft CRs were agreed/endorsed:

  - R4-2103614 TC4: E-UTRAN – NR interruptions at NR SRS carrier based switching (PSCell in FR2, SCell in FR2)

  - R4-2103615 CR to 38.133 on SRS configuration

  - R4-2104085 TC2 - SA interruptions at NR SRS carrier based switching

- CGI reading requirements with autonomous gap

The following agreements were reached and captured in WF R4-2104070:

  - Value for T321 in CGI reading with autonomous gap is 200ms.

  - Do not count LTE power off time in CGI reading delay in both core requirements and test requirements.

  - Only interrupted slots are specified in test requirements of CGI reading test cases.

The following Draft CR / LS was endorsed/approved:

  - R4-2103617	draftCR to CGI reading TC4

  - R4-2103610	LS on CGI reading with autonomous gaps

- FR2 Inter-band CA RRM

The following agreements were reached and captured in WF R4-2103620:

- Issue 2-1-1: scheduling restriction during RLM

  - Agreement: For RLM, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

- Issue 2-1-2: Removal the description for the case when no measurement restriction is allowed

  - Agreement: Keep the descriptions on FR2 SSB BFD/CBD/L1-RSRP for the cases with “no measurement restriction” in the spec.

- Issue 6-1: AoA setup in TCs of inter-band CA requirement for FR2 UE measurement capability of independent Rx beam

  - Agreement: AoA configuration for both Cell 1 and Cell 2 = Setup 3 (with rough beams) and FFS on the other parameter adjustment (e.g. Noc, Es, Es/Iot and etc) in the test cases (R4-2103628).

The following CRs / Draft CRs were agreed/endorsed:

- R4-2100181 CR on maintenance for inter-band FR2 CA RRM R16

- R4-2100182 CR on maintenance for inter-band FR2 CA RRM R17

- R4-2103623 Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16

- R4-2100779 Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R17

- R4-2103628 DraftCR on SCell activation and deactication delay test for FR2 inter-band CA

- Inter-frequency measurement without MG

The following agreements were reached and captured in WF R4-2103620:

- Issue 4-1: Power imbalance condition for inter-frequency measurement requirement without MG

  - Agreement: Do not define conditions

The following CRs / Draft CRs were agreed/endorsed:

- R4-2101220 CR on TS38.133 for inter-frequency measurement requirement without gap R17

- R4-2103624 CR on TS38.133 for inter-frequency measurement requirement without gap R16

- R4-2101693 Correction on inter-frequency measurement without gap test case

- Multiple SCell activation

The following agreements were reached and captured in WF R4-2103620:

- Issue 1-1-1: The RTD (reception timing difference) with contiguous FR1 known cell or active serving cell to the to-be-activated SCell

  - Agreement: The RTD (reception timing difference) with contiguous FR1 known cell or active serving cell to the to-be-activated SCell shall be no larger than 260ns

- Issue 1-1-2: The RPD (reception power difference) with contiguous FR1 known cell or active serving cell to the to-be-activated SCell.

  - Agreement: The RPD (reception power difference) with contiguous FR1 known cell or active serving cell to the to-be-activated SCell shall be no larger than 6dB.

- Issue 1-2-1: Extend the assumption in FR1 multiple SCells activation to single FR1 SCell activation

  - Agreement: Extend the UE requirement (to skip cell detection for unknown FR1 SCell that is intra-band contiguous to active or known serving cell) to single SCell activation.

- Issue 1-2-2: Requirement applicability on the other being-activated SCells during the FR1 multiple SCells activation

  - Agreement: No requirement applies for a FR1 unknown SCells being activated, if no requirements apply for one or more of the FR1 unknown SCell(s) being activated with the same MAC CE

- Issue 1-2-5: Condition of ssb-PositionsInBurst to apply FR2 multiple SCell activation requirement

  - Agreement: Multiple SCell activation requirements for FR2 unknown SCells apply provided that the parameter ssb-PositionsInBurst is same for the known serving cell(s) and the target SCell.

- Issue 7-1: removal of TCs for multiple SCell activation due to the testability of simultaneous FR1+FR2 links

  - Agreement: Keep TC2 but remove the interruption test part for LTE and FR1 PSCell in TC2 and remove TC3 at this stage until the testability of simultaneous FR1+FR2 is clarified.

The following CRs / Draft CRs were agreed/endorsed:

- R4-2102790 CR on multiple SCell activation requirements R17

- R4-2103621 CR on multiple SCell activation requirements

- R4-2103626 draftCR on multiple SCell activation test case

- UE specific CBW change

The following CRs / Draft CRs were agreed/endorsed:

- R4-2100184 CR on UE behavior for UE specific CBW change R17

- R4-2103627 draftCR on the CBW change test case for adding the UL CBW configuration

- BWP switching on multiple CCs

The following agreements were reached and captured in WF R4-2103619:

- Scenario for RRC based BWP switch on multiple CCs

  - Wait for the feedback from RAN2.

- Clarification for N for DCI-based simultaneous BWP switch on multiple CCs

  - A clarification is added to the description of N:

    - The number of CCs, N, on which the UE can simultaneously switch BWPs, while still meeting the requirements, if any, related to allocations on downlink, uplink, or transmission of HARQ-ACK, depend on the UE reported capabilities related to BWP switching, the network configuration and the BWP switch method.

- Whether to define DCI/Timer based FR1+FR2 simultaneous BWP switch test case

  - Do not define DCI/Timer based FR1+FR2 simultaneous BWP switch test case

- Whether to define DCI/Timer based non-simultaneous BWP switch test case

  - Not to define DCI/Timer based non-simultaneous BWP switch test case.

- TC4: DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR2 in SA

  - Cell configuration: NR FR2 PCell + NR FR2 SCell + NR FR2 SCell

The following CRs / Draft CRs were endorsed:

- R4-2101631 CR on interruption requirements of BWP switch on multiple CCs in TS36.133

- R4-2103602 CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs

- R4-2102722 CR on maintenance on BWP switch requirements on multiple CCs

- R4-2101390 CR for test cases for simultaneously DCI/timer based bwp switch over mulitple CCs on FR1 in SA

- R4-2101412 CR on simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs on FR1 in EN-DC (section 4.5.6.x)

- R4-2103605 draftCR to introduce Active BWP Switch on multiple CCs TC2

- R4-2103606 DraftCR 38.133 (A.7.5.X) TC4 BWP switching

- UL Spatial Relation Info Switching requirement

The following agreements were reached and captured in WF R4-2103619:

- Known condition definition update to include PL-RS

  - Do not update known condition

The following CRs / Draft CRs were endorsed:

- R4-2103603 UL spatial relation switching to an unknown DL RS

- R4-2101696 Correction on uplink spatial relation switching test case

- R4-2103607 Test case for RRC based UL spatial relation switch associated with a known DL-RS in SA for periodic SRS

#### 2.1.2	Remaining Open issues

1) Mandatory gap pattern

- Whether R16 UEs can skip some of R15 test cases

2) BWP switching on multiple CCs

- Whether to define RRC based BWP switch on multiple CCs test case. To be discussed after receiving RAN2 LS response and further clarifications in Rel-15 specifications.

Rapporteur note: Remaining issues can be discussed in the maintenance stage.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #98e meeting

- R4-2103700	Email discussion summary for [98e][218] NR_RRM_Enh_RRM_1	Moderator (Intel Corporation)

- R4-2103701	Email discussion summary for [98e][219] NR_RRM_Enh_RRM_2	Moderator (ZTE Corporation)

- R4-2103702	Email discussion summary for [98e][220] NR_RRM_Enh_RRM_3	Moderator (Apple)

- R4-2103619	WF on R16 RRM enhancement part 1	Intel Corporation

- R4-2104070	WF on R16 RRM enhancement part 2	ZTE Corporation

- R4-2103620	WF on R16 RRM enhancement part 3	Apple

- R4-2103610      LS on CGI reading with autonomous gaps ZTE Corporation

- R4-2100052      CGI reading with autonomous gaps ZTE Corporation

- R4-2100181      CR on maintenance for inter-band FR2 CA RRM R16  Apple

- R4-2100182      CR on maintenance for inter-band FR2 CA RRM R17 Apple

- R4-2100183      CR on UE behavior for UE specific CBW change R16 Apple

- R4-2100184      CR on UE behavior for UE specific CBW change R17 Apple

- R4-2100213      Remaining issues for UL spatial relation switching requirements Apple

- R4-2100214      CR to 38.133 on UL spatial relation switch requirements (R16) Apple

- R4-2100215      CR to 38.133 on UL spatial relation switch requirements (R17) Apple

- R4-2100225      Test applicability for mandatory gap patterns Apple

- R4-2100226      TC4: E-UTRAN Apple

- R4-2100623      CGI reading discussion T321 Qualcomm, Inc.

- R4-2100624      SRS carrier switching discussion Qualcomm, Inc.

- R4-2100625      Mandatory MG applicability rule discussion Qualcomm, Inc.

- R4-2100626      Mandatory MG applicability rule CR Qualcomm, Inc.

- R4-2100627      Inter frequency w/o gap power imbalance discussion Qualcomm, Inc.

- R4-2100777      Discussion on Inter-band CA requirement for FR2 MediaTek inc.

- R4-2100778   Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16 MediaTek inc.

- R4-2100779   Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R17 MediaTek inc.

- R4-2100860      Discussion on test cases for mandatory MG patterns CMCC

- R4-2101058      Remaining Issues on multiple SCell Activation MediaTek inc.

- R4-2101059      draftCR on multiple SCell activation test case MediaTek inc.

- R4-2101060      Discussion on multiple BWP switch test case MediaTek inc.

- R4-2101070      draftCR on the CBW change test case for adding the UL CBW configuration MediaTek inc.

- R4-2101219      CR on TS38.133 for inter-frequency measurement requirement without gap MediaTek inc.

- R4-2101220 	CR on TS38.133 for inter-frequency measurement requirement without gap MediaTek inc.

- R4-2101378	 Further considerations on test cases for BWP switch over multiple CCs vivo

- R4-2101390 	CR for test cases for simultaneously DCI/timer based bwp switch over mulitple CCs on FR1 in SA vivo

- R4-2101408 	Discussion on RRC based BWP switching on multiple CC Intel Corporation

- R4-2101409	CR on RRC based BWP switching on multiple CCs (Rel-16) Intel Corporation

- R4-2101410 	CR on RRC based BWP switching on multiple CCs (Rel-17) Intel Corporation

- R4-2101411 	Discussion on test case design for BWP switching on multiple CCs Intel Corporation

- R4-2101412 	Draft CR on DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR1 in EN-DC (section 4.5.6.x) Intel Corporation

- R4-2101416 	Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-16) Intel Corporation, ZTE Corporation, Apple

- R4-2101417 	Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-17) Intel Corporation, ZTE Corporation, Apple

- R4-2101631 	CR on interruption requirements of BWP switch on multiple CCs for EN-DC Huawei, HiSilicon

- R4-2101632 	CR on interruption requirements of BWP switch on multiple CCs for EN-DC Huawei, HiSilicon

- R4-2101633 	Discussion on requirements maintenance for BWP switch on multiple CCs Huawei, HiSilicon

- R4-2101634 	Discussion on performance requirements for BWP switch on multiple CCs

- Huawei, HiSilicon

- R4-2101635 	draftCR to introduce Active BWP Switch on multiple CCs TC2 Huawei, HiSilicon

- R4-2101679 	DraftCR on SCell activation and deactication delay test for FR2 inter-band CA Huawei, HiSilicon

- R4-2101688 	Discussion on SRS carrier switching test case Huawei, HiSilicon

- R4-2101689 	Correction on interruptions of SRS carrier switching Huawei, HiSilicon

- R4-2101690 	Correction on interruptions of SRS carrier switching Huawei, HiSilicon

- R4-2101691 	Clarification on inter-frequency measurement without gap requirements Huawei, HiSilicon

- R4-2101693 	Correction on inter-frequency measurement without gap test case Huawei, HiSilicon

- R4-2101694 	UL spatial relation switching to an unknown DL RS Huawei, HiSilicon

- R4-2101695 	UL spatial relation switching to an unknown DL RS Huawei, HiSilicon

- R4-2101696 	Correction on uplink spatial relation switching test case Huawei, HiSilicon

- R4-2101762 	CR to 38.133 correction on SRS carrier based switching core requirements vivo

- R4-2101764 	Remaining issues for SRS carrier switching test cases vivo

- R4-2101765 	CR to 38.133 on SRS configuration vivo

- R4-2101766 	On CGI reading test cases vivo

- R4-2101922 	On test cases for mandatory gap patterns ZTE Corporation

- R4-2102265 	Test case for RRC based UL spatial relation switch associated with a known DL-RS in SA for periodic SRS Nokia, Nokia Shanghai Bell

- R4-2102354 	On DCI-based simultaneous BWP switch on multiple CCs Ericsson

- R4-2102355 	CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs Ericsson

- R4-2102356 	CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs Ericsson

- R4-2102361	Test case applicability for mandatory measurement gaps in R15/R16 Ericsson

- R4-2102362 	On TC4 for simultaneous BWP switch on multiple CCs Ericsson

- R4-2102363 	DraftCR 38.133 (A.7.5.X) TC4 BWP switching Ericsson

- R4-2102533 	TC2 - SA interruptions at NR SRS carrier based switching Ericsson

- R4-2102634 	Test cases for BWP switching on multiple CCs for FR1+FR2 Ericsson

- R4-2102686 	SRS carrier switching interruption CR Qualcomm, Inc.

- R4-2102687 	SRS carrier switching interruption CR Qualcomm, Inc.

- R4-2102722 	CR on maintenance on BWP switch requirements on multiple CCs Huawei, HiSilicon

- R4-2102723 	CR on maintenance on BWP switch requirements on multiple CCs (cat A) Huawei, HiSilicon

- R4-2102788 	Discussion on remaining issues in multiple SCell activation Huawei, HiSilicon

- R4-2102789 	CR on multiple SCell activation requirements Huawei, HiSilicon

- R4-2102790 	CR on multiple SCell activation requirements R17 Huawei, HiSilicon

- R4-2102791 	CR on CGI reading requirements 38.133 Huawei, HiSilicon, MediaTek

- R4-2102792 	CR on CGI reading requirements 38.133 R17 Huawei, HiSilicon, MediaTek

- R4-2102793 	CR on CGI reading report delay 36.133 Huawei, HiSilicon

- R4-2102794 	CR on CGI reading report delay 36.133 R17 Huawei, HiSilicon

- R4-2102795 	draftCR to CGI reading TC4 Huawei, HiSilicon

- R4-2102888 	Updates on Test Configuration for FR2 Inter-band CA IBM UEs Qualcomm Incorporated

- R4-2103602 	CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs Ericsson

- R4-2103603 	UL spatial relation switching to an unknown DL RS Huawei, HiSilicon

- R4-2103605 	draftCR to introduce Active BWP Switch on multiple CCs TC2 Huawei, HiSilicon

- R4-2103606 	DraftCR 38.133 (A.7.5.X) TC4 BWP switching Ericsson

- R4-2103607 	Test case for RRC based UL spatial relation switch associated with a known DL-RS in SA for periodic SRS Nokia, Nokia Shanghai Bell RAN4

- R4-2103611 	CR to 38.133 correction on SRS carrier based switching core requirements vivo, Qualcomm, Huawei, HiSilicon, MediaTek Inc., Apple, Nokia

- R4-2103613 	CR on CGI reading requirements 38.133 Huawei, HiSilicon, MediaTek

- R4-2103614 	TC4: E-UTRAN Apple

- R4-2103615 	CR to 38.133 on SRS configuration vivo

- R4-2103616 	TC2 - SA interruptions at NR SRS carrier based switching Ericsson

- R4-2103617 	draftCR to CGI reading TC4Huawei, HiSilicon

- R4-2103618 	Mandatory MG applicability rule CR Qualcomm, Inc.

- R4-2103621 	CR on multiple SCell activation requirements Huawei, HiSilicon

- R4-2103622 	CR on UE behavior for UE specific CBW change R16 Apple

- R4-2103623 	Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16 MediaTek inc.

- R4-2103624 	CR on TS38.133 for inter-frequency measurement requirement without gap MediaTek inc.

- R4-2103626 	draftCR on multiple SCell activation test case MediaTek inc.

- R4-2103627 	draftCR on the CBW change test case for adding the UL CBW configuration MediaTek inc.

- R4-2103628 	DraftCR on SCell activation and deactication delay test for FR2 inter-band CA Huawei, HiSilicon

- R4-2104085 	TC2 - SA interruptions at NR SRS carrier based switching Ericsson

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template