3GPP TSG-RAN4 Meeting #99-e	R4-2110752

# Electronic Meeting, 19th May– 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2057 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CSI-RS reference configuration_R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2021-05-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CAT.A CR of R4-2110751 1, For TDD/FDD + 15kHz SCS test configuration, reference configuration of CSI-RS for CSI reporting and TRS are incompatible. As depicted in following figure, resource # 0 of CSI-RS.1.1 FDD/TDD overlaps with resource #3 of TRS.1.1 FDD/TDD. We suggest change firstOFDMSymbolInTimeDomain to 4 for CSI-RS.1.1 FDD/TDD to avoid overlapping, as shown in the following figure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - firstOFDMSymbolInTimeDomain for CSI-RS.1.1 FDD/TDD is changed to 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 CR 1190 TS/TR 38.508-1 CR 1900 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

## A.3.14	CSI-RS configurations

### A.3.14.1	FDD

Table A.3.14.1-1: CSI-RS Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.1.1 FDD | CSI-RS.1.2 FDD | CSI-RS.1.3 FDD | CSI-RS.1.4 FDD | CSI-RS.1.5 FDD |
| --- | --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on | off |
| aperiodicTriggeringOffset | n.a. | n.a. | 6 | 4 | 4 |
| trs-Info | n.a. | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |  |
|  |  | 10 for resource #0 | 20 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |  |
|  |  |  |  | 2 for resource #2 |  |
|  |  |  |  | 3 for resource #3 |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 11 for resource #1 | 21 for resource #1 | 4 for resource #4 | 1 for resource #1 |
|  |  |  |  | 5 for resource #5 |  |
|  |  |  |  | 6 for resource #6 |  |
|  |  |  |  | 7 for resource #7 |  |
| powerControlOffset | 0 | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | n.a. | n.a. | n.a. |
| Offset | 1 | 1 | n.a. | n.a. | n.a. |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. | n.a. |
|  |  | TCI.State.1 |  |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 | 000001 |
| nrofPorts | 2 | 1 | 1 | 1 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 | Specified in the test case for resource #0 |
|  |  |  |  | 1 for resource #1 |  |
|  |  |  |  | 2 for resource #2 |  |
|  |  |  |  | 3 for resource #3 |  |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 | n.a. |
|  |  |  |  | 5 for resource #5 |  |
|  |  |  |  | 6 for resource #6 |  |
|  |  |  |  | 7 for resource #7 |  |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |  |

<End of modified section 1>

<Start of modified section 2>

### A.3.14.2	TDD

Table A.3.14.2-1: CSI-RS Reference Measurement Channels for SCS=15kHz

|  | CSI-RS.1.1 TDD | CSI-RS.1.2 TDD | CSI-RS.1.3 TDD | CSI-RS.1.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | n.a. | off | off | on |
| aperiodicTriggeringOffset | n.a. | n.a. | 4 | 4 |
| trs-Info | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |
|  |  | 10 for resource #0 | 20 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 11 for resource #1 | 21 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | n.a. | n.a. |
| Offset | 1 | 1 | n.a. | n.a. |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | n.a. | n.a. |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
|  |  | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |

<End of modified section 1>