3GPP TSG-RAN Meeting #92e 											   					RP-21xxxx

Electronic Meeting, June 14-18, 2021

Agenda item:			9.7.1.2

Source:	Moderator (Qualcomm)

Title:	Summary of email discussions on Extending NR operation to 71GHz

Document for:	Information

# 1	Introduction

The SR for the WI on Extending NR operation to 71GHz can be found in [0].

Contributions [1]-[15] have been submitted to RAN#92e for discussion in RAN#92e. In additions, LSs from RAN1 and RAN4 have been received in [16] and [17], respectively, with corresponding WG views on the designation of this frequency range in the context of the existing FR2. Finally, another LS from RAN4 [18] has been received on OTA test method in 52.6-71GHz.

The topics that are discussed in these contributions can be summarized as follows:

- Designation of 52.6-71GHz frequency range

- SCS for SSB for Initial Access

- SCS for ANR

- Channel Access

- RAN2 workplan and scope

- OTA Testing methods

- Any other issues

# 2	Initial Round Discussion

In this initial phase of discussions, the goal is collecting company views on the topics identified in section 1 aiming at RAN Plenary guidance.

## 2.1	Issue 1: Frequency range designation

LSs from RAN1 and RAN4 have been received in [16] and [17], respectively, with corresponding WG views on the designation of this frequency range in the context of the existing FR2.

RAN1 and RAN2 have indicated that they can adapt to other WGs decision on the designation of this frequency range. As a result, we consider RAN4 input for further discussion in this email discussion.

RAN4 LS [17] offers the following options:

- Option A:

  - Introduce FR2-1 (or FR2.1) for 24.25 – 52.6 GHz, and FR2-2 (or FR2.2) for 52.6 – 71 GHz,

  - The above two ranges to be introduced under the FR2 common range.

|  | Option A |
| --- | --- |
| Frequency range designation | Corresponding frequency range |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | FR2-1 (or FR2.1): 24250 MHz – 52600 MHz |
|  | FR2-2 (or FR2.2): 52600 MHz – 71000 MHz |
| NOTE: Whenever the FR2 is referred, both FR2.1 and FR2.2 frequency sub-ranges shall be considered, unless otherwise stated. |  |

- Option B:

  - All UE RF/demodulation requirements defined as function of band, BW, PC or band combo within FR2,

  - BS requirements can be updated to cater for an extension of FR2 to include 52.6 – 71 GHz,

  - All RRM requirements for higher SCS applicable for 52.6 – 71 GHz can be defined as function of SCS within FR2.

- Option C: in addition to reusing the existing FR2 for 24.25 – 52.6GHz:

  - Introduce FR2-2 (or FR2x) for 52.6 – 71 GHz,

  - Possibly introduce FR2-comb for 24.25 – 71 GHz.

|  | Option C |
| --- | --- |
| Frequency range designation | Corresponding frequency range |
| FR1 | 410 MHz – 7125 MHz |
| “FR2-comb”, or no common term | FR2: 24250 MHz – 52600 MHz |
|  | FR2-2 (or FR2x): 52600 MHz – 71000 MHz |

### 2.1.1 Company inputs on Issue 1

While the issue of frequency range designation could wait for further RAN4 discussions on the same, it is our understanding that all the facts, as well as, pros and cons of each of the options have already been identified. Therefore, it is a good opportunity for RAN to intervene and save some necessary valuable time for RAN4 to discuss other technical matters.

Please, indicate your preferred Option (A, B or C) along with any relevant comment or clarification on the same. Feel free to pick more than one option if your company is fine with the formulation from more than one option.

| Company | Comments |
| --- | --- |
| Samsung | We are ok with either of the options, with the understanding that proper modification to current specification and UE capability signalling needs to be taken care of by the corresponding working groups. |

### 2.1.2 Proposal for Issue 1

xxx

## 2.2	Issue 2: SCS for SSB for Initial Access

RAN1 has debated the issue of SCS for SSB for Initial Access. Currently, only 120kHz is supported. In this email discussion we consider the possibility of adding other SCS(s) for this purpose.

### 2.2.1 Company inputs on Issue 2

Please, indicate your preference re. SCS for Initial Access in addition to 120kHz along with any relevant information or restrictions for the applicability of this additional SCS(s).

| Company | Comments |
| --- | --- |
| Samsung | We support both 480 kHz and 960 kHz SCS for SSB in initial access case, and possibly comprise to support one of them as a way forward, with the restrictions figured out in RAN1. More precisely, we support the following proposal. In addition to 120kHz, support both 480 and 960 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints. - Limited sync raster entry numbers - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n261 is 602). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz. - only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS i.e., (480,480) and (960,960). - SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical - Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible |

### 2.2.2 Proposal for Issue 2

xxx

## 2.3	Issue 3: SCS for ANR

RAN1 has debated the issue of SCS for ANR. Currently, 480/960 kHz SSB is only agreed to be supported for the case when CORESET#0/Type0 PDCCH is not provided by the SSB.

### 2.3.1 Company inputs on Issue 3

Please, indicate your preference re. SCS for ANR along with any relevant information or restrictions for the additional support.

| Company | Comments |
| --- | --- |
| Samsung | We support 480 and 960 kHz SCS for SSB used for ANR purpose, and CORESET#0/Type0-PDCCH configuration provided in MIB of 480 and 960kHz SSB. More precisely, we support the following RAN1 proposal: To support ANR and PCI confusion detection for 480/960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 480 and 960kHz SSB - FFS: additional method(s) to enable support to obtain neighbor cell SIB1 contents related to CGI reporting - Only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (480,480) and (960,960). - Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification. |

### 2.3.2 Proposal for Issue 3

xxx

## 2.4	Issue 4: Channel Access

RAN1 has been discussing a large number of channel access techniques for unlicensed bands in the 52.6-71GHz frequency range. As there are only three remaining RAN1 meetings to define the channel access for unlicensed bands in this frequency range, focusing on essential aspects of channel access is necessary to guarantee on-time completion of the WI.

### 2.4.1 Company inputs on Issue 4

Please, indicate your preference re. RAN guidance channel access for unlicensed bands in the 52.6-71GHz (e.g., no guidance, items to prioritize, items to deprioritize, etc.).

| Company | Comments |
| --- | --- |
| Samsung | We didn’t see a strong need for RAN guidance on the channel access aspects, and RAN1 can figure out the essential items and prioritize them by normal work in RAN1. If deem necessary, down-scoping can be considered in the next RAN. |

### 2.4.2 Proposal for Issue 4

xxx

## 2.5	Issue 5: RAN2 scope

The WID has the following RAN2 objective:

- Radio interface protocol architecture and procedures [RAN2]: - For operation in this frequency range: Introduce higher layer support of enhancements listed above that are agreed to be specified.

and RAN2 has not started related discussions.

### 2.5.1 Company inputs on Issue 5

In an attempt to provide RAN2 guidance on their work, as well as, to preempt possible RAN2 optimizations on non-essential aspects of the design of NR between 52.6 and 71GHz, the discussion here is meant to provide recommendations on RAN2 work plan. Companies are invited to share their views on this.

| Company | Comments |
| --- | --- |
| Samsung | We agree with the moderator, given the very limited TUs for the item in RAN2. |

### 2.5.2 Proposal for Issue 5

xxx

## 2.6	Issue 6: OTA testing

RAN4 LS in [18] has the following recommendation for consideration at RAN:

RAN4 recommends to expand the scope of the Rel-17 NR FR2 Test Methods Enhancements SI (FS_FR2_enhTestMethods). It is important to complete the NR 52.6-71GHz UE OTA test methods within the Rel-17 timeframe to ensure that RAN5 has the necessary information to complete the conformance test cases.

### 2.6.1 Company inputs on Issue 6

Companies are invited to share their views on RAN4 recommendation on dealing with Test Methods for NR in 52.6-71GHz.

| Company | Comments |
| --- | --- |
| Samsung | We support the LS from RAN4 to include test method in the existing SI. We also agreed that it is important to complete test method in Rel-17 timeframe. |

### Proposal for Issue 6

xxx

## 2.7	Issue 7: Any other issues

### 2.7.1 Company inputs on Issue 7

Companies are invited to share their views on any other issues re. RAN Plenary guidance for the project on extending NR operation to 71GHz.

| Company | Comments |
| --- | --- |

### Proposal for Issue 7

xxx

## 2.8	Initial Round Summary

xxx

# References

SR:

[0] RP-211267	Status report for WI Extending current NR operation to 71GHz; rapporteur: Qualcomm,	RAN1

RAN contributions:

[1]	RP-211197	On the subcarrier spacing of SS/PBCH block for initial access in 52.6 to 71 GHz, Samsung

[2] RP-211211	Discussion on Frequency Range for 52.6 to 71 GHz, CATT

[3] RP-211264	On the progress in RAN1 to extend current NR operation to 71GHz, AT&T

[4] RP-211268	Views on WI extending NR operation to 71GHz, Qualcomm CDMA Technologies

[5] RP-211307	Discussion on Updates to WID for extending current NR operation to 71 GHz, Intel Corporation

[6] RP-211358	Designation of frequencies in the range 52.6-71 GHz	, Huawei, HiSilicon

[7] RP-211365	FR definition on extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

[8] RP-211366	On initial access aspects for extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

[9] RP-211413	Views on the NR 52.6-71 GHz frequency range definition, Intel Corporation

[10] RP-211414	OTA Test Methods for NR 52.6 - 71 GHz, Intel Corporation

[11] RP-211424	Proposal on FR terminology for 52.6-71GHz, Ericsson

[12] RP-211429	On the frequency range designation for 52.6 - 71 GHz, Apple

[13] RP-211437	Discussion on frequency range definition for 52.6-71GHz, ZTE, Sanechips

[14] RP-211450	Testability aspects for the 52.6 GHz to 71 GHz frequency range, Apple Inc.

[15] RP-211459	Way Forward on FRx naming, MediaTek Inc.

WG LSs:

[16] RP-210959	LS on how to introduce the 52.6-71GHz frequency range (R1-2106277; to: RAN; cc: RAN2, RAN4; contact: Lenovo)

[17] RP-210965	LS on RAN4 recommendation for the 52.6 - 71 GHz frequency range designation (R4-2107879; to: RAN; cc: RAN1, RAN2, RAN5; contact: Huawei)

[18] RP-210964	LS on UE OTA test method in 52.6 – 71 GHz (R4-2107878; to: RAN; cc: -; contact: Apple)