3GPP TSG-RAN Meeting #92e 											   					RP-21xxxx

Electronic Meeting, June 14-18, 2021

Agenda item:			9.7.1.2

Source:	Moderator (Qualcomm)

Title:	Summary of email discussions on Extending NR operation to 71GHz

Document for:	Information

# 1	Introduction

The SR for the WI on Extending NR operation to 71GHz can be found in [0].

Contributions [1]-[15] have been submitted to RAN#92e for discussion in RAN#92e. In additions, LSs from RAN1 and RAN4 have been received in [16] and [17], respectively, with corresponding WG views on the designation of this frequency range in the context of the existing FR2. Finally, another LS from RAN4 [18] has been received on OTA test method in 52.6-71GHz.

The topics that are discussed in these contributions can be summarized as follows:

- Designation of 52.6-71GHz frequency range

- SCS for SSB for Initial Access

- SCS for ANR

- Channel Access

- RAN2 workplan and scope

- OTA Testing methods

- Any other issues

# 2	Initial Round Discussion

In this initial phase of discussions, the goal is collecting company views on the topics identified in section 1 aiming at RAN Plenary guidance.

## 2.1	Issue 1: Frequency range designation

LSs from RAN1 and RAN4 have been received in [16] and [17], respectively, with corresponding WG views on the designation of this frequency range in the context of the existing FR2.

RAN1 and RAN2 have indicated that they can adapt to other WGs decision on the designation of this frequency range. As a result, we consider RAN4 input for further discussion in this email discussion.

RAN4 LS [17] offers the following options:

- Option A:

  - Introduce FR2-1 (or FR2.1) for 24.25 – 52.6 GHz, and FR2-2 (or FR2.2) for 52.6 – 71 GHz,

  - The above two ranges to be introduced under the FR2 common range.

|  | Option A |
| --- | --- |
| Frequency range designation | Corresponding frequency range |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | FR2-1 (or FR2.1): 24250 MHz – 52600 MHz |
|  | FR2-2 (or FR2.2): 52600 MHz – 71000 MHz |
| NOTE: Whenever the FR2 is referred, both FR2.1 and FR2.2 frequency sub-ranges shall be considered, unless otherwise stated. |  |

- Option B:

  - All UE RF/demodulation requirements defined as function of band, BW, PC or band combo within FR2,

  - BS requirements can be updated to cater for an extension of FR2 to include 52.6 – 71 GHz,

  - All RRM requirements for higher SCS applicable for 52.6 – 71 GHz can be defined as function of SCS within FR2.

- Option C: in addition to reusing the existing FR2 for 24.25 – 52.6GHz:

  - Introduce FR2-2 (or FR2x) for 52.6 – 71 GHz,

  - Possibly introduce FR2-comb for 24.25 – 71 GHz.

|  | Option C |
| --- | --- |
| Frequency range designation | Corresponding frequency range |
| FR1 | 410 MHz – 7125 MHz |
| “FR2-comb”, or no common term | FR2: 24250 MHz – 52600 MHz |
|  | FR2-2 (or FR2x): 52600 MHz – 71000 MHz |

### 2.1.1 Company inputs on Issue 1

While the issue of frequency range designation could wait for further RAN4 discussions on the same, it is our understanding that all the facts, as well as, pros and cons of each of the options have already been identified. Therefore, it is a good opportunity for RAN to intervene and save some necessary valuable time for RAN4 to discuss other technical matters.

Please, indicate your preferred Option (A, B or C) along with any relevant comment or clarification on the same. Feel free to pick more than one option if your company is fine with the formulation from more than one option.

| Company | Comments |
| --- | --- |
| Samsung | We are ok with either of the options, with the understanding that proper modification to current specification and UE capability signalling needs to be taken care of by the corresponding working groups. |
| Charter Communications Inc. | We have a question for clarification. We have a strong requirement that decisions made in frequency range designation for 52.6-71 GHz shall not restrict common design considerations (LBT framework, numerology, scs, etc…) with other unlicensed bands below 52.6 GHz. In the US, there are other unlicensed bands under 52.6 GHz in mmW (37 GHz). Based on this strong requirement, it is our understanding that option A or option B should be able to meet such requirement. Is this correct understanding? If so, we can either agree in proposals in option A or B. |
| NTT DOCOMO | We agree that it would be good to decide it in this RAN plenary. Our slight preference is option A or C since such sub-labelling of FR could be beneficial in terms of e.g., UE feature and capability definition. |
| Futurewei | We are OK with Option A or C, which are more in line with the RAN1 conclusion that easy distinction in the specifications is useful. Whether to use A or C would depend on which would require fewer changes to the existing specs. |
| CATT | We prefer to leave this to RAN4 for discussion. Technically our first choice is option C but we are also fine with option A. |
| AT&T | We agree that it is better to decide at this RAN Plenary meeting as the designation will be clear to all WGs early enough to take the outcome into account without the need for further discussion on the topic in RAN4. We prefer Option B but can also support Option A. Each of these options will allow RAN4 to use the existing 38.101-2 specification for the frequency range extension and minimize the impact to specifications that generically refer to Frequency Range 2 such as the 38.101-3 specification. |
| InterDigital | We support either option A (1st preference) or option C (2nd preference). |
| Apple | We support picking an option at this meeting instead of asking RAN4 to further discuss it, as all the pros and cons have been discussed in RAN4. Among the three remaining options, we have a slight preference for option A, with detailed analysis provided in our contribution RP-211429. |
| Intel | We support option A. For option B has backward compatibility issue, although we understand the motivation. In terms of timeline, RAN and RAN4 already spent multiple meetings and we think technical aspects have been discussed and identified clearly as in the RAN4 LS. We think another round of WG meeting does not help a lot to conclude. We prefer to conclude in this meeting. |
| Huawei, HiSilicon | Our preference is Option A. Option B is the only option that doesn’t offer a designation specifically for 24-52 and 52-71 GHz. If the selected option has impact on writing RAN1 specifications then a decision should be made at the latest by RAN#93 since RAN1 Rel-17 specification will be provided to RAN#94. |
| ZTE | We think introducing FR2-2 or FR2x is useful as the requirements of BS RF, BS demod, UE RF and UE demod for 52.6-71GHz would be different from those of legacy FR2, so option A or C serves the purpose. Option C is slightly preferred over option A to minimize the editorial changes, since the existing FR2 for 24.25-52.6GHz can remain unchanged which has been widely used in public. |
| Qualcomm | We are fine with either Option A or Option C with a slight preference for Option C. |

### 2.1.2 Proposal for Issue 1

xxx

## 2.2	Issue 2: SCS for SSB for Initial Access

RAN1 has debated the issue of SCS for SSB for Initial Access. Currently, only 120kHz is supported. In this email discussion we consider the possibility of adding other SCS(s) for this purpose.

### 2.2.1 Company inputs on Issue 2

Please, indicate your preference re. SCS for Initial Access in addition to 120kHz along with any relevant information or restrictions for the applicability of this additional SCS(s).

| Company | Comments |
| --- | --- |
| Samsung | We support both 480 kHz and 960 kHz SCS for SSB in initial access case, and possibly comprise to support one of them as a way forward, with the restrictions figured out in RAN1. More precisely, we support the following proposal. In addition to 120kHz, support both 480 and 960 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints. - Limited sync raster entry numbers - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n261 is 602). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz. - only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS i.e., (480,480) and (960,960). - SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical - Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible |
| NTT DOCOMO | We share Samsung’s view. Under the restriction captured in RAN1 agreement above, we think there would be no problem to support both of 480 and 960 kHz SCS in terms of UE complexity, while we are also ok with supporting only one of 480 or 960 kHz as a compromise. |
| Futurewei | We think that supporting two additional SCS for the initial access is not necessary and it could result in too much additional complexity. As a compromise if needed we can accept at most one optional SCS for the initial access on top of the mandatory 120 kHz SCS. If one optional SCS is added to the initial access our preference between 480 and 960 is 480 kHz. |
| CATT | We are fine with one (or two) optional SCS for initial access. |
| AT&T | We share the same view as NTT Docomo. Either one of the two proposed agreements from RAN1 #105-e (see RP-211264 for reference) are fine with us. We can go with the one copied by Samsung above or specify either 480 kHz or 960 kHz SCS for SSB for initial access. |
| InterDigital | We also share the same view as Samsung including the possible compromise to support only one of 480 and 960kHz SCS as a way forward. |
| Apple | We support to introduce one more SCS in addition to 120kHz SCS for SSB in initial access case to enable single numerology operation for this frequency band. Between 480kHz/960kHz, our preference is 480kHz SCS. |
| Intel | In the last RAN1 there was near consensus on agreements for support for which SCS for initial access. However, due to sustained objection from 2 company further agreement and progress was not achieved. Given that there is strong interest from large number of the companies (based on RAN1 discussion summary at least 16 companies) to discuss the support of additional SCS (either 480, or 960, or 480+960kHz) for initial access, we strongly recommend to discuss the proposal and get a conclusion in the RAN Plenary so that further progress could be made. We believe this issue should be resolved together with issue 3 (SCS for ANR). We suggest agreeing to the following proposal and making appropriate update to the WID based on the agreement. If it helps, we can put a clarification note that support of 480kHz and 960kHz for initial access is optional. Proposal: - In addition to 120kHz, support both 480 and 960 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints. - Limited sync raster entry numbers - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz. - only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS i.e., (480,480) and (960,960). - SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical - Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: support of 480 and 960 kHz SSB for initial access is optional. - To support ANR and PCI confusion detection for 480/960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 480 and 960kHz SSB - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification. |
| Huawei, HiSilicon | We would first like to note that even if network implementation could be simpler with a single numerology, implementing 120 kHz SCS for initial access and another numerology for non-initial access is feasible and already constitutes a workable system, as per current WG agreements. So the proposals for additional numerologies for initial access are optimizations. Even though a large majority of companies support defining additional numerologies for initial access, this doesn’t mean that there is no drawback to this proposal. We would like to provide our understanding of the implications and risks of adding one or two optional numerologies for initial access in addition to 120 kHz SCS. #1 Market fragmentation if multiple numerologies are supported in the specifications for initial access. Different networks could choose to implement different numerologies for initial access, causing UEs that only support 120 kHz not being able to access networks operating with 480 kHz or 960 kHz for initial access. It cannot be assumed that all Ues would support all optional initial access numerologies. #2 Increased initial search blind detection complexity (UE power consumption, initial access delay) compared to single numerology for initial access. The number of synch raster entries for 120+480 kHz would be 665, to be compared with 344 for n259 (for 120 kHz and 240 kHz). The number of synch raster entries would be even larger with 120+480+960 for initial access. Considering the above, it should be clear that the concerns and risks increase if both 480 and 960 kHz are to be supported for initial access in addition to 120 kHz. That being said, since a large majority of companies are willing to accept the above risks, and with the goal to minimize those risks, we can compromise to accept defining only 480 kHz as an optional numerology for initial access, with the condition that a UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access), and task RAN4 to minimize the number of synch raster entries for initial access. Other details can be left to RAN1. |
| ZTE | We share the similar view as Samsung to support either one or two optional SCS. We also agree with Intel’s comment that it can be resolved together with issue 3. |
| Qualcomm | Suggesting to support 480KHz SCS for initial access SSB, under the conditions that it is optional for the UE and there is reduced spec impact leveraging as much as possible 120KHz designs on CORESET#0/Type0 PDCCH multiplexing rules. |

### 2.2.2 Proposal for Issue 2

xxx

## 2.3	Issue 3: SCS for ANR

RAN1 has debated the issue of SCS for ANR. Currently, 480/960 kHz SSB is only agreed to be supported for the case when CORESET#0/Type0 PDCCH is not provided by the SSB.

### 2.3.1 Company inputs on Issue 3

Please, indicate your preference re. SCS for ANR along with any relevant information or restrictions for the additional support.

| Company | Comments |
| --- | --- |
| Samsung | We support 480 and 960 kHz SCS for SSB used for ANR purpose, and CORESET#0/Type0-PDCCH configuration provided in MIB of 480 and 960kHz SSB. More precisely, we support the following RAN1 proposal: To support ANR and PCI confusion detection for 480/960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 480 and 960kHz SSB - FFS: additional method(s) to enable support to obtain neighbor cell SIB1 contents related to CGI reporting - Only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (480,480) and (960,960). - Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification. |
| NTT DOCOMO | We share Samsung’s view. ANR to avoid PCI confusion is important from operator’s perspective, as described in GTW. Thus we believe all the SCSs supported in a band should support ANR function by reusing the existing method. |
| Futurewei | We agree that the ANR function should be addressed and resolved in the RAN1. We think that the discussion should continue in RAN1 after the initial access SCS controversial decision is taken, and therefore no RAN guidance is necessary for this topic at this time. |
| CATT | We prefer to close this issue as we already have sufficient technical discussion. So we are fine to conclude this issue in this meeting, or guide RAN1 to conclude this issue next WG meeting. |
| AT&T | We strongly support the views by Samsung and NTT Docomo. This issue has been discussed at length for NR-U in RAN1, RAN2, and RAN (see RP-211264 for reference) and Alt. 1 in the last RAN1 agreement does not work for scenarios where inter-operator PCI confusion resolution is required as is the case for shared unlicensed spectrum. The proposed agreement from RAN1 #105-e (see comment by Samsung or RP-211264) should be confirmed by RAN #92-e. |
| Apple | The following has been agreed in RAN1 105 e-meeting to support ANR function with 480/960 kHz SCS after discussions in the past two meetings. Hence, whether 480/960 kHz SCS can be used for ANR should NOT be of concern. Agreement: For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access) - Support configuring CORESET#0/Type0-PDCCH for the purpose of ANR/PCI confusion detection by down selecting from the following two alternatives - Alt 1) Using dedicated signaling - Alt 2) Using configuration in MIB - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification. The remaining open issue is which alternative is to be used to indicate the location of CORESET#0. We are open to down selecting one in the RAN plenary. From our side, we prefer Alt.2 to reuse the Rel-16 NRU design as much as possible. On the other hand, leaving RAN1 to down-select in next meeting is also fine with us. |
| Intel | In the last RAN1 there was near consensus on agreements for support for ALT 2 of the agreement listed by Apple. However, due to sustained objection from 1 company further agreement and progress was not achieved. Similarly as SCS for SSB in initial access, there is strong interest from large number of the companies and we strongly recommend to discuss the proposal and get a conclusion in the RAN Plenary so that further progress could be made. We believe this issue should be resolved together with issue 2 (SCS for initial access). We suggest agreeing to the proposal made in Issue 2 and making appropriate update to the WID based on the agreement. |
| Huawei, HiSilicon | This issue is dependent on issue 2, since a numerology used for initial access will automatically provide support for ANR. For a numerology not usable for initial access but usable for SSB, ANR and PCI confusion detection can be provided by configuring CORESET#0/Type0-PDCCH in MIB of the same numerology. |
| ZTE | We share the similar view as Samsung and support the RAN1 proposal using configuration in MIB. |
| Qualcomm | Support CORESET#0/Type0 PDCCH configuration in MIB of 480 and 960kHz SSB, under the conditions that it is optional for the UE and there is reduced spec impact leveraging as much as possible 120KHz designs on CORESET#0/Type0 PDCCH multiplexing rules. |

### 2.3.2 Proposal for Issue 3

xxx

## 2.4	Issue 4: Channel Access

RAN1 has been discussing a large number of channel access techniques for unlicensed bands in the 52.6-71GHz frequency range. As there are only three remaining RAN1 meetings to define the channel access for unlicensed bands in this frequency range, focusing on essential aspects of channel access is necessary to guarantee on-time completion of the WI.

### 2.4.1 Company inputs on Issue 4

Please, indicate your preference re. RAN guidance channel access for unlicensed bands in the 52.6-71GHz (e.g., no guidance, items to prioritize, items to deprioritize, etc.).

| Company | Comments |
| --- | --- |
| Samsung | We didn’t see a strong need for RAN guidance on the channel access aspects, and RAN1 can figure out the essential items and prioritize them by normal work in RAN1. If deem necessary, down-scoping can be considered in the next RAN. |
| NTT DOCOMO | While we agree down-scoping could be beneficial, we are not so sure if it is deemed necessary at this stage. |
| Futurewei | We agree with Samsung and NTT DoCoMo that currently there is no need for guidance from RAN on channel access. |
| CATT | No need for RAN guidance. |
| AT&T | We prefer not to engage in any down-scoping discussions before RAN 93.e, i.e., to give working groups one more quarter to make progress before deciding any items for deprioritization. |
| InterDigital | We also think RAN guidance is not needed for this topic at this point |
| Apple | Although progress has been relatively slow, we do not think there is a need for down-scoping at this point. The subject could be raised in RAN 93-e if there is no progress in RAN1. |
| Intel | We are okay to prioritize unlicensed band to guarantee on-time completion of the WI. From RAN4 perspective, licensed band work except system parameters will start when regulations becomes clear. |
| Huawei, HiSilicon | We think RAN1 can continue discussing the various options for channel access mechanism and aim for decisions by RAN#93. We don’t agree that any prioritization between unlicensed operation and licensed operation is needed at this point, since licensed operation support is a subset of unlicensed operation support in RAN1 and RAN2, and RAN4 will not work on defining a licensed band until regulations are available and this is already clear without RAN TSG guidance. |
| ZTE | We are ok with the intention, but maybe it is better to further check it in the next Plenary, based on the RAN1 progress in Q3. |
| Qualcomm | OK to postpone the guidance one more Quarter. |

### 2.4.2 Proposal for Issue 4

xxx

## 2.5	Issue 5: RAN2 scope

The WID has the following RAN2 objective:

- Radio interface protocol architecture and procedures [RAN2]: - For operation in this frequency range: Introduce higher layer support of enhancements listed above that are agreed to be specified.

And RAN2 has not started related discussions.

### 2.5.1 Company inputs on Issue 5

In an attempt to provide RAN2 guidance on their work, as well as, to preempt possible RAN2 optimizations on non-essential aspects of the design of NR between 52.6 and 71GHz, the discussion here is meant to provide recommendations on RAN2 work plan. Companies are invited to share their views on this.

| Company | Comments |
| --- | --- |
| Samsung | We agree with the moderator, given the very limited Tus for the item in RAN2. |
| NTT DOCOMO | We agree to prioritize the aspects related to RAN1 design. |
| Futurewei | We agree to give priority to the RAN1 design. |
| CATT | We don’t see urgent need for RAN guidance on this issue. |
| Apple | While we agree to prioritize RAN1 related aspects, we think that the current WID description reflects this and see no need to change, or revise it. |
| Intel | We agree with QC on this. Since there are only 2 meetings for RAN2 discussion, RAN2 should focus on the essential change. |
| Huawei, HiSilicon | We agree with the intent of the guidance to avoid unnecessary optimizations in RAN2, including RAN2 optimizations related to RAN1 features. The work in RAN2 can be triggered by contributions. RAN1 reports can be referred to, there is no need for RAN1 to trigger the work by LS to RAN2. |
| ZTE | We agree with the moderator as clarified online. |
| Qualcomm | RAN2 to prioritize protocol support of RAN1 design without optimizations on items not discussed in RAN1. |

### 2.5.2 Proposal for Issue 5

xxx

## 2.6	Issue 6: OTA testing

RAN4 LS in [18] has the following recommendation for consideration at RAN:

RAN4 recommends to expand the scope of the Rel-17 NR FR2 Test Methods Enhancements SI (FS_FR2_enhTestMethods). It is important to complete the NR 52.6-71GHz UE OTA test methods within the Rel-17 timeframe to ensure that RAN5 has the necessary information to complete the conformance test cases.

### 2.6.1 Company inputs on Issue 6

Companies are invited to share their views on RAN4 recommendation on dealing with Test Methods for NR in 52.6-71GHz.

| Company | Comments |
| --- | --- |
| Samsung | We support the LS from RAN4 to include test method in the existing SI. We also agreed that it is important to complete test method in Rel-17 timeframe. |
| NTT DOCOMO | We support the LS from RAN4. |
| Telecom Italia | RAN4 is overloaded. The activity should be kept within the current activity on FR2 and no new time units required. Moreover, in several Work Items it was decided to move the RAN4 aspects in Rel 18. This could be another good example where to postpone the RAN4 activity to Rel 18 |
| Futurewei | We support the LS from RAN4. |
| CATT | Support RAN4 LS. |
| AT&T | We support the LS from RAN4 to expand the scope of the existing SI on FR2 test methods to include the frequency range extension. It is important that any OTA test method aspects for the extended frequency range are completed in the Rel-17 timeframe to allow RAN5 to develop the conformance test cases if RAN4 specifies core performance requirements in Rel-17. |
| Apple | As we suggested in our contribution RP-211450, we would like to follow the RAN4 recommendation and expand the scope of the Rel-17 NR FR2 Test Methods Enhancements SI (FS_FR2_enhTestMethods) with 60 GHz testability objectives. The contribution RP-211450 contains an annex with suggested revisions of the FS_FR2_enhTestMethods SID as well as a TU budget proposal. Our recommendation is to endorse RP-211450 in order to allow a formal revision of the SID as the next step during this meeting. |
| Intel | It is important to study OTA testability aspects within the Rel-17 timeframe, so we support the LS from RAN4. Additionally, we propose the following: Proposal 1: Study and define NR 52.6-71 GHz BS OTA methods within the scope of Rel-17 NR 52.6 – 71 GHz WI, as a part of RAN4 BS conformance requirements objectives Proposal 2: Further study NR > 52GHz UE OTA testability aspects within the Rel-17 timeframe - Add NR 52.6-71GHz UE OTA test methods objectives into the scope of the ongoing RAN4-led Rel-17 NR FR2 Test Methods Enhancements SI - Studies can start in August 2021 We proposed a list of study objectives for the UE OTA test methods in our contribution (RP-211414), but we are open to further discussion to align on the specifics for the objectives. |
| Huawei, HiSilicon | We agree with the intent of the RAN4 LS to update the Rel-17 NR FR2 Test Methods Enhancements SI (FS_FR2_enhTestMethods) |
| ZTE | We support the LS from RAN4. |
| Qualcomm | We support RAN4 LS. |

### Proposal for Issue 6

xxx

## 2.7	Issue 7: Any other issues

### 2.7.1 Company inputs on Issue 7

Companies are invited to share their views on any other issues re. RAN Plenary guidance for the project on extending NR operation to 71GHz.

| Company | Comments |
| --- | --- |

### Proposal for Issue 7

xxx

## 2.8	Initial Round Summary

xxx

# References

SR:

[0] RP-211267	Status report for WI Extending current NR operation to 71GHz; rapporteur: Qualcomm,	RAN1

RAN contributions:

[1]	RP-211197	On the subcarrier spacing of SS/PBCH block for initial access in 52.6 to 71 GHz, Samsung

[2] RP-211211	Discussion on Frequency Range for 52.6 to 71 GHz, CATT

[3] RP-211264	On the progress in RAN1 to extend current NR operation to 71GHz, AT&T

[4] RP-211268	Views on WI extending NR operation to 71GHz, Qualcomm CDMA Technologies

[5] RP-211307	Discussion on Updates to WID for extending current NR operation to 71 GHz, Intel Corporation

[6] RP-211358	Designation of frequencies in the range 52.6-71 GHz	, Huawei, HiSilicon

[7] RP-211365	FR definition on extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

[8] RP-211366	On initial access aspects for extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

[9] RP-211413	Views on the NR 52.6-71 GHz frequency range definition, Intel Corporation

[10] RP-211414	OTA Test Methods for NR 52.6 - 71 GHz, Intel Corporation

[11] RP-211424	Proposal on FR terminology for 52.6-71GHz, Ericsson

[12] RP-211429	On the frequency range designation for 52.6 - 71 GHz, Apple

[13] RP-211437	Discussion on frequency range definition for 52.6-71GHz, ZTE, Sanechips

[14] RP-211450	Testability aspects for the 52.6 GHz to 71 GHz frequency range, Apple Inc.

[15] RP-211459	Way Forward on FRx naming, MediaTek Inc.

WG LSs:

[16] RP-210959	LS on how to introduce the 52.6-71GHz frequency range (R1-2106277; to: RAN; cc: RAN2, RAN4; contact: Lenovo)

[17] RP-210965	LS on RAN4 recommendation for the 52.6 - 71 GHz frequency range designation (R4-2107879; to: RAN; cc: RAN1, RAN2, RAN5; contact: Huawei)

[18] RP-210964	LS on UE OTA test method in 52.6 – 71 GHz (R4-2107878; to: RAN; cc: -; contact: Apple)