3GPP TSG RAN Meeting #92-e	RP-21xxxx

- Electronic Meeting, June 14 – 18, 2021

Source:	Moderator (Huawei)

Title:	Moderator Summary of [92-e-29-newLTE-UECat-1Rx]

Agenda item:	10.11

Document for:	Discussion

# Introduction

CAICT, China Telecom, China Unicom, CMCC, Guangdong Genius, Honor, Huawei, HiSilicon, OPPO, Samsung, Spreadtrum, vivo and Xiaomi have provided a joint proposal in RP-211493 (revised from RP-211466).

The proposals are:

Proposal 1: Define new LTE UE DL categories 3bis and 4bis with single Rx antenna based on UE category 3 and 4 in TS 36.306 and TS 36.331.

Proposal 2: For LTE Cat 3bis and Cat 4bis, reuse the existing RF, RRM and performance requirements specified for DL 1bis to verify single Rx antenna performance, and add additional SDR tests to verify the peak date rate.

Proposal 3: Task RAN2 and RAN4 to finalize the work for LTE Cat 3bis and Cat 4bis in TEI17

Proposal 4: UE reporting Category 3bis/4bis can report and therefore fall back to Cat 1bis when it roams to other regions if needed

For the initial email discussion companies are asked to provide their comments in Section 2. Companies are encouraged to provide comments with detailed reasoning (not just “support” or “object”) in order to facilitate the next round of discussion.

Please also add the contact person for your company in Section 4 when responding.

Please update the document’s name following this model before uploading:

RP-21xxxx [92-e-29-newLTE-UECat-1Rx] v001-Moderator

RP-21xxxx [92-e-29-newLTE-UECat-1Rx] v002-Moderator-CompanyA

RP-21xxxx [92-e-29-newLTE-UECat-1Rx] v003-CompanyA-CompanyB

https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_92e/Inbox/Drafts/92-e-29-newLTE-UECat-1Rx

# Initial email discussion (Mon 08:00h UTC – Tues 11:00h UTC)

Proposal 1: Define new LTE UE DL categories 3bis and 4bis with single Rx antenna based on UE category 3 and 4 in TS 36.306 and TS 36.331.

| Company | Comments and questions on proposal 1 in RP-211493 (e.g. support/no support with detailed motivations/reasons) |
| --- | --- |
| Lenovo | Which UL category should be signaled by UEs supporting the new DL categories 3bis and 4bis? How can the peak rate of DL 100/150 Mbps be achieved w/o CA and MIMO? |
| Apple | We support the proposal to define new LTE UE categories, which can help to ensure consistent user experiences (e.g. peak data rate >50Mbps), when 1Rx UE is roaming between LTE and NR networks. Also, the expected standardization efforts can be limited too. |
| DOCOMO | We are not supportive for introducing new categories which overlaps with Rel-17 NR RedCap from usecase/requirement perspectives. |

Proposal 2: For LTE Cat 3bis and Cat 4bis, reuse the existing RF, RRM and performance requirements specified for DL 1bis to verify single Rx antenna performance, and add additional SDR tests to verify the peak date rate.

| Company | Comments and questions on proposal 2 in RP-211493 (e.g. comments on the expected specification impact) |
| --- | --- |
| Apple | Same understanding as the proposal 2 that RAN4 related work can be limited. |

Proposal 3: Task RAN2 and RAN4 to finalize the work for LTE Cat 3bis and Cat 4bis in TEI17

| Company | Comments and questions on proposal 3 in RP-211493 |
| --- | --- |
| Lenovo | For RAN2 it should be feasible to complete the work in just one 1 meeting since the impacts may be only on new UE category signaling. For RAN4 we are not sure whether the work can be completed in one meeting. |
| Apple | OK with TEI17 considering the scope of the work. |

Proposal 4: UE reporting Category 3bis/4bis can report and therefore fall back to Cat 1bis when it roams to other regions if needed

| Company | Comments and questions on proposal 4 in RP-211493 |
| --- | --- |
| Lenovo | Proposal looks bit odd since UE capability signaling is normally not dependent on the region where the UE is located. But to ensure backwards-compatibility and allow the UE to be operated in legacy networks it looks ok that the UE reporting Category 3bis/4bis shall also report Cat 1bis. |
| Apple | Fallback mode can be further discussed in working group level. No hurry to make decision now. |
| DOCOMO | Fallback category to Cat1bis should be supported irrespective of when roaming or not to avoid any impacts on legacy NW. Current proposal is not sufficient to ensure NW-UE alignment on fallback category as NW does not know whether the UE fallbacks or not. If there is misalignment between NW and UE, rate matching for turbo coding is affected as it is determined based on the soft-buffer size, which depends on UE category. |

# Intermediate email discussion (Tue 15:00h UTC – Wed 11:00h UTC)

# Final email discussion (Wed. 15:30h UTC – Thu 11:00h UTC)

# Summary

# Contact information

| Company | Contact person | Email address |
| --- | --- | --- |
| Lenovo | Hyung-Nam Choi | hchoi5@lenovo.com |
| Apple | Yang Tang | yang.tang@apple.com |
| DOCOMO | Shinya Kumagai | shinya.kumagai@docomo-lab.com |