3GPP TSG-RAN WG5 Meeting #92-e	R5-216063

# Electronic meeting, 16th - 27th Aug, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0602 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 5.5A.1 for intra-band contiguous CA configuration table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The uplink CA configurations for intra-band contiguous CA in Table 5.5A.1-1 is incomplete. The (BWChannel,block) for CA_n260(E-P) is not correct in Table 5.5A.2-2. Furthermore, some notes in Table 5.5A.2-1 and 5.5A.2-2 should be updated according to TS 38.101-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the lower order configurations of uplink CA configurations for intra-band contiguous CA in Table 5.5A.1-1. - Add fallback groups for CA configurations in Table 5.5A.1-1. - Remove the “Note 1” for CA_n261C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intra-band contiguous CA configuration table will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.1, 5.5A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-215455r1: (1) Correct the issue of release inconsistency between 3GU and CR coversheet. R5-215455r2: (1) Remove the uplink CA configurations for CA_n257E, CA_n257F, CA_n257H, CA_n257I, CA_n257J, CA_n257K, CA_n257L, CA_n257M, CA_n260E, CA_n260F, CA_n260H, CA_n260I, CA_n260J, CA_n260K, CA_n260L, CA_n260M, CA_n260P, CA_n260Q, CA_n261E, CA_n261F, CA_n261H, CA_n261I, CA_n261J, CA_n261K, CA_n261L, CA_n261M, CA_n261P and CA_n261Q with no operator interesting. (2) Merge R5-215454 into this CR which are both related to Chapter 5. (3) Modify the sequence of notes in Table 5.5A.2-1 and Table 5.5A.2-2. (4) Reason for changes and affected clauses in coversheet are modified due to merge R5-215454. |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

5.5	Configurations

5.5A	Configurations for CA

5.5A.1	Configurations for intra-band contiguous CA

Table 5.5A.1-1: NR CA configurations, bandwidth combination sets, and fallback group defined for intra-band contiguous CA

| NR CA configuration | Uplink CA configurations | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | BWChannel (MHz) | Maximum aggregated BW (MHz) | BCS | Fallback group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n257B | CA_n257B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n257D | CA_n257D | 50, 100, 200, | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n257E | CA_n257E | 50, 100, 200, | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n257F | CA_n257F | 50, 100, 200, | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n257G | CA_n257G | 50, 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n257H | CA_n257H | 50, 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n257I | CA_n257I | 50, 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n257J | CA_n257J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n257K | CA_n257K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n257L | CA_n257L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n257M | CA_n257M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n260B | CA_n260B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n260C | CA_n260B | 50, 100, 200, 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 |  |
| CA_n260D | CA_n260D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n260E | CA_n260E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n260F | CA_n260F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n260G | CA_n260G | 50, 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n260H | CA_n260H | 50, 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n260I | CA_n260I | 50, 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n260J | CA_n260J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n260K | CA_n260K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n260L | CA_n260L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n260M | CA_n260M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n260O | CA_n260O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n260P | CA_n260P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n260Q | CA_n260Q | 50, 100 | 50, 100 | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| CA_n261B | CA_n261B | 50, 100, 200, 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| CA_n261C | CA_n261B | 50 | 400 | 400 |  |  |  |  |  | 850 | 0 |  |
| CA_n261D | CA_n261D | 50, 100, 200 | 200 |  |  |  |  |  |  | 400 | 0 | 2 |
| CA_n261E | CA_n261E | 50, 100, 200 | 200 | 200 |  |  |  |  |  | 600 | 0 |  |
| CA_n261F | CA_n261F | 50, 100, 200 | 200 | 200 | 200 |  |  |  |  | 800 | 0 |  |
| CA_n261G | CA_n261G | 50, 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n261H | CA_n261H | 50, 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n261I | CA_n261I | 50, 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n261J | CA_n261J | 50, 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n261K | CA_n261K | 50, 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n261L | CA_n261L | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n261M | CA_n261M | 50, 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |
| CA_n261O | CA_n261O | 50, 100 | 50, 100 |  |  |  |  |  |  | 200 | 0 | 4 |
| CA_n261P | CA_n261P | 50, 100 | 50, 100 | 50, 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n261Q | CA_n261Q | 50, 100 | 50, 100, | 50, 100 | 50, 100 |  |  |  |  | 400 | 0 |  |
| NOTE 1: Void. NOTE 2: For the NR CA configuration with more than two component carries, the bandwidths in a BCS which may introduce combinations more than requested unintentionally should be listed in a row separately. |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

5.5A.2	Configurations for intra-band non-contiguous CA

Configurations listed in this clause apply to downlink carrier aggregation only.

NOTE:	Sub-blocks belonging to a CA configuration can be in any order. In other words certain CA configuration acronym includes all sub-block arrangements which have exactly the same sub-block set. As an example, CA_n260(2G-3O) denotes CA_n260(2O-2G-O), CA_n260(G-3O-G) etc. but these are not listed in tables separately.

Table 5.5A.2-1: NR CA configurations with single CA bandwidth class defined for intra-band non-contiguous CA

| NR configuration | Uplink CA configurations | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | (BWChannel,block) (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n257(2A) | - | n257A | n257A |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2A) | - | n260A | n260A |  |  |  |  |  |  | 800 | 0 |
| CA_n260(3A) | - | n260A | n260A | n260A |  |  |  |  |  | 1200 | 0 |
| CA_n260(4A) | - | n260A | n260A | n260A | n260A |  |  |  |  | 1600 | 0 |
| CA_n261(2A) | - | n261A | n261A |  |  |  |  |  |  | 800 | 0 |
| CA_n261(3A) | - | n261A | n261A | n261A |  |  |  |  |  | 800 | 0 |
| CA_n261(4A) | - | n261A | n261A | n261A | n261A |  |  |  |  | 800 | 0 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: Channel bandwidth per operating band defined in Table 5.3.5-1. NOTE 5: Void. NOTE 6: Void. NOTE 7: (BWChannel,block) denotes the maximum total bandwidth from the summation of the sub-block bandwidths and shall be less than the bandwidth of the operating band. NOTE 8: Unless otherwise stated, BCS0 is referred in each constituent CA configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5A.2-2: NR CA configurations with multiple CA bandwidth classes defined for intra-band non-contiguous CA

| CA configuration | Uplink CA configurations | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | (BWChannel,block) (MHz) | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n260(A-I) | CA_n260I | n260A | CA_n260I |  |  |  |  |  | 800 | 0 |
| CA_n260(D-G) | CA_n260D CA_n260G | CA_n260D | CA_n260G |  |  |  |  |  | 600 | 0 |
| CA_n260(D-H) | CA_n260D CA_n260H | CA_n260D | CA_n260H |  |  |  |  |  | 700 | 0 |
| CA_n260(D-I) | CA_n260D CA_n260I | CA_n260D | CA_n260I |  |  |  |  |  | 800 | 0 |
| CA_n260(D-O) | CA_n260D CA_n260O | CA_n260D | CA_n260O |  |  |  |  |  | 600 | 0 |
| CA_n260(D-P) | CA_n260D CA_n260P | CA_n260D | CA_n260P |  |  |  |  |  | 700 | 0 |
| CA_n260(D-Q) | CA_n260D CA_n260Q | CA_n260D | CA_n260Q |  |  |  |  |  | 800 | 0 |
| CA_n260(E-O) | CA_n260E CA_n260O | CA_n260O | CA_n260E |  |  |  |  |  | 800 | 0 |
| CA_n260(E-P) | CA_n260E CA_n260P | CA_n260E | CA_n260P |  |  |  |  |  | 800 | 0 |
| CA_n260(E-Q) | CA_n260E CA_n260Q | CA_n260E | CA_n260Q |  |  |  |  |  | 1000 | 0 |
| CA_n260(G-I) | CA_n260G CA_n260I | CA_n260G | CA_n260I |  |  |  |  |  | 600 | 0 |
| CA_n261(D-G) | CA_n261D CA_n261G | CA_n261D | CA_n261G |  |  |  |  |  | 600 | 0 |
| CA_n261(D-H) | CA_n261D CA_n261H | CA_n261D | CA_n261H |  |  |  |  |  | 700 | 0 |
| CA_n261(D-I) | CA_n261D CA_n261I | CA_n261D | CA_n261I |  |  |  |  |  | 800 | 0 |
| CA_n261(D-O) | CA_n261D CA_n261O | CA_n261D | CA_n261O |  |  |  |  |  | 600 | 0 |
| CA_n261(D-P) | CA_n261D CA_n261P | CA_n261D | CA_n261P |  |  |  |  |  | 700 | 0 |
| CA_n261(D-Q) | CA_n261D CA_n261Q | CA_n261D | CA_n261Q |  |  |  |  |  | 800 | 0 |
| CA_n261(E-O) | CA_n261E CA_n261O | CA_n261E | CA_n261O |  |  |  |  |  | 800 | 0 |
| CA_n261(E-P) | CA_n261E CA_n261P | CA_n261E | CA_n261P |  |  |  |  |  | 800 | 0 |
| CA_n261(E-Q) | CA_n261E CA_n261Q | CA_n261E | CA_n261Q |  |  |  |  |  | 8001 | 0 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Unless otherwise stated, BCS0 is referred in each constituent CA configuration. NOTE 4: Void. NOTE 5: Void. NOTE 6: Void. NOTE 7: (BWChannel,block) denotes the maximum total bandwidth from the summation of the sub-block bandwidths and shall be less than the bandwidth of the operating band. NOTE 8: Channel bandwidth per operating band defined in Table 5.3.5-1. NOTE 9: Configurations for intra-band contiguous CA defined in Table 5.5A.1-1. NOTE 10: Configurations for intra-band non-contiguous CA definced in Table 5.5A.2-1. |  |  |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>