3GPP TSG-RAN5 Meeting #92-e	R5-215961

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0239 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of n24 and n99 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specifications for Rel-17 n24 and n99 bands in RAN4 have been completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Included n24 as part of FR1 RF baseline implementation capabilities and 2Rx support Included n99 SUL as part of FR1 RF baseline implementation capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ICS for n24 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.1, A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: i) Revised item # from 8 to 9 for n99 in Table A.4.3.1-5 due to overlap issue with R5-215310; ii) Also, checkmark against ME on the coversheet is removed; ii) Revised CR category from B to F; iii) Revised release column for n99 to Rel-17 (from Rel-16) in Table A.4.3.1-5 |  |  |  |  |  |  |  |  |

### A.4.3.1	RF Baseline Implementation Capabilities

NOTE:	The values indicated in column "Release" in tables A.4.3.1-1 and A.4.3.1-2 below are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 38.307 [19].

Table A.4.3.1-1: NR FDD FR1 RF Baseline Implementation Capabilities

| Item | NR FDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1920-1980 MHz (UL), 2110-2170 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand1_Supp | NR FDD FR1 Band 1 |
| 2 | NR Frequency band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand2_Supp | NR FDD FR1 Band 2 |
| 3 | NR Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand3_Supp | NR FDD FR1 Band 3 |
| 4 | NR Frequency band: 824-849 MHz (UL), 869-894 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand5_Supp | NR FDD FR1 Band 5 |
| 5 | NR Frequency band: 2500-2570 MHz (UL), 2620-2690 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand7_Supp | NR FDD FR1 Band 7 |
| 6 | NR Frequency band: 880-915 MHz (UL), 925-960 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand8_Supp | NR FDD FR1 Band 8 |
| 6a to 6c | Reserved |  |  |  |  |
| 6d | NR Frequency band: 699-716 MHz (UL), 729-746 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand12_Supp | NR FDD FR1 Band 12 |
| 6e | Reserved |  |  |  |  |
| 6f | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand14_Supp | NR FDD FR1 Band 14 |
| 6g to 6i | Reserved |  |  |  |  |
| 6j | NR Frequency band: 815-830 MHz (UL), 860-875 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand18_Supp | NR FDD FR1 Band 18 |
| 6k | Reserved |  |  |  |  |
| 7 | NR Frequency band: 832-862 MHz (UL), 791-821 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand20_Supp | NR FDD FR1 Band 20 |
| 7a to 7c | Reserved |  |  |  |  |
| 7d | NR Frequency band: 1626.5-1660.5 MHz (UL), 1525-1559 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand24_Supp | NR FDD FR1 Band 24 |
| 7e | NR Frequency band: 1850-1915 MHz (UL), 1930- 1995 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand25_Supp | NR FDD FR1 Band 25 |
| 7f | NR Frequency band: 814-849 MHz (UL), 859-894 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand26_Supp | NR FDD FR1 Band 26 |
| 7g | Reserved |  |  |  |  |
| 8 | NR Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand28_Supp | NR FDD FR1 Band 28 |
| 8a | Reserved |  |  |  |  |
| 8b | NR Frequency band: 2305-2315 MHz (UL), 2350-2360 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand30_Supp | NR FDD FR1 Band 30 |
| 8c to 8d | Reserved |  |  |  |  |
| 8e | NR Frequency band: 1920-2010 MHz (UL),2110-2200 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand65_Supp | NR FDD FR1 Band 65 |
| 9 | NR Frequency band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand66_Supp | NR FDD FR1 Band 66 |
| 9a to 9c | Reserved |  |  |  |  |
| 10 | NR Frequency band: 1695-1710 MHz (UL), 1995-2020 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand70_Supp | NR FDD FR1 Band 70 |
| 11 | NR Frequency band: 663-698 MHz (UL), 617-652 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand71_Supp | NR FDD FR1 Band 71 |
| 12 to 13 | Reserved |  |  |  |  |
| 14 | NR Frequency band: 1427-1470 MHz (UL), 1475-1518 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand74_Supp | NR FDD FR1 Band 74 |
| 15 | NR Frequency band: 832-862 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand91_Supp | NR FDD FR1 Band 91 |
| 16 | NR Frequency band: 832-862 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand92_Supp | NR FDD FR1 Band 92 |
| 17 | NR Frequency band: 880-915 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand93_Supp | NR FDD FR1 Band 93 |
| 18 | NR Frequency band: 880-915 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand94_Supp | NR FDD FR1 Band 94 |

<Unchanged text skipped>

Table A.4.3.1-5: NR SUL FR1 RF Baseline Implementation Capabilities

| Item | NR SUL FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1710-1785 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand80_Supp | NR SUL FR1 Band 80 |
| 2 | NR Frequency band: 880-915 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand81_Supp | NR SUL FR1 Band 81 |
| 3 | NR Frequency band: 832-862 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand82_Supp | NR SUL FR1 Band 82 |
| 4 | NR Frequency band: 703-748 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand83_Supp | NR SUL FR1 Band 83 |
| 5 | NR Frequency band: 1920-1980 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand84_Supp | NR SUL FR1 Band 84 |
| 6 | NR Frequency band: 1710-1780 MHz | 38.101-1, 5.2 | Rel-15 | pc_nrBand86_Supp | NR SUL FR1 Band 86 |
| 6a to 6b | Reserved |  |  |  |  |
| 6c | NR Frequency band: 824-849 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand89_Supp | NR SUL FR1 Band 89 |
| 7 | NR Frequency band: 2010-2025 MHz | 38.101-1, 5.2 | Rel-16 | pc_nrBand95_Supp | NR SUL FR1 Band 95 |
| 9 | NR Frequency band: 1626.5-1660.5 MHz | 38.101-1, 5.2 | Rel-17 | pc_nrBand99_Supp | NR SUL FR1 Band 99 |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-4c: 2 Rx antenna ports Capabilities

| Item |  | Band |  | Ref. |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | FDD Band n1 |  | 38.101-1, 7.3.2 |  |  |  |
| 2 |  | FDD Band n2 |  | 38.101-1, 7.3.2 |  |  |  |
| 3 |  | FDD Band n3 |  | 38.101-1, 7.3.2 |  |  |  |
| 4 |  | FDD Band n5 |  | 38.101-1, 7.3.2 |  |  |  |
| 5 |  | FDD Band n7 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 6 |  | FDD Band n8 |  | 38.101-1, 7.3.2 |  |  |  |
| 7 |  | FDD Band n12 |  | 38.101-1, 7.3.2 |  |  |  |
| 7b |  | FDD Band n14 |  | 38.101-1, 7.3.2 |  |  |  |
| 8 |  | FDD Band n20 |  | 38.101-1, 7.3.2 |  |  |  |
| 8d |  | FDD Band n24 |  | 38.101-1, 7.3.2 |  |  |  |
| 9 |  | FDD Band n25 |  | 38.101-1, 7.3.2 |  |  |  |
| 9a |  | FDD Band n26 |  | 38.101-1, 7.3.2 |  |  |  |
| 10 |  | FDD Band n28 |  | 38.101-1, 7.3.2 |  |  |  |
| 10a |  | FDD Band n29 |  | 38.101-1, 7.3A.2.4 |  |  |  |
| 10b |  | FDD Band n30 |  | 38.101-1, 7.3.2 |  |  |  |
| 11 |  | TDD Band n34 |  | 38.101-1, 7.3.2 |  |  |  |
| 12 |  | TDD Band n38 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 13 |  | TDD Band n39 |  | 38.101-1, 7.3.2 |  |  |  |
| 14 |  | TDD Band n40 |  | 38.101-1, 7.3.2 |  |  |  |
| 15 |  | TDD Band n41 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 16 |  | TDD Band n48 |  | 38.101-1, 7.3.2 |  |  |  |
| 17 |  | TDD Band n50 |  | 38.101-1, 7.3.2 |  |  |  |
| 18 |  | TDD Band n51 |  | 38.101-1, 7.3.2 |  |  |  |
| 18a |  | Reserved |  |  |  |  |  |
| 18b |  | TDD Band n53 |  | 38.101-1, 7.3.2 |  |  |  |
| 19 |  | FDD Band n65 |  | 38.101-1, 7.3.2 |  |  |  |
| 20 |  | FDD Band n66 |  | 38.101-1, 7.3.2 |  |  |  |
| 21 |  | FDD Band n70 |  | 38.101-1, 7.3.2 |  |  |  |
| 22 |  | FDD Band n71 |  | 38.101-1, 7.3.2 |  |  |  |
| 23 |  | FDD Band n74 |  | 38.101-1, 7.3.2 |  |  |  |
| 24 |  | TDD Band n77 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 25 |  | TDD Band n78 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 26 |  | TDD Band n79 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/1. NOTE 2: Support of 2 Rx for this band is allowed only for vehicular UEs i.e. if support has been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------