3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210921

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1759 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.3.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.3.8 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.3.8 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

3	Branches executed	7

4	Execution Log Files	7

4.1	Samsung SM-G996B	7

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.3.8 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.3.8.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Samsung SM-G996B

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_11_3_8_TestBody() |
| --- | --- |
| Reason for change | - Incorrect cell ID used to change power levels at T2. - At step 2, the UE’s application layer may initiate the voice call on its UI, even though the UE RRC will not allow it to connect because of the barring. It is upto the UE’s implementation on how long it may try to connect this call (on other RATs if possible). This may result in the UE responding the call attempt at step 4 with ‘BUSY’, thus failing the TC. - At step 5B, the UE sends the SERVICE REQUEST message in the RRC message to establish the PDU session, and it will send it again after receiving RRC Release at step 6, as this Release is sent without any proper cause, and the UE may still try to connect the voice call on NR Cell 3 which is still available. This will caue an unexpected received message, thus failing the TC. - Step 5 does not have the preliminary pass verdict. |
| Summary of change | - Corrected the cell id to cell 3. - A call hang-up command is sent to the UE at step 3A that will ensure the UE UI is not BUSY when the call is dialed at step 4. - The SERIVCE REQUEST is handled by the SS and a SERVICE REJECT is sent in response with cause ‘congestion’, so that the UE does not make any further attempts for SERVICE REQUEST. - Added the preliminary pass verdict for TP2 at step 5 NOTE: Changes 2 and 2 are aligned with the prose CR R5-213501 agreed at RAN5#91 |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_3_8_TestBody() runs on NR_BASE_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var boolean v_IsFR1; var float v_TimerValue := 5.0; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell1, -99, -91), cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //@siclog "Step 1" siclog@ //Make the UE attempt an MTSI MO Speech Call //Note 1: MO MMTEL Voice call is triggered by AT commands. f_UT_RequestIMSCall(UT); //Timer T390 is started //Note 2: T390 timer value is calculated by "T390 = (0.7+ 0.6 * rand) * uac-BarringTime". With uac-BarringTime as s256 the worst-case timer value is 180 seconds which covers both FR1 and FR2 reselection scenario. //@siclog "Step 2" siclog@ //Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-VoiceCall' in the next 5 sec? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, mo_VoiceCall, "Step 2"); //@siclog "Step 3" siclog@ //Set the power levels according to "T2" as per Table 11.3.8.3.2-1/2 for UE to reselect to NR Cell 3. f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Step 4" siclog@ //FR1: Wait for 34 seconds after step 3 and within 20 seconds make the UE attempt an MTSI MO Speech Call //FR2: Wait for 130 seconds after step 3 and within 20 seconds make the UE attempt an MTSI MO Speech Call. //Note 1: MO MMTEL Voice call is triggered by AT commands. //Note 3: The wait time for reselection to a newly detected inter frequency cell is selected to cover Tdetect,NR_Inter (25*1280ms=32s for FR1 and 25*4*1280ms=128s for FR2) + TSI-NR (1.28s for FR1 and FR2) = 33.28s rounded up to 34s for FR1 and 129.28s rounded up to 130s for FR2. 20 seconds is an arbitrary value chosen to allow the user to make the MTSI MO Speech Call. v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { // FR1 f_Delay(34.0); } else { // FR2 f_Delay(130.0); } //@siclog "Step 5" siclog@ //Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-VoiceCall' on NR Cell 3? f_NR_RRC_SetupRequest_Def(nr_Cell3, mo_VoiceCall); //@siclog "Step 5A" siclog@ //@sic R5-204019 sic@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell3); //@siclog "Step 5B" siclog@ //@sic R5-204019 sic@ //The UE transmits an RRCSetupComplete. f_NR_RRCSetupComplete_Def(nr_Cell3, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //@siclog "Step 6" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell3); }

After Change:

… … import from UpperTesterCommon all; import from Common4G5G_Templates all; … … function fl_TC_11_3_8_TestBody() runs on NR_BASE_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var boolean v_IsFR1; var float v_TimerValue := 5.0; var Ut_CnfReq_Type v_Cnf := CNF_REQUIRED; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell1, -99, -91), cs_NR_CellPower (nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //@siclog "Step 1" siclog@ //Make the UE attempt an MTSI MO Speech Call //Note 1: MO MMTEL Voice call is triggered by AT commands. f_UT_RequestIMSCall(UT); //Timer T390 is started //Note 2: T390 timer value is calculated by "T390 = (0.7+ 0.6 * rand) * uac-BarringTime". With uac-BarringTime as s256 the worst-case timer value is 180 seconds which covers both FR1 and FR2 reselection scenario. //@siclog "Step 2" siclog@ //Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-VoiceCall' in the next 5 sec? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, mo_VoiceCall, "Step 2"); //@siclog "Step 3" siclog@ //Set the power levels according to "T2" as per Table 11.3.8.3.2-1/2 for UE to reselect to NR Cell 3. f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Step 3A" siclog@ //FR1: Wait for 34 seconds after step 3 and within 20 seconds make the UE attempt an MTSI MO Speech Call //FR2: Wait for 130 seconds after step 3 and within 20 seconds make the UE attempt an MTSI MO Speech Call. //Note 1: MO MMTEL Voice call is triggered by AT commands. //Note 3: The wait time for reselection to a newly detected inter frequency cell is selected to cover Tdetect,NR_Inter (25*1280ms=32s for FR1 and 25*4*1280ms=128s for FR2) + TSI-NR (1.28s for FR1 and FR2) = 33.28s rounded up to 34s for FR1 and 129.28s rounded up to 130s for FR2. 20 seconds is an arbitrary value chosen to allow the user to make the MTSI MO Speech Call. v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) { // FR1 f_Delay(34.0); } else { // FR2 f_Delay(130.0); } //@siclog "Step 4 - To disconnect call initiated at step2 if any" siclog@ f_UT_HangupIMSCall(UT,-,v_Cnf); f_Delay(30.0); f_UT_RequestIMSCall(UT); //@siclog "Step 5" siclog@ //Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-VoiceCall' on NR Cell 3? f_NR_RRC_SetupRequest_Def(nr_Cell3, mo_VoiceCall); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 Setup Req received from UE with cause mo-VoiceCall"); //@siclog "Step 5A" siclog@ //@sic R5-204019 sic@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell3); //@siclog "Step 5B" siclog@ //@sic R5-204019 sic@ //The UE transmits an RRCSetupComplete. f_NR_RRCSetupComplete_Def(nr_Cell3, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //@siclog "Step 6" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. //f_NR_RRCRelease(nr_Cell3); //MOVED BELOW //@siclog "Step7 - Additional step to handle IMS call initiated by UE " siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell3, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_SERVICE_REJECT(cs_GMM_GSM_Cause (omit, '00010110'B), omit, cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00001'B), omit, omit, omit)))); f_NR_RRCRelease(nr_Cell3); //MOVED HERE }

## Change 2

| Function name | f_UT_HangupIMSCall |
| --- | --- |
| Reason for change | For change 1.2, A confirmation for the AT command to hang up the call is needed to ensure that the next steps are executed only after executing the AT command |
| Summary of change | Add a confirmation required parameter to the functio |
| TTCN module | UpperTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_UT_HangupIMSCall(UT_PTC_MTC_PORT p_Port, integer p_CallId := 1) { fl_UT_ApplyCommand(p_Port, cas_UT_Req(RELEASE_IMS_CALL, NO_CNF_REQUIRED, cs_CallIdNumber(int2str (p_CallId)))); }

After Change:

function f_UT_HangupIMSCall(UT_PTC_MTC_PORT p_Port, integer p_CallId := 1, Ut_CnfReq_Type p_CnfRequired := NO_CNF_REQUIRED) { fl_UT_ApplyCommand(p_Port, cas_UT_Req(RELEASE_IMS_CALL, p_CnfRequired, cs_CallIdNumber(int2str (p_CallId)))); }

Executio

# Branches executed

The test case was executed on NR band n78

# Execution Log Files

## Samsung SM-G996B

The Samsung SM-G996B UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_3_8_LOG.html

- PICS/PIXIT parameter file:

TC_11_3_8_PIXIT.xml

# References

| [1] | R5s210629 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.3.8 |
| --- | --- |