3GPP TSG RAN Meeting #93e															RP-211675

Electronic Meeting, September 13 - 17, 2021

3GPP TSG-RAN WG4 Meeting # 100-e	R4-2114750

# Electronic Meeting, August 16-27, 2021

# Title:	LS on the progress of the study item on extended 600MHz NR band

# Response to:	-

# Release:	Rel-17

# Work Item:	FS_NR_600MHz_ext

Source:	3GPP RAN WG4

To:	Asia-Pacific Telecommunity Wireless Group (AWG)

Cc:	3GPP TSG RAN

Contact Person:

#### Name: Mansoor Shafi

#### E-mail Address: Mansoor.Shafi@spark.co.nz

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

## 1. Overall Description:

In response to a request from APT Wireless Group on 3GPP RAN WG4 feedback on technical feasibilities of various frequency arrangements in 612 - 703 MHz frequency range, 3GPP RAN WG4 would like to inform on AWG, that the 3GPP Rel-17 Study Item (SI) on the on extended 600MHz NR band for APT region has been completed during the RAN WG4 meeting #100-e (August 2021).

During that SI, a Technical Report (TR) was created in TR 38.860. The TR 38.860 can be also found at: https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3893

RAN4 would like to note that the Technical Report will be considered a draft until it is formally approved by TSG RAN#93-e. AWG should only consider version 17.0.0 the final approved version of the technical report.

The TR 38.860 provides feasibility analyses for band arrangement options B1 and B2, as requested by AWG. Additionally, band arrangement B2a has been defined by RAN WG4 and related feasibility analyses of all arrangements were also captured in the TR 38.860.

## 2. Operating band and channel bandwidth

The band plans for the extended 600 MHz NR band are shown below in Tables 1 to 7.  Option B1 may have a single duplexer. Option B2 has split duplexers, partially overlapping, but part of the same band. Option B2a has two duplexers, but one is band n71 and the other is a new band.

The Tx-Rx is "reverse-duplex"; in other words, the downlink frequency band is below the duplex gap while the uplink frequency band is above the duplex gap. This arrangement is opposite to conventional notation; however, for this band, it provides the benefit of aligning the uplink band adjacent to 3GPP band 28 thereby minimizing interference conditions at the 703 MHz boundary.

Table 1: NR operating band (option B1)

| Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

The above could be implemented as a single or overlapping duplexers. In case of overlapping duplexers, the following option is studied:

Table 2: Duplexer arrangements (option B1)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| Full | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |
| Dual 35 + 35 | 663 MHz 668 MHz | – – | 698 MHz 703 MHz | 612 MHz 617 MHz | – – | 647 MHz 652 MHz | FDD |
| n71 + 25 Dual 35 + 25 | 663 MHz 678 MHz | – – | 698 MHz 703 MHz | 617 MHz 612 MHz | – – | 652 MHz 637 MHz | FDD |

Table 3.: NR operating band (option B2)

| Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 663 MHz | – | 703 MHz | 617 MHz | – | 657 MHz | FDD |

The above could be implemented as a single or overlapping duplexers. In case of overlapping/split duplexers, three sub options are studied.

Table 4: Duplexer arrangements (option B2 35 + 35)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| Duplexer 1 Duplexer 2 | 663 MHz 668 MHz | – – | 698 MHz 703 MHz | 617 MHz 622 MHz | – – | 652 MHz 657 MHz | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |  |  |  |  |

Table 5: Duplexer arrangements (option B2 35+30)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| Duplexer 1 Duplexer 2 | 663 MHz 673 MHz | – – | 698 MHz 703 MHz | 617 MHz 627 MHz | – – | 652 MHz 657 MHz | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |  |  |  |  |

Table 6: Duplexer arrangements (option B2 35+25)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| Duplexer 1 Duplexer 2 | 663 MHz 678 MHz | – – | 698 MHz 703 MHz | 617 MHz 632 MHz | – – | 652 MHz 657 MHz | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |  |  |  |  |

Table 7: NR operating bands (option B2a)

| Operating Bands | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| nX | 673 MHz | – | 703 MHz | 627 MHz | – | 657 MHz | FDD |
| NOTE : These are two bands, band n71 plus band nX. A UE that complies with NR requirements in this specification shall also comply with NR Band 71 minimum requirements |  |  |  |  |  |  |  |

## 3. Additional Information

Any updates on the regulatory requirements for the APT region in the frequency range of interest, possibly including normative regulatory updates which could impact the 600MHz band plan in APT region, including but not limited to, DTV and RAS services, would be of interest to 3GPP for any possible further work on the APT 600 MHz arrangements by 3GPP.

4. Actions:

None

5. Date of Next TSG RAN Meetings:

TSG RAN meeting #93-e 	13 - 17 September 2021			Online meeting

TSG RAN meeting #94-e 	13 - 16 December 2021			Online meeting

6. Date of Next RAN WG4 Meetings:

RAN WG4 Meeting #101-e 	1st – 12th November 2021		Online meeting

RAN WG4 Meeting #102		21st – 25th February 2021		Athens, Greece