# 3GPP TSG-RAN Meeting #93-e	RP-212219

# e-meeting, September 13 – 17, 2021

# Title:	NTN features for Rel-18

# Source:	Eutelsat

# Type:	Discussion

# Document for: 	Discussion

# Agenda Item:	9.0.1 General

# Introduction

This sets out the Eutelsat view on the proposed R18 candidate features, in particular focusing on points that were considered contentious by the moderator in [1]. In addition to the points below, it is assumed that any R17 ‘leftovers’ would also be treated / considered for R18.

# Discussion

## Candidate Rel-18 NR-NTN enhancements

### DL PAPR reduction

We support work on DL PAPR reduction. The power consumption, size and thermal impact of RF high power amplifier stages are extremely important aspects of many satellite payload designs. High amplifier efficiency whilst achieving the required RF performance (Power output, OOB, Vector-Error, etc.) depends heavily on the waveform characteristics. Techniques that can improve the link budget at a given average power (e.g., reduce the DL PAPR) are therefore a high priority to improving satellite payload efficiency, thus enabling a wider adoption of NR over satellite networks.

Proposal 1: Study ways to reduce DL PAPR for NTN Rel-18.

### Support of NTN-NTN asynchronous multi connectivity & Carrier Aggregation

We support work on multi-connectivity. This technique will increase user throughput and QoE by using spatial diversity between NGSO & NGSO, GEO & NGSO and add resilience/ improved availability to the network. Achieving reliable connections at higher frequencies can be improved through providing diverse paths. Even at lower frequencies (e.g., S-band) scintillation at certain latitudes and times of year can be significantly higher than assumed in [2].

An example NR dual-connectivity scenario is where a single user equipment is connected simultaneously via a GEO satellite and a LEO satellite to a 5G CN assuming transparent satellite payloads.

Similar schemes can be envisaged with regenerative payloads in one or both satellites.

Gateway locations could be the same for both and gNodeB basebands could then be shared, however, using different gateway locations provides an additional degree of resilience to the network.

The use of multiple connectivity to satellites with different coverage areas (and durations) also allows a layered functionality so that, for example, general information can be provided in the larger area and information of more specific interest in the smaller area. The nature and throughput of signalling and user traffic of the two links can also be different and complementary.

For a R18 study, we would propose the baseline assumption to support at least dual-connectivity operation with satellite nodes in the same orbits and different orbits (e.g., LEO-LEO, LEO-GEO). Transparent and regenerative payloads should also be considered.

Carrier Aggregation could be beneficial to further improve throughput but raises significant challenges if there are significant differences in delay; it is considered a lower priority in R18 than dual-connectivity, or multi-connectivity. It would be a desirable addition to any R18 study.

Proposal 2: Study the technical requirements to support the introduction of Dual/Multi-connectivity in NTN networks.

### Support of MBS

This would be a new service in NTN and, in line with normal procedures, we would prefer to see this first discussed in SA1. A use-case/ business justification for any work, at least in the R18 timeframe, needs to be established prior to RAN dedicating TU to this feature. We can see some potentially useful applications for Alerts, and other Public Safety applications.

Proposal 3: Do not introduce MBS in NTN R18 until service definition requirements are established.

### Network based UE location (network verified/provided)

We support this activity to the extent that it is required to achieve regulatory compliance.

Proposal 4: Study the technical requirements to meet regulatory compliance for network-based UE location.

## Candidate Rel-18 IoT-NTN enhancements

### Support for store-and-forward on-board NTN  payload

The primary benefit of Store-Forward is the reduction in ground segment – it reduces the number of required ground-stations, which therefore can be positioned in ‘friendly’ locations. This architecture offers a new optimized way to implement an existing (R17) service (support non-delay critical IoT applications via NTN); we therefore support this item.

Clearly it is a major specification impact and performance trade-off to be considered (e.g., paging, disconnection timers, etc.). There will be impact on specifications outside the RAN scope so identifying all the relevant points in SA2 and impact on the CN (EPC/ 5GC) on these cases is important.

Proposal 5: A study of store-forward satellite architecture should be part of a R18 IoT-NTN Study.

### Further uplink improvements

There has been some discussion in RAN1 for R17 already on this feature. PUR would be interesting (if it can be made to work reliably, e.g. for GEO) if it could benefit capacity and reduce power consumption (increase battery life). However, our own evaluation of the techniques is that PUR, even if implemented as in TN, would only bring marginal benefits in the IoT-NTN use-cases of most interest, therefore, we would not propose to focus on PUR.

Instead we propose a study which takes a broader view on uplink improvements. Rel-18 will have to include a number of [leftover] capabilities that would e.g. further reduce power consumption, improve spectral efficiency, resources usage and more generally IoT service provision over NTN for the considered deployment scenarios. Technical Feasibility of any options should be established early in the study for LEO, MEO, GEO to allow time for them to be integrated into the R18 release.

Proposal 6: A study which takes a broader view on uplink improvements than just PUR, with  the introduction of enhancements to further reduce power consumption, improve spectral efficiency and limit resource usage.

### UE without GNSS

GNSS for system operation is imposed in the R17 assumptions both in the network and in the devices. However, total reliance on GNSS is a concern to many in the industry and alternative approaches should be considered for resilience and potential power reduction.

Proposal 7: We propose there should be consideration of a joint NR/IoT-NTN study on the potential of removing device reliance on GNSS.

# Conclusions and Proposals

In this document we have set out areas for which we see has having potentially high value to study for NTN in R18. Our specific proposals are set out below.

Proposal 1: Study ways to reduce DL PAPR for NTN Rel-18

Proposal 2: Study the technical requirements to support the introduction of Dual/Multi-connectivity in NTN networks.

Proposal 3: Do not introduce MBS in NTN R18 until service definition requirements are established.

Proposal 4: Study the technical requirements to meet regulatory compliance for network-based UE location.

Proposal 5: A study of store-forward satellite architecture should be part of a R18 IoT-NTN Study.

Proposal 6: A study which takes a broader view on uplink improvements than just PUR, with the introduction of enhancements to further reduce power consumption, improve spectral efficiency and limit resource usage.

Proposal 7: We propose there should be consideration of a joint NR/IoT-NTN study on the potential of removing device reliance on GNSS.

# References

[1] RP-211658,  Moderator's summary for discussion [RAN93e-R18Prep-08] NTN evolution

[2]  3GPP TR 38.821 Solutions for NR to support non-terrestrial networks (NTN) (Release 16)