3GPP TSG-RAN5 Meeting #93-e	R5-216761

# Electronic Meeting, 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2053 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PDCCH Search Space Ext configuration in 38.508-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PDCCH Search Space Ext configuration is absented in 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDCCH Search Space Ext configuration is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work item will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.4.2.0	Parameters common to all Demod and CSI tests

Physical layer parameters

<Unchanged Sections Skipped>

PDCCH-config

Table 5.4.2.0-6: PDCCH-ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | ControlResourceSetId |  |  |
| frequencyDomainResources | Table 5.2-2 for tested channel bandwidth and subcarrier spacing |  | FR1 |
| frequencyDomainResources | Table 7.2-2 for tested channel bandwidth and subcarrier spacing |  | FR2 |
| Duration | 2 | SearchSpace duration of 2 symbols | DEMOD_FR1, CSI_FR1, CSI_FR2 |
|  | 1 | SearchSpace duration of 1 symbols | DEMOD_FR2, PDCCH_FR1, PDCCH_FR2 |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | Null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 0 | entry 1 TCI State #0 |  |
|  | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DEMOD_FR1 | DEMOD testing in FR1. |
| DEMOD_FR2 | DEMOD testing in FR2. |
| PDCCH_FR1 | PDCCH testing in FR1 |
| PDCCH_FR2 | PDCCH testing in FR2 |
| CSI_FR1 | CSI testing in FR1 |
| CSI_FR2 | CSI testing in FR2 |

Table 5.4.2.0-7: PDCCH Search Space

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Starting symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  |  |
| ue-Specific SEQUENCE { |  |  |  |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.4.2.0-7a: PDCCH Search Space Ext

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | ControlResourceSetId |  |  |
| searchSpaceType-r16 | Not present |  |  |
| searchSpaceGroupIdList-r16 | Not present |  |  |
| freqMonitorLocations-r16 | Not present |  |  |
| } |  |  |  |