c3GPP TSG-RAN5 Meeting #93-e	R5-216499

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2482 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of cell power level for FR2 in Radio Link Failure TC 8.1.6.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Cell power level for FR2 is FFS in the test case. 2. In current NR power level configuration of T0 in Table 8.1.6.2.3.3.2-1, the condition B2-1 is not fulfilled, so the UE will not report the MR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Updated the Cell power level for FR2 in Table 8.1.6.2.3.3.2-2. 2. Corrected the NR power level of T0 for entering event B2 in Table 8.1.6.2.3.3.2-1. 3. Changed the NR power level of T2 as -88, namely “Serving cell” in Table 8.1.6.2.3.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test purpose of Inter-RAT MDT in FR2 could not be covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

8.1.6.2.3	Inter-RAT MDT / Radio Link Failure / Reporting at E-UTRA Inter-RAT handover

8.1.6.2.3.1	Test Purpose (TP)

(1)

with {UE selects the E-UTRA cell after detection of radio link failure in NR cell }

ensure that {
  when { UE receives a HANDOVER FROM E-UTRA COMMAND message }

then { UE transmits an RRCReconfigurationComplete message containing rlf-InfoAvailable and enters NR RRC_CONNECTED state }

}

(2)

with { UE in RRC_CONNECTED having reported that the UE has radio link failure information available }

ensure that {
  when {UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for E-UTRA neighbour cell }

}

8.1.6.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.10.5, 5.4.2.3, 5.3.5.3, and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

…

1>	for each of the configured measObjectNR in which measurements are available:

2>	if the SS/PBCH block-based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell(in case HO failure) or PCell (in case RLF), ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

4>	for each neighbour cell included, include the optional fields that are available;

2>	if the CSI-RS based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure;

4>	for each neighbour cell included, include the optional fields that are available;

1>	for each of the configured EUTRA frequencies in which measurements are available;

2>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, and based on measurements collected up to the moment the UE detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

2>	if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover:

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

3>	else if the last RRCReconfiguration message including the reconfigurationWithSync concerned a handover to NR from E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA:

4>	include the eutraPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the E-UTRA PCell where the last RRCReconfiguration message including reconfigurationWithSync was received embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

…

[TS 38.331, clause 5.4.2.3]

The UE shall:

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	perform RRC reconfiguration procedure as specified in 5.3.5;

NOTE:	If the UE is connected to 5GC of the source E-UTRA cell, the delta configuration for PDCP and SDAP can be used for intra-system inter-RAT handover. For other cases, source RAT configuration is not considered when the UE applies the reconfiguration message of target RAT.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCReconfigurationComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.2.3.3	Test description

8.1.6.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell and E-UTRA Cell 1 is the inter-RAT neighbour cell of NR Cell 1

-	System information combination NR-6 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

-	System information combination 31 as defined in TS 36.508 [7] clause 4.4.3.1 is used in the E-UTRA cell.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.6.2.3.3.2	Test procedure sequence

Table 8.1.6.2.3.3.2-1 and Table 8.1.6.2.3.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 8.1.6.2.3.3.2-1 and Table 8.1.6.2.3.3.2-2. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.2.3.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -90 | - | Power levels are such that entry condition 1 for event B2 and entry condition 2 are both satisfied: Mp + Hys < Thresh1 and Mn + Ofn + Ocn - Hys > Thresh2 |
|  | Cell-specific RS EPRE | dBm/15kHz | - | -73 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | - | Only E-UTRA Cell 1 is available. |
|  | Cell-specific RS EPRE | dBm/15kHz | - | -73 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | - |  |
|  | Cell-specific RS EPRE | dBm/15kHz | - | -85 |  |

Table 8.1.6.2.3.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -92 | - | Power levels are such that entry condition 1 for event B2 and entry condition 2 are both satisfied: Mp + Hys < Thresh1 and Mn + Ofn + Ocn - Hys > Thresh2 |
|  | Cell-specific RS EPRE | dBm/15kHz | - | -73 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | - | Only E-UTRA Cell 1 is available. |
|  | Cell-specific RS EPRE | dBm/15kHz | - | -73 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | - |  |
|  | Cell-specific RS EPRE | dBm/15kHz | - | -85 |  |

Table 8.1.6.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including measConfig to setup inter-RAT measurement on NR Cell1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The UE transmits a MeasurementReport message on NR Cell 1. | --> | MeasurementReport | - | - |
| 4 | The SS changes NR Cell1 and E-UTRA Cell 1 parameters according to the row "T1" in table 8.1.6.2.3.3.2-1/2. | - | - | - | - |
| 5 | Generic test procedure in TS 38.508-1 Table 4.9.7.2.2-1 is performed on E-UTRA Cell 1. | - | - | - | - |
| 6 | The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.2.3.3.2-1/2. | - | - | - | - |
| 7 | The SS transmits a MobilityFromEUTRACommand message on E-UTRA Cell 1. | <-- | E-UTRA RRC: MobilityFromEUTRACommand | - | - |
| 8 | Check: Does the UE transmit a RRCReconfigurationComplete message containing rlf-InfoAvailable on NR Cell 1? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 9 | The UE transmits a ULInformationTransfer message and a REGISTRATION REQUEST message indicating “mobility registration updating” is sent to update the registration of the actual tracking area. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION REQUEST | - | - |
| 10 | SS sends an DLInformationTransfer message and a REGISTRATION ACCEPT message containing a 5G-GUTI. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 11 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 12 | The procedure in table 4.5A.2C.2.2-1 in TS 38.508-1 [4] is performed. | - | - | - | - |
| 13 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 14 | Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.2.3.3.3	Specific message contents

Table 8.1.6.2.3.3.3-1: RRCReconfiguration (step 1, Table 8.1.6.2.3.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 8.1.6.2.3.3.3-2: MeasConfig (Table 8.1.6.2.3.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 | MeasObjectIdNR-f1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| ssbFrequency | ssbFrequency IE equals the ARFCN for NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 | MeasObjectIdE-UTRA-f1 |  |
| measObject CHOICE { |  |  |  |
| measObjectEUTRA SEQUENCE { |  |  |  |
| carrierFreq | ARFCN-ValueEUTRA for E-UTRA Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-EventB2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig with Condition INTER-RAT |  |  |
| measGapConfig | MeasGapConfig |  |  |
| } |  |  |  |

Table 8.1.6.2.3.3.3-3: ReportConfigInterRAT-EventB2 (Table 8.1.6.2.3.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-141 with condition EVENT_B2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 SEQUENCE { |  |  |  |
| rsrp | 73 | -83dBm | FR1 |
|  | 73 | -83dBm | FR2 |
| } |  |  |  |
| b2-Threshold2EUTRA SEQUENCE { |  |  |  |
| rsrp | 58 | -83dBm | FR1 |
|  | 58 | -83dBm | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.2.3.3.3-4: MeasurementReport (step 4, Table 8.1.6.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry | Report NR Cell 1 |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | Physical layer cell identity of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultEUTRA { | 1 entry | Report E-UTRA neighbour cell |  |
| MeasResultEUTRA[1] SEQUENCE { |  | entry 1 |  |
| eutra-PhysCellId | Physical layer cell identity of E-UTRA Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrp | (0..97) |  |  |
| rsrq | (0..34) |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.2.3.3.3-5: MobilityFromEUTRACommand (step 8, Table 8.1.6.2.3.3.2-3)

| Derivation Path: TS 36.508 [7] table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| purpose CHOICE { |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | nr |  |  |
| targetRAT-MessageContainer | RRCReconfiguration |  |  |
| nas-SecurityParamFromEUTRA | Not present |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.2.3.3.3-6: RRCReconfiguration (Table 8.1.6.2.3.3.3-5)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig as per table 4.6.3-132 in TS 38.508-1 [4] with conditions SRB1,SRB2 and DRB1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with conditions SRB1,SRB2 and DRB1 | OCTET STRING (CONTAINING CellGroupConfig) |  |
| fullConfig |  | true |  |  |
| masterKeyUpdate |  | MasterKeyUpdate |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.2.3.3.3-7: RRCReconfigurationComplete (step 9, Table 8.1.6.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |
| rlf-InfoAvailable-r16 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.2.3.3.3-8: UEInformationRequest (step 10, Table 8.1.6.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |
| rlf-ReportReq-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.2.3.3.3-9: UEInformationResponse (step 11, Table 8.1.6.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | Not present |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultListEUTRA-r16 SEQUENCE(SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r16[1] | Same as E-UTRA Cell 1 |  |  |
| measResultList-r16[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| eutra-PhysCellId[1] | Same as E-UTRA Cell 1 |  |  |
| measResult[1] | MeasQuantityResults of E-UTRA Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| c-RNTI-r16 | the value of the C-RNTI of the UE |  |  |
| previousPCellId-r16 | Not present |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 CHOICE { |  |  |  |
| pci-arfcn-r16 | Same as NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 | Not present |  |  |
| timeConnFailure-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | t310-Expiry |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<<< END OF CHANGES >>>