3GPP TSG-RAN5 Meeting #93-e	R5-217931

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2620 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MDT TC 8.1.6.3.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The release information of conformance requirements is missing. - The parameter and unit in power level tables are used for LTE and the FR2 is missing. - The rlf-InfoAvailable-r16 shall be set to true in RRCReestablishmentComplete. - The message contents of RRCReconfiguration and UEInformationResponse don`t align with TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The release information was added. - The power level tables were updated. - The specific message contents were updated. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this case unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-217294 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-217294 -> R5-217931 |  |  |  |  |  |  |  |  |

[Start of change]

8.1.6.3.3.2	Inter-System MDT / Radio Link Failure / Logging and reporting / WLAN measurement collection

8.1.6.3.3.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable  included }

}

(2)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report-r16 including logMeasResultListWLAN }

}

(3)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq and a WLAN AP is not included in wlan-NameListConfig }

then { UE sends the UEInformationResponse message not including the measurement result of the WLAN AP not included in wlan-NameListConfig }

}

8.1.6.3.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.3.5.9, 5.3.7.5, 5.3.10.5, 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.9]

The UE shall:

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

2>	if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover:

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.6.3.3.2.3	Test description

8.1.6.3.3.2.3.1	Pre-test conditions

System Simulator:

- NR Cell 1, NR Cell 2, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

UE:

None

Preamble:

- The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.3.2.3.2	Test procedure sequence

Table 8.1.6.3.3.2.3.2-1/2 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.3.2.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Cell 27 (WLAN AP1) | Cell 27b (WLAN AP2) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 |  |  |  |
|  | BeaconRSSI | dBm |  |  | -60 | -50 |  |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | "Off" | -79 |  |  | Only Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  | -60 | -50 |  |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3. |  |  |  |  |  |  |  |

Table 8.1.6.3.3.2.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Cell 27 (WLAN AP1) | Cell 27b (WLAN AP2) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -94 |  |  |  |
|  | BeaconRSSI | dBm |  |  | -60 | -50 |  |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | "Off" | -82 |  |  | Only Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  | -60 | -50 |  |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 Table 6.2.2.2.2-2. |  |  |  |  |  |  |  |

Table 8.1.6.3.3.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS changes NR Cell 1 and Cell 2 parameter according to the row "T1" in Table 8.1.6.3.3.2.3.2-1/2. | - | - | - | - |
| 4 | The UE transmits an RRCReestablishmentRequest message on NR Cell 2. | --> | RRCReestablishmentRequest | - | - |
| 5 | The SS transmits an RRCReestablishment message on NR Cell 2. | <-- | RRCReestablishment | - | - |
| 6 | Check: Does the UE transmit an RRCReestablishmentComplete message on NR Cell 2 with rlf-InfoAvailable included? | --> | RRCReestablishmentComplete | 1 | P |
| 7 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB on NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS transmits a UEInformationRequest message on NR Cell 2. | <-- | UEInformationRequest | - | - |
| 10 | Check: Does the UE transmit a UEInformationResponse message with rlf-Report-r16 with logMeasResultListWLAN including one entry (WLAN AP 1) measurement result? | --> | UEInformationResponse | 2, 3 | P |

8.1.6.3.3.2.3.3	Specific message contents

Table 8.1.6.3.3.2.3.3-1: RRCReconfiguration (step 1, Table 8.1.6.3.3.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1540 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| idc-AssistanceConfig-r16 | Not present |  |  |
| drx-PreferenceConfig-r16 | Not present |  |  |
| maxBW-PreferenceConfig-r16 | Not present |  |  |
| maxCC-PreferenceConfig-r16 | Not present |  |  |
| maxMIMO-LayerPreferenceConfig-r16 | Not present |  |  |
| minSchedulingOffsetPreferenceConfig-r16 | Not present |  |  |
| releasePreferenceConfig-r16 | Not present |  |  |
| referenceTimePreferenceReporting-r16 | Not present |  |  |
| btNameList-r16 | Not present |  |  |
| wlanNameList-r16 |  |  |  |
| Setup SEQUENCE (SIZE (1..maxWLAN-Name-r16)) OF WLAN-Name-r16 { | 1 entry | WLAN AP1 |  |
| WLAN-Name-r16 | Set as per Table 4.4.8-1 of TS 36.508 | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.2.3.3-2: RRCReestablishmentComplete (step 6, Table 8.1.6.3.3.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| connEstFailInfoAvailable-r16 | Not present |  |  |
| rlf-InfoAvailable-r16 | true |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.2.3.3-3: UEInformationRequest (step 9, Table 8.1.6.3.3.2.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.3.3.2.3.3-4: UEInformationResponse (step 10, Table 8.1.6.3.3.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 | Not checked |  |  |
| measResultNeighCells-r16 | Not checked |  |  |
| c-RNTI-r16 | Not checked |  |  |
| previousPCellId-r16 | Not checked |  |  |
| failedPCellId-r16 | Not checked |  |  |
| reconnectCellId-r16 | Not checked |  |  |
| timeUntilReconnection-16 | Not checked |  |  |
| reestablishmentCellId-r16 | Not checked |  |  |
| timeConnFailure-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Not checked |  |  |
| connectionFailureType-r16 | Not checked |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| locationInfo-r16 SEQUENCE { | Not checked |  |  |
| commonLocationInfo-r16 | Not checked |  |  |
| bt-LocationInfo-r16 | Not present |  |  |
| wlan-LocationInfo-r16 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r16)) OF LogMeasResultWLAN-r16 { |  |  |  |
| LogMeasResultListWLAN-r16[1] SEQUENCE { | 1 entry |  |  |
| wlan-Identifiers-r16 SEQUENCE { |  | Entry 1 |  |
| ssid-r12 | ssid for WLAN AP 1 |  |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| rssiWLAN-r16 | Any allowed value |  |  |
| rtt-WLAN-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| sensor-LocationInfo-r16 | Not present |  |  |
| } |  |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-v1650 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]