3GPP TSG-WG1 Meeting #107	R1-2112926

e-meeting, November 11 – 19, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0564 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new bandwidths for LTE-based 5G terrestrial broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part1 |  |  |  |  |  | Date: |  |  | 2021-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of new bandwidths for LTE-based 5G terrestrial broadcast |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a statement on which bandwidth to use for broadcast |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for 6/7/8 MHz bandwidths |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.5	Physical multicast channel

The physical multicast channel shall be processed and mapped to resource elements as described in clause 6.3 with the following exceptions:

-	No transmit diversity scheme is specified.

-	Layer mapping and precoding shall be done assuming a single antenna port and the transmission shall use antenna port 4.

-	The PMCH can only be transmitted in the MBSFN region. For PMCH with Δf  = 15 kHz, the index  in the first slot in the MBSFN subframe fulfils l≥lPMCHStart  where lPMCHStart is equal to the value given by the higher layer parameter non-MBSFNregionLength [9].

-	The PMCH shall use extended cyclic prefix.

-	The PMCH is not mapped to resource elements used for transmission of MBSFN reference signals.

-	In clause 6.3.1, for Δf = 1.25 kHz and Δf ≈ 0.37 kHz, the scrambling generator shall be initialised at the start of each slot.

-	For Δf=182944Ts≈0.37 kHz the following exception applies to clause 6.3.5:

-	The text "which meet all of the following criteria in the current subframe" shall be replaced by "which meet all of the following criteria in the current slot"

-	The mapping to resource elements k,l on antenna port p not reserved for other purposes shall be in increasing order of first the index k over the assigned physical resource blocks and then the index l.

-	For PMCH symbols belonging to an MBSFN area with NRBPMCH configured, NRBDL shall be replaced by NRBPMCH in clauses 6.3, 6.10.2, and 6.12.