3GPP TSG RAN Meeting #94-e								                                                 RP-213400

Electronic Meeting, December 6-17, 2021

Source: 	ZTE

Title:	Discussion on NR-NTN in Rel-18

Agenda item:	8A.2

Document for:	Discussion

- Introduction

According to rounds of discussion, the continuous evolution of NR-NTN is taken as one of the topic for Rel-18 with the draft WI [1] based on the summary of [RAN94e-R18Prep-13] [2]. Although general consensus on following four aspects are achieved, further discussion to converge the scope of some bullets is needed:

- Coverage enhancement

- NR-NTN deployment in above 10 GHz bands

- NTN-TN and NTN-NTN mobility and service continuity enhancements

- Network verified and Network based UE location

Then, in this contribution, views to further update the scope of WI are proposed with corresponding justification.

- Discussion on the scope of NR-NTN

In general, refinement on description and scope on following aspects are proposed:

- NR-NTN deployment in above 10 GHz bands

For the NR-NTN deployment in above 10 GHz, as highlighted in the assumption below, only VSAT UE is considered:

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

- Note: In Rel-17 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM.

According to the existing regulation defined in ITU, there are currently three types of ESIMs: ESIM on board aircraft (aeronautical ESIM), ESIM on board ships (maritime ESIM) and ESIM on board land vehicles (land ESIM)[3]. And based on the WRC-19’s decision (cited below), for Ka-band, the application should be restricted to the GSO only.

- The typical data rates currently provided by terminals operating in networks serving ESIM are around 100 Mbit/s - much higher, or faster, than those provided historically by satellite networks and systems in the mobile-satellite service (MSS), which use lower frequency bands (e.g. the 1.5 GHz, 1.6 GHz, 2.1 GHz, and 2.4 GHz bands).

- To address the increasing need of ESIM for radio-frequency spectrum, while protecting other and existing services, the ITU World Radiocommunication Conference (WRC-19), which took place in Sharm el-Sheikh, Egypt, from 28 October to 22 November 2019, decided on the regulatory and technical conditions under which the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) can be used by the three types of ESIMs communicating with geostationary (GSO) space stations in the fixed-satellite service (FSS).

Regarding the supports for NSGO in 3GPP, it can be postponed in later release once the decision has been made in WRC-23.  Moreover, as highlighted in previous discussion, the ITU regulation including the scenario for interference evaluation, should also be followed in the co-existence study with terrestrial network.

Proposal 1: Clarification on the definition of ESIM with limited applicability to GSO should be captured in WI.

- Network verified and Network based UE location

For the positioning related issue, by keeping the legacy assumption as UE with GNSS-capability, the main intention for the study is to satisfy the regulation requirement to verify the UE’s location, e.g., to avoid the faked GNSS reporting. Then, as discussed in the previous meeting, following scopes are proposed.

- Study detailed regulatory requirement for network-verified UE location, e.g. accuracy requirement.

- Study and evaluate solutions for network to verify UE reported location information [RAN1,RAN2]

  - For Network based UE location, re-use of Rel-17 UE-specific Timing Advance report can be considered as baseline

- Study network-based positioning solutions suitable for NTN and identify achievable performance considering privacy and regulation requirements and considering that the number of satellites in view can be limited (including single satellite) [RAN1,RAN2]

- Study possible issues of applying existing network-based positioning solutions in NTN and evaluate possible enhancements if needed. [RAN1,RAN2,RAN3]

However, among these bullet-points, two duplicated solutions are proposed with different complexity. In our view, with consideration on the motivation of this aspect along with the potential workload, it’s preferred to only focus on the verification based solution in Rel-18.

Proposal 2: Studying on the solutions for network to verify UE reported location information should be prioritized in Rel-18 instead of network-based positioning solution.

Then, according to the justification above, updates on the scope of SI listed in [1] is expected as listed in the proposal below.

Proposal 3: Refinement of scope for NR-NTN with following updates should be considered.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The work item aims at specifying enhancements for NG-RAN based NTN (non-terrestrial networks) according to the following assumptions [with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios]:

- GSO and NGSO (LEO and MEO) with transparent payload.

  - ESIM device is not applicable for NGSO

- Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO

- FDD mode

- UEs with GNSS capabilities

- Both “VSAT” devices with directive antenna (including fixed and moving platform mounted devices and commercial handset terminals (e.g. Power class 3) are supported in FR1

- Only “VSAT” devices with directive antenna (including fixed and moving platform mounted devices) are supported in above 10 GHz bands.

Note: In Rel-17 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM including ESIM on board aircraft (aeronautical ESIM), ESIM on board ships (maritime ESIM) and ESIM on board land vehicles (land ESIM).

====================<unchanged text omitted>==================

4.1.4	Network verified and Network based UE location

The following items are taken as the objectives for a Study Item or a study phase.

- Study detailed regulatory requirement for network-verified UE location, e.g. accuracy requirement.

- Study and evaluate solutions for network to verify UE reported location information [RAN1,RAN2]

  - For Network based UE location, re-use of Rel-17 UE-specific Timing Advance report can be considered as baseline

RAN to determine by RAN#XX whether the study has identified any need for Network verified and/or Network based UE location specification support in Rel-18.

- Conclusions

In this contribution, the general views to refine the scope of WI for NR-NTN in Rel-18 are elaborated with following proposals:

Proposal 1: Clarification on the definition of ESIM with limited applicability to GSO should be captured in WI.

Proposal 2: Studying on the solutions for network to verify UE reported location information should be prioritized in Rel-18 instead of network-based positioning solution.

Proposal 3: Refinement of scope for NR-NTN with following updates should be considered.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The work item aims at specifying enhancements for NG-RAN based NTN (non-terrestrial networks) according to the following assumptions [with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios]:

- GSO and NGSO (LEO and MEO) with transparent payload.

  - ESIM device is not applicable for NGSO

- Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO

- FDD mode

- UEs with GNSS capabilities

- Both “VSAT” devices with directive antenna (including fixed and moving platform mounted devices and commercial handset terminals (e.g. Power class 3) are supported in FR1

- Only “VSAT” devices with directive antenna (including fixed and moving platform mounted devices) are supported in above 10 GHz bands.

Note: In Rel-17 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM including ESIM on board aircraft (aeronautical ESIM), ESIM on board ships (maritime ESIM) and ESIM on board land vehicles (land ESIM).

====================<unchanged text omitted>==================

4.1.4	Network verified and Network based UE location

The following items are taken as the objectives for a Study Item or a study phase.

- Study detailed regulatory requirement for network-verified UE location, e.g. accuracy requirement.

- Study and evaluate solutions for network to verify UE reported location information [RAN1,RAN2]

  - For Network based UE location, re-use of Rel-17 UE-specific Timing Advance report can be considered as baseline

RAN to determine by RAN#XX whether the study has identified any need for Network verified and/or Network based UE location specification support in Rel-18.

Reference

- RP-212713 NR NTN (Non-Terrestrial Networks) enhancement

- RP-212673 Moderator’s summary of discussion [RAN94e-R18Prep-13] NTN evolution

- Satellite issues: Earth stations in motion (ESIM), https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx.