3GPP TSG RAN Meeting #94e	RP-21xxxx

Electronic Meeting, December 06 - 17, 2021

- Source:	Moderator (MediaTek)

- Title:	New WID: NR further mobility enhancements

- Document for:	Approval

Agenda Item:	8A.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	NR further mobility enhancements

## Acronym: NR_Mob_enh2

## Unique identifier:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

Potential target Release: Rel-18

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x |  |  |
| No |  |  |  |  |  |
| Don't know |  |  |  | x |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: None

## 3	Justification

When the UE moves from the coverage area of one cell to another cell, at some point a serving cell change needs to be performed. Currently serving cell change is triggered by L3 measurements and is done by RRC signalling triggered Reconfiguration with Synchronisation for change of PCell and PSCell, as well as release add for SCells when applicable. All cases involve complete L2 (and L1) resets, leading to longer latency, larger overhead and longer interruption time than beam switch mobility. The goal of L1/L2 mobility enhancements is to enable a serving cell change via L1/L2 signalling, in order to reduce the latency, overhead and interruption time.

In Rel-17 Conditional PSCell change (CPC)/Conditional PSCell change/addition (CPAC), a CPC/CPAC-configured UE has to release the CPC/CPAC configurations when performing random access towards the target PSCell. Hence the UE doesn’t have a chance to perform subsequent CPC/CPAC without prior CPC/CPAC reconfiguration and re-initialization from the network. This will increase the delay for the cell change and increase the signaling overhead, especially in the case of frequent SCG changes when operating FR2. Therefore, MR-DC with selective activation of cell groups aims at enabling subsequent CPC/CPAC after SCG change, without reconfiguration and re-initialization on the CPC/CPAC preparation from the network. This results in a reduction of the signalling overhead and interrupting time for CPC/CPAC.

Currently, CHO and MR-DC cannot be configured simultaneously. This limits the usefulness of these two features when MR-DC is configured. If it is not completed in Rel-17, Rel-18 should specify mechanisms for CHO and MR-DC to be configured simultaneously. However, this alone may not be sufficient to optimise MR-DC mobility, as the radio link quality of the conditionally-configured PSCell may not be good enough or may not be the best candidate PSCell when the UE accesses the target PCell, and this may impact the UE throughput. To mitigate this throughput impact, Rel-18 CHO+MRDC can consider CHO including target MCG and multiple candidate SCG for CPC/CPAC.

## 4	Objective

### 4.1	Objective of Core part WI

The detailed objective of this work item are:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements, including inter-cell beam management, L1 measurement and reporting, beam indication, and for non-synchronized scenario to handle TA management [RAN1, RAN2]

  - Note: early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 1: FR2 specific enhancements are not precluded, if any.

Note 2: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups via L3 enhancements:

- To allow subsequent CPC/CPAC after changing SCG without reconfiguration and re-initiation of CPC/CPAC [RAN2, RAN3, RAN4]

- TBD: whether Rel-17 CPC/CPAC mechanism is used as the baseline

- To specify CHO including target MCG and target SCG if it cannot be completed in Rel-17 [RAN3, RAN2]

- To specify CHO including target MCG and candidate SCG for CPC/CPAC  [RAN3, RAN2]

- CHO including target MCG and target SCG is used as the baseline

### 4.2	Objective of Performance part WI

Core requirements associated with above items [RAN4]

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.331 | NR RRC signalling changes to support mobility enhancements | TBD | Core part |
| 38.306 | Necessary UE capabilities to support mobility enhancements | TBD | Core part |
| 38.300 | Possible changes to NR Stage-2 specification | TBD | Core part |
| 37.340 | Possible changes to MR-DC Stage-2 specification | TBD | Core part |
| 38.321 | Possible changes to NR MAC specification | TBD | Core part |
| 38.413 | Possible changes to NR Stage 3 NG-AP to support mobility enhancements | TBD | Core part |
| 38.420 | Possible changes to NR Stage 2 Xn-AP to support mobility enhancements | TBD | Core part |
| 38.423 | Possible changes to NR Stage 3 Xn-AP to support mobility enhancements | TBD | Core part |
| 38.470 | Possible changes to NR Stage 2 F1-AP to support mobility enhancements | TBD | Core part |
| 38.473 | Possible changes to NR Stage 3 F1-AP to support mobility enhancements | TBD | Core part |
| 38.212 | Possible changes to introduce L1/L2 mobility enhancements | TBD | Core part |
| 38.213 | Possible changes to introduce L1/L2 mobility enhancements | TBD | Core part |
| 38.214 | Possible changes to introduce L1/L2 mobility enhancements | TBD | Core part |
| 38.133 | Requirements to support mobility enhancements | TBD | Performance part |

## 6	Work item Rapporteur(s)

Name: Li-Chuan Tseng

Company: MediaTek

Email: li-chuan dot tseng at mediatek dot com

Name: Hong He

Company: Apple

Email: hhe5 at apple dot com

## 7	Work item leadership

Primary: RAN WG2

Secondary: RAN WG3, RAN WG1, RAN WG4

## 8	Aspects that involve other WGs

None identified yet.

## 9	Supporting Individual Members

Supporting IM name