3GPP TSG RAN Meeting #95e	RP-220053

Electronic Meeting, Mar. 17 - 23, 2022

# Source:	Moderator (CMCC)

# Title:	New WID on Further RF requirements enhancement for NR frequency range 1 (FR1)

# Document for:	Approval

Agenda Item:	8A.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Further RF requirements enhancement for NR frequency range 1 (FR1)

Acronym: TBD

Unique identifier: 	{A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-18.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 Work Item is presented or e.g. the related Study Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. List previous Work Items of earlier releases if relevant.}

{This section is mandatory to be filled out by the rapporteur.}

{Not applicable for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

3	Justification

This work item includes the objectives of UE FR1 requirement focus on evolution for potential RAN4 enhancements for NR frequency range 1. The following working areas are based on the summary of email discussion in [RAN95e-RAN4-R18Prep-01] (RP-220019).

- Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices

- Enable [6Rx and] 8Rx and for CPE/FWA/vehicle/industrial device

- Investigate and enable 6Rx on higher frequency bands targeting at support of smartphone

- Study and specify requirements to support 3Tx considering the use cases of 2Tx simultaneous transmissions for two band inter-band UL CA/ENDC/SUL with 1Tx on one uplink operating band and 2Tx on the other uplink operating band for single carrier with UL MIMO or intra-band contiguous CA+UL MIMO

- Investigate the feasibility of lower MSD for inter-band CA/EN-DC/[DC] combinations

Other topics are also discussed, but no consensus reached for these topics:

- Investigate and define UE requirements with smaller A-MPR

- Study and specify requirements for 3Tx simultaneous transmission for three band inter-band UL CA with 1Tx on each uplink operating band

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objectives of core part for Rel-18 RF FR1 requirement focus evolution include:

- Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices (RAN4)

  - Investigate framework and architecture

  - Example bands: n41, n77 and n78 (other bands to be introduced in the release independent way later from Rel-18)

    - [FFS on FDD example band]

    - [Note 1: the total number of example band should be limited to 3.]

    - [Note 2: if FDD example band is added, study the REFSNES degradation due to 4Tx FDD operation in the example band]

  - Specify the UE RF requirements to support 4Tx

    - First priority: 4x4 UL MIMO

    - Second priority: investigate and if necessary specify TXD requirement to support the same power class in UL MIMO and single antenna port

      - [PA configuration assumption: 23dBm x 4, 26dBm x4]

    - For UE power class

      - first priority: PC 1.5

      - second priority: PC2 and PC3

      - Note: PC1.5 is only applicable for TDD bands

- Enable [6Rx and] 8Rx for CPE/FWA/vehicle/industrial devices (RAN4)

  - Example bands: n41, n77 and n78 (other bands to be introduced in the release independent way later from Rel-18)

  - Specify the UE RF requirements to support [6Rx and] 8Rx

  - [Study the extension to 6/8 antenna SRS antenna switching]

Note: FDD bands are not precluded to be introduced in the release independent way later from Rel-18.

- Investigate 6Rx on higher frequency bands targeting at support of smartphone (RAN4)

  - Investigate the feasibility whether 6Rx can be supported by smartphone

    - Feasibility study includes performance gain and form factor restrictions

  - Example bands are n41, n77 and n78 (other bands to be introduced in the release independent way later from Rel-18)

    - Study the applicable frequency upper bound to support 6Rx.

  - Specify the requirements to support 6Rx subject to the conclusion of feasibility study

    - Specify the UE RF requirements to support 6Rx

    - [Study the extension to 6 antenna SRS antenna switching]

- Study and specify requirements to support 3Tx considering the following use cases (RAN4)

  - 3Tx simultaneous transmissions for two band inter-band UL CA/ENDC/SUL with 1Tx on one uplink operating band and 2Tx on the other uplink operating band for single carrier with UL MIMO or intra-band contiguous CA+UL MIMO

    - Specify Tx switching transmission enhancement [if needed](RAN2, RAN4)

      - Cover inter-band UL CA , ENDC and SUL case

      - Identify and specify RAN2 signaling design is needed

      - Identify and specify the necessary RF requirements if needed

    - [Specify 3Tx simultaneous transmission for inter-band UL CA and EN-DC]

    - Power enhancement

      - Study of the definition of power class starts after the completion of Rel-17 PowerLimitCA_DC WI.

- Investigate the feasibility of lower MSD for inter-band CA/EN-DC/[DC] combinations (RAN4)

  - [Example band combinations: CA_n1-n77, CA_n3-n41]

  - Study how the MSD performance can be improved for example band combinations

    - Study correlation of MSD improvement between different MSD resources (harmonics, IMD2/3/4/5, cross band isolation and harmonic mixing)

  - [Study whether the generic approach can be applied based on UE indication of capability]

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

The objectives of performance part for Rel-18 RF FR1 requirement focus evolution include:

- Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices

  - Specify the BS demodulation performance requirements to support UL 4-layer MIMO UE operation

- Enable [6Rx and] 8Rx for CPE/FWA/vehicle/industrial devices

  - Specify RLM test cases with 8Rx

  - Specify UE demodulation performance and CSI requirements to support [6Rx and] 8Rx

  - Specify the requirements with 8 MIMO layers

- Investigate 6Rx on higher frequency bands targeting at support of smartphone

  - Specify the requirements to support 6Rx subject to the conclusion of feasibility study

    - Specify RLM test cases with 6Rx

    - Specify UE demodulation performance and CSI requirements to support 6Rx

    - [Investigate the feasibility and if necessary define the requirements with MIMO layer larger than 4]

- Specify release independence requirements in TS 38.307 if needed.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {E.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {E.g. "TSG#89"} | {Free text} |
| 38.101-1 | Specify UE RF core requirements | TBD | TBD |
| 38.101-3 | Specify UE RF core requirements related to EN-DC | TBD | TBD |
| 38.307 | Specify release independence | TBD | TBD |
| 38.101-4 | Specify UE demodulation performance and CSI requirements for >4Rx | TBD | TBD |
| 38.104 | Specify BS demodulation performance requirements for >2Tx | TBD | TBD |
| 38.331 | Specify the necessary signalling or capability to support the features | TBD | TBD |
| 38.214 | Specify PHY to support UL 3-layer transmission with 3Tx | TBD | TBD |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>.}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s).}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. Secondary Rapporteur(s) are possible for specific secondary task(s)}.

7	Work item leadership

R4

8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name