3GPP TSG-RAN5 Meeting #94-e	R5-220824

# Electronic Meeting, 21st Feb – 4th Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2612 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 5.1.41 inter-frequency DAPS handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New test case 5.1.41 for DAPS handover is introduced by RAN4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 5.1.41. Since the power levels are exactly the same as 5.1.3, the MU and TT could be leveraged without separate analysis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is missing in test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.41 (new), Annex E, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

5.1.34.5	Test requirement

Tables 5.1.34.4.1-1 and 5.1.34.5-1 define the primary level settings including test tolerances.

Table 5.1.34.5-1: Cell specific test parameters for HD-FDD Intra frequency handover for CE UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 |  |  | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.2 |  | R.11 HD-FDD | R.11 HD-FDD | - | - | - |  |  | R.11 HD-FDD |  |  |
| MPDCCH Reference Channel in clause A.3.1.3.2 |  | R.7 HD-FDD |  |  | R.7 HD-FDD |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH Reference Channel in clause A.3.1.2.3 |  | R.4 HD-FDD |  |  | R.4 HD-FDD |  |  |  |  |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.21 FDD | OP.21 FDD | OP.6 FDD | OP.6 FDD | OP.6 FDD |  |  | OP.21 FDD |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
| Note 3 | dB | 8 | 8 | 8 | -Infinity |  | 12.5 |  |  | 12.5 |  |
|  | dB | 8 | -4.74 | -4.74 | -Infinity |  |  | 3.86 |  |  | 3.86 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity |  | -85.5 |  |  | -85.5 |  |
| Io Note 3 | dBm/9MHz | -61.58 | - 56.22 | - 56.22 | Specified in columns for Cell 1 |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  | 2x1 |  |  |  |  |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | 3 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves |  |  |  |  |  |  |  |  |  |  |  |

The UE shall finish the transmission of all the repetitions of the PRACH to Cell 2 less than 50 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331.

Tinterrupt = 35 ms in the test; Tinterrupt is defined in clause 5.5.2.1.2. in TS 36.133.

This gives a total of 50 ms.

5.1.35-5.1.40	FFS

5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover

5.1.41.1	Test purpose

To verify that the requirement for the FDD – FDD Intra-band Inter-frequency sync DAPS handover including handover delay and interruption length.

5.1.41.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 16 and forward supporting DAPS handover in source PCell and inter-frequency target PCell.

5.1.41.3	Minimum conformance requirements

The purpose of DAPS handover is to change the PCell to another cell.

A DAPS handover is synchronous if it meets the conditions in Table 5.1.41.3-1, otherwise it is asynchronous.

Table 5.1.41.3-1: Synchronous conditions for DAPS handover

| Type of handover | Maximum receive timing difference between source and target cells (µs) for sync DAPS handover | Maximum transmit timing difference between source and target cells (µs) for sync DAPS handover |
| --- | --- | --- |
| Intra-frequencyNote 1,2,3 | 6µs | 8.21 µs |
| Intra-band inter-frequency Note 1,2,3 | 6µs | 8.21 µs |
| Inter-band inter-frequency | 33 µs | 35.21 µs |
| Note 1: For synchronous DAPS handover, if the receive time difference exceeds the cyclic prefix length, demodulation performance degradation is expected for the first symbol of the slot. For asynchronous DAPS handover, if the receive time difference exceeds the cyclic prefix length, interruptions may occur depending on UE implementation. The duration and frequency of occurrence of such interruptions is not specified. Note 2: For DAPS handover on a TDD band, a UE is not expected to transmit in the uplink earlier than 20us after the end of the last received downlink symbol in the same cell. Note 3: For DAPS handover on a TDD band, a UE is not expected to receive in the downlink earlier than 20us after the end of the last transmitted uplink symbol in the same cell. |  |  |

Procedure delays for the procedure that can command a DAPS handover are specified in TS 36.331 [2]. DAPS delay is comprised of Dhandover1 and Dhandover2.

When the UE receives an RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure+ Tsearch + TIU + 20 ms

Where

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [5].

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

A cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in TS 36.133 [4] Clause 8.1.2.2.1 (FDD) and TS 36.133 [4] Clause 8.1.2.2.2 (TDD) for intra-frequency handover and TS 36.133 [4] Clause 8.1.2.3.1 (FDD) and TS 36.133 [4] Clause 8.1.2.3.2 (TDD) for inter-frequency handover.

After successful RACH procedure of the target cell, when the UE receives an RRC message implying source cell release command, the UE shall accomplish the release actions specified in TS 36.331 [5] within Dhandover2.

Dhandover2 = TRRC_procedure+ Tinterrupt2

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [5].

Tinterrupt2 is defined in TS 36.133 [4] clause 5.7.2.1.2.

During Dhandover1 the UE is allowed an interruption of up to Tinterrupt1 on source cell:

-	Tinterrupt1 is 1 ms for synchronous intra-frequency DAPS handover and 2ms for asynchronous intra-frequency DAPS handover, when the bandwidth of target cell is no larger than the bandwidth of source cell,

-	Tinterrupt1 is 2ms for synchronous intra-frequency DAPS handover and 3ms for asynchronous intra-frequency DAPS handover, when the bandwidth of target cell is larger than the bandwidth of source cell,

-	Tinterrupt1 is 5 ms for synchronous intra-band inter-frequency DAPS handover and 6 ms for asynchronous intra-band inter-frequency DAPS handover

-	Tinterrupt1 is 1 ms for synchronous inter-band inter-frequency DAPS handover and 2 ms for asynchronous inter-band inter-frequency DAPS handover.

During Dhandover2 the UE is allowed an interruption of up to Tinterrupt2 on target cell:

-	Tinterrupt2 is 2 ms for synchronous intra-frequency DAPS handover and 3 ms for asynchronous intra-frequency DAPS handover, when the bandwidth of target cell is smaller than the bandwidth of source cell.

-	Tinterrupt2 is 1 ms for synchronous intra-frequency DAPS handover and 2 ms for asynchronous intra-frequency DAPS handover, when the bandwidth of target cell is not smaller than the bandwidth of source cell

-	Tinterrupt2 is 5 ms for synchronous intra-band inter-frequency DAPS handover and 6 ms for asynchronous intra-band inter-frequency DAPS handover.

-	Tinterrupt2 is 1 ms for synchronous inter-band inter-frequency DAPS handover and 2ms for asynchronous inter-band inter-frequency DAPS handover.

The normative reference for this requirement is TS 36.133 [4] clause 5.7.1, 5.7.2.1 and A.5.1.41.

5.1.41.4	Test description

5.1.41.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A figure A.14 for UE with 2Rx RF band and Annex A, Figure A.78a for 4Rx capable UE without any 2Rx RF bands.

2.	The general test parameter settings are set up according to Table 5.1.41.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 5.1.41.4.3.

5.	There are two E-UTRA TDD carriers and two cells on that carrier specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

Table 5.1.41.4.1-1: General test parameters for E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.2.1 |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| E-UTRA RF channel number |  |  | 1, 2 | Two FDD carriers on the same band are used |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| A3-Offset |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | DRX_L | As specified in TS 36.133 [4] clause A.3.3 |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 6 s | synchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | 100 |  |
| T5 |  | ms | 100 |  |

5.1.41.4.2	Test procedure

The test consists of one active cell and one neighbour cell. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would not enter the DRX state. The test consists of three five time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

Before the start of T1, the UE is connected to Cell 1 (source PCell) on radio channel 1 but is not aware of Cell 2 (neighbour cell) on radio channel 2. During T1, the UE shall not have any timing information of Cell 2.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event A3 is configured for neighbour cell (Cell 2), and the UE is configured with the measurement gaps (gap pattern ID # 0). Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing the RRC message implying DAPS handover to Cell 2 (target PCell) is sent to the UE. During T3, the UE shall be continuously scheduled on Cell 1 and shall be able to perform random access to Cell 2. After the RACH procedure is completed, the test system shall send a RRC message to the UE to release Cell 1 (source cell) on radio channel 1.

The start of T4 is the instant when the last TTI containing the RRC message implying source cell release is sent to the UE. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stops to send CSI report to the source cell.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 5.1.41.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	SS shall transmit an RRCConnectionReconfiguration message defined in Table H.3.1-1 to configure the event-triggered reporting with Event A3.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 5.1.41.5-1. T2 starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3.

7.	SS shall transmit an RRCConnectionReconfiguration message defined in Table 5.1.41.4.3-6 implying DAPS handover to Cell 2.

8.	The start of T3 is the instant when the last TTI containing the RRC connection reconfiguration message implying DAPS handover to Cell 2 (target PCell) is sent to the UE. At that instant the SS shall switch the power settings from T2 to T3 as specified in Table 5.1.41.5-1.

9.	The UE shall transmit RRCConnectionReconfigurationComplete message.

10.	If the UE transmits the uplink PRACH channel to Cell 2 less than 50 ms from the beginning of time period T3, and during T3 no more than 4 ACK/NACK loss are detected by SS, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

11.	After the RACH procedure is completed, SS shall transmit an RRCConnectionReconfiguration message defined in Table 5.1.41.4.3-6 to release Cell 1 (source cell) on radio channel 1.

12.	The start of T4 is the instant when the last TTI containing the RRC message implying source cell release is sent to the UE. At that instant the SS shall switch the power settings from T3 to T4 as specified in Table 5.1.41.5-1.

13.	The UE shall transmit RRCConnectionReconfigurationComplete message.

14.	If during T4 no more than 4 ACK/NACK loss are detected by SS, the number of successful tests is increased by one.Otherwise the number of failed tests is increased by one.

15.	When T4 expires, the SS shall switch the power setting from T4 to T5 as specified in Table 5.1.41.5-1. T5 starts.

16.	The UE shall stop sending CSI reports on Cell1 during T5, if the number of successful tests is increased by one. Otherwise the number of failed tests is increased by one.

17.	After T5 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 7.2A.3. Cell 1 is the active cell.

18.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

19.	Repeat step 2-18 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.1.41.4.3	Message contents

Message contents are according to TS 36.508 [7] values 4.6 with the following exceptions:

Table 5.1.41.4.3-1: Common Exception messages for E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover test case

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-3 Table H.3.1-7 Table H.3.2-1 Table H.3.2-3 Table H.3.6-2 |

Table 5.1.41.4.3-2: ReportConfigEUTRA-A3: Additional E-UTRAN FDD-FDD inter frequency handover test requirement

| Derivation Path: TS 36.508 [7] clause 4.6.6, Table 4.6.6-6 ReportConfigEUTRA-A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset | -8 (-4 dB) | -4 is actual value in dB (-8 * 0.5 dB) |  |
| reportOnLeave | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| Hysteresis | 0 (0 dB) | 0 is actual value in dB (0 * 0.5 dB) |  |
| timeToTrigger | 0 (0 ms) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.41.4.3-3: MeasResults: Additional E-UTRAN FDD-FDD inter frequency handover test requirement

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | 1 | Identifies the measurement id for the reporting being performed |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult |  | Set according to specific test |  |
| rsrqResult |  | Set according to specific test |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA | MeasResultListEUTRA |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.41.4.3-4: MeasResultListEUTRA: Additional E-UTRAN FDD-FDD inter frequency handover test requirement

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultListEUTRA::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult |  | Set according to specific test |  |
| rsrqResult |  | Set according to specific test |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.41.4.3-5: RACH-ConfigCommon-DEFAULT: Additional E-UTRAN FDD-FDD inter frequency handover test requirement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-12 RACH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |
| preambleInitialReceivedTargetPower | dBm-90 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.41.4.3-6: RRCConnectionReconfiguration with DAPS HO (step 7)

| Derivation Path: 36.508 clause 4.6.1-8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |  |  |  |
| radioResourceConfigDedicated |  |  | RadioResourceConfigDedicated-HO |  |  |  | HO |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 5.1.41.4.3-7: RadioResourceConfigDedicated-HO (Table 5.1.41.4.3-6)

| Derivation Path: 36.508 clause 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-HO ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | Not present |  |  |
| drb-ToAddMod[1] SEQUENCE { |  |  |  |
| drb-Identity | 1 |  |  |
| daps-HO-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.1.41.4.3-8: RRCConnectionReconfiguration with source cell release (step 11)

| Derivation Path: 36.508 clause 4.6.1-8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v890-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v920-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v1020-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v1130-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v1250-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v1310-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v1430-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v1510-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v1530-IEs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v1610-IEs |  |  |
| daps-SourceRelease-r16 |  |  | true |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

5.1.41.5	Test requirement

Table 5.1.41.5-1 defines the primary level settings including test tolerances for E-UTRAN FDD-FDD Intra-band Inter-frequency DAPS handover test case.

Table 5.1.41.5-1: Cell specific test parameters for E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover test case

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel number |  | 1 |  |  |  |  | 2 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  | 10 |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | OP.1 FDD | OP.1 FDD | OP.1 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD | OP.1 FDD | OP.1 FDD | OP.1 FDD |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 | -Infinity | 7.5 | 7.5 | 7.5 | 7.5 |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 | -Infinity | 7.5 | 7.5 | 7.5 | 7.5 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -94 | -94 | -infinity | -90.5 | -90.5 | -90.5 | -90.5 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 50ms (Dhandover1) from the beginning of time period T3. During Dhandover1 the interruption on Cell 1 shall not exceed 5ms (Tinterrupt1).

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: Dhandover1 = TRRC_procedure + TIU + 20 ms.

The UE shall complete to release Cell 1 less than 20ms (Dhandover2) from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed 5ms (Tinterrupt2).

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

RRC procedure delay = 15 ms and is specified in clause 11.2 in TS 36.331 [5].

Tinterrupt2 = 5 ms in the test; Tinterrupt2 is defined in TS 36.133 [4] clause 5.7.2.1.2.

This gives a total of 20 ms.

5.2	Handover from E-UTRAN to other RATs

5.2.1	E-UTRAN FDD - UTRAN FDD handover

5.2.1.1	Test purpose

To verify the UE’s ability to transfer a connection between the UE and E-UTRAN to UTRAN in RRC_CONNECTED state by meeting the UE RRC procedure delay and interruption time requirements.

<Unchanged Text Skipped>

Annex E (normative):
Cell configuration mapping

The cells used in TS 36.521-3 do not correspond to the cells defined in TS 36.508 [7] section 4.4.2. Tables E-1, E-2, E-3, E-5, E-6, E-7 and E-8 describes the mapping between cells described in TS 36.521-3 and those defined in TS 36.508 [7]. Table E-4 describe the mapping between Ncells described in TS 36.521-3 and those defined in TS 36.508 [7] For each test case the cells as defined in TS 36.508 [7] section 4.4.2 and section 8.1.4.2 are listed in one row. The test case shall apply the RF parameters as defined in TS 36.521-3 according to the column heading.

NOTE:	For example if the second cell in a test case is an inter-frequency cell then Cell3 from TS 36.508 [7] section 4.4.2 is used with the radio parameters as defined for Cell2 in TS 36.521-3.

Table E-1: Cell configuration mapping for RRM testing

| Test Case | Description | 36.521-3 Cell1 | 36.521-3 Cell2 | 36.521-3 Cell3 | 36.521-3 Cell4 | CA Type (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| 4.2.1 | RRC IDLE / E-UTRAN Cell Reselection / FDD - FDD cell re-selection intra frequency case | Cell1 | Cell11 |  |  |  |
| 4.2.2 | RRC IDLE / E-UTRAN Cell Reselection / TDD - TDD cell re-selection intra frequency case | Cell1 | Cell11 |  |  |  |
| 4.2.3 | RRC IDLE / E-UTRAN Cell Reselection / FDD - FDD cell re-selection inter frequency case | Cell6 | Cell23 |  |  |  |
| 4.2.4 | RRC IDLE / E-UTRAN Cell Reselection / FDD - TDD cell re-selection inter frequency case | Cell1 | Cell31 | Dual mode in single PLMN |  |  |
| 4.2.5 | RRC IDLE / E-UTRAN Cell Reselection / TDD - FDD cell re-selection inter frequency case | Cell31 | Cell1 |  |  |  |
| 4.2.6 | RRC IDLE / E-UTRAN Cell Reselection / TDD - TDD cell re-selection inter frequency case | Cell6 | Cell23 |  |  |  |
| 4.2.7 | RRC IDLE / E-UTRAN Cell Reselection / FDD – FDD Inter frequency case in the existence of non-allowed CSG cell | Cell23 | Cell6 | Cell3 |  |  |
| 4.2.8 | RRC IDLE / E-UTRAN Cell Reselection / TDD – TDD Inter frequency case in the existence of non-allowed CSG cell | Cell23 | Cell6 | Cell3 |  |  |
| 4.2.9 | RRC IDLE / E-UTRAN Cell Reselection / E-UTRAN FDD-FDD intra-frequency Cell Re-selection case for 5MHz Bandwidth | Cell1 | Cell11 |  |  |  |
| 4.2.10 | E-UTRAN FDD – FDD reselection using an increased number of carriers | Cell6 | (Note 9) | (Note 10) (Note15) | (Note 10) (Note15) |  |
| 4.2.11 | E-UTRAN TDD – TDD reselection using an increased number of carriers | Cell6 | (Note 9) | (Note 10) (Note15) | (Note 10) (Note15) |  |
| 4.2.12 | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.13 | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.14 | E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage | Cell1 | Cell11 |  |  |  |
| 4.2.15 | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.16 | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.17 | E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage | Cell1 | Cell11 |  |  |  |
| 4.2.20 | E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis | Cell1 | Cell11 |  |  |  |
| 4.2.21 | E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis | Cell1 | Cell11 |  |  |  |
| 4.2.22 | E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Cell1 | Cell11 |  |  |  |
| 4.2.23 | E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Cell1 | Cell11 |  |  |  |
| 4.3.1.1 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA is of higher priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection in fading propagation conditions: UTRA FDD is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.3.1.4 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz Bandwidth | Cell3 | Cell9 |  |  |  |
| 4.3.1.5 | Idle mode FDD to UTRA FDD interRAT reselection | Cell6 | (Note11) | (Note14) (Note15) |  |  |
| 4.3.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN FDD - UTRAN TDD cell re-selection | Cell6 | Cell8 |  |  |  |
| 4.3.2A | E-UTRA FDD to UTRA TDD cell re-selection for IncMon | Cell6 | (Note11) | (Note 16) (Note15) |  |  |
| 4.3.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN FDD cell re-selection | Cell6 | Cell8 |  |  |  |
| 4.3.3A | Idle mode TDD to UTRA FDD interRAT reselection | Cell6 | (Note11) | (Note 14) (Note 15) |  |  |
| 4.3.4.1 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN TDD cell re-selection: UTRA is of higher priority | Cell6 | Cell8 |  |  |  |
| 4.3.4.2 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection / E-UTRAN TDD - UTRAN TDD cell re-selection: UTRA is of lower priority | Cell6 | Cell8 |  |  |  |
| 4.3.4.4 | E-UTRA TDD to UTRA TDD cell re-selection for IncMon | Cell6 | (Note11) | (Note 16) (Note 15) |  |  |
| 4.3.4.3 | RRC IDLE / E-UTRAN to UTRAN Cell re-selection /EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority | Cell3 | Cell9 |  |  |  |
| 4.4.1 | RRC IDLE / E-UTRAN to GSM Cell re-selection / E-UTRAN FDD - GSM cell re-selection | Cell1 | Cell26 |  |  |  |
| 4.4.2 | RRC IDLE / E-UTRAN to GSM Cell re-selection / E-UTRAN TDD - GSM cell re-selection | Cell1 | Cell26 |  |  |  |
| 4.5.1.1 | RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN FDD - HRPD cell re-selection: HRPD is of lower priority | Cell1 | Cell15 |  |  |  |
| 4.5.2.1 | RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN TDD - HRPD Cell Reselection: HRPD is of Lower Priority | Cell1 | Cell15 |  |  |  |
| 4.6.1.1 | RRC IDLE / E-UTRAN to cdma2000 1xRTT Cell re-selection / E-UTRAN FDD - cdma2000 1xRTT cell re-selection: cdma2000 1x is of lower priority | Cell1 | Cell19 |  |  |  |
| 4.6.2.1 | RRC IDLE / E-UTRAN to cdma2000 1xRTT Cell re-selection / E-UTRAN TDD - cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority | Cell1 | Cell19 |  |  |  |
| 5.1.1 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Intra frequency case | Cell1 | Cell2 |  |  |  |
| 5.1.2 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Intra frequency case | Cell1 | Cell2 |  |  |  |
| 5.1.3 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Inter frequency case | Cell6 | Cell3 |  |  |  |
| 5.1.3_2 | E-UTRAN FDD – FDD Inter frequency handover for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.4 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Inter frequency case | Cell6 | Cell3 |  |  |  |
| 5.1.4_2 | E-UTRAN TDD – TDD Inter frequency handover for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.5 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Inter frequency case: unknown target cell | Cell6 | Cell3 |  |  |  |
| 5.1.5_2 | E-UTRAN FDD – FDD Inter frequency handover: unknown target cell for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.6 | RRC CONNECTED / E-UTRAN Handover / TDD - TDD / Inter frequency case: unknown target cell | Cell6 | Cell3 |  |  |  |
| 5.1.6_2 | E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell for UE Category 1bis | Cell6 | Cell3 |  |  |  |
| 5.1.7 | RRC CONNECTED / E-UTRAN Handover / FDD – TDD / Inter frequency case | Cell1 | Cell10 | Dual mode in multiple PLMN |  |  |
| 5.1.7_2 | E-UTRAN FDD – TDD Inter frequency handover for UE Category 1bis | Cell1 | Cell10 | Dual mode in multiple PLMN |  |  |
| 5.1.8 | RRC CONNECTED / E-UTRAN Handover / TDD – FDD / Inter frequency case | Cell10 | Cell1 |  |  |  |
| 5.1.8_2 | E-UTRAN TDD – FDD Inter frequency handover for UE Category 1bis | Cell10 | Cell1 |  |  |  |
| 5.1.9 | RRC CONNECTED / E-UTRAN Handover / FDD - FDD / Intra frequency handover for 5MHz Bandwidth | Cell1 | Cell2 |  |  |  |
| 5.1.10 | E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.11 | E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.12 | E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 | Cell1 | Cell2 |  |  |  |
| 5.1.13 | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.14 | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.15 | E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA | Cell1 | Cell2 |  |  |  |
| 5.1.19 | E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 5.1.20 | E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis | Cell1 | Cell2 |  |  |  |
| 5.1.28 | E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA | Cell6 | Cell3 |  |  |  |
| 5.1.34 | E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition | Cell6 | Cell3 |  |  |  |
| 5.1.41 | E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover | Cell6 | Cell3 |  |  |  |
| 5.2.1 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN FDD - UTRAN FDD handover | Cell3 | Cell9 |  |  |  |
| 5.2.2 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN TDD - UTRAN FDD handover | Cell6 | Cell8 |  |  |  |
| 5.2.3 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to GSM / E-UTRAN FDD - GSM handover | Cell1 | Cell26 |  |  |  |
| 5.2.4 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN TDD - UTRAN TDD handover | Cell3 | Cell9 |  |  |  |
| 5.2.5 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to UTRAN / E-UTRAN FDD - UTRAN TDD handover | Cell6 | Cell8 |  |  |  |
| 5.2.6 | RRC CONNECTED / Handover from E-UTRAN to other RATs / From E-UTRAN to GSM / E-UTRA TDD - GSM handover | Cell1 | Cell26 |  |  |  |
| 5.2.7 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN FDD - UTRAN FDD handover: unknown target cell | Cell3 | Cell9 |  |  |  |
| 5.2.8 | RRC CONNECTED / Handover from E-UTRAN to other RATs / E-UTRAN FDD - GSM handover: unknown target cell | Cell1 | Cell26 |  |  |  |

<Unchanged Text Skipped>

F.1.2	Measurement of RRM requirements

Table F.1.2-1: Maximum Test System Uncertainty for RRM Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 4.2.1 E-UTRA FDD - FDD cell re-selection intra frequency | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 4.2.2 E-UTRA TDD - TDD cell re-selection intra frequency | Same as 4.2.1 |  |
| 4.2.3 E-UTRA FDD - FDD cell re-selection inter frequency | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| < Unchanged lines skipped > |  |  |
| 5.1.2 E-UTRAN TDD-TDD Handover intra frequency case | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.3 E-UTRAN FDD-FDD Handover inter frequency case | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 5.1.3_2 E-UTRAN FDD – FDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.4 E-UTRAN TDD-TDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.4_2 E-UTRAN TDD – TDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.5 E-UTRAN FDD-FDD inter-frequency Handover with unknown target cell | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.5_2 E-UTRAN FDD – FDD Inter frequency handover: unknown target cell for UE Category 1bis | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.6 E-UTRAN TDD-TDD inter-frequency Handover with unknown target cell | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.6_2 E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell for UE Category 1bis | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.7 E-UTRAN FDD-TDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.7_2 E-UTRAN FDD – TDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.8 E-UTRAN TDD-FDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.8_2 E-UTRAN TDD – FDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.9 E-UTRAN FDD-FDD Intra frequency handover for 5MHz bandwidth | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.10 E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.11 E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.12 E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.13 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.14 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.15 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.16 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.17 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.18 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.19 E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.1.20 E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis | Same as 5.1.19 | Same as 5.1.19 |
| 5.1.28 E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.34 E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.41 E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.2.1 E-UTRAN FDD - UTRAN FDD handover | E-UTRA cell Noc ±1.0dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 5.2.2 E-UTRAN TDD - UTRAN FDD handover | Same as 5.2.1 | Same as 5.2.1 |

<Unchanged Text Skipped>

F.3.2	Measurement of RRM requirements

Because the relationships between the Test system uncertainties and the Test Tolerances are often complex, it is not always possible to give a simple derivation of the Test Requirement in this document. The analysis is recorded in 3GPP TR 36 903 [20].

Table F.3.2-1: Derivation of Test Requirements (RRM tests)

| Test | Minimum Requirement in TS 36.133 | Test Tolerance (TT) | Test Requirement in TS 36.521-3 |
| --- | --- | --- | --- |
| 4.2.1 E-UTRA FDD – FDD cell re-selection intra frequency | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: +13.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13.00dB |
| 4.2.2 E-UTRA TDD – TDD cell re-selection intra frequency | Same as 4.2.1 | Same as 4.2.1 | Same as 4.2.1 |
| 4.2.3 E-UTRA FDD - FDD cell re-selection inter frequency | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: -4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +14.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12.00dB | During T0: -1.35dB +0.3dB -1.35dB +0.4dB During T1: -1.35dB +2.4dB -1.35dB +0.3dB During T2: -1.35dB +2.4dB -1.35dB 0dB During T3: -1.35dB +2.4dB -1.35dB +2.4dB | During T0: Noc1: -99.35dBm/15kHz Ês1 / Noc1: -3.70dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: +14.40dB During T1: Noc1: -99.35dBm/15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: -3.70dB During T2: Noc1: -99.35dBm/15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -99.35dBm /15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: +14.40dB |
| < Unchanged lines skipped > |  |  |  |
| 5.1.1 E-UTRAN FDD-FDD Handover intra frequency case | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +11.50dB |
| 5.1.2 E-UTRAN TDD-TDD Handover intra frequency case | Same as 5.1.1 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.3 E-UTRAN FDD-FDD Handover inter frequency case | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.5dB During T3: 0dB 0dB 0dB +0.5dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB |
| 5.1.3_2 E-UTRAN FDD – FDD Inter frequency handover for UE Category 1bis | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |
| 5.1.4 E-UTRAN TDD-TDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.4_2 E-UTRAN TDD – TDD Inter frequency handover for UE Category 1bis | Same as 5.1.3_2 | Same as 5.1.3_2 | Same as 5.1.3_2 |
| 5.1.5 E-UTRAN FDD-FDD inter-frequency Handover with unknown target cell | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |
| 5.1.5_2 E-UTRAN FDD – FDD Inter frequency handover: unknown target cell for UE Category 1bis | Same as 5.1.5 | Same as 5.1.5 | Same as 5.1.5 |
| 5.1.6 E-UTRAN TDD-TDD inter-frequency Handover with unknown target cell | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +5.0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +5.0dB |
| 5.1.6_2 E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell for UE Category 1bis | Same as 5.1.6 | Same as 5.1.6 | Same as 5.1.6 |
| 5.1.7 E-UTRAN FDD-TDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.7_2 E-UTRAN FDD – TDD Inter frequency handover for UE Category 1bis | Same as 5.1.3_2 | Same as 5.1.3_2 | Same as 5.1.3_2 |
| 5.1.8 E-UTRAN TDD-FDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.8_2 E-UTRAN TDD – FDD Inter frequency handover for UE Category 1bis | Same as 5.1.3_2 | Same as 5.1.3_2 | Same as 5.1.3_2 |
| 5.1.9 E-UTRAN FDD-FDD Intra frequency handover for 5MHz bandwidth | Same as 5.1.1 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.10 E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.11 E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.12 E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.13 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB |
| 5.1.14 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.13 | Same as 5.1.13 | Same as 5.1.13 |
| 5.1.15 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.13 | Same as 5.1.13 | Same as 5.1.13 |
| 5.1.16 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -7dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -7dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -6.5dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.00 Ês2 / Noc: -6.5dB |
| 5.1.17 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.16 | Same as 5.1.16 | Same as 5.1.16 |
| 5.1.18 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.16 | Same as 5.1.16 | Same as 5.1.16 |
| 5.1.19 E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +12.50dB |
| 5.1.20 E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis | Same as 5.1.19 | Same as 5.1.19 | Same as 5.1.19 |
| 5.1.28 E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB |
| 5.1.34 E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. |
| 5.1.41 E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T4: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T5: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.5dB During T3: 0dB 0dB 0dB +0.5dB During T4: 0dB 0dB 0dB +0.5dB During T5: 0dB 0dB 0dB +0.5dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T4: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T5: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB |
| 5.2.1 E-UTRAN FDD - UTRAN FDD handover | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB | During T1: 0dB -1.10dB 0dB - During T2: 0dB -1.10dB 0dB 0dB During T3: 0dB -1.10dB 0dB 0dB | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB |
| 5.2.2 E-UTRAN TDD - UTRAN FDD handover | Same as 5.2.1 | Same as 5.2.1 | Same as 5.2.1 |

<End of Changes >