3GPP TSG RAN meeting #95e									RP-220770

Electronic Meeting, Mar 17-23, 2022

## Status Report to TSG

Agenda item:			9.5.4.8

| WI / SI Name | Further enhancements of NR RF requirements for frequency range 2 (FR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RF_FR2_req_enh2 |  |  |  |  |
| Unique ID | 890059 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213666 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 06/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 75% | Performance Part: 25% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Vasenkari, Petri |
|  | Company | Nokia |
|  | Email | Petri.j.vasenkari@nokia.com |

# 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

# 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #101bis-e Agreements

| R4-2200699 | TR38.851 v0.3.0 | Nokia, Nokia Shanghai Bell |
| --- | --- | --- |
| R4-2200700 | TP to TR 38.851: Agreements made for CA configurations between frequency groups using CBM | Nokia, Nokia Shanghai Bell |

| R4-2202342 WF on FR2 DL CA | Nokia |
| --- | --- |
| R4-2202343 WF on FR2 UL CA | Qualcomm |
| R4-2202417 WF on UL gap for FR2 | Apple |
| R4-2202419 Reply LS on UL gap for RAN2 | Apple |
| R4-2202420 LS to RAN2 on UL gap in FR2 RF enhancement | Apple |
| R4-2202346 WF on DC-Location | vivo |
| R4-2202347 WF on FR2 CA BW classes | Xiaomi |
| R4-2202581 WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia |
| R4-2202587 DraftCR on interruption requirements for FR2 inter-band CA with CBM | Huawei, Hisilicon |

RAN4 #102-e Agreements

| R4-2206507 WF for FR2 DL CA | Nokia |
| --- | --- |
| R4-2206508 WF for FR2 UL CA | Qualcomm Inc. |
| Revised to R4-2206600 | Draft CR to 38.101-2 FR2+FR2 ULCA |
| R4-2204787 | TR 38.851 v0.4.0 |
| R4-2206604 | WF on UL gap in FR2 |
| R4-2206608 | LS on UL gap in FR2 |
| R4-2206511 | Draft CR for UL gap for Tx power management RRM aspect |
| R4-2206513 | Draft CR for UL gap for Tx power management RF aspect |
| R4-2206601 | WF on DC location |
| R4-2206602 | Reply LS on DC location for >2CC |
| R4-2206837 WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia |
| R4-2205868 Draft Big CR on RRM requirements for FR2 Inter-band CA | Nokia |
| R4-2206838 | draftCR on CBM inter-band FR2 DL CA |
| R4-2206839 | draftCR on MRTD for CBM inter-band FR2 DL CA |
| R4-2206841 | DraftCR on applicability rules for FR2 inter-band CA with CBM |
| R4-2206842 | Draft CR on scheduling restriction for FR2 inter-band DL CA for CBM UE |
| R4-2206843 | draftCR on measurement restriction for CBM inter-band FR2 DL CA |

RAN4 #101bis-e GTW Agreements

CA

Agreement: for maximum input level, agree on Option 1.

  - Option 1: If max input level of CBM is to be defined per-band, 3dB relaxation per-band is needed

Agreement:

- Agree on Option 1 and Option 5.

- Further discussion Option 2, 3 and 6.

    - Option 1: Configuration and side condition of reference signal of the Band_with_BMRS is as single-band beam correspondence operation

    - Option 5: The reference signal configuration, side condition, power level and QCLed behavior are applied for both “different frequency groups” and “within same frequency group” based on CBM.

    - Option 2: “QCLed with the other CC in Band_with_BMRS” shall be applied for the reference signal of Band_without_BMRS.

    - Option 3: LS to RAN1 to raise the requirement on “SSB QCLed with the other CC in Band_with_BMRS” for the reference signal of Band_without_BMRS.

    - Option 4: Reference signal power level of the two bands, Band_with_BMRS and Band_without_BMRS, shall be equal for CBM.

    - Option 6: No need to specify the BMRS side condition for CBM in R17 and only inform RAN5 that the BMRS type is the same as IBM.

Agreement: Agree on Option 2 and Option 3.

- For the inter-band CA supporting both RAN4 can further discuss the verification rules.

- FFS Option 1.

  - Option 1: if the measured sensitivity of CBM has already satisfied the delta_RIB requirements of IBM, then the IBM sensitivity verification is not necessary

  - Option 2: if the max input level is already met with IBM requirements, then it is not necessary to verify the CBM requirements

  - Option 3: ACS and IBB can be verified with either IBM or CBM

- Agreement: Agree on Option 1 in principle but need check the concrete wording.

  - Option 1: refine the IBM and CBM definition to highlight the per-BC characteristics to avoid confusion.

Agreement: Further discuss the following options:

- Option 2: Fs_Inter capability is introduced. No additional EIS relaxation specific for frequency separation factor is acceptable

- Option 4a: Fs_Inter capability is not introduced. Define Delta_RIB based on worse case of frequency separation

Agreement: Agree on Option1 and define the requirements considering the worst case of frequency separation.

- FFS whether Option 2 is needed depending on the outcome of Issue 3-1-1 for Fs_inter_capability.

UL GAPS

Agreement:

- Support measuring UE in-band Tx power during the gaps

  - The maximum value is TX_OFF power

- Agreement: When UL gap is activated or de-activated and non-zero P-MPR is applied, the peak EIRP measurement should be averaged across UL slots with PUSCH transmission over 4s.

- Agreement: UL gap UE capability should be defined per band.

- Agreement: When UGL is shorter than a slot length with respect to an activated UL BWP’s SCS on a serving cell where UL gap is configured and activated, the configured UGL and UGRP are adjusted. For ULGP#3, when an SCS of active BWP is 60kHz, UGL and UGRP are adjusted to Option-A in Table 1

| ULGP #3 | 120kHz | 0.125 | 1 | 5 | 40 | 2.5% |
| --- | --- | --- | --- | --- | --- | --- |
|  | Option-A: 60kHz | 0.25 | 1 | 5 | 20 | 5% |
|  | Option-B: 60kHz | 0.25 | 1 | 10 | 40 | 2.5% |

Agreement: Regarding Procedures to be prioritized over UL gap,

- All the RACH procedure should be prioritized

- FFS for other procedure

- Agreement: further discuss whether all the gaps will be optional in the next meeting.

DC location

Agreement: down-select to Option 2 and Option 4

- FFS which options among 2 and 4 should be chosen or further simplified, considering the number of new band combinations.

  - Option 2: It is proposed to define UE DC offset ranges as +/-20MHz from the default DC location.

  - Option 4: Others

RRM:

- Agreement

  - Performance degradation due to network driven Rx beam switch e.g. TCI state change (Case 1)

    - Performance degradation will be specified as a note in MRTD clause

    - Option 1: If the receive time difference exceeds [X] of that SCS, demodulation performance degradation is expected for the first or the last symbol of the slot in the in the SCells of the other band, where X is defined in Table 7.6.4.

    - Option 2: If the receive time difference exceeds [X] of that SCS, demodulation performance degradation is expected for the first symbol of the SCell of the other band, where X is defined in Table 7.6.4.

RAN4 #102-e GTW Agreements

DL CA:

Agreement:

- All the reference signals in Band_without_BMRS shall traces its QCL type-D dependence to SSB and/or CSI-RS in Band_with_BMRS by certain manner.

Agreement:

- Apply the in-gap exemption for the CBM requirements of ACS and IBB for inter-band CA within the same frequency group

  - Refer to R4-2114960

- For IBM requirements, the following changes in R4-2204789 are agreeable

- Agreement: For CBM, Rx beam switch value is 200ns.

UL CA:

- Agreement: RAN4 recommends to include UL CA_n260-n261 is included in this WI in addition to CA_n257-n259.

Agreements: Focus on the common requirements (i.e., MPR and power control) of PC1/2/3/4/5 and Delta_TIB values of PC1/2/4/5, and afterwards discuss the PC3 specific requirements (i.e., Delta_TIB values and total power issue).

- The power class cannot be supported without finalized the requirements including Delta_TIB.

Agreement: The total power concept is not applied for power classes such as PC1/2/5.

- FFS include new power class

- Further check the MPE regulation for FWA/CPE.

- Agreement: Focus on inter-band UL CA across the different frequency group for power control.

UL GAPS:

Agreement:

- Agree on Option 2.

- No need to mandate P-MPR reporting when UL gap is not configured/activated (P bis is 1)

  - Option 2: delta P-MPR should not be tested with the already agreed P bit setting before and after the UL GAP in conformance tests. (OPPO)

Agreement:

- Introduce the UE capability for inter-band UL CA on whether UL transmission in different FR2 within gap is feasible when UL gap is activated.

  - For CBM, the capability is per band per band combination

  - For IBM, FFS whether the capability is per band per band combination or per UE.

- Agreement: When maxUplinkDutyCycle-FR2 is less than 20, or not reported, Z=20 in the test as agreed in previous meeting. When maxUplinkDutyCycle-FR2 is equal to or greater than 20, then Z should be larger than maxUplinkDutyCycle-FR2 as proposed.

Agreement:

- [UE is mandated to support at least one of patterns #1 and #3].

- The other two gap patterns except for #1 and #3 are optional

Agreement: The capability to support the UL gap configuration which is defined as optional should be per-UE based for FR2 only.

- The gap for FR2 has no impact on FR1.

Agreement: Further discuss the following options in order to conclude in this meeting

- Symbol used for calculation

  - Option 1: DMRS+Data symbols (Huawei)

  - Option 2: DMRS RE (Anritsu)

- Average window for relative phase and power error

  - Option 1: The relative phase and power errors for each slot should be an average over a slot. (Huawei)

  - Option 2: The “relative phase error” and “relative amplitude” shall be calculated in frequency domain. There should not be then mention of “instantaneous” or “average over a slot”. (Anritsu)

DC Location:

Agreement: The frequency component type should be the same for the two default DC locations in Rel-17.

CA BW Class:

Agreement:

- Alternative 1: Approve Option 2c or Option 2b with the following clarification in the meeting minutes as the common understanding

  - Capture that the interlacing CC bandwidth is not allowed.

  - Limit the maximum aggregated bandwidth to 1600MHz.

- Alternative 2: Approve Option 3.

- For both Alternative 1 and Alternative 2, 50MHz channel bandwidth is not supported

- For options see meeting report.

RRM:

- Agreements

  - Performance degradation due to network driven Rx beam switch (e.g. TCI state change) (Case 1)

    - Add a note “If the receive time difference exceeds [X] of that SCS, demodulation performance degradation is expected for the first or the last symbol of the slot in the SCells of the other band, where X is defined in Table 7.6.4. This may result in performance degradation for the slot, where impacted symbols belong to, if PDCCH/PDSCH is scheduled in these symbols.”

  - Performance degradation due to UE autonomous Rx beam switch (Case 2)

    - Do not define requirements (e.g., performance degradation) for Case 2 when receive time difference exceeds [X], where X is defined in Table 7.6.4

  - The time uncertainty due to TCI state indication on SCell can be skipped:

    - Option 1: RAN4 to agree that MAC-CE to activate TCI are sent along with SCell activation MAC CE itself. There is no uncertainty term required in the SCell activation delay timeline as the beam information is known at the time of SCell activation command indication.

    - Option 2: The TCI state of target SCell can be assumed to be same as that for SpCell, there is no need to wait TCI indication on SCell

  - The time uncertainty due to activation/configuration of CSI reporting on SCell can be skipped.

    - Define the requirements only for the case that:

      - MAC-CEs to activate PDCCH TCI, PDSCH TCI (when applicable) and CSI reporting are sent along with SCell activation MAC-CE, if semi-persistent CSI-RS is used for CSI reporting.

      - MAC-CEs to activate PDCCH TCI, PDSCH TCI (when applicable) and RRC configuration message for TCI of periodic CSI for CQI reporting are sent along with SCell activation MAC-CE, if periodic CSI-RS is used for CSI reporting.

Email discussion summaries (For information)

RAN4 #101bis-e

| R4-2202219 | Email discussion summary for [101-bis-e][119] NR_RF_FR2_enh2_Part_1 |
| --- | --- |
| R4-2202319 | Email discussion summary for [101-bis-e][119] NR_RF_FR2_enh2_Part_1 |
| R4-2202220 | Email discussion summary for [101-bis-e][120] NR_RF_FR2_enh2_Part_2 |
| R4-2202320 | Email discussion summary for [101-bis-e][120] NR_RF_FR2_enh2_Part_2 |
| R4-2202221 | Email discussion summary for [101-bis-e][121] NR_RF_FR2_enh2_Part_3 |
| R4-2202321 | Email discussion summary for [101-bis-e][121] NR_RF_FR2_enh2_Part_3 |
| R4-2202553 | Email discussion summary for [101-bis-e][202] NR_RF_FR2_enh2_RRM |
| R4-2202719 | Email discussion summary for [101-bis-e][202] NR_RF_FR2_enh2_RRM |

RAN4 #102-e

| R4-2206325 | Email discussion summary for [102-e][125] NR_RF_FR2_enh2_Part_1 |
| --- | --- |
| R4-2206425 | Email discussion summary for [102-e][125] NR_RF_FR2_enh2_Part_1 |
| R4-2206326 | Email discussion summary for [102-e][126] NR_RF_FR2_enh2_Part_2 |
| R4-2206426 | Email discussion summary for [102-e][126] NR_RF_FR2_enh2_Part_2 |
| R4-2206327 | Email discussion summary for [102-e][127] NR_RF_FR2_enh2_Part_3 |
| R4-2206427 | Email discussion summary for [102-e][127] NR_RF_FR2_enh2_Part_3 |
| R4-2206753 | Email discussion summary for [102-e][210] NR_RF_FR2_req_enh2_RRM |
| R4-2207051 | Email discussion summary for [102-e][210] NR_RF_FR2_req_enh2_RRM |

#### 2.4.2	Remaining Open issues

RF:

DL interband CA:

- UE requirements for DL CA with CBM for CA_n258-n261, n257-n259, n258-n260, n260-n261,

- UE relaxation values for DL CA with IBM for CA_n258-n261

UL interband CA:

- UL CA MPR, relaxation values and Pcmax handling for CA_n257A-n259A and CA_n260-n261

UL gaps for self-calibration and monitoring:

DC location:

- Offset range from a default DC location

- Single CC handling within or not within CA

- Handling of DL only CC

CA BW classes:

- New FR2 CA BW Classes to enable CA operation for mix of 100 and 200 MHz CCs.

RRM:

- Define the SCell activation delay requirements for inter-band CA common beam management (CBM)

# 4.	References

RAN4 #101-bis-e contributions submitted

| R4-2200699 | TR38.851 v0.3.0 | Nokia, Nokia Shanghai Bell |
| --- | --- | --- |
| R4-2202219 | Email discussion summary for [101-bis-e][119] NR_RF_FR2_enh2_Part_1 | Moderator (Nokia) |
| R4-2202319 | Email discussion summary for [101-bis-e][119] NR_RF_FR2_enh2_Part_1 | Moderator (Nokia) |
| R4-2202342 | WF on FR2 DL CA | Nokia |
| R4-2202343 | WF on FR2 UL CA | Qualcomm |
| R4-2200361 | Sensitivity requirements for inter-band CA with CBM | NTT DOCOMO, INC. |
| R4-2200940 | Discussion on introducation of Fs, inter_CBM | vivo |
| R4-2201970 | dCR to 38.101-2 on requirements for UEs that support inter-band CA with CBM | Nokia, Nokia Shanghai Bell, Qualcomm Incorporated |
| R4-2200362 | UE capability for CA within same frequency group with CBM | NTT DOCOMO, INC. |
| R4-2200466 | UE requirements for CBM for the same frequency group | Sony, Ericsson |
| R4-2200554 | Discussion on CBM based inter-band DL CA within same frequency group | LG Electronics |
| R4-2200579 | Reference signal and Fs_intern_CBM of FR2 inter-band DL CA within same frequency group based on CBM | MediaTek Beijing Inc. |
| R4-2200939 | Discussion on CBM within same frequency group | vivo |
| R4-2201275 | R17 FR2 CBM inter-band DL CA | OPPO |
| R4-2201337 | Discussion on CBM for FR2 Inter-band DL CA | ZTE Corporation |
| R4-2201968 | On delta(RIB) for n258+n261 DL inter-CA | Qualcomm Incorporated |
| R4-2200439 | Views on FR2 inter-band DL CA CBM for different band groups | Apple |
| R4-2200467 | Requirements for CBM UEs between different frequency group | Sony, Ericsson |
| R4-2200577 | Reference signal of FR2 inter-band DL CA between different frequency groups based on CBM | MediaTek Beijing Inc. |
| R4-2200700 | TP to TR 38.851: Agreements made for CA configurations between frequency groups using CBM | Nokia, Nokia Shanghai Bell |
| R4-2200735 | Discussion on requirements of FR2 inter-band DL CA | Samsung |
| R4-2200941 | Discussion on CBM between different frequency group | vivo |
| R4-2201299 | Rx requirements for inter-band DL CA with CBM | Xiaomi |
| R4-2201969 | On delta(RIB) for DL inter-CA with CBM in n260+n261 | Qualcomm Incorporated |
| R4-2200701 | Discussion on UE Rx beam switch delay | Nokia, Nokia Shanghai Bell |
| R4-2200945 | Discussion on Rx beam switch time | vivo |
| R4-2201594 | Discussion on UE Rx beam switch delay | Nokia |
| R4-2200345 | FR2 inter-band UL CA framework | Nokia, Nokia Shanghai Bell |
| R4-2200468 | UE UL CA requirements based on IBM | Sony, Ericsson |
| R4-2200736 | Discussion on requirements of FR2 inter-band UL CA | Samsung |
| R4-2200942 | Discussion on inter-band UL CA | vivo |
| R4-2201276 | R17 FR2 Inter-band UL CA requirements | OPPO |
| R4-2201291 | Discussion on inter-band UL CA requirements | ZTE Corporation |
| R4-2201300 | Tx requirements for inter-band UL CA between different frequency groups based on IBM | Xiaomi |
| R4-2200346 | TP to TR 38.851 to introduce FR2 UL CA_n257A-n259A | Nokia, Nokia Shanghai Bell |
| R4-2200555 | RF requirements for CA_n257A_n259A based on IBM | LG Electronics |
| R4-2200569 | View on factor of FR2 inter-band UL CA relaxation | MediaTek Beijing Inc. |
| R4-2201292 | Discussion on relaxation value X&Y for CA_n257A_n259A | ZTE Corporation |
| R4-2201967 | On UL power for FR2 inter-band ULCA | Qualcomm Incorporated |
| R4-2200353 | Discussion of UL gap in FR2 RF enhancement for RAN2 LS | Mediatek India Technology Pvt. |
| R4-2201693 | Discussion on LS on UL gap in FR2 RF enhancement | Nokia, Nokia Shanghai Bell |
| R4-2202220 | Email discussion summary for [101-bis-e][120] NR_RF_FR2_enh2_Part_2 | Moderator (Apple) |
| R4-2202320 | Email discussion summary for [101-bis-e][120] NR_RF_FR2_enh2_Part_2 | Moderator (Apple) |
| R4-2202344 | WF on UL gap for FR2 | Apple |
| R4-2202345 | Reply LS to RAN2 on UL gap in FR2 RF enhancement | Apple |
| R4-2202417 | WF on UL gap for FR2 | Apple |
| R4-2202419 | Reply LS to RAN2 on UL gap in FR2 RF enhancement | Apple |
| R4-2202420 | LS to RAN2 on UL gap in FR2 RF enhancement | Apple |
| R4-2200253 | UL gaps for Tx power management RF aspect | Apple |
| R4-2200255 | Draft CR for UL gap for Tx power management RF aspect | Apple |
| R4-2200383 | Requirements and test cases of UE FR2 UL Gap for UE Tx power enhancement | Nokia, Nokia Shanghai Bell |
| R4-2200589 | Discussion on UL gap for Tx power management | ZTE Corporation |
| R4-2200856 | On UE Tx power management for MPE compliance | Ericsson, Sony |
| R4-2200943 | Discussion and reply LS on FR2 UL gap | vivo |
| R4-2201274 | R17 FR2 UL gap for power management | OPPO |
| R4-2201443 | Discussion on Tx power management | Huawei,HiSilicon |
| R4-2200254 | UL gaps for coherent UL MIMO | Apple |
| R4-2201442 | Draft CR to 38.101-2 on requirements for coherent UL MIMO | Huawei,HiSilicon |
| R4-2201444 | Discussion on UL coherent MIMO | Huawei,HiSilicon |
| R4-2200333 | Further details and optimizations on DC location | Qualcomm Incorporated |
| R4-2200456 | Handling of multiple DC locations for intra-band configuration | Nokia, Nokia Shanghai Bell |
| R4-2200944 | Discussion on DC location | vivo |
| R4-2201273 | R17 FR2 DC reporting | OPPO |
| R4-2201959 | Further study on DC location reporting | Huawei, HiSilicon |
| R4-2202221 | Email discussion summary for [101-bis-e][121] NR_RF_FR2_enh2_Part_3 | Moderator (Vivo) |
| R4-2202321 | Email discussion summary for [101-bis-e][121] NR_RF_FR2_enh2_Part_3 | Moderator (Vivo) |
| R4-2202346 | WF on DC-Location | vivo |
| R4-2202347 | WF on FR2 CA BW classes | Xiaomi |
| R4-2200857 | FR2 bandwidth classes covering up to 1600 MHz aggregated bandwidth with mixed carrier bandwidths | Ericsson |
| R4-2200858 | FR2 CA BW classes up to 1600 MHz aggregated BW with mixed channel bandwidths | Ericsson |
| R4-2200909 | Alternatives of FR2 new CA BW classes | Apple |
| R4-2201297 | Draft CR for TS 38.101-2 to introduction of FR2 new CA BW classesV, MF,ME, MD and MA | Xiaomi |
| R4-2200302 | FR2 bandwidth class and fallback group | Verizon, Qualcomm, MediaTek |
| R4-2200620 | Further considerations on FR2 fallback group | ZTE Corporation |
| R4-2201298 | Discussion on fallback group for FR2 new CA BW classes | Xiaomi |
| R4-2201536 | Draft Big CR: RRM requirements for Rel-17 NR FR2 Inter-band CA | Nokia, Nokia Shanghai Bell |
| R4-2202553 | Email discussion summary for [101-bis-e][202] NR_RF_FR2_enh2_RRM | Moderator (Nokia) |
| R4-2202719 | Email discussion summary for [101-bis-e][202] NR_RF_FR2_enh2_RRM | Moderator (Nokia) |
| R4-2200391 | Further considering on remaining issues for RRM requirements for inter-band DL CA in NR FR2 | vivo |
| R4-2200425 | MRTD requirements for CBM based Inter-band DL CA | Qualcomm Incorporated |
| R4-2200559 | Discussion on MRTD for FR2 inter-band CA based on CBM | LG Electronics |
| R4-2200927 | Discussion on CBM MRTD requirement for FR2 inter-band DL CA | MediaTek inc. |
| R4-2201129 | MRTD requirements for FR2 inter-band DL CA enhancements | OPPO |
| R4-2201537 | discussion on MRTD for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2201538 | draftCR on MRTD for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2201588 | Timing requirements for inter-band DL CA | Ericsson |
| R4-2201589 | Timing requirements for inter-band DL CA | Ericsson |
| R4-2201607 | Discussion on MRTD requirements for FR2 inter-band DL CA with CBM | Huawei, Hisilicon |
| R4-2202581 | WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia |
| R4-2202582 | draftCR on MRTD for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2202583 | Timing requirements for inter-band DL CA | Ericsson |
| R4-2200426 | RRM requirements for CBM based Inter-band DL CA | Qualcomm Incorporated |
| R4-2200742 | Other RRM requirements for FR2 inter-band DL CA requirements for CBM | ZTE Corporation |
| R4-2200928 | Discussion on CBM RRM requirements for FR2 inter-band DL CA | MediaTek inc. |
| R4-2200929 | Introduction of SCell activation delay requirement for FR2 inter-band CA with common beam management | MediaTek inc. |
| R4-2201130 | Other RRM requirements for FR2 inter-band DL CA enhancements | OPPO |
| R4-2201373 | Discussion on RRM requirements of inter-band DL CA for CBM UE | Ericsson |
| R4-2201374 | Draft CR on scheduling restriction for FR2 inter-band DL CA for CBM UE | Ericsson |
| R4-2201539 | discussion on other RRM requirements for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2201540 | draftCR on measurement restriction for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2201608 | Discussion on other RRM requirements for FR2 inter-band DL CA with CBM | Huawei, Hisilicon |
| R4-2201609 | DraftCR on interruption requirements for FR2 inter-band CA with CBM | Huawei, Hisilicon |
| R4-2202584 | Introduction of SCell activation delay requirement for FR2 inter-band CA with common beam management | MediaTek inc. |
| R4-2202585 | Draft CR on scheduling restriction for FR2 inter-band DL CA for CBM UE | Ericsson |
| R4-2202586 | draftCR on measurement restriction for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2202587 | DraftCR on interruption requirements for FR2 inter-band CA with CBM | Huawei, Hisilicon |
| R4-2201131 | Inter-band UL CA requirements for IBM | OPPO |
| R4-2201375 | Discussion on RRM requirements for inter-band UL CA for IBM UE | Ericsson |
| R4-2201376 | Draft CR on RRM requirements for FR2 inter-band UL CA for IBM UE | Ericsson |
| R4-2201541 | draftCR on RRM requirements for IBM inter-band FR2 UL CA | Nokia, Nokia Shanghai Bell |
| R4-2202588 | Draft CR on RRM requirements for FR2 inter-band UL CA for IBM UE | Ericsson |
| R4-2202589 | draftCR on RRM requirements for IBM inter-band FR2 UL CA | Nokia, Nokia Shanghai Bell |
| R4-2200256 | UL gaps for Tx power management RRM aspect and draft reply LS | Apple |
| R4-2200257 | Draft CR for UL gap for Tx power management RRM aspect | Apple |
| R4-2200384 | Network impact of UE FR2 UL Gap for UE Tx power enhancements | Nokia, Nokia Shanghai Bell |
| R4-2200427 | UL gaps for self-calibration and monitoring | Qualcomm Incorporated |
| R4-2200590 | Discussion on RRM impact of UL gap for Tx power management | ZTE Corporation |
| R4-2200605 | Discussion on RRM impacts of UL gaps for self-calibration and monitoring | vivo |
| R4-2201377 | Discussion on UL gaps for self calibration and monitoring | Ericsson |

RAN4 #102e contributions submitted

| R4-2204787 | TR 38.851 v0.4.0 | Nokia, Nokia Shanghai Bell |
| --- | --- | --- |
| R4-2206325 | Email discussion summary for [102-e][125] NR_RF_FR2_enh2_Part_1 | Moderator (Nokia) |
| R4-2206425 | Email discussion summary for [102-e][125] NR_RF_FR2_enh2_Part_1 | Moderator (Nokia) |
| R4-2204361 | Sensitivity requirements for inter-band DL CA with CBM | NTT DOCOMO, INC. |
| R4-2204789 | Addition of downlink CA feature for CBM UEs and one band combination for IBM UEs | Nokia, Qualcomm |
| R4-2204035 | UE requirements for CBM for the same frequency group | Sony, Ericsson |
| R4-2204143 | Discussion on CBM based inter-band DL CA within same frequency group | LG Electronics |
| R4-2204229 | Fs_inter and view on FR2 inter-band DL CA within same frequency group based on CBM | MediaTek Beijing Inc. |
| R4-2204927 | R17 FR2 CBM inter-band DL CA | OPPO |
| R4-2204940 | Discussion on requirement of n258-n261 | vivo |
| R4-2205122 | Discussion on inter-band DL CA with CBM | Xiaomi |
| R4-2205598 | On RF requirements for FR2 Inter-band DL CA with CBM | Huawei, HiSilicon |
| R4-2206055 | On delta(RIB) for n258+n261 DL inter-CA | Qualcomm Incorporated |
| R4-2203699 | FR2 Sensitivity requirements for inter-band CBM | Apple |
| R4-2204036 | Requirements for CBM UEs between different frequency group | Sony, Ericsson |
| R4-2204230 | Reference signal, and relaxation value about FR2 inter-band DL CA between different frequency groups based on CBM | MediaTek Beijing Inc. |
| R4-2204575 | Discussion on requirements of FR2 inter-band DL CA | Samsung |
| R4-2204612 | Introduction of requirements for DL inter-band CA for CBM-capable UEs | Ericsson, Sony |
| R4-2204941 | Discussion on CBM between different frequency group | vivo |
| R4-2206056 | On delta(RIB) for DL inter-CA with CBM in n260+n261 | Qualcomm Incorporated |
| R4-2204790 | Discussion on UE Rx beam switch delay | Nokia, Nokia Shanghai Bell |
| R4-2203814 | Introduce FR2 n260 and n261 uplink CA | Verizon Denmark |
| R4-2204037 | UE UL CA requirements based on IBM | Sony, Ericsson |
| R4-2204576 | Discussion on MOP relaxation of FR2 inter-band UL CA | Samsung |
| R4-2205123 | Tx requirements for inter-band UL CA between different frequency groups based on IBM | Xiaomi |
| R4-2205599 | On RF requirements for FR2 inter-band UL CA | Huawei, HiSilicon |
| R4-2206057 | Draft CR to 38.101-2 FR2+FR2 ULCA | Qualcomm, Nokia, Verizon, Docomo |
| R4-2204228 | View on FR2 inter-band UL CA relaxation | MediaTek Beijing Inc. |
| R4-2204928 | R17 FR2 Inter-band UL CA requirements | OPPO |
| R4-2204942 | Discussion on iinter-band UL CA | vivo |
| R4-2205109 | Discussion on relaxation value X&Y for CA_n257A_n259A | ZTE Corporation |
| R4-2206054 | delta(TIB) for FR2+FR2 ULCA | Qualcomm Incorporated |
| R4-2204943 | Discussion on UL gap | vivo |
| R4-2203557 | Requirements and test cases of UE FR2 UL Gap for UE Tx power enhancement | Nokia Denmark |
| R4-2203749 | UL gaps for Tx power management RF aspect | Apple |
| R4-2203751 | Draft CR for UL gap for Tx power management RF aspect | Apple |
| R4-2204613 | More on UE Tx power management for MPE compliance | Ericsson, Sony |
| R4-2204925 | R17 FR2 UL gap for power management | Guangdong OPPO Mobile Telecom. |
| R4-2205005 | Discussion on Tx power management | Huawei,HiSilicon |
| R4-2206326 | Email discussion summary for [102-e][126] NR_RF_FR2_enh2_Part_2 | Moderator (Apple) |
| R4-2206426 | Email discussion summary for [102-e][126] NR_RF_FR2_enh2_Part_2 | Moderator (Apple) |
| R4-2203750 | UL gaps for coherent UL MIMO | Apple |
| R4-2205004 | Draft CR to 38.101-2 on requirements for coherent UL MIMO | Huawei,HiSilicon |
| R4-2205006 | Discussion on UL coherent MIMO | Huawei,HiSilicon |
| R4-2205611 | FR2 UL coherent MIMO | Anritsu Limited |
| R4-2203698 | DC location for intra-band UL CA with more than 2 CCs | Apple |
| R4-2204198 | Handling of multiple DC locations for intra-band configuration | Nokia, Nokia Shanghai Bell |
| R4-2204822 | Further study on DC location reporting | Huawei, HiSilicon |
| R4-2204922 | R17 FR2 DC reporting | OPPO |
| R4-2204944 | Discussion and draft reply LS on DC location | vivo |
| R4-2205883 | Two DC location and RAN2 LS discussion | Qualcomm Incorporated |
| R4-2206327 | Email discussion summary for [102-e][127] NR_RF_FR2_enh2_Part_3 | Moderator (Vivo) |
| R4-2206427 | Email discussion summary for [102-e][127] NR_RF_FR2_enh2_Part_3 | Moderator (Vivo) |
| R4-2203697 | New CA BW classes | Apple |
| R4-2206062 | On new contiguous BW classes for legacy networks | Qualcomm Incorporated |
| R4-2203812 | FR2 bandwidth class and fallback group | Verizon |
| R4-2203990 | Discussion on new FR2 CA BW classes in hybrid FBG | ZTE Corporation |
| R4-2204220 | View on new FR2 CA BW class options | MediaTek Beijing Inc. |
| R4-2204614 | FR2 bandwidth classes covering up to 1600 MHz aggregated bandwidth with mixed carrier bandwidths | Ericsson |
| R4-2204615 | FR2 CA BW classes up to 1600 MHz aggregated BW with mixed channel bandwidths | Ericsson |
| R4-2204788 | Solution to FBG3+2 topic | Nokia, Nokia Shanghai Bell |
| R4-2205124 | Discussion on FR2 new CA BW classes | Xiaomi |
| R4-2205125 | LS on release independence aspects of newly introduced FR2 CA BW Classes | Xiaomi |
| R4-2205126 | Draft CR for TS 38.101-2 to introduction of FR2 new CA BW classes V, AF, GF, HF, IF, JF, KF, LF, MF,ME, MD, MA | Xiaomi |
| R4-2205868 | Draft Big CR on RRM requirements for FR2 Inter-band CA | Nokia, Nokia Shanghai Bell |
| R4-2206753 | Email discussion summary for [102-e][210] NR_RF_FR2_req_enh2_RRM | Moderator (Nokia) |
| R4-2206837 | WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia |
| R4-2207051 | Email discussion summary for [102-e][210] NR_RF_FR2_req_enh2_RRM | Moderator (Nokia) |
| R4-2205869 | draftCR on CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2203860 | MRTD requirements for CBM based Inter-band DL CA | Qualcomm Incorporated |
| R4-2204149 | Discussion on MRTD for FR2 inter-band CA based on CBM | LG Electronics |
| R4-2204182 | Discussion on CBM MRTD requirement for FR2 inter-band DL CA | MediaTek inc. |
| R4-2204271 | MRTD requirements for FR2 inter-band DL CA enhancements | OPPO |
| R4-2205326 | Discussion on MRTD requirements for FR2 inter-band DL CA with CBM | Huawei, HiSilicon |
| R4-2205423 | Timing requirements for inter-band DL CA | Ericsson |
| R4-2205424 | Timing requirements for inter-band DL CA | Ericsson |
| R4-2205870 | discussion on MRTD for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2205871 | draftCR on MRTD for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2203861 | RRM requirements for CBM based Inter-band DL CA | Qualcomm Incorporated |
| R4-2204183 | Discussion on CBM RRM requirements for FR2 inter-band DL CA | MediaTek inc. |
| R4-2204184 | Introduction of SCell activation delay requirement for FR2 inter-band CA with common beam management | MediaTek inc. |
| R4-2204272 | Other RRM requirements for FR2 inter-band DL CA enhancements | OPPO |
| R4-2205327 | Discussion on other RRM requirements for FR2 inter-band DL CA with CBM | Huawei, HiSilicon |
| R4-2205328 | DraftCR on applicability rules for FR2 inter-band CA with CBM | Huawei, HiSilicon |
| R4-2205657 | On network driven Rx beam switch for CBM | Apple |
| R4-2205830 | Discussion on RRM requirements of inter-band DL CA for CBM UE | Ericsson |
| R4-2205831 | Draft CR on scheduling restriction for FR2 inter-band DL CA for CBM UE | Ericsson |
| R4-2205872 | discussion on other RRM requirements for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2205873 | draftCR on measurement restriction for CBM inter-band FR2 DL CA | Nokia, Nokia Shanghai Bell |
| R4-2205832 | Discussion on RRM requirements for inter-band UL CA for IBM UE | Ericsson |
| R4-2205833 | Draft CR on number of UL CC support for FR2 and interruption requirements for FR2 UL CA for IBM UE | Ericsson |
| R4-2205874 | Discussion on supportedserving carriers for IBM inter-band FR2 UL CA | Nokia, Nokia Shanghai Bell |
| R4-2205875 | draftCR on RRM requirements for IBM inter-band FR2 UL CA | Nokia, Nokia Shanghai Bell |
| R4-2206069 | Discussion on RRM requirements of FR2 inter-band DL and UL CA for IBM UE | Ericsson |
| R4-2203752 | UL gaps for Tx power management RRM aspect | Apple |
| R4-2203753 | Draft CR for UL gap for Tx power management RRM aspect | Apple |
| R4-2203862 | UL gaps for self-calibration and monitoring | Qualcomm Incorporated |
| R4-2205013 | Discussion on RRM impact of UL gap for Tx power management | ZTE Corporation |
| R4-2205649 | Network impact of FR2 UL gaps - RRM | Nokia, Nokia Shanghai Bell |
| R4-2205650 | LS on UL gap in FR2 RF enhancement | Nokia, Nokia Shanghai Bell |
| R4-2205834 | Discussion on UL gaps for self calibration and monitoring | Ericsson |
| R4-2205835 | Draft CR on UL gaps for TX power management | Ericsson |