3GPP TSG RAN Meeting #97-e    	            RP-222064

E-meeting, September 12th – 16th, 2022

Source:	Qualcomm Incorporated

Title:	Views on support of XR with 2Rx

Agenda Item:	9.2.11

Document for: 	Discussion and Decision

# Introduction

At RAN#96 a contribution [1] was submitted, proposing to add a study (if found necessary) of 2Rx devices in the Rel-18 XR Study Item.

In this contribution, we give our views on this topic.

# Discussion

We fully agree that some XR devices, such as glasses will have size limitation making it not possible to fit 4Rx in some of those devices.

For example, due to the trace losses involved with high operating frequencies, running RF cables between the two temples of the glass may not be feasible. In that case, all antennas must be placed on one side, where there is no sufficient space to fit four antennas.

Therefore, we agree that there is a need to allow XR devices equipped with 2Rx in FR1 high bands.

We also agree that some XR devices will require more than 20MHz BW for supporting the necessary data rates for operation.

On the other hand, we do not agree that all XR devices will need more than 20MHz to operate, or that RedCap modems cannot function to support lower category of XR devices. Therefore, we make the following proposals:

Proposal 1:  Agree to introduce support of 2Rx devices for XR in FR1 with up to 100MHz operating BW.

Proposal 2:  Do not make any conclusion implying that RedCap devices with 20MHz BW limitation cannot be used for XR.

Based on past discussions, it can be expected that there will be concerns on applying such device definitions also to devices other than XR. To address such concerns, it should be discussed how to define the device type for which the 2Rx requirement applies. One option is to apply similar restrictions as in RedCap / eRedCap, such as non-NSA, non-CA/DC/SUL. Of course, such restriction would not apply to XR in general, only to those devices that comply with the 2Rx requirement.

Proposal 3:   Discuss and decide how to define the applicable device types for which 100MHz BW and 2Rx requirement applies.  One option is that the devices inherit all the RedCap limitations, e.g. non-NSA, non-CA/DC/SUL, except for the BW limitation​.

Lastly, in contribution [1] it was proposed to conduct an extensive study (if found necessary) of the impact and performance of the 2Rx XR devices in the XR SI. We do not think that such an extensive study is necessary. In addition, there is unlikely to have any room for adding such study in Rel-18. Therefore, we make the following proposal.

Proposal 4:   Make a Plenary decision on adding support of 2Rx 100MHz devices for operation in FR1 high bands, without up-scoping the Rel-18 XR SI with additional studies. Discuss and decide also whether this should be based on an extension of the RedCap device type or based on defining a suitable XR device type.

# Conclusion

In this contribution, we gave our views on the topic of 100MHz 2Rx XR devices. The following proposals were made.

Proposal 1:  Agree to introduce support of 2Rx devices for XR in FR1 with up to 100MHz operating BW.

Proposal 2:  Do not make any conclusion implying that RedCap devices with 20MHz BW limitation cannot be used for XR.

Proposal 3:  Discuss and decide how to define the applicable device types for which 100MHz BW and 2Rx requirement applies.  One option is that such devices inherit all the RedCap limitations, e.g. non-NSA, non-CA/DC/SUL, except for the BW limitation​.

Proposal 4:  Make a Plenary decision on adding support of 2Rx 100MHz devices for operation in FR1 high bands, without up-scoping the Rel-18 XR SI with additional studies. Discuss and decide also whether this should be based on an extension of the RedCap device type or based on defining a suitable XR device type.

# Reference

- RP-221692, “Views on Rel-18 XR”, Apple