|  | ASIA-PACIFIC TELECOMMUNITY | Document No: |
| --- | --- | --- |

Technical Considerations for new band options (B1 and B2)

- Contents

1	Scope	3

2	References	3

3	Introduction	4

4	Proposed bands and characteristics	4

5	Analysing B1 and B2	5

5.1	Frequency ranges studied in TR 38 860 v1:	5

5.2	Analysis of BS aspects of Option B1 and Option B2	7

5.2.1	Unwanted Emissions generic for B1 and B2	7

5.2.2	Reference sensitivity level	11

5.2.3	Adjacent Channel Selectivity (ACS), in-band blocking and out-of-band blocking	12

5.2.4	Protection to RAS	12

5.2.5	Summary	13

5.3	Review of Guard Band Requirements	13

5.3.1	Indicative Guard Band, Co-Existence and Interference Analysis for Digital TV (DTV) Transmitters into Mobile User Equipment (UE) Receivers	13

5.4	Analysis of B1 Ue Aspects	15

5.5	Analysis of B2 Ue aspects	18

5.6	Network Architecture Impacts of option A,B1 and B2	20

5.6.1	5G System Architecture	20

5.6.2	Impacts to Antenna Ports	20

5.6.3	NSA Impacts from Option A / B1 / B2	21

5.6.4	Core / Architectural Impacts – MR DC	22

5.6.5	Summary	22

6	Comparison of B1 and B2 from a UE Perspective	23

6.1	Option B1	23

6.2	Option B2	23

6.3	Compatibility with legacy Band n71	23

6.4	Option B1 compatibility with n71	23

7	Example implementation considerations	24

7.1	Example 1	24

7.1.1	600 MHz (B71) vs 700 MHz (B28) Comparison	24

7.2	Example 2	25

7.3	Example 3	25

8	Options for Preferred band(s) plans clarified	26

9	Transition Scenarios	26

10	Recommendations	26

11	Correspondence Group Involvement	26

# Scope

The purpose of the Informal correspondence activity is to investigate options for the APT 600 MHz band and Prepare an input contribution to AWG-29 on the technical aspects of the two new band options B1 and B2.

Quantifying the radio performance

Implementation complexity

Possible regional device market dynamics

Impact of network infrastructure architecture

The following topics have been investigated:

- Radio performance - including receiver sensitivity and out-of-band emission performance

- Implementation complexity to achieve those performance levels

- Option B1 specifics – guard band to BC, RAS, OOBE for one duplexer 2x40MHz (compare n71 & FCC)

- Option B2 specifics - guard band to BC, RAS, OOBE for n71 2x35MHz duplexer + one upper duplexer 2x25/2x20/2x15MHz (compare n71 & FCC)

- Regional device market dynamics (LAM vs APAC & Region 1); [cost issues], ecosystem over time, option A, option B1, option B2

- Network infrastructure architecture

- DTV Co-existence and guard bands

The scope of the report is to summarise the key aspects of the band(s), for the complete details of the bands and technical implementation please see the AWG and 3GPP documents referenced.

# References

AWG- documents

AWG-28-INP-43_IAFI-08-_Frequency_arrangement_for_IMT

AWG-28-INP-70_600_MHz_band_plans

AWG-28-INP-76_GSA-SparkNZ-GSMA_600_MHz

AWG-28-OUT-15_WG-SPEC_Meeting_Report-v1

AWG-28-OUT-17_Reply_liaison_statement_to_3GPP_RAN_3GPP_RAN4 :(3GPP ref RP-212540)

AWG-28-TMP-x1_600MHz_3GPP_LS final

3GPP LS

RP-212629 : LS to APT on frequency arrangements for IMT in the band 470 – 703 MHz

R4-2114750: LS on the progress of the study item on extended 600MHz NR band

3GPP technical reports

3GPP 38.860: Study on Extended 600 MHz NR band

3GPP TR 36.755: US 600 MHz Band for LTE

GSA

GSA-National-Spectrum-Positions-from-600-September-2021

# Introduction

- The AWG-27 tasked 3GPP to study two new band plan options (B1 and B2) for APT which add 5 MHz to the 35 MHz USA band to create a 40 MHz band

- AWG-28 received a LS from 3GPP on feasibility of both bands and needed to select the APT 600MHz band/s to be finalised by 3GPP.

- At the AWG-28, there was support for each band option and proposals for sending a LS to 3GPP with the request of finalisation of both bands for APT.

- AWG-28 decided to “continue keeping these options under review with the objective to decide on a single option at our next AWG-29 meeting as more information becomes available.”

- AWG agreed that at least one additional band should be considered for APT 600MHz.

# Proposed bands and characteristics

The figures below depict the different options being studied for the APT 600 MHz band, with the US 600 Band being considered as Option A. Option B1 has 1 proposed configuration, while B2 has 3.

Option A

An Analysis of band plans, duplex arrangements and the implementation options required to enable 3GPP to complete the band specifications is contained in the following sections.

Detailed analysis of the Frequency band arrangements, regulatory requirements, coexistence with DTV and ACS can be found in the APT and 3GPP documents.

While the initial assumption was for a guard band of 9 MHz (the same as for B71) further studies have been carried out to confirm if other viable bandwidths may be suitable given the frequency arrangement options in the APT region.

Due to various DTV channel arrangements across national regulations, the actual frequency separation will be decided by administrators.

# Analysing B1 and B2

## Frequency ranges studied in TR 38 860 v1:

The band plans for the extended 600 MHz NR band are shown below in Tables 6.1-1 to 6.1-6 and 6.1-9.  For ease of cross referencing the Table numbers from 3GPP 38.860 have been kept.

Options B1 and B2 have split duplexers, partially overlapping, but part of the same band.

The Tx-Rx is “reverse-duplex”; in other words, the downlink frequency band is below the duplex gap while the uplink frequency band is above the duplex gap. This arrangement is opposite to conventional notation; however, for this band, it provides the benefit of aligning the uplink band adjacent to 3GPP band 28 thereby minimizing interference conditions at the 703 MHz boundary.

Table 6.1-1: NR operating band (option B1)

| Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

The above could be implemented as a single or overlapping duplexers. In case of overlapping duplexers, the following option is studied:

Table 6.1-2: Duplexer arrangements (option B1)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| Full | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |
| 35 + 35 | 663 – 698 MHz 668 – 703 MHz |  |  | 612 – 647 MHz 617 – 652 MHz |  |  | FDD |
|  |  |  |  |  |  |  | FDD |
| n71 + 25 Dual 35+25 | 663 – 698 MHz 678 – 703 MHz |  |  | 617 – 652 MHz 612 – 637 MHz |  |  | FDD. FDD. |

Table 6.1-3.: NR operating band (option B2)

| Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 663 MHz | – | 703 MHz | 617 MHz | – | 657 MHz | FDD |

The above could be implemented as a single or overlapping duplexers. In case of overlapping/split duplexers, three sub options are studied.

Table 6.1-4: Duplexer arrangements (option B2 35 + 35)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive | Duplex Mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| Duplex 1 Duplex 2 | 663 MHz – 698 MHz 668 MHz – 703 MHz | 617MHz – 652 MHz 622MHz – 657 MHz | FDD |
|  |  |  | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |

Table 6.1-5: Duplexer arrangements (option B2 35+30)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive | Duplex Mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| Duplex1 Duplex 2 | 663 MHz – 698 MHz 673 MHz – 703 MHz | 617MHz – 652 MHz 627MHz – 657 MHz | FDD |
|  |  |  | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |

Table 6.1-6: Duplexer arrangements (option B2 35+25)

| Duplexer type | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive | Duplex Mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| Duplex 1 Duplex 2 | 663 MHz – 698 MHz 678 MHz – 703 MHz | 617MHz – 652 MHz 632MHz – 657 MHz | FDD |
|  |  |  | FDD |
| NOTE: Both duplexers will be part of the same band |  |  |  |

Tx-Rx separation is shown in Table 6.1-9 and is dependent upon the filter options.

Table 6.1-9: Default UE TX-RX frequency separation

| Filter Option | TX – RX carrier centre frequency separation |
| --- | --- |
| B1 (Full band) | -51 MHz |
| B1 (duplexer 1) 35 + 35 B1 (duplexer 2) 35 + 35 | -51 MHz -51 MHz |
| B1 (duplexer 1) n71 + 25 | -46 MHz |
| B1 (duplexer 2) n71 +25 | 66 MHz |
| B2 (duplexer 1) B2 (duplexer 2) | 46 MHz |
|  | 46 MHz |

In the detailed analysis of the various duplexer configurations, TR 38 860 suggests:

B1 : Full band implementation only.

B2 : three cases were studied:

- B2 single duplexer- not recommended further.

- 35 + 35 (one duplexer is same as n 71)- assumes n71 is implemented as a single duplexer

- 35 + 25 (one duplexer is same as n 71)- assumes n 71 is implemented as a full duplexer

For Option B2 the 35+35 MHz duplexer implementation is preferred for the following reasons.

  - The Lower duplexer can be the same as the n71 duplexer and the upper duplexer is just frequency shifted, this may provide some cost efficiencies relative to other dual duplexer implementations.

  - Maximises the possible bandwidth available (for Option B2) for an individual operator in either the lower or upper duplexer.

  - Different operators will have similar performance that is not dependent on the frequency allocation within the band.

  - Minimises the likelihood of an operator requiring intra-band CA and the significant implementation complexities and limitations for both Network and Ue.

It is recognised that using an upper duplexer with a bandwidth of less than 35 MHz may have some improved sensitivity however this was considered secondary to the above points.

## Analysis of BS aspects of Option B1 and Option B2

This analysis refers to:

- 3GPP TS 38.104 v17.4.0

- 3GPP TS 38.860 v17.0.0

- 3GPP TS 36.755 v15.0.0

### Unwanted Emissions generic for B1 and B2

The discussions below are from 3GPP TS 38.104. Table numbers refer to the tables in TS 38.104 by way of reference.

Unwanted emissions consist of out-of-band emissions and spurious emissions according to ITU definitions.

In ITU terminology:

- Out of band emissions are unwanted emissions immediately outside the BS channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions.

- Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The out-of-band emissions requirement for the BS transmitter is specified both in terms of Adjacent Channel Leakage Power Ratio (ACLR) and operating band unwanted emissions (OBUE).

The maximum offset of the operating band unwanted emissions mask from the operating band edge is via the ΔfOBUE parameter. The Operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band.  Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement.

The values of ΔfOBUE are defined in table 6.6.1-1 for the NR operating bands.

Note: FDL,high – FDL,low < 100 MHz  is the case for B1 and B2, therefore, OBUE region is 10 MHz outside of the band edge regardless of the band options (B1 or BS) and BS types (1-H or 1-C).

Table 6.6.1-1: Maximum offset of OBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-H | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz FDL,high – FDL,low 900 MHz | 40 |
| BS type 1-C | FDL,high – FDL,low 200 MHz | 10 |
|  | 200 MHz < FDL,high – FDL,low 900 MHz | 40 |

#### ACLR Limits generic to both B1 and B2

These are specified in Table 6.6.3.2-1 and 6.6.3.2-2 of TS 38.104 and are applicable to both B1 and B2 and are specified as a limit and a basic limit. These are common to both B1 and B2

For operation in paired and unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.2-1 in any operating band.

Table 6.6.3.2-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 45 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 45 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |

The ACLR absolute basic limit is specified in table 6.6.3.2-2.

Table 6.6.3.2-2: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

#### Operating band out of band emission requirements for B1 and B2

Basic limits (unwanted emission limits) for both B1 and B2 are specified in the tables below, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE outside the downlink operating band, where ΔfOBUE is defined in table 6.6.1-1.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Basic limits for Wide Area BS (Category A)

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n71, n85, basic limits are specified in table 6.6.4.2.1-1.

Table 6.6.4.2.1-1: Wide Area BS operating band unwanted emission limits 
(NR bands below 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -13 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -13 dBm/100 kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Basic limits for Wide Area BS (Category B)

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, the basic limits are specified in table 6.6.4.2.2.1-1:

Table 6.6.4.2.2.1-1: Wide Area BS operating band unwanted emission limits 
(NR bands below 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -16 dBm/100 kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Note: Similar limits but different values are also specified for medium range and local area BS

The difference between wide area, medium area and local BS is in terms of rated output BS power.

Specific out of band emission requirements for B1

The above tables also apply to B1.

Specific out of band emission requirements for B2

The frequency range for B2 towards the DTV side is same as n71.  Therefore, the operating band unwanted emissions for n71 can be fulfilled by Option B2.

#### Transmitter spurious emissions

The transmitter spurious emission limits  are  also common to B1 and B2 , they shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table 6.6.1-1.

Unless otherwise stated, all requirements are measured as mean power (RMS).

##### General transmitter spurious emissions requirements

The basic limits of either table 6.6.5.2.1-1 (Category A limits) or table 6.6.5. 2.1-2 (Category B limits) shall apply. The application of either Category A or Category B limits shall be the same as for operating band unwanted emissions in clause 6.6.4.

Table 6.6.5.2.1-1: General BS transmitter spurious emission limits in FR1, Category A

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -13 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | -13 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies only for band n46 and n96. |  |  |  |

Table 6.6.5.2.1-2: General BS transmitter spurious emission limits in FR1, Category B

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz | -36 dBm | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz |  | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | - 30 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies only for band n46. |  |  |  |

##### Protection of the BS receiver of own or different BS

This requirement shall be applied for NR FDD operation in order to prevent the receivers of the BSs being desensitised by emissions from a BS transmitter. It is measured at the transmit antenna connector for BS type 1-C or at the TAB connector for BS type 1-H for any type of BS which has common or separate Tx/Rx antenna connectors / TAB connectors.

The spurious emission basic limits are provided in table 6.6.5.2.2-1.

Table 6.6.5.2.2-1: BS spurious emissions basic limits for protection of the BS receiver

| BS class | Frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| Wide Area BS | FUL,low – FUL,high | -96 dBm | 100 kHz |  |
| Medium Range BS | FUL,low – FUL,high | -91 dBm | 100 kHz |  |
| Local Area BS | FUL,low – FUL,high | -88 dBm | 100 kHz |  |

The above emission requirement shall be fulfilled regardless of option B1 or B2 to not allow self de-sensitization.

### Reference sensitivity level

BS reference sensitivity level is specified in clause 7.2 of TS 38.104.

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for BS type 1-C or TAB connector for BS type 1-H at which a throughput requirement shall be met for a specified reference measurement channel.

Minimum requirements for BS type 1-C and BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-1 for Wide Area BS, in table 7.2.2-2 for Medium Range BS and in table 7.2.2-3 for Local Area BS .

Only wide area BS levels are shown below; the following sensitivity level shall be fulfilled regardless of option B1 and B2.

Table 7.2.2-1: NR Wide Area BS reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -101.7 |
|  |  | G-FR1-A1-10 (Note 3) | -101.7 (Note 2) |
| 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -101.8 |
| 10, 15 | 60 | G-FR1-A1-3 (Note 1) | -98.9 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 (Note 1) | -95.3 |
|  |  | G-FR1-A1-11 (Note 4) | -95.3 (Note 2) |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 (Note 1) | -95.6 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 (Note 1) | -95.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The requirements apply to BS that supports NB-IoT operation in NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. NOTE 5: Void. |  |  |  |

### Adjacent Channel Selectivity (ACS), in-band blocking and out-of-band blocking

BS receivers shall fulfil the following common requirement to both B1 and B2 for in-band and out-of-band coexistence.

- Adjacent channel selectivity (ACS) is specified in clause 7.4.1 of TS 38.104 for the adjacent channel coexistence.

- In-band blocking is specified in clause 7.4.2 of TS 38.104 for coexistence in-band region, i.e., up to ΔfOOB = 20 MHz outside of the operating band.

- Out-of-band blocking is specified in clause 7.5 of TS 38.104, more than ΔfOOB = 20 MHz outside of the operating band.

### Protection to RAS

As per 3GPP TR 36.755, Clause 5.3.2.2 mentions the following to protect RAS:

To protect RAS from harmful interference, FCC requires 600 MHz wireless BSs located within 25 kilometres of VLBA observatories to coordinate with the National Science Foundation (NSF) prior to commencing operation; while those located close to dish radio astronomy installations are required to notify technical details of the proposed operations according to section 1.924 of Part 47 CFR.

No additional emission limits need to be included in BS RF requirements for the 600 MHz band.

### Summary

From the point of view of out of band emissions, both B1 and B2 are no different and will be subject to the same requirements. There are additional requirements for n71 that could also apply to B2 but this is a local Administration issue.

## Review of Guard Band Requirements

### Indicative Guard Band, Co-Existence and Interference Analysis for Digital TV (DTV) Transmitters into Mobile User Equipment (UE) Receivers

APT countries have multiple TV channel spacings - 6, 7 and 8 MHz, respectively. Guard band and co-existence studies were conducted for two options:

- B1 – Full band;

- B2 – 35 + 35 MHz.

For each of these options, co-existence studies were done for DTV transmitters on three different TV channel spacings in use in APT countries and mobile B1 and B2 options. Figure 1 depicts an overall preview of the co-existence issues studied for the aforementioned bands with digital TV systems in the APT region.

Figure 1. Mobile and DTV co-existence in the 600 MHz, with an illustration of the interference from DTV transmitter into the mobile UE receivers.

The results presented in this summary document are a result of a detailed Monte-Carlo simulation which can be presented probabilistically as cumulative distribution function (CDFs) to determine the interference levels from a DTV transmitter to a mobile UE, and to determine guard band requirements so that the interference remains below the blocking level for 95% of the time.

The CDFs are determined by the following simulation layout and key assumptions:

- An 83 dBm (200 kW) DTV transmitter is placed 300 m high (effective antenna height). In the DTV coverage area, both DTV and mobile base stations, as well as the associated mobile UE receivers are uniformly distributed (located) with respect to the mobile and DTV coverage areas. This is based on the DVB-T, DVB-T2 assumptions from Report ITU-R BT.2383.

- Density of users for DTV and mobile systems is consistent with Recommendations ITU-R M.2292-0 and ITU-R BT.2383, respectively.

- 50% of the mobile UEs are considered to be indoor, in which case a fixed outdoor-to-indoor propagation loss of 15 dB is considered in estimating field strengths. This is obtained from the Recommendation ITU-R M.2292-0.

- DTV transmitter pattern and DTV receiver antenna patterns are derived from Recommendations ITU-R BT.2383 and ITU-R BT 419-3, respectively (see Section 7.1.1).

- Propagation model used for the analysis is derived from Recommendation ITU-R P.1546-6 with parameters for rural/suburban land paths for a 50% exceedance window, and a log-normally distributed shadow fading standard deviation of 5.5 dB on top of the fixed path loss. The 20-step procedure is followed with the relevant parameters from Annex 6 of ITU-R P.1546-6.

- Mobile system parameters are largely derived from ITU-R M.2292-0 and ITU-R F.1336-5 for a rural setting.

- Blocking level of a mobile UE is specified in 3GPP TS 38.101-1 Table 7.6.2-2 as -44 dBm for +/- 15 MHz either side of the desired (UE receive band).  This level is defined as a value for which the throughput of the wanted signal is >= 95% of the maximum throughput of the reference measurement channels.  The simulations have made a simplified, yet conservative, assumption to ensure that the interferer level should be less than -44 dBm 95% of the time.

- An integrated isolation provided by the UE duplex filter is assumed to be 15- 35 dB - based on information sought from different UE duplex filter vendors. Results shown below use a value of 20 dB as an average representation. However, the blocking level is also respected for the lower value of 15 dB.

The CDFs for DTV interference to a mobile UE, within the UE receive band are given below for option B1 (Figure 2) and B2 (Figure 3), respectively using a 4 MHz guard band:

Figure 2. Interference CDFs at the mobile UE from the applicable APT region DTV channels, considering option B1 with a 4 MHz guard band.

Figure 3. Interference CDFs at the mobile UE from the applicable APT region DTV channels, considering option B2 with a 4 MHz guard band.

Using the 4 MHz guard band, the impacted TV channels in the APT region are shown in Figure 4:

Figure 4. A 4 MHz guard band example of the impacted DTV channels in the APT region with 6/7/8 MHz spacings for co-existence with 600 MHz mobile systems considering band options B1 and B2, respectively.

To this end, for a minimum guard band of 4 MHz, the following DTV channels must be cleared/removed.

Table 1. DTV channels in the APT regions to be cleared with a 4 MHz guard band for options B1 and B2.

## Analysis of B1 Ue Aspects

The UE aspects related to B1 are discussed in 3GPP TR 38.830 with the key aspects highlighted below.

UE Tx power

Co-existence issues should be considered with DTV. Co-existence with band 28 is not a problem as duplex directions are reversed.

UE power class 3 – Table 6.2.1-1 of 38 101-1 – copied below- should be maintained as is also the case for n71

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance dB | Class 2 (dBm) | Tolerance dB | Class 3 (dbm) | Tolerance dB |
| --- | --- | --- | --- | --- | --- | --- |
| N 71 |  |  |  |  | 23 | +/- 2.5 |
| B1 |  |  |  |  | 23 | +/- 2.5 |

Insertion Loss

TR 38 860 states that:

One advantage of option B1 compared to other filtering options is that B1 is conducive to a single full-band filter implementation. The potential advantage of single filter is reduced size and bill-of-materials of the required front-end and reduced complexity for intra-band carrier aggregation across the filters and inter-band carrier aggregation with another band in close frequency proximity when quadplexing is needed are limited. On the other hand, the performance of a dual filter solution in insertion loss and stop band attenuation may be superior to a single filter.

For option B1, extending the 35 MHz filter passband to 40 MHz increases the relative bandwidth from 5.5% to 6.3% at 600 MHz. At the time that Band 28 was defined, such relative bandwidths were not feasible. However, since that time with technological advances in filter design and materials, wider relative bandwidths have now become available. Therefore, from a relative bandwidth perspective option B1 is regarded as feasible.

On the transmit side, the insertion loss is approaching 3 dB but still meets the Band 71 filter specification. In comparison to a narrower Band 71 filter using the same generation of filter technology and the same constraints, an increase in Tx insertion loss towards the band edge may be expected. Moreover, the maximum output power for Band n71 provides a lower tolerance of 2.5 dB so the insertion loss of the Tx filter is not expected to be a problem.

On the receiver side, the insertion loss is approaching 2 dB. The steep drop-off of the filter at the upper edge of the band at 652 MHz is likely due to the relatively narrow duplex gap and the need to provide sufficient Tx isolation. In comparison to a narrower Band 71 filter using the same generation of filter technology and the same constraints, an increase in Rx insertion loss may be expected. But since the reference sensitivity requirement for Band 71 is relatively relaxed to accommodate the noise and spurious products from the transmitter, it is not expected that the increase in Rx IL will hinder a device implementing the B1 filter from meeting Band 71 receiver requirements. Moreover, while the duplex gap is relatively narrow for Band 71 at 11 MHz, the Tx-Rx separation is 46 MHz so the Tx isolation at 11 MHz offset may be slightly compromised if band edge Rx insertion loss needs to be improved.

NS values and Spectrum emission mask

3GPP TR 38.860 notes that NS values could also be used for other purposes, e.g. for facilitating compliance with the standard UE-UE band coexistence requirements across the APT 600 MHz duplex gap.

This is common for both B1 and B2 options

UE Receiver requirements

UE RFSENS requirements are given in table 7.3.2-1 of 38 101-1 and for band n 71 they are:

| NR band | SCS KHz | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| N71 | 15 | -97.2 | -94 | -91.6 | -86 |
|  | 30 |  | -94.3 | -91.9 | -87.4 |

Note: The UL is limited to 20 MHz in case of n71 and this is likely to be the case for B1.

TR 38.860 mentions the duplexer Tx/RX isolation should be 50 dB

The following combinations shown in Fig 6.4.1.1-5 will meet the isolation:

- Tx- ant B1 full-band with RX antenna B1 full-band

Figure 6.4.1.1-5: Duplexer evaluation for option B1

In band blocking

Rx blocking interference from broadcast TV sources needs to be studied.

For band n71 the power of the TV broadcast at 600 MHz is up to 1 MW a blocking signal level of -15 dBm is assumed in TR 36 755 and is deemed as a necessary requirement to withstand TV blocking interference. There is also a 9 MHz guard band as we discussed in the base station doc and further isolation provided by the Rx filter in the duplexer. A minimum attenuation of 29 dB is discussed in TR 36.755 and data from various filter vendors show this to be feasible.

The duplexer option B1 is most likely to respect the above conditions.

Summary

The feasibility of duplexer B1 was shown in TR 38.860. The following conclusion was noted in TR 38.860:

Option B1 requires that the adjacent broadcast spectrum is repurposed below 612 MHz. The main advantage of this option is that it could be supported with a single 40 MHz filter (channel bandwidths up to 40 MHz could be supported with a single duplex filter, the bill of materials for the UE is reduced, software complexity is reduced, interband CA and EN-DC configurations are more easily accommodated) while still maintaining Band n71 filter requirements). Preliminary data show that compliance with the n71 filter requirements including CH36 blocking requirement would be feasible. With this option B1, allowing n71 UEs in the network by multiple FBI requires further investigations since the duplex spacings of band n71 and option B1 are different.

Band n71 is partially overlapping with arrangement B1. Use of multiple FBI for allowing n71-capable UE in cells of a B1 frequency plan is not straightforward for the B1 and n71 duplex spacings (TX-RX separation) are different. This could possibly be resolved by use of asymmetric bandwidths as supported by n71 could possibly be used (asymmetric bandwidths implicitly modify the TX-RX spacing) if the operator blocks permit.

## Analysis of B2 Ue aspects

This section discusses the UE aspects for B2. The following options of B2 are discussed. Referring  to Tables 6.1.3, 6.1.4 and 6.1.6 above, (Table numbers refer to TR 38 860).

Duplexer bandwidths compare:

|  | UE Tx- MHz |  | UE Rx- Mhz |  | Gap- MHz | BW - MHz | Gap/fc | BW/fc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 full | 663 | 698 | 617 | 652 | 11 | 35 | 1.7% | 5.5% |
| B2 split ( 35+35) | 668 | 703 | 622 | 657 | 11 | 35 | 1.7% | 5.5% |
| Band 28 full | 703 | 748 | 758 | 803 | 10 | 45 | 1.2% | 4% |
| Band 5 | 824 | 849 | 869 | 894 | 20 | 25 | 2.3% | 2.8% |
| Band 8 | 880 | 915 | 925 | 960 | 10 | 35 | 1.1% | 3.7% |

Band n71 (full) and B2 split are the same and slightly higher in terms of gap/fc and BW/fc as compared to band 28. Therefore, they are similar from a complexity aspect.

UE Tx power

Co-existence issues should be considered with DTV and are shown in this report. Co-existence with band 28 is not a problem as duplex directions are reversed.

UE power class 3 – Table 6.2.1-1 of 38 101-1 – copied below- should be maintained as is also the case for n 71

| NR band | Class 1 ( dBm) | Tolerance dB | Class 2 ( dBm) | Tolerance dB | Class 3 ( dbm) | Tolerance dB |
| --- | --- | --- | --- | --- | --- | --- |
| N 71 |  |  |  |  | 23 | +/- 2.5 |
| B2 |  |  |  |  | 23 | +/- 2.5 |

Insertion Loss

TR 38 860, in section 6.4.2.1 Band plan 6.4.2.1 Band plan, states that:

“Band 71 was originally specified as a dual duplexer band, since at that time filter vendors considered a 2x 35 MHz duplexer as not implementable with reasonable performance concerning insertion loss as well as stop band attenuation. Meanwhile technology has improved and there are single duplexers available for band 71/n71 that can cover the 2x 35 MHz with reasonable performance, however, they are still worse than usual duplexers with lower bandwidth. So having 2x 35 MHz as a duplexer is still marginal, but it can be used as one duplexer/band of a dual duplexer/band solution. The other advantage is that meanwhile band 71/n71 is mainstream and implemented in a huge number of smartphones, therefore it is useful to re-use it.”

B2 is only shown to be feasible by the 3GPP with a dual duplexer. Whilst there are a number of duplexer options- relating to the bandwidth of the second duplexer we have only considered the 35+ 35 option. This is because from an implementation point of view having two duplexers with the same bandwidth but only offset by the centre frequency is simpler.  To date, the dual duplexer options in other bands ( i.e. band 28 and band n 71) both have the two duplexers with the same bandwidth. A smaller bandwidth of the second duplexer- say 25 MHz may improve the UE sensitivity but the implementation simplicity outweighs this improvement.

NS values and Spectrum emission mask

NS values for indicating UE additional requirements

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value.  In addition to the NS value additional max power reduction A-MPR can also be specified. For n 71 A-MPR is 0 dB. This will also be the case for B2.

New network signalling (NS) values for indicating the unwanted emissions requirements applicable for the APT 600 MHz band must be specified unless the applicable UE unwanted emissions limits in different geographical regions are either accommodated by either

- the general NR unwanted emissions requirement according to sub-clause 6.5 of 38 101-1 indicated by “NS_01” or

- the FCC requirements for Band n71 according to sub-clause 6.5.2.3.1 of 38 101-1  indicated by “NS_35”.

The latter requirements are repeated below for convenience:

Table 7.2.1-1: Additional requirements for "NS_35"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 |  |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| NOTE 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

This could allow use of n71-capable UEs in an APT 600 MHz deployment for channel bandwidths up to 20 MHz in case the APT 600 MHz unwanted emissions requirements for the region of operation are identical or less stringent than the FCC requirements above.

TR 38 860 notes that NS values could also be used for other purposes, e.g. for facilitating compliance with the standard UE-UE band coexistence requirements across the APT 600 MHz duplex gap.

UE Receiver requirements

UE RFSENS requirements are given in table 7.3.2-1 of 38 101-1 and for band n 71 they are:

| band | SCS KHz | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz (dBm) | 10 MHz ( dBm) | 15 MHz ( dBm) | 20 MHz ( dBm) |
| N 71 | 15 | -97.2 | -94 | -91.6 | -86 |
|  | 30 |  | -94.3 | -91.9 | -87.4 |

Note: The UL is limited to 20 MHz in case of n 71 and this is likely to be the case for B2.

TR 38 860 mentions the duplexer Tx/RX isolation should be 50 dB

The following combinations shown in Fig 6.4.2.2-2 will meet the isolation:

- Tx- ant B2B25 with RX antenna B2B 25

The case of Tx antenna B2B 35 with RX antenna B2B 35 is similar as n 71- where the isolation is degraded. The case of B2A Tx 25 with corresponding Rx appears to have much better isolation ( see attached ppt)

In band blocking

Rx blocking interference from broadcast TV sources needs to be studied.

For band n71 the power of the TV broadcast at 600 MHz is up to 1 MW a blocking signal level of -15 dBm is assumed in TR 36 755 and is deemed as a necessary requirement to withstand TV blocking interference. There is also a 9 MHz guard band as we discussed in the base station doc and further isolation provided by the Rx filter in the duplexer. A minimum attenuation of 29 dB is discussed in TR 36 755 and data from various filter vendors show this to be feasible.

The DTV co-existence studies shown in this report confirm that TV channel clearance for  B2 is the same as for n 71 and a 4MHz guard band is required.

Summary

The feasibility of duplexer B2 was shown in TR 38 860. The following conclusion was noted in TR 38.860:

Option B2 requires that the broadcast spectrum is repurposed below 617 MHz. For option B2, a single UE filter does not appear to be technically feasible with state-of-the art technology at this time though may be achievable in the future as technology advances. Option B2 can be implemented with dual duplexers, e.g. either 35+35 or 35+25 configuration. UEs supporting B2 could also support band n71 and thus meet band n71 requirements. Some degradation in the protection of the own DL band might be expected for the full range due to the narrow 6 MHz duplex gap. With option B2, network could allow legacy n71 UEs by using multiple FBI within the n71 range of B2 (as long as option B2 SEM can be accommodated by either NR or n71).

Commonality with n 71 can be achieved via the use of frequency band indicators- FBI. A network indicating the FBIs for both the APT 600 band and n71 also supports operation of n71 UEs in the lower 35 MHz of the B2 arrangements irrespective of the n71 implementation (split duplexers of full band). Now, UEs supporting the APT 600 MHz band are likely to be implemented with a n71 lower filter with up to 35 MHz passband and an additional overlapping duplexer covering the remaining part of B2. Two duplexers will increase the bill of materials, but B2 25 MHz option may be slightly cheaper than the B2 35 Mhz option.

Determining the cost impact is relative as one needs to consider the supporting customer base. Asia has countries very large populations and economies of scale over these large populations could be substantial.

## Network Architecture Impacts of option A,B1 and B2

### 5G System Architecture

In considering the various components from RAN and Core that may be impacted through Option A, B1 or B2, only the Radio Functions are materially impacted, with other functions transparently handling band specific IEs (information entities) if required.

### Impacts to Antenna Ports

Using B28 dual duplexer as an example we’re able to combine multiple duplexers- in B28 there is typically a single duplexer implemented.

Example shown below is an extremely simplified case where 4 MIMO streams are sent- BS Tx is shown. The diagram shows 4 combiners, but there are also a total of four sets of duplexers and PAs – not shown.

A dual duplexer implementation does NOT need additional antenna ports over a single duplexer implementation

### NSA Impacts from Option A / B1 / B2

Adapted from TS37.340 -  NSA / SA MR-DC Principles.

Most common NSA architecture – Option 3X

Impacts on RU/RRH bands irrelevant to architectural principles, and indeed existing NSA infra. can support Option A, B1, and B2.

Adapted from TS37.340 -  NSA / SA MR-DC Principles.

Most common SA architecture – Option 2 – showing NR CA as well -

Impacts on RU/RRH bands irrelevant to architectural principles, and indeed existing SA infra. can support Option A, B1, and B2.

Adapted from TS37.340 -  NSA / SA MR-DC Principles.

Most common SA architecture – Option 2 – showing NR DC as well –

Impacts on RU/RRH bands irrelevant to architectural principles, and indeed existing SA infra. can support Option A, B1, and B2.

### Core / Architectural Impacts – MR DC

RRC Related aspects – UE Capability information – handled by AMF/MME however transparent containers to support RAN functions. Proxied MN/SN RRC messages will transparently support various RU band combinations.

Bearer mode changes – handled by both Control-Plane and User-Plane functions – however transparent to specific RAT frequency.

### Summary

The options A, B1 and B2 have no impact on the network architecture. These options are internal to the radio.

5G radios are deployed as NSA or SA and will need other bands

The only architectural dependence (if we are to call it this way) is what band combinations are standardised in 3GPP 38 101 and implemented in the base station and in devices.

CA needs to be supported by both duplexers in case of dual duplexers

Option A was designed for the US market and the band combinations allowed mostly follow the US bands that are not available in Europe and Asia.

# Comparison of B1 and B2 from a UE Perspective

This section provides a comparison of the different options A, B1 and B2 from the user equipment (UE) perspective.

## Option B1

•Advantages

•Single filter design is highly preferred.  The vast majority of 3GPP bands are single duplexer, only by exception are a few bands dual duplexer.

•Single filter advantage:  Lower bill-of-materials, software complexity, simpler aggregation

•Potentially this single filter becomes the 600 MHz de facto standard (for n71and APT600)

•No technical showstoppers identified so far, assuming upper edge of highest DTV channel is ≤ 608 MHz

•Maximize FDD spectrum usage in the 612 –703 MHz frequency range (assuming 4 MHz guard band to DTV)

•40 MHz channel bandwidth available, no restriction in CBW or placement

•Disadvantages

•More challenging to share same hardware with n71, requires improved filter design (not necessarily new technology) but feedback from filter designers indicate that parity can be achieved with n71.

•May require uppermost DTV channels above 608 MHz to be cleared.

## Option B2

•Advantages

•One of the filters may reuse n71, so n71 performance can be maintained without any change needed to filter design

•Less dependency on improved filter designs; complexity of new filter is approximately same as n71 filter assuming no new coexistence requirements

•Disadvantages

•Requires two filters.  Increased BOM, less flexibility, risk of partial implementation, even more difficult for CA especially with low band+low band quadplexing (ex: CA_n20_n28, CA_18-28, CA_19-28 are restricted), limitations in variable duplex support (UL and DL have to be in the same duplexer)

•Narrow duplex gap, so UE coexistence may not be met.  Interference in the upper block of DL.

•May require that the n71 filter be redesigned so benefits of reuse could be lost

•Restricted placement of large channels

•Channels must be fully contained within one filter, cannot straddle across two filters

•For example, channels larger than the second filter (35, 30, or 25 MHz) can only be placed within the first filter’s frequency range

•Intra-band CA especially NC is restricted

•DTV is still restricted to 608 MHz (upper DTV channel edge) and below when reusing the existing n71filter

## Compatibility with legacy Band n71

•Option B1

•Possibly compatible over 30 MHz that overlaps with n71.

•Best to deploy with new UE’s.  New UE’s can work in both n71 and new band.

•Option B2

•Fully compatible over the 35 MHz that overlaps with n71

•Not operable over the uppermost 5 MHz, only new UE’s can use this

•Best to deploy with new UE’s.  New UE’s can work in both n71and new band.

## Option B1 compatibility with n71

While it is expected new UE’s will be developed to support Option B1, a UE implementation that supports variable Rx-Tx separation for n71 and the network utilising blocks 2-8 (or a subset) could support the same frequency range as band n71.

•UL block 8 and DL block 1 are outside of n71 filter range and cannot be used by legacy device.

The difficulty is that Band n71 utilizes a different Tx-Rx separation than Option B1, even if the frequency ranges overlap.  This can be partially addressed as described below.

• UL 2-7 can be paired with DL 2-7 if the UE implementation supports variable Rx-Tx separation

•Operators that own blocks 1 and 8 cannot take advantage of this

•Band n71 UE may not have been tested with other than default n71 Rx-Tx separation, but reading the SIB is mandatory for UE’s

•With this approach, it is possible to enable a UE implementation for n71 (over 30 MHz limited frequency range) and new option B1 UE’s

# Example implementation considerations

With various DTV band plans across different regions in the Asia pacific a couple of examples are included below to understand the considerations different administrations may need to consider.

## Example 1

A preliminary study was conducted looking at how the proposed APT600 band options would perform against a future B71 and existing B28 network deployments in an Australian context

### 600 MHz (B71) vs 700 MHz (B28) Comparison

Base station antenna gain: For a typical B28 macro site uses 10 element panel antennas.  These have wideband elements and are quoted by the antenna vendor as having ~ 0.3 dB lower gain at 600 MHz compared to 700 MHz.

Base station fixed losses: In most cases, a ground mounted base station UL sensitivity would be the same for each band since a TMA will be normally be in use, however the DL has approximately 0.2 dB higher losses at 700 MHz assuming a 50m feeder length (LCF78 feeder).   For a tower mounted RRU, losses are the same for each band (immeasurable difference).

UE sensitivity: Based on input contributions from 3GPP members, it has been identified that there are issues with UE Tx noise impacting the UE receiver sensitivity with increasing channel bandwidth (for B71 UE). Thus, the relative impairment (compared to B28) varies from 1.3 to 3.5 dB as the bandwidth increases from 5 MHz to 20 MHz.

PREFSENS for various channel bandwidth.

This table is derived from 3GPP 36.101 17.3.0 Section 7.3.1 Table 7.3.1-1

Path Loss: The free space path loss is 2.14 dB lower at 610 MHz vs 780 MHz (both mid-band DL frequencies)

Summary: The table below compares the key parameters for a 20 MHz wide 700 MHz carrier (Band 28) with those for a Band 71 (600 MHz) carrier having either a 15 or 20 MHz bandwidth.

The table shows that based on the current 3GPP specification, we could expect that for a 20 MHz channel bandwidth, the 600 MHz cell would have a 1.46 dB less coverage, thus reducing the footprint by approximately 6 %.

As 600 MHz performance is improved with lower channel bandwidths, a 15 MHz wide carrier in the 600 MHz band is likely to show a net 3.04 dB improvement in coverage compared to a 20 MHz 700 MHz carrier.  This would increase the footprint by 17 %.

#### Australian DTV Band Plan Impact

The Australian DTV environment is shown in the diagram below with

+87dBm EIRP TV transmissions

-15dBm RX Blocking limit is met once the UE is 5km from the broadcast transmitter within the current B71 band configurations.

There are potential UE blocking concerns due to high power DTV TX up to 610 MHz These may be addressed through various options by the administration.

Options include, among others,

downshifting the DTV band to increase the guard band to 4 MHz,

Geographical band planning to place the higher power DTV channels in block B.

## Example 2

The spectrum utilization in India is depicted as shown in the figure below. There is no existing or planned DTV utilization beyond 582 MHz.

The full utilization of 40 + 40 MHz in 612 – 703 MHz is possible in India.

## Example 3

New Zealand is considering the opportunity to use the 600 MHz band for mobile and is looking at technical aspects that include those around DTV and wireless microphones. The spectrum is presently occupied by DTV and wireless microphones. The 600 MHz band was among proposals in Preparing New Zealand for 5G consultation is part of the recently published Draft five-year spectrum outlook. The  timing and next steps on band still need to be determined.

# Options for Preferred band(s) plans clarified

|  | Option B1 (single filter) | Option B2 (n71 + 35 MHz) |
| --- | --- | --- |
| Time to Market |  |  |
| Spectrum utilization | Allows 40 MHz channels, no restrictions on placement of other channel bandwidths | 40 MHz is available, but max channel bandwidth is 35 MHz |
| DTV | Recent simulations show a guard band of 4 MHz is needed. Therefore, the TV channel clearance is the same as B2 and n 71 | Lower edge of 617 MHz allows greater separation to DTV. However recent simulations show a guard band of 4 MHz is needed. Therefore, the TV channel clearance is the same as B1 and n 71 |
| Implementation complexity | Filter is more difficult. Modem is simpler. Front end is simpler | Filter is simpler. Modem is more difficult. Overall front-end complexity is increased |
| Compatibility with n71 | A UE implementation that supports variable Rx-Tx separation for n71, and the network utilising blocks 2-8 (or a subset) could support the same frequency range as band n71. | Possible with network signalling (one of the duplexers same as n71) |
| Ecosystem | if a single filter can meet both B1 and n71 performance*then n71 is fully leveraged for these devices. | n71 filter may be reused with redesign for narrow duplex gap, a second filter is unique to this band and would need to be populated by OEM |

* A vendor indicated confidence that single filter B1 is feasible without adverse effect to Band n71

# Transition Scenarios

- A B1 or B2 base station (BS) with corresponding B1 or B2 UE’s is the recommended deployment approach. A B1 or B2 BS is expected to be able to support an n71 UE from the start.

- If an operator installs a B1 or B2 base station and starts with n71 handsets ahead of B1 or B2 handsets, for B1 this will require a UE implementation that supports variable Rx-Tx separation for n71. If a UE doesn’t support variable Rx-Tx separation for n71 it will use a different band. Eventually we can expect all handsets to be B1 or B2 capable globally.

- If an operator installs n71 BS first and then needs to support B1 or B2 later, then this n71 base station may need to be swapped with a new B1 or B2 BS; or if the system software upgrade is possible, only some parts of hardware, such as RF and antenna (without full B1 or B2 support) may need to be replaced with new ones capable of B1 or B2.

# Recommendations

The informal correspondence group arrived at the following conclusion after its deliberations and the industry therefore suggests:

- That B1 is the preferred option for APT and be referred to as the ‘APT 600 MHz’ band.

- That AWG invites 3GPP to immediately start work on the technical specifications to support Option B1.

- That B2 may be considered as an option for later standardization should it be required (in a 35 MHz + 35 MHz configuration).

# Correspondence Group Involvement

The Informal Correspondence group included participants from the following APT Wireless Group members

Apple, Ericsson Vietnam Co Ltd, GSMA (Hong Kong), Huawei Technologies Co. Limited, IAFI, Nokia Pte Ltd, Qualcomm Inc, Samsung, Spark NZ Ltd and Telstra.

https://trai.gov.in/sites/default/files/Additional_information_30122021_0.pdf