3GPP TSG RAN Meeting #98-e														RP-222725

Electronic Meeting, December 12 - 16, 2022

3GPP TSG RAN WG4 Meeting #105	R4-2220437

Toulouse, France, November 14 – November 18, 2022

Title:	LS on BWP operation without bandwidth restriction

Response to:

Release:	Rel-18

Work Item:

Source:	RAN4

To:	RANP

Cc:	RAN2, RAN1

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

In the RAN#97 meeting, RAN asks RAN4 to do a high-level analysis of the options in RAN4’s answer to Q2 in RP-221911 and report it to RAN#98 for RAN decision. RAN4 has done the high-level analysis in the two RAN4 meetings and RAN4 would like to provide report as follows.

In the RAN4#104-bis-e meeting, RAN4 agreed to perform high-level analysis from the following 4 aspects/criteria.

- Agreements

- RAN4 works on the below aspects/criteria for highest-level analysis on options for UE performing RLM/BFD/BM when CD-SSB is outside active BWP

  - RRM requirements impact (Spec impact) / workload in RAN4

  - Mobility performance impact

  - Throughput impact (Data interruption)

  - UE power consumption / UE complexity

In the RAN4#105 meeting, RAN4 agreed to deprioritize some of the options and the final candidate options for high-level analysis and for RAN decision are as below.

- Agreement

- Candidate options

  - Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP

  - Option B) Perform BM/RLM/BFD based on SSB outside active BWP

    - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

      - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions

      - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions

    - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps

      - Option B-2-2) Dedicated MG or NCSG for RLM/BFD/BM measurements

  - Option C) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

RAN4 high-level analysis for candidate options for UE performing RLM/BFD/BM when CD-SSB is outside active BWP is summarized in Table 1, Table 2, Table 3, and Table 4, respectively.

Table 1: High-level analysis of candidate options on RRM requirements impact/workload in RAN4

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | - CSI-RS based RLM/BFD/BM requirements are already specified. - Further study is needed to decide on whether timing requirements may need to be updated | Low |
| Option B-1-1) | - Existing SSB based RLM/BFD/BM measurement requirements will apply. - The applicability rule of existing requirements is to be updated. - Depending on RF BW and BB BW (FFT BW) assumption, at least intra-frequency measurement with gap need to be revisited | Low |
| Option B-1-2) | - Same as Option B-1-1). Additionally, - Interruption requirements need to be developed additionally to allow UE for switching. | Low or Medium |
| Option B-2-2) | - Existing requirements need to be updated for SSB based RLM/BFD/BM with dedicated NCSG. - Existing requirements need to be updated for SSB based RLM/BFD/BM with dedicated MG. | NCSG: Low or Medium MG: Medium |
| Option C) | - Existing requirements for SSB based RLM/BFD/BM can be re-used. - Clarification on the requirement applicability might be needed. | Low |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium RRM requirements impact.

Table 2: High-level analysis of candidate options on UE power consumption / UE complexity

| Options | Technical analysis | Summary |  |
| --- | --- | --- | --- |
|  |  | Power consumption | UE complexity |
| Option A) | - UE works in active BWP. - No additional implementation complexity is needed | Low | Low |
| Option B-1-1) | - Depending on RF BW and BB BW (FFT BW) assumption, UE can consume much higher power than other options - If BB BW is assumed adapted to BWP BW, it should be FFS how interruption can be avoided and its impact on UE complexity | High | Low to medium |
| Option B-1-2) | - UE works in larger RF BW than active BWP periodically and UE may consume larger power than other options except Option B-1-1. | Medium | Low to Medium |
| Option B-2-2) | - UE works in active BWP. | Low | Medium |
| Option C) | - UE works in active BWP | Low | Low |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium negative impact on UE power consumption and UE complexity comparing to the legacy solution.

Table 3: High-level analysis of candidate options on Mobility performance impact

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | - Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-1-1) | - No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement - Can also help L3 intra-frequency measurement | Low |
| Option B-1-2) | - No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-2-2) | - Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option C) | - Measurement gap may or may not be needed for intra-frequency L3 measurement depending on whether or not NCD-SSBs from all neighbor cells are present within UE active BWP. - Measurement gap is needed for inter-frequency L3 measurement. | Low |

Note: “Low” - means a limited impact on mobility performance comparing to the legacy solution.

Table 4: High-level analysis of candidate options on Throughput impact (Data interruption)

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |
| Option B-1-1) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |
| Option B-1-2) | Interruption is needed for BM/RLM/BFD measurements | Low to Medium |
| Option B-2-2) | NCSG or MG is needed for BM/RLM/BFD measurements | NCSG: Low to Medium MG: Medium |
| Option C) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |

Note 1: Throughput impact due to L3 measurements is not considered.

Note 2: “Low”/ “Medium” correspond to the low/medium negative impact on throughput comparing to the legacy solution.

2. Actions:

To: RAN Plenary

ACTION: 	RAN4 kindly asks RAN plenary to take the above RAN4 report on high-level analysis for options for BWP operation without restriction into consideration.

3. Reference:

- RP-222630	Moderator's summary for discussion [97e-05-BWP-WithoutRestriction], Vodafone

4. Date of Next TSG RAN WG4 Meetings:

TSG-RAN4 Meeting #106			Feb. 27 – Mar. 03, 2023	Athens

TSG-RAN4 Meeting #106-bis-e		Apr. 17 – Apr. 26, 2023	Online